3GPP TSG-RAN5 Meeting #108	R5-254469

# Bengaluru, India, August 25-29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3464 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of connection diagram for 2Tx TxD test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A majority of 2Tx TxD test cases are referred to 1Tx test cases in terms of test description without the exception of connection diagram for 2Tx. Moreover, there are some mistakes in 6.3G.3.1 general ON/OFF time mask and 6.5G.4 Transmit modulation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the exception of connection diagram for 2Tx TxD test cases. - Adding ON power requirements for PC2 and PC1.5 in 6.3G.3.1. - Updating the measurement of wanted signal to the sum of power from all antenna connectors in 6.5G.4 Transmit modulation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 2Tx TxD test cases may not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.1, 6.2G.2, 6.2G.3, 6.3G.1, 6.3G.3.1, 6.3G.3.2, 6.3G.3.3, 6.3G.4.1, 6.3G.4.2, 6.3G.4.3, 6.4G.1, 6.4G.2.1, 6.4G.2.2, 6.4G.2.3, 6.4G.2.4, 6.5G.1, 6.5G.2.1, 6.5G.2.2, 6.5G.2.3.2, 6.5G.3.1, 6.5G.3.2, 6.5G.3.3, 6.5G.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2G.1	UE maximum output power for Tx Diversity

Editor’s Note: The following aspects are either missing or not yet determined:

- No test points are defined for Power Class 1.5 non-FWA UEs since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1. Testing with minimum MPR has been covered in 6.2G.2.

- MU and TT for >6GHz are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

6.2G.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2G.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 FWA UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.2G.1.3	Minimum conformance requirements

For UE supporting Tx Diversity, the maximum output power as indicated by UE power class in Table 6.2G.1.3-1 is defined as the sum of the maximum output power from all UE transmit antenna connectors. The period of measurement shall be at least one sub frame (1 ms).

When a UE indicates PC1.5 for a given band it achieves maximum power by means of TxD in the current version of the spec. Therefore, Tx Diversity is implied for PC1.5 even if the UE does not indicate Tx diversity capability.

Table 6.2G.1.3-1: Void

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else if the UE does not support a power class with higher maximum output power than PC2; or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [6] is provided and set to the maximum output power of the power class 2 or lower;

-	shall apply all requirements for power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

-	else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2.4.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.1 and 6.2G.1.

6.2G.1.4	Test description

6.2G.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2G.1.4.1-1 and Table 6.2G.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2G.1.4.1-1: Test Configuration Table for Power Class 2 indicating TxD support

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range (NOTE 4) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For P-Max test (Step 4 and Step 5 in Section 6.2G.1.4.2) and maxUplinkDutyCycle test (Step 6, Step 7, Step 8 and Step 9 in Section 6.2G.1.4.2), only “Normal” test environment, “Low range” test frequencies, “Lowest” Test Channel Bandwidth, “Lowest” test SCS and “Inner Full” RB allocation to be used. NOTE 4: For NR bands with ∆MPR > 0dB in Table 6.2.2.3-3, the test channel bandwidth is replaced by the closest one among all the possible channel bandwidths excluding those defined for ∆MPR > 0dB in Table 6.2.2.3-3 in order to test with MPR = 0 dB. If there are two channel bandwidths that have same distance, the higher one is selected. |  |  |  |

Table 6.2G.1.4.1-2: Test Configuration Table for Power Class 1.5 non-FWA UE

NOTE 1:	No test points are defined for Power Class 1.5 non-FWA UE since there is no configuration satisfying MPR=0dB requirements in TS 38.101-1.

Table 6.2G.1.4.1-3: Test Configuration Table for Power Class 3 indicating TxD support

NOTE 2:	Test points defined for 1Tx Power Class 3 in Table Table 6.2.2.3-1 to be reused as test points defined for Power Class 3 indicating TxD support since 1Tx Power Class 3 and TxD Power Class 3 share the same MPR requirements.

Table 6.2G.1.4.1-4: Test Configuration Table for Power Class 1.5 FWA UE

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for maximum output | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | power test case | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Left |
| 3 |  | DFT-s-OFDM PI/2 BPSK | Inner 1RB Right |
| 4 |  | DFT-s-OFDM QPSK | Inner Full |
| 5 |  | DFT-s-OFDM QPSK | Inner 1RB Left |
| 6 |  | DFT-s-OFDM QPSK | Inner 1RB Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2G.1.4.1-1 and Table 6.2G.1.4.1-4.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.1.4.3.

<Unchanged Text Skipped>

6.2G.2	UE maximum output power reduction for Tx Diversity

Editor’s note: The following aspects are either missing or not yet determined:

- MU and TT for >6GHz are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

6.2G.2.1	Test purpose

The number of RB identified in Table 6.2D.2.3-1 and Table 6.2D.2.3-2 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2G.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5G.2.3.1 is executed.

6.2G.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the allowed MPR for the maximum output power is specified in Table 6.2.2.3-1 for UE power class 3, in Table 6.2D.2.3-1 for UE power class 2, Table 6.2D.2.3-2 and Table 6.2D.2.3-3 for UE power class 1.5 with 2Tx, in Table 6.2D.2.3-4 and 6.2D.2.3-5 for UE power class 1.5 with 4 Tx. For UE power class 1.5, the allowed maximum power reduction (MPR) defined in Table 6.2D.2.3-3 is in accordance with the indicated modifiedMPR-Behaviour specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The maximum output power is defined as the sum of the maximum output power at each UE antenna connector. If a UE that supports PC1.5 has to apply the requirements of PC2 according to the rules in clause 6.2.1, the MPR requirements in Table 6.2.2.3-2 apply.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2G.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2G.2.

6.2G.2.4	Test description

6.2G.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, and Table 6.2G.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2G.2.4.1-1: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2G.2.4.1-2: Test Configuration Table for power class 1.5 UE and power class 1.5 FWA (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Default |  | DFT-s-OFDM_64_QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 17 | Default |  | DFT-s-OFDM_256_QAM | Inner Full |
| 18 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 19 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 20 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 21 | Default |  | CP-OFDM QPSK | Inner Full |
| 22 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 23 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM QPSK | Outer Full |
| 25 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 26 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 27 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 28 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 29 | Default |  | CP-OFDM 64 QAM | Inner Full |
| 30 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 33 | Default |  | CP-OFDM 256 QAM | Inner Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. |  |  |  |  |

Table 6.2G.2.4.1-2a: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 374,5 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,5 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,5 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |

Table 6.2G.2.4.1-3: Test Configuration Table for power class 2 and 3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4, 6.2.2.4.1-4a and 6.2.2.4.1-4b. NOTE 2: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, Table 6.2G.2.4.1-2a and Table 6.2G.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.2.4.3.

6.2G.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2G.2.4.1-1, Table 6.2G.2.4.1-2, Table 6.2G.2.4.1-2a and Table 6.2G.2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	For UEs supporting Power Class 1.5 and Power Class 2, repeat steps 1~3 for Test ID 16 and 22 in Table 6.2G.2.4.1-2a on the applicable bands with message exception of P-Max defined in Table 6.2G.2.4.3-2.

5.	For UEs supporting Power Class 1.5, repeat steps 1~3 for all Test IDs in Table 6.2G.2.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.2G.2.4.3-3.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2G.2.4.1-1 and Table 6.2G.2.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

<Unchanged Text Skipped>

6.2G.3	UE additional maximum output power reduction for Tx Diversity

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_47 for Power Class 2 and Power Class 1.5 with contiguous allocation is complete.

- Tests for network signalling values NS_55 for Power Class 2 and, NS_55 for Power Class 1.5 are FFS.

- Tests for Power Class 3 are FFS.

6.2G.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2G.2. Outer and inner allocation notation used in clause 6.2G.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2G.3.2	Test applicability

The requirements of this test apply in test case 6.5G.2.2 Additional Spectrum Emission mask for Tx Diversity for network signalling value NS_04 to all types of NR Power Class 1.5, Power Class 2 reporting Tx diversity UE release 15 and forward.

The requirements of this test apply in test case 6.5G.3.3 Additional Spurious Emissions Tx Diversity for network signalling values NS_04, NS_47 and NS_50 to all types of NR Power Class 2 and Power Class 1.5 reporting Tx diversity UE release 15 and forward.

6.2G.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2.1-1, and the maximum output power is defined as the sum of the maximum output power at each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2G.3.3-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2G.3.3-2.

Table 6.2G.3.3-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_04 | 6.5G.2.3.3.2, 6.5G.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_47 | 6.5G.3.3.3.15 | n41 (Note 3) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_50 | 6.5G.3.3.3.16 | n39, n98 | 10, 15, 20, 25, 30, 35, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 4 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz, 10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 4: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. |  |  |  |  |  |

Table 6.2G.3.3-2: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n98 | NS_01 | NS_50 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1 and 6.2G.3.

6.2G.3.4	Test description

6.2G.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.3.4.1-2 for NS_04 and Table 6.2.3.4.1-18 for NS_47 PC2 and PC3, and in Table 6.2G.3.4.1-1 for NS_47 PC1.5, and in Table 6.2G.3.4.1-2 for NS_50 PC1.5. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2G.3.4.1-1: Test Configuration table for NS_47 power class 1.5

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 30 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK | Edge_1RB_Left (A1) |  |  |  |
| 2 | Default | 30 | Default |  |  | PI/2 BPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 3 | Default | 30 | Default |  |  | PI/2 BPSK | Edge_1RB_Right (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | PI/2 BPSK | Outer_Full (A2) |  |  |  |
| 5 | Default | 30 | Default |  |  | PI/2 BPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 6 | Default | 30 | Default |  |  | PI/2 BPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 7 | Default | 30 | Default |  |  | PI/2 BPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 8 | Default | 30 | Default |  |  | PI/2 BPSK | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 9 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 10 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 11 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 12 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 13 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 14 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 15 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 16 | Default | 30 | Default |  |  | QPSK | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 17 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 18 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 19 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 20 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 21 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 22 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 23 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 24 | Default | 30 | Default |  |  | 16 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 25 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 26 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 27 | Default | 30 | Default |  |  | 64 QAM | Edge 1RB_Right (A3) |  |  |  |
| 28 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 29 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 30 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 31 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 32 | Default | 30 | Default |  |  | 64 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 33 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 34 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 35 | Default | 30 | Default |  |  | 256 QAM | Edge 1RB_Right (A3) |  |  |  |
| 36 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 37 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 38 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 39 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 40 | Default | 30 | Default |  |  | 256 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 41 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 42 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 43 | Default | 30 | Deafult |  |  | QPSK | Edge 1RB_Right (A3) |  |  |  |
| 44 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 45 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 46 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 47 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 48 | Default | 30 | Default |  |  | QPSK | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 49 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 50 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 51 | Default | 30 | Default |  |  | 16 QAM | Edge 1RB_Right (A3) |  |  |  |
| 52 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 53 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 54 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 55 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 56 | Default | 30 | Default |  |  | 16 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 57 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |

| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
| 58 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 64 QAM | Edge 1RB_Right (A3) |  |  |
| 60 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |
| 61 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 64 | Default | 30 | Default |  |  | 64 QAM | 80@35 (A5) | 40@18 (A5) | 20@9 (A5) |
| 65 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |
| 66 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) | 1@15 (A2) | 1@8 (A2) |
| 67 | Default | 30 | Default |  |  | 256 QAM | Edge 1RB_Right (A3) |  |  |
| 68 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 69 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) | 54@0 (A4) | 27@0 (A4) |
| 70 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) | 40@0 (A4) | 20@0 (A4) |
| 71 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) | 27@0 (A2) | 12@0 (A2) |
| 72 | Default | 30 | Default |  |  | 256 QAM | 80@35 (A5) | 40@18 (A5) | 20@9 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |

Table 6.2G.3.4.1-2: Test Configuration table for NS_50 power class 1.5

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-32a, 6.2.3.4.1-32b and 6.2.3.4.1-32c |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10MHz, 15MHz, 20MHz, 25 MHz, 30MHz, 35MHz, 40MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |  |  |
| A-MPR test parameters for NS_50 (Power Class 1.5) |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | Default | 10 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 2 | Default | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |  |
| 3 | Default | 10 | Default |  |  | QPSK | 40@11 (A4) | 18@6 (A4) |  | / |  |
| 4 | Default | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 5 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 6 | Default | 15 | Default |  |  | QPSK | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |  |
| 7 | Default | 15 | Default |  |  | QPSK | 16@1 (A10) | 9@1 (A10) |  | 5@1 (A10) |  |
| 8 | Default | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 9 | Default | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 10 | Default | 20 | Default |  |  | QPSK | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |  |
| 11 | Default | 20 | Default |  |  | QPSK | 21@1 (A10) | 10@1 (A10) |  | 6@1 (A10) |  |
| 12 | Default | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 13 | Default | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 14 | Default | 25 | Default |  |  | QPSK | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |  |
| 15 | Default | 25 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 16 | Default | 25 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 17 | Default | 25 | Default |  |  | QPSK | 38@11 (A10) | 33@6 (A10) |  | 30@3 (A10) |  |
| 18 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 19 | Default | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 20 | Default | 30 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 21 | Default | 30 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 22 | Default | 30 | Default |  |  | QPSK | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |  |
| 23 | Default | 30 | Default |  |  | QPSK | 38@11 (A10) | 33@6 (A10) |  | 30@3 (A10) |  |
| 24 | Default | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 25 | Default | 35 | Default |  |  | QPSK | 50@10 (A3) | 30@5 (A3) |  | 12@3 (A3) |  |
| 26 | Default | 35 | Default |  |  | QPSK | 178@10 (A8) | 87@5 (A8) |  | 40@3 (A8) |  |
| 27 | Default | 35 | Default |  |  | QPSK | 111@77 (A4) | 52@39 (A4) |  | 24@20 (A4) |  |
| 28 | Default | 35 | Default |  |  | QPSK | 16@171 (A9) | 6@86 (A9) |  | 1@43 (A9) |  |
| 29 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 30 | Default | 40 | Default |  |  | QPSK | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |  |
| 31 | Default | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |  |
| 32 | Default | 40 | Default |  |  | QPSK | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |  |
| 33 | Default | 40 | Default |  |  | QPSK | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |  |
| 34 | Default | 40 | Default |  |  | QPSK | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |  |
| 35 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 36 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |  |
| 37 | Default | 10 | Default |  |  | 256 QAM | 40@11 (A4) | 18@6 (A4) |  | / |  |
| 38 | Default | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 39 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 40 | Default | 15 | Default |  |  | 256 QAM | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |  |
| 41 | Default | 15 | Default |  |  | 256 QAM | 16@1 (A10) | 9@1 (A10 |  | 5@1 (A10) |  |
| 42 | Default | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 43 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 44 | Default | 20 | Default |  |  | 256 QAM | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |  |
| 45 | Default | 20 | Default |  |  | 256 QAM | 21@1 (A10) | 10@1 (A10) |  | 6@1 (A10) |  |
| 46 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 47 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 48 | Default | 25 | Default |  |  | 256 QAM | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |  |
| 49 | Default | 25 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 50 | Default | 25 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 51 | Default | 25 | Default |  |  | 256 QAM | 38@11 (A10) | 33@6 (A10) |  | 30@3 (A10) |  |
| 52 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 53 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 54 | Default | 30 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 55 | Default | 30 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 56 | Default | 30 | Default |  |  | 256 QAM | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |  |
| 57 | Default | 30 | Default |  |  | 256 QAM | 38@11 (A10) | 33@6 (A10) |  | 30@3 (A10) |  |
| 58 | Default | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 59 | Default | 35 | Default |  |  | 256 QAM | 50@10 (A3) | 30@5 (A3) |  | 12@3 (A3) |  |
| 60 | Default | 35 | Default |  |  | 256 QAM | 178@10 (A8) | 87@5 (A8) |  | 40@3 (A8) |  |
| 61 | Default | 35 | Default |  |  | 256 QAM | 111@77 (A4) | 52@39 (A4) |  | 24@20 (A4) |  |
| 62 | Default | 35 | Default |  |  | 256 QAM | 16@171 (A9) | 6@86 (A9) |  | 1@43 (A9) |  |
| 63 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 64 | Default | 40 | Default |  |  | 256 QAM | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |  |
| 65 | Default | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |  |
| 66 | Default | 40 | Default |  |  | 256 QAM | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |  |
| 67 | Default | 40 | Default |  |  | 256 QAM | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |  |
| 68 | Default | 40 | Default |  |  | 256 QAM | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |  |
| 69 | Default | 10 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 70 | Default | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |  |
| 71 | Default | 10 | Default |  |  | QPSK | 41@11 (A4) |  | 18@6 (A4) |  | / |
| 72 | Default | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 73 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 74 | Default | 15 | Default |  |  | QPSK | 60@19 (A4) |  | 28@10 (A4) |  | 13@5 (A4) |
| 75 | Default | 15 | Default |  |  | QPSK | 16@1(A10) |  | 9@1(A10) |  | 5@1(A10) |
| 76 | Default | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 77 | Default | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 78 | Default | 20 | Default |  |  | QPSK | 80@26 (A4) |  | 38@13 (A4) |  | 17@7 (A4) |
| 79 | Default | 20 | Default |  |  | QPSK | 21@1 (A10) |  | 10@1 (A10) |  | 6@1 (A10) |
| 80 | Default | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 81 | Default | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 82 | Default | 25 | Default |  |  | QPSK | 74@47 (A4) |  | 37@24 (A4) |  | 19@13 (A4) |
| 83 | Default | 25 | Default |  |  | QPSK | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 84 | Default | 25 | Default |  |  | QPSK | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 85 | Default | 25 | Default |  |  | QPSK | 38@11(A10) |  | 33@6(A10) |  | 30@3(A10) |
| 86 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 87 | Default | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 88 | Default | 30 | Default |  |  | QPSK | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 89 | Default | 30 | Default |  |  | QPSK | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 90 | Default | 30 | Default |  |  | QPSK | 88@61 (A4) |  | 44@31 (A4) |  | 22@16 (A4) |
| 91 | Default | 30 | Default |  |  | QPSK | 38@11(A10) |  | 33@6(A10) |  | 30@3(A10) |
| 92 | Default | 35 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 93 | Default | 35 | Default |  |  | QPSK | 50@10 (A3) |  | 30@5 (A3) |  | 12@3 (A3) |
| 94 | Default | 35 | Default |  |  | QPSK | 178@10 (A8) |  | 87@5 (A8) |  | 41@3 (A8) |
| 95 | Default | 35 | Default |  |  | QPSK | 111@77(A4) |  | 53@39(A4) |  | 24@20(A4) |
| 96 | Default | 35 | Default |  |  | QPSK | 17@171(A9) |  | 6@86(A9) |  | 1@43(A9) |
| 97 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 98 | Default | 40 | Default |  |  | QPSK | 191@25 (A8) |  | 93@13 (A8) |  | 44@7 (A8) |
| 99 | Default | 40 | Default |  |  | QPSK | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 100 | Default | 40 | Default |  |  | QPSK | 91@25 (A7) |  | 46@13 (A7) |  | 23@7 (A7) |
| 101 | Default | 40 | Default |  |  | QPSK | 115@101 (A4) |  | 55@51 (A4) |  | 25@26 (A4) |
| 102 | Default | 40 | Default |  |  | QPSK | 39@177 (A9) |  | 17@89 (A9) |  | 6@45 (A9) |
| 103 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 104 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |  |
| 105 | Default | 10 | Default |  |  | 256 QAM | 41@11 (A4) |  | 18@6 (A4) |  | / |
| 106 | Default | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 107 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 108 | Default | 15 | Default |  |  | 256 QAM | 60@19 (A4) |  | 28@10 (A4) |  | 13@5 (A4) |
| 109 | Default | 15 | Default |  |  | 256 QAM | 16@1(A10) |  | 9@1(A10) |  | 5@1(A10) |
| 110 | Default | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 111 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 112 | Default | 20 | Default |  |  | 256 QAM | 80@26 (A4) |  | 38@13 (A4) |  | 17@7 (A4) |
| 113 | Default | 20 | Default |  |  | 256 QAM | 21@1 (A10) |  | 10@1 (A10) |  | 6@1 (A10) |
| 114 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 115 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 116 | Default | 25 | Default |  |  | 256 QAM | 74@47 (A4) |  | 37@24 (A4) |  | 19@13 (A4) |
| 117 | Default | 25 | Default |  |  | 256 QAM | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 118 | Default | 25 | Default |  |  | 256 QAM | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 119 | Default | 25 | Default |  |  | 256 QAM | 38@11(A10) |  | 33@6(A10) |  | 30@3(A10) |
| 120 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 121 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 123 | Default | 30 | Default |  |  | 256 QAM | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 124 | Default | 30 | Default |  |  | 256 QAM | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 125 | Default | 30 | Default |  |  | 256 QAM | 88@61 (A4) |  | 44@31 (A4) |  | 22@16 (A4) |
| 126 | Default | 30 | Default |  |  | 256 QAM | 38@11(A10) |  | 33@6(A10) |  | 30@3(A10) |
| 127 | Default | 35 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 128 | Default | 35 | Default |  |  | 256 QAM | 50@10 (A3) |  | 30@5 (A3) |  | 12@3 (A3) |
| 129 | Default | 35 | Default |  |  | 256 QAM | 178@10 (A8) |  | 87@5 (A8) |  | 41@3 (A8) |
| 130 | Default | 35 | Default |  |  | 256 QAM | 111@77(A4) |  | 53@39(A4) |  | 24@20(A4) |
| 131 | Default | 35 | Default |  |  | 256 QAM | 17@171(A9) |  | 6@86(A9) |  | 1@43(A9) |
| 132 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 133 | Default | 40 | Default |  |  | 256 QAM | 191@25 (A8) |  | 93@13 (A8) |  | 44@7 (A8) |
| 134 | Default | 40 | Default |  |  | 256 QAM | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 135 | Default | 40 | Default |  |  | 256 QAM | 91@25 (A7) |  | 46@13 (A7) |  | 23@7 (A7) |
| 136 | Default | 40 | Default |  |  | 256 QAM | 115@101 (A4) |  | 55@51 (A4) |  | 25@26 (A4) |
| 137 | Default | 40 | Default |  |  | 256 QAM | 39@177 (A9) |  | 17@89 (A9) |  | 6@45 (A9) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2G.3.4.1-1 to Table 6.2G.3.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.3.4.3.

6.2G.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2G.3.4.1-1 to Table 6.2G.3.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD, only slots consisting of only UL symbols are under test.

4.	For network signalling value “NS_04” and UEs supporting Power Class 2 and Power Class 1.5, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2G.3.4.3.1-2.

5.	For network signalling value “NS_04” and UEs supporting Power Class 1.5, repeat steps 1~3 on the applicable bands with message exception of P-Max defined in Table 6.2G.3.4.3.1-3.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2G.3.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

<Unchanged Text Skipped>

6.3G	Output power dynamics for Tx Diversity

6.3G.1	Minimum output power for Tx Diversity

6.3G.1.1	Test purpose

Same test purpose as in 6.3.1.1.

6.3G.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1 ms). The minimum output power shall not exceed the values specified in Table 6.3.1.3-1.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.1.

6.3G.1.4	Test description

Same test description as specified in clause 6.3.1.4 with following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

Step 3 of Test procedure as in 6.3.1.4.2 is replaced by:

3.	Measure the sum of mean power at each antenna connector in the associated measurement channel bandwidth specified in Table 6.3G.1.5-1 for the specific channel bandwidth under test. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

6.3G.1.5	Test requirement

The minimum output power, derived in step 3 shall not exceed the values specified in Table 6.3G.1.5-1.

Table 6.3G.1.5-1: Minimum output power

| Minimum output powerChannel bandwidth(MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 5 | -40+TT | 4.515 |
| 10 | -40+TT | 9.375 |
| 15 | -40+TT | 14.235 |
| 20 | -40+TT | 19.095 |
| 25 | -39+TT | 23.955 |
| 30 | -38.2+TT | 28.815 |
| 35 | -37.6+TT | 33.855 |
| 40 | -37+TT | 38.895 |
| 45 | -36.5+TT | 43.575 |
| 50 | -36+TT | 48.615 |
| 60 | -35.2+TT | 58.35 |
| 70 | -34.6+TT | 68.07 |
| 80 | -34+TT | 78.15 |
| 90 | -33.5+TT | 88.23 |
| 100 | -33+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3G.1.5-2 |  |  |

Table 6.3G.1.5-2: Test Tolerance (Minimum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.3 dB |
| 40MHz < BW ≤ 100MHz | 1.3 dB | 1.3 dB |

<Unchanged Text Skipped>

6.3G.3.1	General ON/OFF time mask for Tx Diversity

6.3G.3.1.1	Test purpose

Same test purpose as in 6.3.3.2.1.

6.3G.3.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.3.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the general ON/OFF time mask requirements in clause 6.3.3.2.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.1.4	Test description

Same test description as in clause 6.3.3.2.4 with following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.1.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.1.5-1.

Table 6.3G.3.1.5-1: General ON/OFF time mask

|  |  |  |  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 |  | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 43.575 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  | Same as Test ID 5 of Tables 6.2G.2.5-1 to 6.2G.2.5-3 Same as Test ID 9 of Tables 6.2G.2.5-5 to 6.2G.2.5-8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.1.5-2 NOTE 2: TT of ON power for each frequency and channel bandwidth is specified in the referred ON power test requirement tables. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.1.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

Table 6.3G.3.1.5-3: Void

<Unchanged Text Skipped>

6.3G.3.2	PRACH time mask for Tx Diversity

6.3G.3.2.1	Test purpose

Same test purpose as in 6.3.3.4.1.

6.3G.3.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.3.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the PRACH time mask requirements in clause 6.3.3.4.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.2.4	Test description

Same test description as in clause 6.3.3.4.4 with following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.2.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.2.5-1.

Table 6.3G.3.2.5-1: PRACH time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Expected PRACH Transmission ON Measured Power for PRACH Format 0 and PRACH Format A3 for SCS 30kHz | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm | -1 dBm |
| Expected PRACH Transmission ON Measured Power for PRACH Format A3 for SCS 15kHz and SCS 60kHz | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm | -2 dBm |
| ON Power Tolerance | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.2.5-2 NOTE 2: TT of ON power for each frequency and channel bandwidth is specified in Table 6.3.3.4.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.2.5-2: Test Tolerance (Transmit OFF power and PRACH time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3G.3.3	SRS time mask for Tx Diversity

6.3G.3.3.1	Test purpose

Same test purpose as in 6.3.3.6.1.

6.3G.3.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.3.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the SRS time mask requirements in clause 6.3.3.6.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.3.4	Test description

Same test description as in clause 6.3.3.6.4 with following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.3.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.3.5-1.

Table 6.3G.3.3.5-1: SRS time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 33.855 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power | Same as Table 6.2G.1.5-4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.3.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.3.5-2: Test Tolerance (Transmit OFF power and SRS time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

6.3G.4	Power control for Tx Diversity

6.3G.4.1	Absolute power tolerance for Tx Diversity

6.3G.4.1.1	Test purpose

Same test purpose as in 6.3.4.2.1.

6.3G.4.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.4.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the power control tolerance applies to the sum of output power at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.4

6.3G.4.1.4	Test description

Same test description as in clause 6.3.4.2.4 with following exceptions:

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

-	The UE’s transmit power is measured as sum of both transmit antenna connectors.

6.3G.4.1.5	Test requirement

The requirement for the power measured in step (2) of the test procedure is not to exceed the values specified in Table 6.3G.4.1.5-1 and 6.3G.4.1.5-2.

Table 6.3G.4.1.5-1: Absolute power tolerance: test point 1

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -17.6 | -14.4 | -12.6 | -11.3 | -10.4 | -9.6 | -8.9 | -8.3 | -7.8 | -7.3 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -18.2 | -14.8 | -12.8 | -11.5 | -10.5 | -9.7 | -9 | -8.3 | -7.8 | -7.4 | -6.5 | -5.8 | -5.2 | -4.7 | -4.2 |
|  | SCS60 |  | -15.2 | -13 | -11.8 | -10.7 | -9.8 | -9.1 | -8.5 | -7.9 | -7.5 | -6.6 | -5.9 | -5.3 | -4.8 | -4.3 |
| Power tolerance |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3G.1.5. Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3G.4.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.4.1.5-2: Absolute power tolerance: test point 2

|  |  | Channel bandwidth / expected output power (dBm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Expected Measured power | SCS15 | -3.6 | 0.4 | 1.4 | 2.7 | 3.6 | 4.4 | 5.1 | 5.7 | 6.2 | 6.7 | N/A | N/A | N/A | N/A | N/A |
|  | SCS30 | -4.2 | -0.8 | 1.2 | 2.5 | 3.5 | 4.3 | 5 | 5.7 | 6.2 | 6.6 | 7.5 | 8.2 | 8.8 | 9.3 | 9.8 |
|  | SCS60 | N/A | -1.2 | 1 | 2.2 | 3.3 | 4.2 | 4.9 | 5.5 | 6.1 | 6.5 | 7.4 | 8.1 | 8.7 | 9.2 | 9.7 |
| Power tolerance |  |  | ± (9+TT)dB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The higher power limit shall not exceed the maximum output power requirements defined in sub-clause 6.2G.1.5. Note 2: TT for each duplex, Sub-Carrier Spacing, frequency and channel bandwidth is specified in Table 6.3G.4.1.5-3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.4.1.5-3: Test Tolerance

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB1 1.2 dB2 | 1.4 dB | 1.4 dB |
| 40MHz < BW ≤ 100MHz | 1.4 dB | 1.4 dB | 1.4 dB |
| Note 1: Applicable for test point with UL power ≥ 0 dBm. Note 2: Applicable for test point with UL power < 0 dBm. |  |  |  |

6.3G.4.2	Relative power tolerance for Tx Diversity

6.3G.4.2.1	Test purpose

Same test purpose as in 6.3.4.3.1.

6.3G.4.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.4.2.3	Minimum conformance requirement

For UE supporting Tx diversity, the relative power tolerance applies to the sum of output power at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.4.

6.3G.4.2.4	Test description

Same test description as in clause 6.3.4.3.4 with the output power is measured as the sum of both antenna connectors.

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

6.3G.4.2.5	Test requirement

Each UE power step measured in the test procedure 6.3G.4.2.4 should satisfy the test requirements specified in Table 6.3G.4.2.5-1 through 6.3G.4.2.5-7.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each of ramping up and ramping down test patterns. For these exceptions the power tolerance limit is a maximum of ± (6.0 + TT) dB. If there is an exception in the power step caused by the RB change for all test patterns (A, B, C) then fail the UE.

Table 6.3G.4.2.5-1: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 15 RBs | TPC=+1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 15 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-2: Test Requirements Relative Power Tolerance for Transmission, channel BW 5MHz, ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 |  | Sub-frames before RB change | Fixed = 15 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 15 RBs to 1 RB | TPC=-1dB | 12.76 | 10dB ≤ ΔP < 15dB | 12.76 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Sub-frames before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Sub-frames before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Sub-frames after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Sub-frames Pattern B the position of RB uplink allocation change is after 20 active uplink Sub-frames Pattern C the position of RB uplink allocation change is after 30 active uplink Sub-frames Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-3: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 50MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 1RB to 20 RBs | TPC=+1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 20 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 50 RBs | TPC=+1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 50 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80+/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 10 RBs | TPC=+1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 10 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-4: Test Requirements Relative Power Tolerance for Transmission, channel BW 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 35MHz, 40MHz, 50MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1RBs | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 20 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 15 | 2 | RB change | 20 RBs to 1 RB | TPC=-1dB | 14.01 | 10dB ≤ ΔP < 15dB | 14.01 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 50 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 50 RBs to 1 RB | TPC=-1dB | 17.99 | 15dB ≤ ΔP | 17.99 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 |  |  |  |  |  |  |  |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 10 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 2 | RB change | 10 RBs to 1 RB | TPC=-1dB | 11.00 | 10dB ≤ ΔP < 15dB | 11.00 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-5: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp up sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 1RB to 24 RBs | TPC=+1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 24 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 1RB to 81 RBs | TPC=+1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 81 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 1RB to 5 RBs | TPC=+1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 5 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | 1RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 1RB to 75 RBs | TPC=+1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 75 | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-6: Test Requirements Relative Power Tolerance for Transmission, channel BW 60MHz, 70MHz, 80MHz, 90MHz, 100MHz ramp down sub-test

| Test SCS [kHz] | Sub-test ID | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Down) | Power step size range (Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/-0.7 + TT |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 24 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 30 | 2 | RB change | 24 RBs to 1 RB | TPC=-1dB | 14.80 | 10dB ≤ ΔP < 15dB | 14.80 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 81 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 3 | RB change | 81 RBs to 1 RB | TPC=-1dB | 20.08 | 15dB < ΔP | 20.08 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 1 | RB change | 5 RBs to 1 RB | TPC=-1dB | 7.99 | 4dB ≤ ΔP < 10dB | 7.99 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 |  | Subframes before RB change | Fixed = 75 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 2 | RB change | 75 RBs to 1 RB | TPC=-1dB | 19.75 | 15dB < ΔP | 19.75 +/- (5 + TT) |
|  |  | Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: The starting resource block shall be RB# 0. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

Table 6.3G.4.2.5-7: Test Requirements Relative Power Tolerance for Transmission, alternating sub-test

| BW | Test SCS [kHz] | Sub-test ID | Uplink RB allocation | TPC command | Expected power step size (Up or Down) | Power step size range (Up or Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 15 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
| 5 |  | 3 | Alternating 1 and 15 | TPC=0dB | 11.76 | 10dB ≤ ΔP < 15dB | 11.76 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  |  | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 15 | 3 | Alternating 1 and 20 | TPC=0dB | 13.01 | 10dB ≤ ΔP < 15dB | 13.01 +/- (4 + TT) |
|  |  | 4 | Alternating 1 and 50 | TPC=0dB | 16.99 | 15dB ≤ ΔP | 16.99 +/- (5 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
| 10,15,20, 25,30,40,50 | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 24 | TPC=0dB | 13.80 | 10dB ≤ ΔP < 15dB | 13.80 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  | 60 | 2 | Alternating 1 and 10 | TPC=0dB | 10.00 | 10dB ≤ ΔP < 15dB | 10.00 +/- (4 + TT) |
|  |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 30 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 81 | TPC=0dB | 19.08 | 15dB < ΔP | 19.08 +/- (5 + TT) |
| 60, 70,80,90,100 |  | 1 | Alternating 1 and 2 | TPC=0dB | 3.01 | 3dB ≤ ΔP < 4dB | 3.01 +/- (3 + TT) |
|  | 60 | 2 | Alternating 1 and 5 | TPC=0dB | 6.99 | 4dB ≤ ΔP < 10dB | 6.99 +/- (3.5 + TT) |
|  |  | 3 | Alternating 1 and 75 | TPC=0dB | 18.75 | 15dB < ΔP | 18.75 +/- (5 + TT) |
| Note 1: The starting resource block shall be RB# 0. Note 2: TT=0.7dB Note 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3G.1. |  |  |  |  |  |  |  |

6.3G.4.3	Aggregate power tolerance for Tx Diversity

6.3G.4.3.1	Test purpose

Same test purpose as in 6.3.4.4.1.

6.3G.4.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.4.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the absolute power tolerance applies to the sum of output power at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.4.

6.3G.4.3.4	Test description

Same test description as in clause 6.3.4.4.4 with the output power is measured as the sum of both antenna connectors.

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

6.3G.4.3.5	Test requirement

The requirement for the power measurements made in step 1.3 and 2.3 of the test procedure shall not exceed the values specified in Table 6.3G.4.3.5-1. The power measurement period shall be 1 sub-frame(1ms).

Table 6.3G.4.3.5-1: Power control tolerance

| TPC command | UL channel | Test requirement measured power |
| --- | --- | --- |
| 0 dB | PUCCH | Given 5power measurements in the pattern, the 2nd, and later measurements shall be within ± (2.5 + TT) dB of the 1st measurement. |
| 0 dB | PUSCH | Given 5 power measurements in the pattern, the 2nd, and later measurements shall be within ± (3.5 + TT) dB of the 1st measurement. |
| Note 1: For SCS 30kHz 1 sub-frame corresponds to 2 slots and for SCS 60kHz 1 sub-frame corresponds to 4 slots, so 2 TPC commands will be sent for a single measurement period. Note 2: TT=0.7dB. |  |  |

<Unchanged Text Skipped>

6.4G.1	Frequency error for Tx Diversity

6.4G.1.1	Test purpose

Same test purpose as in 6.4.1.1.

6.4G.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.4G.1.3	Minimum conformance requirements

For UE(s) supporting Tx diversity, the basic measurement interval of modulated carrier frequency is 1 UL slot. The mean value of basic measurements of UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to the carrier frequency received from the NR Node B.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4G.1

6.4G.1.4	Test description

Same test description as in clause 6.4.1.4 with the measurement performed at each transmit antenna connector.

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

6.4G.1.5	Test requirement

For each transmit antenna connector, the each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

<Unchanged Text Skipped>

6.4G.2	Transmit modulation quality for Tx Diversity

For UE supporting Tx diversity, the transmit modulation quality requirements are specified at each transmit antenna connector. The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage (caused by IQ offset)

-	In-band emissions for the non-allocated RB

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [6]), carrier leakage measurement requirement in clause 6.4G.2.2 and 6.4G.2.3 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

6.4G.2.1	Error Vector Magnitude for Tx Diversity

6.4G.2.1.1	Test Purpose

Same test purpose as in 6.4.2.1.1.

6.4G.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.4G.2.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the Error Vector Magnitude requirements specified in clause 6.4.2.1.3. The total EVM requirement is derived based on the measurement at each antenna connector according to Annex E.4.8.

The normative reference for this requirement is TS 38.101 [2] clause 6.4G.2.1.

6.4G.2.1.4	Test description

6.4G.2.1.4.1	Initial conditions

Same initial conditions as in 6.4.2.1.4.1 with the following exceptions:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

6.4G.2.1.4.2	Test procedure

Test procedure for PUSCH:

1.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4.2.1.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

1.2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

1.3.	Measure the mean power at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

1.4.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E). For TDD, only slots consisting of only UL symbols are under test.

-	If the UE’s transmit power measured in step 1.3 is higher than the minimum output power requirement in 6.3G.1.5 at both antenna connectors, the composite EVM is measured according to E.4.8.

-	If UE’s transmit power measured in step 1.3 is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.4.2 and E.4.6.

1.5.	For modulations except 256QAM, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Pmin, where:

-	Pmin is the minimum output power according to Table 6.3.1.3-1.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

For 256 QAM, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Pmin + 10 dB, where Pmin, MU and Uplink power control window size are defined above.

1.6.	Measure the mean power at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

1.7.	Measure the EVM and  using Global In-Channel Tx-Test (Annex E). For TDD, only slots consisting of only UL symbols are under test.

-	If the UE’s transmit power measured in step 1.6 is higher than the minimum output power requirement in 6.3G.1.5 at both antenna connectors, the composite EVM is measured according to E.4.8.

-	If UE’s transmit power measured in step 1.6 is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.4.2 and E.4.6.

NOTE1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.4.2.1.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE2:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.2.

Test procedure for PUCCH:

2.1.	PUCCH is set according to Table 6.4.2.1.4.1-2.

2.2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 6.4.2.1.4.1-2. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH. There is no PUSCH transmission.

2.3.	SS send appropriate TPC commands for PUCCH to the UE until the UE transmit PUCCH at PUMAX level. Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

2.4.	Measure the mean power at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

2.5.	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E).

-	If the UE’s transmit power measured in step 2.4 is higher than the minimum output power requirement in 6.3G.1.5 at both antenna connectors, the composite EVM is measured according to E.5.9.4.

-	If UE’s transmit power measured in step 2.4 is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.5.9.2.

2.6.	Send uplink power control commands for PUCCH to the UE using 1dB power step size to ensure that the UE PUCCH output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Pmin, where:

-	Pmin is the minimum output power according to Table 6.3.1.3-1.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 2.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 2.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

2.7.	Measure the mean power at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

2.8.	Measure PUCCH EVM using Global In-Channel Tx-Test (Annex E).

-	If the UE’s transmit power measured in step 2.7 is higher than the minimum output power requirement in 6.3G.1.5 at both antenna connectors, the composite EVM is measured according to E.5.9.4.

-	If UE’s transmit power measured in step 2.7 is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.5.9.4.

NOTE1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.4.2.1.4.1-2, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE2:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.2.

Test procedure for PRACH:

3.1.	The SS shall set RS EPRE according to Table 6.4.2.1.4.1-3.

3.2.	PRACH is set according to Table 6.4.2.1.4.1-3.

3.3.	The SS shall signal a Random Access Preamble ID via a PDCCH order to the UE and initiate a Non-contention based Random Access procedure.

3.4.	The UE shall send the signalled preamble to the SS.

3.5.	In response to the preamble, the SS shall transmit a random access response not corresponding to the transmitted random access preamble, or send no response.

3.6.	The UE shall consider the random access response reception not successful then re-transmit the preamble with the calculated PRACH transmission power.

3.7.	Measure the output power of the transmitted PRACH preamble at each antenna connector.

3.8.	Repeat step 3.5 ~ 3.7 until the SS collect enough PRACH preambles. Measure the EVM in PRACH channel using Global In-Channel Tx-Test (Annex E).

-	If the UE’s transmit power measured in step 3.7 is higher than the minimum output power requirement in 6.3G.1.5 at both antenna connectors, the composite EVM is measured according to E.6.9.3.

-	If UE’s transmit power measured in step 3.7 is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the EVM is measured only on this antenna connector according to E.6.9.2.

6.4G.2.1.4.3	Message contents

Same message contents as specified in 6.4.2.1.4.3.

6.4G.2.1.5	Test requirement

The PUSCH EVM, derived in steps 1.4 and 1.7, shall not exceed the values in Table 6.4G.2.1.5-1.

The PUSCH, derived in steps 1.4 and 1.7, shall not exceed the values in Table 6.4G.2.1.5-1 when embedded with data symbols of the respective modulation scheme.

Table 6.4G.2.1.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 + TT |
| QPSK | % | 17.5 + TT |
| 16 QAM | % | 12.5 + TT |
| 64 QAM | % | 8 + TT |
| 256 QAM | % | 3.5 + TT |
| Note 1: TT is defined in Table 6.4G.2.1.5-2. |  |  |

Table 6.4G.2.1.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 0 |
| QPSK | % | 0 |
| 16 QAM | % | 0 |
| 64 QAM | % | 0 |
| 256 QAM | % | 0.3 for 15 dBm < PUL 0.8 for -25 dBm < PUL≤ 15 dBm 1.1 for -40dBm ≤ PUL ≤ -25dBm |

The PUCCH EVM derived in steps 2.5 and 2.8 shall not exceed 17.5 %.

The PRACH EVM derived in step 3.8 shall not exceed 17.5%.

6.4G.2.2	Carrier leakage for Tx Diversity

6.4G.2.2.1	Test purpose

Same test purpose as in 6.4.2.2.1.

6.4G.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.4G.2.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the Relative Carrier Leakage Power requirements specified in Table 6.4.2.2.3-1 which is defined in clause 6.4.2.2.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4G.2.2.

6.4G.2.2.4	Test description

Same test description as in clause 6.4.2.2.4 with the following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

For test procedures:

-	Adding the following test steps:

10. Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

11. Measure carrier leakage using Global In-Channel Tx-Test (Annex E) with the measurement performed at each transmit antenna connector. For TDD, only slots consisting of only UL symbols are under test.

-	If the UE’s transmit power at both antenna connectors are higher than the minimum output power requirement in 6.3G.1.5, the carrier leakage is measured on each transmit antenna. Pass the UE if the carrier leakage on each antenna connector meets the requirements in 6.4G.2.2.5.

-	If UE’s transmit power is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the carrier leakage is measured only on this antenna connector. Pass the UE if the carrier leakage on this antenna connector meets the requirements in 6.4G.2.2.5.

6.4G.2.2.5	Test requirement

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE, carrier leakage measurement requirement shall be waived. Otherwise, for each transmit antenna connector, each of the n carrier leakage results, derived in Annex E.3.1, shall not exceed the values in Table 6.4G.2.2.5-1. Allocated RBs are not under test. n is 10 for 15kHz SCS, 20 for 30kHz SCS and 30 for 60kHz SCS.

Table 6.4G.2.2.5-1: Test requirements for Relative Carrier Leakage Power

| Parameters UE output power | Relative limit (dBc) |
| --- | --- |
| 10 + MU to 10 + (MU + Uplink power control window size) dBm | -28 + TT |
| 0 + MU to 0 + (MU + Uplink power control window size) dBm | -25 + TT |
| -30 + MU to -30 + (MU + Uplink power control window size) dBm | -20 + TT |
| Pmin + MU to Pmin + (MU + Uplink power control window size) dBm | -10 + TT |
| NOTE 1: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 2: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RBs containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 3: is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 4: MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW. NOTE 5: Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1. NOTE 6: Test tolerance TT = 0.8 dB. NOTE 7: Pmin is the minimum output power according to Table 6.3.1.3-1. |  |

6.4G.2.3	In-band emissions for Tx Diversity

6.4G.2.3.1	Test purpose

Same test purpose as in 6.4.2.3.1.

6.4G.2.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.4G.2.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the In-band Emission requirements specified in Table 6.4.2.3.3-1 which is defined in clause 6.4.2.3.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4G.2.3.

6.4G.2.3.4	Test description

Same test description as in clause 6.4.2.3.4 with the following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

For test procedures:

-	Adding the following test steps:

Test procedure for PUSCH:

1.10 Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

1.11 Measure In-band emission using Global In-Channel Tx-Test (Annex E) with the measurement performed at each transmit antenna connector. For TDD, only slots consisting of only UL symbols are under test.

Test procedure for PUCCH:

2.10 Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

2.11 Measure In-band emission using Global In-Channel Tx-Test (Annex E) with the measurement performed at each transmit antenna connector.

-	If the UE’s transmit power at both antenna connectors are higher than the minimum output power requirement in 6.3G.1.5, the In-band emissions are measured on each transmit antenna. Pass the UE if the In-band emissions on each antenna connector meets the requirements in 6.4G.2.3.5.

-	If UE’s transmit power is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the In-band emissions are measured only on this antenna connector. Pass the UE if the carrier leakage on this antenna connector meets the requirements in 6.4G.2.3.5.

6.4G.2.3.5	Test requirement

For each transmit antenna connector, the averaged In-band emission result, derived in Annex E.4.3 shall not exceed the corresponding values in Table 6.4G.2.3.5-1.

Table 6.4G.2.3.5-1: Test requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General (NOTE 12) | dB | + TT |  | Any non-allocated (NOTE 2) |
| IQ Image (NOTE 12) | dB | -28 + TT | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 + TT | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage (NOTE 12) | dBc | -28 + TT | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 + TT | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 + TT | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 + TT | -40 dBm ≤ Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RBs containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Section 5.3). NOTE 7: is the Transmission Bandwidth Configuration (see Section 5.3). NOTE 8: is the limit specified in Table 6.4G.2.1.3-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: Test tolerance TT = 0.8 dB. NOTE 12: In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE, IQ Image and Carrier leakage limit do not apply and General limit applies for all non-allocated frequencies. |  |  |  |  |

6.4G.2.4	EVM equalizer spectrum flatness for Tx Diversity

6.4G.2.4.1	Test purpose

Same test purpose as in 6.4.2.4.1.

6.4G.2.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.4G.2.4.3	Minimum conformance requirements

For UE supporting Tx diversity, the EVM Equalizer Spectrum Flatness requirements specified in Table 6.4.2.4.3-1 and Table 6.4.2.4.3-2 which are defined in clause 6.4.2.4.3. The composite EVM equalizer EC(f) is defined as

EC(f)=P1 ∙|EC1f|+P2 ∙|EC2(f)|P1+P2

where

ECn(f) represents equalizer coefficient for each antenna connector,  ，f is the allocated subcarriers within the transmission bandwidth ((|F|=12*);

P1 and P2 denote the linear power measured at each antenna connector respectively.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4G.2.4.

6.4G.2.4.4	Test description

Same test description as specified in clause 6.4.2.4.4 with following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

Step 3 of Test procedure as in 6.4.2.4.4.2 is replaced by:

3.	Measure the mean power at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

4.	Measure spectrum flatness using Global In-Channel Tx-Test (Annex E). For TDD, only slots consisting of only UL symbols are under test.

-	If the UE’s transmit power at both antenna connectors are higher than the minimum output power requirement in 6.3G.1.5, spectrum flatness is measured at both antenna connectors and the composite EVM equalizer EC(f) is determined by applying

EC(f)=P1 ∙|EC1f|+P2 ∙|EC2(f)|P1+P2

where

ECn(f) represents equalizer coefficient for each antenna connector,  ，f is the allocated subcarriers within the transmission bandwidth ((|F|=12*);

P1 and P2 denote the linear power measured at each antenna connector respectively.

-	If UE’s transmit power is higher than the minimum output power requirement in 6.3G.1.5 at only one antenna connector, the spectrum flatness is measured only on this antenna connector using Global In-Channel Tx-Test (Annex E).

<Unchanged Text Skipped>

6.5G.1	Occupied bandwidth for Tx Diversity

6.5G.1.1	Test purpose

To verify that the UE occupied bandwidth for Tx Diversity for all transmission bandwidth configurations supported by the UE are less than their specific limits.

6.5G.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for occupied bandwidth apply to the transmitted spectrum as measured as the sum of the power from all UE transmit antenna connectors. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.1.

6.5G.1.4	Test description

Same test description as specified in clause 6.5.1.4 with following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

Step 3 of Test procedure as in 6.5.1.4.2 is replaced by:

3.	Measure the power spectrum distribution as the sum of the power from all UE transmit antenna connectors within two times or more range over the requirement for Occupied Bandwidth specification centring on the current carrier frequency. The characteristics of the filter shall be approximately Gaussian (typical spectrum analyser filter). Other methods to measure the power spectrum distribution are allowed. The measuring duration is at least 1ms over consecutive active uplink slots.

6.5G.1.5	Test requirement

The measured Occupied Bandwidth shall not exceed values in Table 6.5.1.5-1.

<Unchanged Text Skipped>

6.5G.2.1	Spectrum emission mask for Tx Diversity

6.5G.2.1.1	Test purpose

Same test purpose as in 6.5.2.2.1.

6.5G.2.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.2.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters apply to the sum of the emissions from all UE transmit antenna connectors.

The requirements specified in clause 6.5.2 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.1

6.5G.2.1.4	Test description

6.5G.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.5G.2.1.4.1-1, 6.5G.2.1.4.1-2, 6.5G.2.1.4.1-2a and Table 6.5G.2.1.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5G.2.1.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Spectrum Emission Mask test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 23 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 33 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 44 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Left |
| 54 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Edge_1RB_Right |
| 64 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM QPSK | Edger_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 13 | Low |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 16 | Low |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  |  |  | DFT-s-OFDM 256 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 22 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 23 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 24 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 25 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 26 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 27 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 28 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 29 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 30 | Default |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| 315,6 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 325,6 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 335,6 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: For Power Class 3 testing, UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. NOTE 6: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. NOTE 7: It is essential that all test points in this table also exist in table 6.2G.2.4.1-2a. |  |  |  |  |  |  |

Table 6.5G.2.1.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: It is essential that all test points in this table also exist in table 6.2G.2.4.1-1. |  |  |  |  |

Table 6.5G.2.1.4.1-3: Test Configuration Table for power class 1.5 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Spectrum Emission Mask test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 2 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 3 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 4 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 5 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 7 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 10 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 16 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  | CP-OFDM QPSK | Outer Full |
| 19 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 22 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 25 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 28 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 29 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 30 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. NOTE 4: It is essential that all test points in this table also exist in table 6.2G.2.4.1-2. |  |  |  |  |

Table 6.5G.2.1.4.1-4: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2G.2.4.1-4. NOTE 2: It is essential that all test points in this table also exist in table 6.2G.2.4.1-3. NOTE 3: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5G.2.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5G.2.1.4.3.

<Unchanged Text Skipped>

6.5G.2.2	Additional spectrum emission mask for Tx Diversity

Editor’s Note: The following aspects are either missing or not yet determined:

- Tests for network signalling values other than NS_04 are incomplete due to lack of test requirement in 6.2G.3.

6.5G.2.2.1	Test purpose

The same test purpose as in 6.5.2.3.1.

6.5G.2.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.2.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters apply to the sum of the emissions from all UE transmit antenna connectors.

If UE indicates IE Tx diversity capability, Adjacent Channel Leakage power Ratio (ACLR) is defined as the ratio of sum of the filtered mean power at each antenna connector centred on the assigned channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent channel frequency.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.

6.5G.2.2.4	Test description

Same test description as specified in clause 6.5.2.3.4 with following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

Step 3 and 4 of Test procedure as in 6.5.2.3.4.2 is replaced by:

3.	Measure the mean power of the UE as the sum of the power from all UE transmit antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in applicable table of clause 6.2G.3.5. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal as the sum of the power from all UE transmit antenna connectors with a measurement filter of bandwidths according to applicable test configuration tables in subclause 6.5G.2.2.5 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the applicable test requirement table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

<Unchanged Text Skipped>

6.5G.2.3.2	UTRA ACLR for Tx Diversity

6.5G.2.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5G.2.3.2.2	Test applicability

This test case applies for network signalling values NS_03U, NS_05U, NS_43U, and NS_100 to all types of NR Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.2.3.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters apply to the sum of the emissions from all UE transmit antenna connectors.

If UE indicates IE txDiversity-r16, Adjacent Channel Leakage power Ratio (ACLR) is defined as the ratio of sum of the filtered mean power at each antenna connector centred on the assigned channel frequency to sum of the filtered mean power at each antenna connector centred on an adjacent channel frequency.

The requirements specified in clause 6.5.2 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.2.

6.5G.2.3.2.4	Test description

Same test description as specified in clause 6.5.2.4.2.4 with following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2 for TE diagram and section A.3.2 for UE diagram.

Step 3, 4 and 5 of Test procedure as in 6.5.1.4.2 is replaced by:

3.	Measure the mean power of the UE as the sum of the power from all UE transmit antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in 6.2.3.5 as appropriate. The period of the measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD slots with transient periods are not under test.

4.	Measure the rectangular filtered mean power as the sum of the power from all UE transmit antenna connectors for the assigned NR channel using a rms detector. If the sweep count is higher than one, the trace mode shall be average.

5.	Measure the RRC filtered mean power as the sum of the power from all UE transmit antenna connectors of the first and the second UTRA adjacent channel on both lower and upper side of the NR channel using a rms detector, respectively. If the sweep count is higher than one, the trace mode shall be average.

6.5G.2.3.2.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements described in clause 6.2G.3.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured UTRA ACLR, derived in step 6, shall be higher than the limits in table 6.5G.2.3.2.5-1.

Table 6.5G.2.3.2.5-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB -TT |
| UTRAACLR2 | 36 dB - TT |
| NOTE 1: TT = 0.8 dB |  |

6.5G.3.1	General spurious emissions for Tx Diversity

6.5G.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

6.5G.3.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.3.1.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products apply to the sum of the emissions from all UE transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5G.3.

6.5G.3.1.4	Test description

Same test description as specified in clause 6.5.3.1.4 with following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

Step 3 of Test procedure as in 6.5.3.1.4.2 is replaced by:

3.	Measure the sum power of the transmitted signal at all antenna connectors with a measurement filter of bandwidths according to Table 6.5G.3.1.5-1. The centre frequency of the filter shall be stepped in contiguous steps according to Table 6.5G.3.1.5-1. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5G.3.1.5	Test requirement

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.5G.3.1.5-1.

Table 6.5G.3.1.5-1: General spurious emissions for Tx Diversity test requirements

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz | 4 |
|  | -25 dBm | 1 MHz | 3 |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 2.55 GHz and less than or equal to 5.2 GHz NOTE 2: Applies for Band that the upper frequency edge of the UL Band more than 5.2 GHz NOTE 3: Applies for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in sub-clause 5.2B of 38.101-3 [4] when NS_04 is signalled. NOTE 4: Does not apply for Band n41, CA configurations including Band n41, and EN-DC configurations that include n41 specified in subclause 5.2B of TS 38.101-3 [4] when NS_04 is signalled. |  |  |  |

<Unchanged Text Skipped>

6.5G.3.2	Spurious emissions for UE co-existence for Tx Diversity

6.5G.3.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5G.3.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.3.2.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products apply to the sum of the emissions from all UE transmit antenna connectors.

The requirements specified in clause 6.5.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5G.3.

6.5G.3.2.4	Test description

Same test description as specified in clause 6.5.3.2.4 with following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

Step 3 of Test procedure as in 6.5.3.2.4.2 is replaced by:

3.	Measure the sum power of the transmitted signal at all antenna connectors with a measurement filter of bandwidths according to table 6.5.3.2.3-1 to 6.5.3.2.3-3. The centre frequency of the filter shall be stepped in contiguous steps according to table 6.5.3.2.3-1 to 6.5.3.2.3-3. The measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

6.5G.3.2.5	Test requirement

Test requirements for Spurious Emissions UE Co-existence are the same as the minimum requirements and are not repeated in this section.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Tables 6.5.3.1.3-1 from the edge of the channel bandwidth. The spurious emission limits in Tables 6.5.3.2.3-1 to 6.5.3.2.3-3 apply for all transmitter band configurations (NRB) and channel bandwidths.

The measured average power of spurious emission, derived in step 3, shall not exceed the described value in Table 6.5.3.2.3-1 to 6.5.3.2.3-3 for difference releases.

The requirements for the UE are release specific and can be found in Tables 6.5.3.2.3-1 to 6.5.3.2.3-3. If the UE support a band, which is not defined in the table corresponding UE’s release, the requirements for this band are taken from the table of earliest release where requirements for this band are defined. This has been described in following Table 6.5G.3.2.5-1.

Table 6.5G.3.2.5-1: UE Requirements according to UE NR release and supported E-UTRA and NR band

| UE Requirements per release |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR Band | Rel-15 | Rel-16 | R17 | R18 |
| n1 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n2 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n3 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n5 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n7 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n8 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n12 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n13 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 |  |
| n14 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n18 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n20 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n24 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n25 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n26 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n28 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n30 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n34 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n38 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n39 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n40 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n41 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n48 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n50 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n51 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n53 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n65 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n66 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n70 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n71 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n74 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n77, n78 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n79 | Table 6.5.3.2.3-1 | Table 6.5.3.2.3-2 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n85 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 |  |
| n104 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-3 | Table 6.5.3.2.3-4 |
| n105 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 | Table 6.5.3.2.3-4 |
| NOTE 1: The frequency range applicable with network signalling values of NS_04, NS_17, NS_18, NS_05, NS_43, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_45 and NS_56 are covered in subclause 6.5.3.3 Additional Spurious Emissions NOTE 2: The restriction on the maximum uplink transmission to 54 RB in Notes 21 and 22 of Tables 6.5G.3.2.3-1 to 6.5G.3.2.3-3 is intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future. |  |  |  |  |

<Unchanged Text Skipped>

6.5G.3.3	Additional spurious emissions for Tx Diversity

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values other than NS_04 and NS_47 are incomplete due to lack of test requirement in 6.2G.3.

6.5G.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5G.3.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.3.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products apply to the sum of the emissions from all UE transmit antenna connectors.

The requirements specified in clause 6.5.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.5G.3.

6.5G.3.3.4	Test description

For Power Class 2 and Power Class 3 UE, same test description as specified in clause 6.5.3.3.4 with following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.2 for TE diagram and section A.3.2 for UE diagram.

Steps 3 and 4 of Test procedure as in 6.5.3.3.4.2 is replaced by:

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode, which shall meet the requirements described in Clauses  6.2G.3.5. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum power of the transmitted signal at all antenna connectors with a measurement filter of bandwidths according to Tables 6.5.3.3.3.1 to 6.5.3.3.3.27 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table the measured power shall be verified for each step. The measurement period shall capture the active time slots. During measurement the spectrum analyser shall be set to 'Detector' = RMS. If the sweep count is higher than one, the trace mode shall be average.

For Power Class 1.5 UE, same test configuration as listed in Table 6.2G.3.4.1-2 shall be used for NS_50 (Power Class 1.5) and Table 6.2G.3.4.1-1 for NS_47 (Power Class 1.5).

6.5G.3.3.5	Test requirement

This clause specifies the requirements for the specified NR band for an additional spectrum emission requirement with protected bands as indicated from Table 6.5.3.3.5.1 to Table 6.5.3.3.5.27 for different NS_values.

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements specified as 6.2G.3.5 for different NS_values.

The measured power derived in step 4 shall meet the requirements for the specified NR band for an additional spurious emission requirement with protected bands as indicated in clause 6.5.3.3.5 for different NS values.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5G.4	Transmit intermodulation for Tx Diversity

6.5G.4.1	Test purpose

To verify that the UE transmit intermodulation for Tx Diversity does not exceed the described value in the test requirement.

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

6.5G.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.5G.4.3	Minimum conformance requirements

For UE supporting Tx diversity, the transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output power from all UE transmit antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5G.4.

6.5G.4.4	Test description

Same test description as specified in clause 6.5.4.4 with following exceptions:

For initial condition:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.3.2 for TE diagram and section A.3.2 for UE diagram.

Test procedure as in 6.5.4.4.2 is replaced by:

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands to the UE until the UE transmits at its PUMAX level.

3.	Measure the sum of rectangular filtered mean power from all antenna connectors of the UE. For TDD, only slots consisting of only UL symbols are under test for the wanted signal and for the intermodulation product.

4.	Set the interference signal frequency below the UL carrier frequency using the first offset in table 6.5.4.5-1.

5.	Set the interference CW signal level according to table 6.5.4.5-1.

6.	Search the intermodulation product signals below and above the UL carrier frequency at each UE antenna connector, then measure the rectangular filtered mean power at each antenna connector of transmitting intermodulation for both signals, and calculate the ratios with the power measured in step 3.

7.	Set the interference signal frequency above the UL carrier frequency using the first offset in table 6.5.4.5-1.

8.	Search the intermodulation product signals below and above the UL carrier frequency at each UE antenna connector, then measure the rectangular filtered mean power at each antenna connector of transmitting intermodulation for both signals, and calculate the ratios with the power measured in step 3.

9.	Repeat the measurement using the second offset in table 6.5.4.5-1.

10.	Repeat step 3) until 9) for each of transmit antenna of the UE.

<End of Changes>