3GPP TSG-RAN5 Meeting #108	R5-254862

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1187 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of FR2 RF testability issue list to specification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Need to have consolidated list of FR2 tests with testability issues in the test specification for certification and regulatory ecosystem reference |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added list of FR2 tests with testability issues from TR 38.903 into test specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing consolidated list of such FR2 testability related issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated discussion paper in R5-254861 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanges sections skipped>

# A.6	FR2 RF tests with testability issues

## A.6.1	FR2 RF tests with testability issues not related to Measurement Uncertainty (MU)

Editor’s note: The list of FR2 RF testability issues not related to MU and listed in the table below is incomplete and ongoing updates.

Table A.6-1: FR2 RF test cases and testability issues not related to MU

| Clause | Requirement | FR2 RF Testability issue not related to MU |
| --- | --- | --- |
| 6.2.2 | UE maximum output power reduction | How to deal with power classes reusing PC3 MPR requirements, especially those defined from Release 17 and forward, and then the relationship with 6.2.2_1 test is FFS. |
| 6.2D.1.1 | UE maximum output power - EIRP and TRP for UL MIMO | No test points are defined for 2-layer UL MIMO since there is no configuration satisfying MPR=0dB requirements in RAN4. |
| 6.3.2 | Transmit OFF Power | Test Procedure aspects for UE indicating ul-GapFR2-r17 is FFS |
| 6.3.3.1 | PRACH Time Mask | Further investigation is essential that how does beamforming affect the initial access procedure |
| 6.3D.3.1 | General ON/OFF time mask for UL MIMO | Testability of OFF power needs further study OTA test procedure for UL-MIMO is still under investigation |
| 6.4.2.3 | In-band emissions | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) | This test is incomplete due to lack of RRC framework for LO position retrieval |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) |  |
| 6.4A.2.2.2 | Carrier leakage for CA (3UL CA) |  |
| 6.4A.2.2.3 | Carrier leakage for CA (4UL CA) |  |
| 6.4A.2.2.4 | Carrier leakage for CA (5UL CA) |  |
| 6.4A.2.2.5 | Carrier leakage for CA (6UL CA) |  |
| 6.4A.2.2.6 | Carrier leakage for CA (7UL CA) |  |
| 6.4A.2.2.7 | Carrier leakage for CA (8UL CA) |  |
| 6.4A.2.3.1 | In-band emissions for CA (2UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.2 | In-band emissions for CA (3UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.3 | In-band emissions for CA (4UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.4 | In-band emissions for CA (5UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.5 | In-band emissions for CA (6UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.6 | In-band emissions for CA (7UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4A.2.3.7 | In-band emissions for CA (8UL CA) | Testing of the general in-band emission requirement and if yes at which UE Tx power level and with which relaxation applied to the requirement is FFS. |
| 6.4.2.5 | EVM spectral flatness for pi/2 BPSK modulation | Whether and, if yes, how to test the requirement on shaping filter is FFS. |
| 7.9 | Spurious Emissions | Connection diagram between SS and UE in TS 38.508-1 [10] Annex A is FFS. |

## A.6.2	FR2 RF tests with testability issues related to Measurement Uncertainty (MU)

Editor’s note: Please note that the list of FR2 RF testability issues related to MU and listed in the table below is incomplete and ongoing updates. This list is also maintained in parallel in TR 38.903 Annex B.26

Table B.26-1: FR2 RF test cases with known testability issues related to MU

| Clause | Requirement | FR2 RF Testability issues related to MU | Notes |
| --- | --- | --- | --- |
| 6.3.1 | Minimum output power | Low UL power | For several test points (details in sub-clause 6.3.1.5), Core requirement cannot be tested due to testability issue and test requirement includes relaxation to achieve impact from test system noise to measurement result = 1.0 dB (Minimum requirement + relaxation). |
| 6.3.2 | Transmit OFF power | Low UL power | For all FR2 bands and channel bandwidths, Core requirement cannot be tested due to testability issue and test requirement includes relaxation to achieve impact from test system noise to measurement result = 1.0 dB (Minimum requirement + relaxation). |
| 6.3.4.3 | Relative Power Tolerance | Starting power at ramp up/ramp down/alternating sub-test is TBD (6.3.4.3 MU dependent) | Testability issue due to narrow range for 1 dB TPC step core requirement and therefore testing is not recommended. |
| 6.3D.2 | Transmit OFF power for UL MIMO | The testability of this test case is pending further analysis on relaxation of the requirement for other than Band n257. | For several test points (see 6.3D.2.5 for details), Core requirement cannot be tested due to testability issue and test requirement includes relaxation to achieve impact from test system noise to measurement result = 1.0 dB (Minimum requirement + relaxation). |
| 6.5.1 | Occupied bandwidth | High SNR required for measurement | To avoid testability issues, MBW has been reduced from 2*CBW (typical used value in FR1) down to 1.5*CBW in general and even down to 1.3*CBW for PC3 400MHz in FR2c. |
| 6.5.2.1 | Spectrum Emission Mask | Low Spurious Emission power | Testability in FR2b for PC1 is FFS. For PC1 in FR2a, to avoid defining relaxations, accepted to use influence of noise remarkable higher than 1dB in the MTSU calculation. |
| 6.5.2.3 | Adjacent channel leakage ratio | Low adjacent channel power | Relaxation due to testability limits applied for several TC IDs and MPR values as defined in subclause 6.5.2.3.5 |
| 6.5.3.2 | Additional spurious emissions | Low Spurious Emission power | Relaxation due to testability limit applied to test requirements as per subclause 6.5.3.2.5 |
| 6.5D.2.1 | Occupied bandwidth for UL MIMO | High SNR required for measurement | Testability is FFS for PC3 FR2b and FR2c 400 MHz, PC3 FR2c 200 MHz and other power classes. |
| 7.4 | Maximum input power | High DL power | The test requirements deviate from minimum requirements by 26dB relaxation for 24.25 ~ 29.5 GHz and 34 dB relaxation for 37 ~ 40 GHz. |
| 7.5 | Adjacent channel selectivity (case 1) | High DL power | For several test points, Core requirement cannot be tested due to testability issue and test requirement for wanted signal and interferer includes relaxation to achieve feasible interferer power level. See details in subclause 7.5.5 |
| 7.6.2 | In-band blocking | High DL power | For several test points, Core requirement cannot be tested due to testability issue and test requirement for wanted signal and interferer includes relaxation to achieve feasible interferer power level. See details in subclause 7.6.2.5 |
| 7.9 | Receiver spurious emissions | Low Spurious Emission power | The testability of this test case is pending further analysis on relaxation of the requirement for band other than n257, n258, n259, n260 and n261 |

<End of changes>