3GPP TSG-RAN5 Meeting #109	R5-255560

# Dallas, US, 17th – 21st November, 2025

Agenda item:	5.3.39.4

Source:	Keysight Technologies UK Ltd

Title:	FR2 MU - Discussion on MU and TT update for NS_202 and NS_205 additional spurious requirements

Document for:	Discussion and Endorsement

# Introduction

As part of the NR_mmWave_protect-UEConTest WI, additional spurious requirements for NS_202 will be updated and requirements for new NS_205 will be added into RAN5 specs. This contribution will analyse the MTSU, TT and relaxation to apply to these new and updated requirements.

# Discussion

## Requirements

The requirements were introduced in the core specs by [1] under NR_mmWave_protect-Core WI. Main changes to be considered in this analysis are reproduced below:

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR) Network Signalling label; Requirements (clause); NR Band; Channel bandwidth (MHz); Resources Blocks (NRB); A-MPR (dB). NS_200; N/A. NS_201 (NOTE 1); 6.5.3.2.2; n258; 6.2.3.2. NS_202; 6.5.3.2.3; n257, n258; 50, 100, 200, 400; Table 5.3.2-1; 6.2.3.3. NS_203; 6.5.3.2.4; n258; 50, 100, 200, 400; Table 5.3.2-1; 6.2.3.4. NS_205; 6.5.3.2.6; n257, n258; 50,100, 200, 400; Table 5.3.2-1; 6.2.3.6. NOTE 1: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable.. Table 6.2.3.1-2: Mapping of Network Signalling labe NR Band; Value of additionalSpectrumEmission. 0; 1; 2; 3; 4; 5; 6; 7. n257; NS_200; NS_202; NS_205. n258; NS_200; NS_201 (NOTE 2); NS_202; NS_203; NS_205. n259; NS_200. n260; NS_200. n261; NS_200. n263; NS_200; NS_204. NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in sub-clause 6.3.2 of TS 38.331 [13]. NOTE 2: NS_201 is obsolete, the associated additional spurious emission requirements are not applicable..

From previous changes it can be observed that both former NS_202 and new NS_205 apply to the same bands n257 and n258.

Observation 1. Former NS_202 and new NS_205 apply to the same bands n257 and n258.

Table 6.5.3.2.3-1: Additional requirements (NS_202) Frequency Range; Maximum Level; Measurement bandwidth; NOTE. 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band; -10 dBm; 100 MHz. 23.6 GHz f 24.0 GHz; -5 dBm; 200 MHz; 1. NOTE 1: This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services.. <<Unchanged Omitted>> ##### 6.5.3.2.6 Additional spurious emission requirements for NS_205 When "NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth. Table 6.5.3.2.6-1: Additional requirements (NS_205) Frequency band (GHz); Spectrum emission limit (dBm); Measurement bandwidth. 23.6 f 24.0; -5; 200 MHz.

Regarding the additional spurious requirements, NS_202 requirement in the range 23.6 GHz f 24.0 GHz has been hardened by 6dB, from 1dBm/200MHz to -5dBm/200MHz. At the same time, the additional spurious requirement for the new NS_205 has been defined as a subset of the requirement for NS_202, restricted to the range 23.6 GHz f 24.0 GHz and requiring the same limit of -5dBm/200MHz.

Observation 2. Additional spurious requirement for NS_202 in the range 23.6 GHz f 24.0 GHz has been updated and become 6dB harder, from +1dBm/200MHz to -5dBm/200MHz.

Observation 3. Additional spurious requirement for new NS_205 is the same as defined for NS_202 in the range 23.6 GHz f 24.0 GHz, -5dBm/200MHz.

## MTSU, Relaxations and TT analysis

As per observations made above, the protected frequency range, 23.6 GHz f 24.0 GHz, and the applicable bands, n257 and n258, have not changed. Therefore, the same noise floor should be assumed.

Proposal 1. For additional spurious requirement for new NS_205 and updated NS_202 in the range 23.6 GHz f 24.0 GHz, assume the same noise floor already assumed for NS_202 in such a frequency range.

With such a noise floor, the former additional spurious requirement for NS_202 in that frequency already required to apply certain relaxation as captured in [3], section B.2.2.27. With such a relaxation, the influence of noise is limited to 1dB. Observations made in previous section 2.1 indicate that the requirement is now 6dB harder. Therefore, in order to maintain the influence of noise limited to 1dB, the required relaxation must be increased by 6dB.

Proposal 2. For additional spurious requirement for new NS_205 and updated NS_202 in the range 23.6 GHz f 24.0 GHz, in order to maintain the influence of noise limited to 1dB, increase by 6dB the relaxation formerly assumed for NS_202 in such a frequency range.

If previous proposals are endorsed, the resulting MTSU will not require any update other than adding references to the new NS_205.

Observation 4. If Proposal 1 and Proposal 2 are endorsed, no changes will be required in the MTSU definition other than adding references to the new NS_205.

Regarding test tolerances, TT = 0dB is applied for all spurious test cases in [2], annex F.3. There is no reason to proceed differently for new additional spurious requirements analysed in this document so same value will be proposed.

Proposal 3. For additional spurious requirement for new NS_205 and updated NS_202 in the range 23.6 GHz f 24.0 GHz, apply TT=0dB.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. Former NS_202 and new NS_205 apply to the same bands n257 and n258.

Observation 2. Additional spurious requirement for NS_202 in the range 23.6 GHz f 24.0 GHz has been updated and become 6dB harder, from +1dBm/200MHz to -5dBm/200MHz.

Observation 3. Additional spurious requirement for new NS_205 is the same as defined for NS_202 in the range 23.6 GHz f 24.0 GHz, -5dBm/200MHz.

Observation 4. If Proposal 1 and Proposal 2 are endorsed, no changes will be required in the MTSU definition other than adding references to the new NS_205.

Proposal 1. For additional spurious requirement for new NS_205 and updated NS_202 in the range 23.6 GHz f 24.0 GHz, assume the same noise floor already assumed for NS_202 in such a frequency range.

Proposal 2. For additional spurious requirement for new NS_205 and updated NS_202 in the range 23.6 GHz f 24.0 GHz, in order to maintain the influence of noise limited to 1dB, increase by 6dB the relaxation formerly assumed for NS_202 in such a frequency range.

Proposal 3. For additional spurious requirement for new NS_205 and updated NS_202 in the range 23.6 GHz f 24.0 GHz, apply TT=0dB.

# References

- R4-2501408, “Big CR on UE RF requirements for WI NR_mmWave_protect”, China Unicom, Huawei, HiSilicon, RAN4#114

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 2 Standalone”, v19.0.0 (2025-09)

- TR 38.903, “NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance tests”, v19.0.0 (2025-09)