3GPP TSG-RAN5 Meeting #109	R5-255857

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0076 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Element, Ericsson, FirstNet, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case, 6.4.3 On-network / Emergency Alert / Group Geographic Area included in the Work Plan is not implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case, 6.4.3 On-network / Emergency Alert / Group Geographic Area, according to the Work Plan |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case, 6.4.3 On-network / Emergency Alert / Group Geographic Area, included in the Work Plan will not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 6.4.2.3.3-5: MCData-Info in SIP MESSAGE (Table 6.4.2.3.3-4)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-group-id | Encrypted <mcdata-calling-group-id> with mcdataURI set to px_MCData_Group_A_ID | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-3 |  |  |
| alert-ind | Encrypted <alert-ind> with mcdataBoolean set to false | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-3 |  |  |

### 6.4.3	On-network / Emergency Alert / Group geographic area

6.4.3.1	Test Purpose (TP)

(1)

with { UE (MCData Client) registered and authorized for MCData Service }

ensure that {
  when { the MCData Server notifies the UE (MCData Client) of entry into a group geographic area by sending the UE (MCData Client) a SIP MESSAGE }

then { UE (MCData Client) acknowledges the entry into a group geographic area notification by sending a SIP 200 (OK) response and notifies the user of the entry into the group geographic area }

}

(2)

with { UE (MCData Client) having been previously notified of entry into a group geographic area }

ensure that {
  when { MCData User requests to affiliate to the received MCData group }

then { UE (MCData Client) requests to subscribe to affiliation status changes for the MCData User by sending the SS a SIP SUBSCRIBE message and starts informing the MCData User of any affiliation status changes for the MCData User after the subscription is accepted }

}

(3)

with { UE (MCData Client) affiliated with a group geographic area  }

ensure that {
  when { the MCData Server notifies the UE (MCData Client) of exit from a group geographic area by sending the UE (MCData Client) a SIP MESSAGE }

then { UE (MCData Client) acknowledges the exit from a group geographic area notification by sending a SIP 200 (OK) response and notifies the user of the exit from the group geographic area }

}

(4)

with { UE (MCData Client) having been previously notified of exit from a group geographic area }

ensure that {
  when { MCData User requests to de-affiliate from the received MCData group }

then { UE (MCData Client) requests to de-affiliate from an MCData group by sending the SS a SIP PUBLISH message }

}

6.4.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: 24.282 clause 16.2.1.4. Unless otherwise stated, these are Rel-17 requirements.

[24.282 clause 16.2.1.4]

Upon receipt of a "SIP MESSAGE request for notification of entry into or exit from a group geographic area", the MCData client:

1)	shall send a SIP 200 (OK) to the participating MCData function that sent the SIP MESSAGE request; and

2)	if the <group-geo-area-ind> element of the application/vnd.3gpp.mcdata-info+xml MIME body is:

a)	set to "true":

i)	may display to the MCData user an indication that a group geographic area has been entered; and

ii)	shall execute the procedure in clause 8.2.2 to affiliate to the group indicated by the participating MCData function; and

b)	set to "false":

i)	may display to the MCData user an indication that a group geographic area has been exited; and

ii)	shall execute the procedure in clause 8.2.2 to de-affiliate from the group indicated by the participating MCData function.

6.4.3.3	Test description

6.4.3.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.4.3.3.2	Test procedure sequence

Table 6.4.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to receive notification of entry into a group geographic area? | - | - | 1 | P |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication that a group geographic area has been entered. | - | - | - | - |
| 2a1 | IF pc_MCX_DisplayInfoGroupGeographicArea THEN Check: Does the UE (MCData client) notify the user that a group geographic area has been entered? (NOTE 1) | - | - | 1 | P |
| 3-8 | Check: Does the UE (MCData client) correctly perform steps 2-7 of procedure 'MCX Group Affiliation status change' as described in TS 37.579-1 [2] Table 5.3.34.3-1 to have the target user affiliate with group A? |  |  | 2 | P |
| 9 | Check: Does the UE (MCData client) correctly perform procedure 'MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to receive notification of exit from a group geographic area? | - | - | 3 | P |
| - | EXCEPTION: Step 10a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication that a group geographic area has been exited. |  |  |  |  |
| 10a | IF pc_MCX_DisplayInfoGroupGeographicArea THEN Check: Does the UE (MCData client) notify the user that a group geographic area has been exited? (NOTE 1) |  |  | 3 | P |
| 11-16 | Check: Does the UE (MCData client) correctly perform steps 2-7 of procedure 'MCX Group Affiliation status change' as described in TS 37.579-1 [2] Table 5.3.34.3-1 to de-affiliate with group A? |  |  | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.4.3.3.3	Specific message contents

Table 6.4.3.3.3-1: SIP MESSAGE from the SS (step 1, Table 6.4.3.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO, ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.3.3.3-2 |  |  |  |

Table 6.4.3.3.3-2: MCData-Info in SIP MESSAGE (Table 6.4.3.3.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| associated-group-id | px_MCData_Group_A_ID |  |  |  |
| anyExt |  |  |  |  |
| group-geo-area-ind | true |  |  |  |

Table 6.4.3.3.3-3: SIP PUBLISH from the UE (step 3, Table 6.4.3.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.34.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.11-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.3.3.3-4 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 6.4.3.3.3-5 |  |  |  |

Table 6.4.3.3.3-4: MCData-Info in SIP PUBLISH (Table 6.4.3.3.3-3)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | px_MCData_ID_User_A |  |  |  |

Table 6.4.3.3.3-5: PIDF in SIP PUBLISH (Table 6.4.3.3.3-3)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.5.1-2, condition AFFILIATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| Entity attribute | px_MCData_ID_User_A |  |  |  |
| tuple |  |  |  |  |
| Id attribute | px_MCX_Client_A_ID |  |  |  |
| p-id | any allowed value | p-id shall be present acc. to TS 24.282 [31] clause 8.3.2.5 and clause 8.3.3.5 |  |  |

Table 6.4.3.3.3-6: SIP MESSAGE from the SS (step 9, Table 6.4.3.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition LOCATION-INFO, ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.3.3.3-2 |  |  |  |

Table 6.4.3.3.3-7: MCData-Info in SIP MESSAGE (Table 6.4.3.3.3-6)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| associated-group-id | px_MCData_Group_A_ID |  |  |  |
| anyExt |  |  |  |  |
| group-geo-area-ind | false |  |  |  |

Table 6.4.3.3.3-8: SIP PUBLISH from the UE (step 11, Table 6.4.3.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3.34.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.11-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| delta-seconds | "0" |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.3.3.3-9 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF as described in Table 6.4.3.3.3-10 |  |  |  |

Table 6.4.3.3.3-9: MCPTT-Info in SIP PUBLISH (Table 6.4.3.3.3-8)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | px_MCData_ID_User_A |  |  |  |

Table 6.4.3.3.3-10: PIDF in SIP PUBLISH (Table 6.4.3.3.3-8)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.5.1-2, condition AFFILIATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| presence |  |  |  |  |
| tuple |  |  |  |  |
| status | Not present |  |  |  |
| p-id | any allowed value | p-id shall be present acc. to TS 24.282 [31] clause 8.3.2.5 and clause 8.3.3.5 |  |  |