3GPP TSG-RAN5 Meeting #109	R5-255890

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5183 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UAV event A3H1 reporting test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Atheros, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There were some typos identified in the Test procedure sequence for Test cases 8.1.3.5.3, reportConfigNR Table reference is wrong for test case 8.1.3.5.4 and 8.1.3.5.5. measGapConfig was missing for TC 8.1.3.5.5. Simulated aerial heights previously defined may not work with +/-90m allowance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected Test procedure sequence for Test cases 8.1.3.5.3. Table reference updated for TC 8.1.3.5.4 and 8.1.3.5.5 for reportConfigNR. measGapConfig added for TC 8.1.3.5.5. Simulated aerial heights are updated. Hysteresis, Threshold values are updated in ReportConfigNR for both the test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose for test cases 8.1.3.5.3, 8.1.3.5.4 and 8.1.3.5.5 will remain incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.5.2.3.2, 8.1.3.5.4.3.3 and 8.1.3.5.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 8.1.3.5.3	NR UAV / Measurement configuration control and reporting / Event A3H1 / Measurement of Neighbour NR cell / Intra-frequency measurements

8.1.3.5.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and intra-frequency measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes higher than the absolute threshold plus hysteresis }

then { UE sends MeasurementReport with correct measdID for event A3H1}

}

(2)

with { UE in RRC_CONNECTED state and intra-frequency measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes lower than the absolute threshold minus hysteresis }

then { UE stops sending MeasurementReport }

}

(3)

with { UE in RRC_CONNECTED state and intra-frequency measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes lower than serving cell and the Aerial UE altitude becomes lower than the absolute threshold minus hysteresis }

then { UE does not send MeasurementReport }

}

8.1.3.5.3.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.23 and 5.5.5.1 Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch:

. . .

1>	if the RRCReconfiguration message includes the aerial-Config:

2>	(re)configure the aerial parameters in accordance with the included aerial-Config;

[TS 38.331, clause 5.5.4.1]

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	else if the reportType is set to eventTriggered and if the eventId is set to eventD1 or eventD2 or eventH1 or eventH2 and if the entering condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

initiate the measurement reporting procedure, as specified in 5.5.5;

…

[TS 38.331, clause 5.5.4.23]

The UE shall:

1>	consider the entering condition for this event to be satisfied when both condition A3H1-1 and condition A3H1-2, as specified below, are fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3H1-3 or condition A3H1-4, i.e. at least one of the two, as specified below, is fulfilled;

1>	use the SpCell for Mp, Ofp and Ocp.

NOTE 1:	The cell(s) that triggers the event has reference signals indicated in the measObjectNR associated to this event which may be different from the NR SpCell measObjectNR.

Inequality A3H1-1 (Entering condition 1)

Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off

Inequality A3H1-2 (Entering condition 2)

Ms – Hys2 > Thresh

Inequality A3H1-3 (Leaving condition 1)

Mn + Ofn + Ocn + Hys1 < Mp + Ofp + Ocp + Off

Inequality A3H1-4 (Leaving condition 2)

Ms + Hys2 < Thresh

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the measurement object specific offset of the reference signal of the neighbour cell (i.e. offsetMO as defined within measObjectNR corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the frequency of the neighbour cell, or cellIndividualOffset as defined within reportConfigNR), and set to zero if not configured for the neighbour cell.

Mp is the measurement result of the SpCell, not taking into account any offsets.

Ofp is the measurement object specific offset of the SpCell (i.e. offsetMO as defined within measObjectNR corresponding to the SpCell).

Ocp is the cell specific offset of the SpCell (i.e. cellIndividualOffset as defined within measObjectNR corresponding to the SpCell), and is set to zero if not configured for the SpCell.

Hys1 is the hysteresis parameter for this event (i.e. a3-Hysteresis as defined within reportConfigNR for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigNR for this event).

Ms is the Aerial UE altitude relative to the sea level.

Hys2 is the hysteresis parameter for this event (i.e. h1-Hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. h1-Threshold as defined within reportConfigNR for this event).

Mn, Mp are expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Ofn, Ocn, Hys1, Ofp, Ocp, Off are expressed in dB.

Ms, Hys2, Thresh are expressed in meters.

[TS 38.331, clause 5.5.5.1]

Figure 5.5.5.1-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation.

The UE shall, for each entry in the VarMeasReportList:

1>	if the eventH1 or eventH2 is configured in the corresponding reportConfig associated with the measId that triggered the measurement reporting:

2>	for all the entries in the VarMeasReportList for which the measurement reporting procedure was triggered and the corresponding reportConfig is configured with the same eventID and simulMultiTriggerSingleMeasReport set to true:

3>	except for the entry corresponding to the event for which the difference between the configured altitude threshold and the altitude of the UE is the smallest, remove all other measurement reporting entries from the VarMeasReportList, if any, and stop the associated periodical reporting timer(s), if running;

1>	else if the eventA3H1 or eventA3H2 or eventA4H1 or eventA4H2 or eventA5H1 or eventA5H2 is configured in the corresponding reportConfig associated with the measId that triggered the measurement reporting:

2>	for all the entries in the VarMeasReportList associated with the same measObjectNR for which the measurement reporting procedure was triggered and the corresponding reportConfig is configured with the same eventID and simulMultiTriggerSingleMeasReport set to true:

3>	except for the entry corresponding to the event for which the difference between the configured altitude threshold and the altitude of the UE is the smallest, remove all other measurement reporting entries from the VarMeasReportList, if any, and stop the associated periodical reporting timer(s), if running;

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

. . .

1>	if the reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventB1, or eventB2, or eventA3H1, or eventA3H2, or eventA4H1, or eventA4H2, or eventA5H1, or eventA5H2:

2>	if the UE is in NE-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListEUTRA-SCG to include an entry for each E-UTRA SCG serving frequency with the following:

4>	include carrierFreq of the E-UTRA serving frequency;

4>	set the measResultServingCell to include the available measurement quantities that the UE is configured to measure by the measurement configuration associated with the SCG;

4>	if reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas:

5>	set the measResultServFreqListEUTRA-SCG to include within measResultBestNeighCell the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	if reportConfig associated with the measId that triggered the measurement reporting is set to eventTriggered and eventID is set to eventA3, or eventA4, or eventA5, or eventA3H1, or eventA3H2, or eventA4H1, or eventA4H2, or eventA5H1, or eventA5H2:

2>	if the UE is in NR-DC and the measurement configuration that triggered this measurement report is associated with the MCG:

3>	set the measResultServFreqListNR-SCG to include for each NR SCG serving cell that is configured with servingCellMO, if any, the following:

4>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

5>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available according to the measurement configuration associated with the SCG:

6>	set the measResultServingCell within measResultServFreqListNR-SCG to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

8.1.3.5.3.3	Test description

8.1.3.5.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell and NR cell 2 is the intra-frequency neighbor cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

-	1 GNSS simulator

UE:

-	None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.3.5.3.3.2	Test procedure sequence

Table 8.1.3.5.3.3.2-1 illustrates the simulated aerial height above sea level at various time instants of the test execution along with the downlink power levels to be applied for NR Cell 1 and NR Cell 2 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1", "T2" and "T3" are to be applied subsequently at the point indicated in the Main behaviour description in Table 8.1.3.1.2.3.2-2.

Table 8.1.3.5.3.3.2-1: Time instances of cell power level and parameter changes for NR Cell 1 and NR Cell 2 in conducted test environment

| Time | Parameter | Unit | Value (Note 1) | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Simulated aerial height above sea level (Ms) | m | 700 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | Simulated aerial height above sea level (Ms) and Power levels are such that neither of the entry conditions A3H1-1 and A3H1-2 are satisfied: A3H1-1: Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off A3H1-2: Ms – Hys2 > Thresh |
| T1 |  |  | 1100 |  |  | -85 | -79 | Simulated aerial height above sea level (Ms) and Power levels are such that both the entry conditions A3H1-1 and A3H1-2 are satisfied: A3H1-1: Mn + Ofn + Ocn – Hys1 > Mp + Ofp + Ocp + Off A3H1-2: Ms – Hys2 > Thresh |
| T2 |  |  | 300 |  |  | -85 | -79 | Simulated aerial height above sea level (Ms) and Power levels are such that Leaving Condition conditions A3H1-4 is satisfied: A3H1-4: Ms – Hys2 < Thresh |
| T3 |  |  | 300 |  |  | -85 | -91 | Simulated aerial height above sea level (Ms) and Power levels are such that Leaving Condition conditions A3H1-3 and A3H1-4 are satisfied: A3H1-3: Mn + Ofn + Ocn + Hys1 < Mp + Ofp + Ocp + Off A3H1-4: Ms – Hys2 < Thresh |
| Note 1: An allowance of +/-90m has been used to generate the aerial height values. This allowance takes into account both the UE measurement accuracy and the test tolerances in the simulation. Note 2: Value of Hys correspond to values of h1-Hysteresis-r18 in Table 8.1.3.5.3.3.3-4. Value of Thresh corresponds to value of h1-Threshold-r18 in Table 8.1.3.5.3.3.3-4. |  |  |  |  |  |  |  |  |

Table 8.1.3.5.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger the UE to reset UTC time in order that the UE accepts the time generated by the GNSS simulator and also uses only GNSS for position calculation in the subsequent test procedure, as defined in TS 38.509 [25] clause 5.6.3. (Note1) | - | - | - | - |
| 2 | The SS configures the GNSS simulator for step 1 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate a location with the height above sea level given for time T0 in Table 8.1.3.5.3.3.2-1. | - | - | - | - |
| 3 | The SS transmits a RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for event A3H1. | <-- | RRCReconfiguration | - | - |
| 4 | The UE transmits a RRCReconfigrationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 5 | The SS configures GNSS simulator for step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level and readjusts the cell-specific reference signal to row "T1" in Table 8.1.3.5.3.3.2-1. | - | - | - | - |
| 6 | Check: Does the UE transmit a MeasurementReport message to report event A3H1 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| - | EXCEPTION: Step 7 below is repeated until 3 MeasurementReport messages are received from the UE. | - | - | - | - |
| 7 | Check: Does the UE transmit a MeasurementReport message to report event A3H1 with the measured aerial height value? | --> | MeasurementReport | 1 | P |
| 8 | The SS configures GNSS simulator for step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level and readjusts the cell-specific reference signal to row "T2" in Table 8.1.3.5.3.3.2-1. | - | - | - | - |
| 9 | Wait and ignore MeasurementReport messages for 50s (as detailed in TS 36.508 [7] Table 4.12.2-1) to allow simulated change of aerial height value to be measured by the UE. | - | - | - | - |
| 10 | Check: Does the UE transmit a MeasurementReport message within the next 10s? | - | - | 2 | F |
| 11 | The SS configures GNSS simulator for step 2 of Scenario #1 as defined in TS 36.508 [7] Table 4.12.2-1 to simulate the UE altitude moving to a height above sea level and readjusts the cell-specific reference signal to row "T3" in Table 8.1.3.5.3.3.2-1. | - | - | - | - |
| 12 | Wait for 50s (as detailed in TS 36.508 [7] Table 4.12.2-1) to allow simulated change of aerial height value to be measured by the UE. | - | - | - | - |
| 13 | Check: Does the UE transmit a MeasurementReport message within the next 10s? | - | - | 3 | F |
| Note 1: The UTC time reset may be performed by MMI or AT command (+CUTCR). |  |  |  |  |  |

8.1.3.5.3.3.3	Specific message contents

Table 8.1.3.5.3.3.3-1: RRCReconfiguration (step 3, Table 8.1.3.5.3.3.2-2)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS and UAV

Table 8.1.3.5.3.3.3-2: MeasConfig (step 3, Table 8.1.3.5.3.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.3.5.3.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3H1 | Table 8.1.3.5.3.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.3.3.3-3: MeasObjectNR (Table 8.1.3.5.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.3.5.3.3.3-4: ReportConfigNR-EventA3H1 (Table 8.1.3.5.3.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3H1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3H1-r18 SEQUENCE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| h1-Threshold-r18 | 900 (900m) |  |  |
| h1-Hysteresis-r18 | 5 (5m) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.5.3.3.3-5: MeasurementReport (step 6, Table 8.1.3.5.3.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry | Measurement report for NR Cell 1 |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry | Measurement report for NR Cell 2 |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| altitudeUE-r18 | (-420..10000) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.3.5.4	NR UAV / Measurement configuration control and reporting / Event A3H1 / Measurement of Neighbour NR cell / Inter-frequency measurements

8.1.3.5.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and inter-frequency measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes higher than the absolute threshold plus hysteresis }

then { UE sends MeasurementReport with correct measdID for event A3H1}

}

(2)

with { UE in RRC_CONNECTED state and inter-frequency measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes lower than the absolute threshold minus hysteresis }

then { UE stops sending MeasurementReport }

}

(3)

with { UE in RRC_CONNECTED state and inter-frequency measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes lower than serving cell and the Aerial UE altitude becomes lower than the absolute threshold minus hysteresis }

then { UE does not send MeasurementReport }

}

8.1.3.5.4.2	Conformance requirements

Same as 8.1.3.5.2.2 with the following difference:

[TS 38.331, clause 5.5.2.9]

The UE shall:

. . .

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration configured by gapUE is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the gapUE in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

8.1.3.5.4.3	Test description

8.1.3.5.4.3.1	Pre-test conditions

Same as 8.1.3.5.3.3.1 with the following differences

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

8.1.3.5.4.3.2	Test procedure sequence

Same as test case 8.1.3.5.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.3.5.4.3.3	Specific message contents

Same as test case 8.1.3.5.2 with the following difference:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

Table 8.1.3.5.4.3.3-1: MeasConfig (step 3, Table 8.1.3.5.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 | Table 8.1.3.5.4.3.3-2 |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 | Table 8.1.3.5.4.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3H1 | Table 8.1.3.5.3.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig |  |  |
| } |  |  |  |

Table 8.1.3.5.4.3.3-2: MeasObjectNR-f1 (Table 8.1.3.5.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.3.5.4.3.3-3: MeasObjectNR-f1 (Table 8.1.3.5.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 3 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

##### 8.1.3.5.5	NR UAV / Measurement configuration control and reporting / Event A3H1 / Measurement of Neighbour NR cell / Inter-band measurements

8.1.3.5.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and inter-band measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes higher than the absolute threshold plus hysteresis }

then { UE sends MeasurementReport with correct measdID for event A3H1}

}

(2)

with { UE in RRC_CONNECTED state and inter-band measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes better than serving cell and the Aerial UE altitude becomes lower than the absolute threshold minus hysteresis }

then { UE stops sending MeasurementReport }

}

(3)

with { UE in RRC_CONNECTED state and inter-band measurement configured for event A3H1 }

ensure that {

when { Neighbour cell becomes lower than serving cell and the Aerial UE altitude becomes lower than the absolute threshold minus hysteresis }

then { UE does not send MeasurementReport }

}

8.1.3.5.5.2	Conformance requirements

Same as 8.1.3.5.2.2 with the following difference:

[TS 38.331, clause 5.5.2.9]

The UE shall:

...

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration configured by gapUE is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the gapUE in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	apply the specified timing advance mgta to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

8.1.3.5.5.3	Test description

8.1.3.5.5.3.1	Pre-test conditions

Same as 8.1.3.5.3.3.1 with the following differences

-	Cells configuration: NR Cell 10 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

8.1.3.5.5.3.2	Test procedure sequence

Same as test case 8.1.3.5.2.3.2 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.3.5.5.3.3	Specific message contents

Same as test case 8.1.3.5.2.3.3 with the following difference:

-	Cells configuration: NR Cell 10 replaces NR Cell 2.

Table 8.1.3.5.5.3.3-1: MeasConfig (step 3, Table 8.1.3.5.5.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 | Table 8.1.3.5.5.3.3-2 |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 | Table 8.1.3.5.5.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3H1 | Table 8.1.3.5.3.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig |  |  |
| } |  |  |  |

Table 8.1.3.5.5.3.3-2: MeasObjectNR-f1 (Table 8.1.3.5.5.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.3.5.5.3.3-3: MeasObjectNR-f1 (Table 8.1.3.5.5.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 10 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

==============End of change==============