3GPP TSG WG RAN5 Meeting #109	R5-256286r1

# Dallas, USA, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-7 | CR | 0020 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new MCData test case 6.8.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 6.8.5 On-network / MCData Message Store / Notification Channel / Create / Open / Update / Delete have not been added to the specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case 6.8.5 On-network / MCData Message Store / Notification Channel / Create / Open / Update / Delete has been added according to the Work Plan. Added a reference to the OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 6.8.5 will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.8.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.579-4 CR 0007 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Added reference to the XML schema for the Notification Channel API. |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 37.579-1: "Mission Critical (MC) services; Part 1: Common test environment".

[3]	Void.

[4]	3GPP TS 37.579-4: " Mission Critical (MC) services; Part 4: Test Applicability and Implementation Conformance Statement (ICS).

[5]	3GPP TS 37.579-5: "Mission Critical (MC) services; Part 5: Abstract test suite (ATS)".

[6]	3GPP TS 36.523-1: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[7]	Void

[8]	Void

[9]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification".

[10]	Void

[11]	3GPP TS 24.481: "Mission Critical Services (MCS) group management; Protocol specification".

[12]	3GPP TS 24.482: "Mission Critical Services (MCS) identity management; Protocol specification".

[13]	3GPP TS 24.483: "Mission Critical Services (MCS) Management Object (MO)".

[14]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management; Protocol specification".

[15]	3GPP TS 33.179: " Security of Mission Critical Push To Talk (MCPTT) over LTE ".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[18]	Void

[19]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Special conformance testing functions for User Equipment (UE)".

[20]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRAN); Common Test Environments for User Equipment (UE) Conformance Testing".

[21]	OpenID Connect 1.0: "OpenID Connect Core 1.0 incorporating errata set 1", http://openid.net/specs/openid-connect-core-1_0.html.

[22]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[23]	Void

[24]	3GPP TS 37.579-2: "Mission Critical (MC) services; Part 2: Mission Critical Push To Talk (MCPTT) User Equipment (UE) Protocol conformance specification".

[25]	3GPP TS 37.579-6: "Mission Critical (MC) services; Part 6: Mission Critical Video (MCVideo) User Equipment (UE) Protocol conformance specification ".

[26]	Void

[27]	Void

[28]	Void

[29]	Void

[30]	3GPP TS 33.180: "Security of the mission critical service".

[31]	3GPP TS 24.282: "Mission Critical Data (MCData) signalling control; Protocol specification".

[32]	3GPP TS 24.582: "Mission Critical Data (MCData) media plane control; Protocol specification".

[33]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[34]	3GPP TS 22.282: "Mission Critical Data over LTE. Status: Under change control".

[35]	Void

[36]	IETF RFC 4826 (May 2007): "Extensible Markup Language (XML) Formats for Representing Resource Lists".

[37]	IETF RFC 4122 (July 2005): "A Universally Unique IDentifier (UUID) URN Namespace".

[38]	IETF RFC 2046: "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[39]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[40]	OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C: "RESTful Network API for Network Message Storage".

[41]	IETF RFC 4567: "Key Management Extensions for Session Description Protocol (SDP) and Real Time Streaming Protocol (RTSP)"

[42]	OMA-TS-REST_NetAPI_Common-V1_0-20120417-C: "Common definitions for RESTful Network APIs"

[43]	IETF RFC 2388: "Returning Values from Forms: multipart/form-data".

[44]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[45]	3GPP TS 38.523-1: " 5GS; User Equipment (UE) conformance specification;Part 1: Protocol"

[46]	OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C: “RESTful Network API for Notification Channel”.

[47]	OMA-SUP-XSD_rest_netapi_notificationchannel-V1_0: “XML data type definitions for the RESTful Network API for Notification Channel”, https://www.openmobilealliance.org/release/NotifChanREST/V1_0-20200319-C/OMA-SUP-XSD_rest_netapi_notificationchannel-V1_0-20200319-C.xsd

### <<< TEXT SKIPPED >>>

### 6.8.5	On-network / MCData Message Store / Notification Channel / Create / Open / Update / Delete

6.8.5.1	Test Purpose (TP)

(1)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to create a notification channel for PULL notification delivery method }

then { UE (MCData Client) requests to create a notification channel by sending an HTTP POST message }

}

(2)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to update a notification channel }

then { UE (MCData Client) requests to update a notification channelby sending an HTTP PUT message }

}

(3)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to open a notification channel }

then { UE (MCData Client) requests to open a notification channel by sending an HTTP POST message }

}

(4)

with { UE (MCData Client)registered and authorized for MCData service }

ensure that {

when { the MCDATA User requests to delete a notification channel }

then { UE (MCData Client) requests to delete a notification channel by sending an HTTP DELETE message }

}

6.8.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 21.2.19.1, 21.2.20.1, 21.2.21.1 and 21.2.22.1. Unless otherwise stated, these are Rel-17 requirements.

[TS 24.282 clause 21.2.19.1]

To create a notification channel, the Message notification client, acting as an HTTP client shall follow the procedure described in clause 6.1 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] as follows:

1)	shall generate an HTTP POST request as specified in clause 6.1.5 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] as follows:

a)	shall set the Host header field to a hostname identifying the MCData Notification server;

b)	shall include a valid MCData access token in the Authorization header; and

c)	shall send the HTTP POST request towards the MCData Notification server.

Upon receipt of an HTTP response, the Message notification client should follow the procedure as described in clause 6.1.2 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76].

[TS 24.282 clause 21.2.20.1]

To delete a notification channel, the Message notification client, acting as an HTTP client shall follow the procedure described in clause 6.2 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] as follows:

1)	shall generate an HTTP DELETE request as specified in clause 6.2.6 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] with following the clarifications:

a)	shall set the Host header field to a hostname identifying the MCData Notification server;

b)	shall include a valid MCData access token in the Authorization header; and

c)	shall send the HTTP DELETE request towards the MCData Notification server.

Upon receipt of a HTTP response, the Message notification client should follow the procedure as described in clause 6.2.2 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76].

NOTE:	When the notification channel is deleted, the Message store client normally removes the notification subscription in the MCData Message store function as described in clause 21.2.13A.

[TS 24.282 clause 21.2.21.1]

To update a notification channel's lifetime, the Message notification client, acting as an HTTP client shall follow the procedure described in clause 6.4 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] as follows:

1)	shall generate an HTTP PUT request as specified in clause 6.4.4 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] as follows:

a)	shall set the Host header field to a hostname identifying the MCData Notification server;

b)	shall include a valid MCData access token in the Authorization header; and

c)	shall send the HTTP PUT request towards the MCData Notification server.

Upon receipt of an HTTP response, the Message notification client should follow the procedure as described in clause 6.4.2 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76].

NOTE:	A successful HTTP response includes the new Channel's lifetime duration which can be used by the Message store client to update the lifetime of the notification subscription in the MCData message store function as described in clause 21.2.14A.

[TS 24.282 clause 21.2.22.1]

Based on the channel type created as part of the notification channel creation procedure (see clause 21.2.19. "Create notification channel"), the Message notification client would determine if and how it needs to open (interact with) the created channel for notification flow (i.e. using PULL or PUSH).

To open the notification channel for a PULL notification delivery method (i.e. created channel is of type LongPolling), the Message notification client, acting as an HTTP client shall follow the procedure described in clauses 6.3 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] as follows:

1)	shall generate an HTTP POST request as specified in clause 6.3.5 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] as follows:

a)	shall set the Host header field to a hostname identifying the Notification server extracted from the channelURL received from the Notification server during channel creation (see clause 21.2.19. "Create notification channel");

b)	shall include a valid MCData access token in the Authorization header; and

c)	shall send the HTTP POST request towards the MCData Notification server using the channelURL received from the MCData Notification server during channel creation procedure (see clause 21.2.19. "Create notification channel").

Upon receipt of a HTTP response, the Message notification client should follow the procedure as described in clause 6.3.2 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76]; and

1)	either use the notification content and the reported "restartToken" and "index" as specified in clause 5.1.5.1 of OMA-TS-REST_NetAPI_NMS-V1_0-20190528-C [66] to have the client's local message store updated accordingly; or

2)	use the notification as a trigger to subsequently search the MCData message store for the list of changes as specified in clause 21.2.17;

NOTE:	The notifications about changes in the MCData message store can be used by the message store client to synchronize its local message store with the MCData message store in two distinguished ways which are listed in bullets "1" and "2" above.

To open the notification channel for a PUSH notification delivery method over WebSocket (i.e. created channel is of type WebSocket), the Message notification client shall follow the procedure described in Appendix I of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] and use the channelURL received from the MCData Notification server during the channel creation procedure (see clauses 21.2.19 ) to create a WebSocket connection with the MCData Notification server. The process of creating a WebSokect connection between the Message notification client and the MCData Notification server through which the MCData Notification server can send notifications to the Message notification client is not RESTful.

If the created channel is of type NativeChannel, the Message notification client, is not required to invoke the "Open notification channel" procedure as defined in this clause. See clauses 5, 5.3.13, 5.3.14 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [76] for description on receiving notification over a NativeChannel.

6.8.5.3	Test description

6.8.5.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 36.879-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 36.879-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.8.5.3.2	Test procedure sequence

Table 6.8.5.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) create a notification channel for a PULL notification delivery methods. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Object Upload using HTTP' as described in TS 37.579-1 [2] Table 5.3C.14.2-1 to create a notification channel? | - | - | 1 | P |
| 3 | Make the UE (MCData client) update the lifetime of the notification channel created at step 2 to 10800 seconds. (NOTE 1) | - | - | - | - |
| 4 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Put Request using HTTP' as described in TS 37.579-1 [2] Table 5.3C.18.2-1 to update the lifetime of the notification channel? | - | - | 2 | P |
| 5 | Make the UE (MCData client) open the notification channel. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Post Request using HTTP' as described in TS 37.579-1 [2] Table 5.3C.17.2-1 to open the notification channel? | - | - | 3 | P |
| 7 | Make the UE (MCData client) delete the notification channel created at step 2. (NOTE 1) | - | - | - | - |
| 8 | Check: Does the UE (MCData client) correctly perform procedure 'Message Store Function Delete using HTTP' as described in TS 37.579-1 [2] Table 5.3C.15.2-1 to delete a notification channel? | - | - | 4 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.8.5.3.3	Specific message contents

Table 6.8.5.3.3-1: HTTP POST from the UE (Step 2, Table 6.8.5.3.2-1; step 2, TS 37.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.4.3-1, condition NOTIFICATION_CH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_NS_BaseURL & px_MCData_ID_User_A& "/channels" |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NC notificationChannel |  |  | RESTful Network API for NC [46], clause 6.1 and XML schema defined in [47]. |  |
| clientCorrelator | any value if present |  |  |  |
| applicationTag | any value if present |  |  |  |
| channelType | LongPolling |  |  |  |
| channelData |  |  |  |  |
| maxNotifications | 1 |  |  |  |
| channelLifetime | any value if present |  |  |  |

Table 6.8.5.3.3-2: HTTP 201 Created from the SS (step 2, Table 6.8.5.3.2-1; step 3, TS 37.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.4.7-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Location |  |  |  |  |
| uri | tsc_MCData_NS_BaseURL & px_MCData_ID_User_A& "/channels/ch123" |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NC notificationChannel |  |  | RESTful Network API for NC [46], clause 6.1 and XML schema defined in [47]. |  |
| clientCorrelator | Same value as provided by the client if present in the POST request otherwise not present |  |  |  |
| applicationTag | Same value as provided by the client if present in the POST request otherwise not present |  |  |  |
| channelType | LongPolling |  |  |  |
| channelData |  |  |  |  |
| channelURL | tsc_MCData_NS_BaseURL & px_MCData_ID_User_A& "/channels/ch123/notifications" |  |  |  |
| maxNotifications | 1 |  |  |  |
| channelLifetime | Same value as provided by the client if present in the POST request otherwise not present |  |  |  |
| resourceURL | tsc_MCData_NS_BaseURL & px_MCData_ID_User_A & "/channels/ch123" | same as in Location |  |  |

Table 6.8.5.3.3-3: HTTP PUT from the UE (Step 4, Table 6.8.5.3.2-1; step 2, TS 37.579-1 [2] Table 5.3C.18.2-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.4.4-1, condition NOTIFICATION_CH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_NS_BaseURL & px_MCData_ID_User_A& "/channels/ch123/channelLifetime" |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NC notificationChannelLifetime |  |  | RESTful Network API for NC [46], clause 6.4 and XML schema defined in [47]. |  |
| channelLifetime | “10800” |  |  |  |

Table 6.8.5.3.3-4: HTTP 200 (OK) from the SS (step 4, Table 6.8.5.3.2-1; step 3, TS 37.579-1 [2] Table 5.3C.18.2-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NC notificationChannelLifetime |  |  | RESTful Network API for NC [46], clause 6.4 and XML schema defined in [47]. |  |
| channelLifetime | "10800" |  |  |  |

Table 6.8.5.3.3-5: HTTP POST from the UE (Step 6, Table 6.8.5.3.2-1; step 2, TS 37.579-1 [2] Table 5.3C.14.2-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.4.3-1, condition NOTIFICATION_CH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_NS_BaseURL & px_MCData_ID_User_A& "/channels/ch123/notifications" | Same value as received in the channelURL IE in Table 6.8.5.3.3-2 |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NC longPollingRequestParameters |  |  | RESTful Network API for NC [46], clause 6.3 and XML schema defined in [47]. |  |

Table 6.8.5.3.3-6: HTTP 200 (OK) from the SS (step 6 Table 6.8.5.3.2-1; step 3, TS 37.579-1 [2] Table 5.3C.18.2-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.4.6-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/xml" |  |  |  |
| Message-body: NC notificationList |  |  | RESTful Network API for NC [46], clause 6.3 and XML schema defined in [47]. |  |

Table 6.8.5.3.3-7: HTTP DELETE from the UE (Step 8, Table 6.8.5.3.2-1; step 2, TS 37.579-1 [2] Table 5.3C.15.2-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.4.5-1, condition NOTIFICATION_CH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Request-Line |  |  |  |  |
| Request-URI |  |  |  |  |
| uri | tsc_MCData_NS_BaseURL & px_MCData_ID_User_A& "/channels/ch123" | The notification channel to be deleted |  |  |