3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210100

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0664 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS over 5GS test case 7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR TC 7.2 MTSI MO Voice Call / 504 Server Time-out / 5GS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GS IMS test case 6.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches Executed	5

4	Execution Log Files	5

4.1	Qualcomm SM8350 + SDX60	5

5	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 7.2 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	7.2

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8350

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_7_2_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Since the bearers for voice call have not been setup, attempt to release them would cause runtime error in f_NR5GC_ModifyPDUSession_RemoveIMSVoice during building Reconfiguration message with empty v_SS_Config. Therefore the postamble function needs to be called with input parameter IPCAN_MO_IMS_Signalling instead of IPCAN_MO_SpeechCall. |
| Summary of change | Changed the postamble function so it is called with input parameter IPCAN_MO_IMS_Signalling instead of IPCAN_MO_SpeechCall. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_2_NR5GC_IMS1() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (value) MessageHeader v_MessageHeader504; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) MessageHeader v_MessageHeaderACK; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2 - 7" siclog@ Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 // @siclog "Step 8" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(); // @siclog "Step 9" siclog@ 504 Server Time-out v_MessageHeader504 := f_IMS_CommonResponse_504_MessageHeaderTX(v_InviteRequestWithSdp.Invite.msgHeader); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine504, v_MessageHeader504, cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Restoration)))); // @siclog "Step 9A" siclog@ ACK !!!! PROSE ISSUE: not in the prose yet !!!! v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequestWithSdp.Invite, c_statusLine504); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_InviteRequestWithSdp.Invite.requestLine.requestUri, v_MessageHeaderACK))); f_IMS_GlobalRelease(oldestProtectedContextOnly); // release security context but keep unprotected context (TCP) f_IMS_CC_AbortCall(); // abort call establishment procedure at the IPCAN PTC // @siclog "Step 10 - 17" siclog@ f_IMS_Registration(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_SpeechCall); }

After Change:

function f_TC_7_2_NR5GC_IMS1() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (value) MessageHeader v_MessageHeader504; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) MessageHeader v_MessageHeaderACK; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2 - 7" siclog@ Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 // @siclog "Step 8" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(); // @siclog "Step 9" siclog@ 504 Server Time-out v_MessageHeader504 := f_IMS_CommonResponse_504_MessageHeaderTX(v_InviteRequestWithSdp.Invite.msgHeader); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine504, v_MessageHeader504, cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Restoration)))); // @siclog "Step 9A" siclog@ ACK !!!! PROSE ISSUE: not in the prose yet !!!! v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequestWithSdp.Invite, c_statusLine504); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_InviteRequestWithSdp.Invite.requestLine.requestUri, v_MessageHeaderACK))); f_IMS_GlobalRelease(oldestProtectedContextOnly); // release security context but keep unprotected context (TCP) f_IMS_CC_AbortCall(); // abort call establishment procedure at the IPCAN PTC // @siclog "Step 10 - 17" siclog@ f_IMS_Registration(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling); }

# Branches Executed

The test case was executed on NR band n41.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 IMS UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_2_LOGS.xml

- PICS/PIXIT parameter file:

TC_7_2_PICS_PIXIT.txt

# References

| [1] | R5s210101 |
| --- | --- |