3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210112

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0665 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for 5GS IMS SMS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The TP-MMS field should be set to 0 for all segments according to the agreed prose (34.229-5 tables 9.4.3.3.-1, 9.4.3.3-5 and 9.4.3.3-6) - Following implementation of agreed prose CR R5-205345, TTCN implementation of SMS DELIVERY REPORT is incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change setting of TP-MMS to 0 - Correct the template so that expected value of tp_ParameterIndicator corresponds to ‘00000000’B and tp_ProtocolIdentifier is allowed omitted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.NR5GC, 9.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | Template cs_SMS_DELIVER_ConcSMS |
| --- | --- |
| Reason for change |  |
| Summary of change | Change setting of TP-MMS to 0 Note : This change was previously proposed and accepted as part of R5s201329 but not implemented. |
| TTCN module | IMS_SMS_Messages.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) SMS_DELIVER_Type cs_SMS_DELIVER_ConcSMS (octetstring p_Digits, O1_Type p_MsgRef, O1_Type p_SN_SMS) := { /* @status APPROVED (IMS) */ tP_ReplyPath := '0'B, tP_UserDataHeaderIndicator := '1'B, // concatenated SMS tP_StatusReportIndication := '0'B, spare2 := '00'B, tP_MoreMessagesToSend := '1'B, // @sic R5s201329 sic@ tP_MessageTypeIndicator := tsc_MT_SMS_DELIVER, tP_OriginatingAddress := cs_TP_Address (p_Digits), tP_ProtocolIdentifier := cs_TP_ProtocolIdentifier, tP_DataCodingScheme_Type := cs_TP_DataCodingScheme, tP_ServiceCentreTimeStamp := f_BCD_TimestampWithTimezone(), tP_UserDataLength := int2oct(160,1), // as the user data are septets the overall length is provided in septets tP_UserData := f_TP_UserDataWithHeader_Encode(cs_TP_UserDataHeader_ConcSMS (p_MsgRef, p_SN_SMS, f_CharPacking_IA5toGsm7Bit_WithFiller(tsc_FoxShort, SMS_Packing, 1))) // @sic R5s201446 sic@ };

After Change:

template (value) SMS_DELIVER_Type cs_SMS_DELIVER_ConcSMS (octetstring p_Digits, O1_Type p_MsgRef, O1_Type p_SN_SMS) := { /* @status APPROVED (IMS) */ tP_ReplyPath := '0'B, tP_UserDataHeaderIndicator := '1'B, // concatenated SMS tP_StatusReportIndication := '0'B, spare2 := '00'B, tP_MoreMessagesToSend := '0'B, // @sic R5s201329 sic@ tP_MessageTypeIndicator := tsc_MT_SMS_DELIVER, tP_OriginatingAddress := cs_TP_Address (p_Digits), tP_ProtocolIdentifier := cs_TP_ProtocolIdentifier, tP_DataCodingScheme_Type := cs_TP_DataCodingScheme, tP_ServiceCentreTimeStamp := f_BCD_TimestampWithTimezone(), tP_UserDataLength := int2oct(160,1), // as the user data are septets the overall length is provided in septets tP_UserData := f_TP_UserDataWithHeader_Encode(cs_TP_UserDataHeader_ConcSMS (p_MsgRef, p_SN_SMS, f_CharPacking_IA5toGsm7Bit_WithFiller(tsc_FoxShort, SMS_Packing, 1))) // @sic R5s201446 sic@ };

## Change 2

| Function name | Template cs_SMS_DELIVER_ConcSMS |
| --- | --- |
| Reason for change | Following implementation of agreed prose CR R5-205345, TTCN implementation of SMS DELIVERY REPORT is incorrect: The agreed prose has: Message-body; RP-ACK message with RP-User Data including SMS-DELIVERY-REPORT: TP-MTI=‘00’B (SMS-DELIVERY-REPORT) TP-PI=‘00000000’B TP-SCTS=set by the SS (encoded as specified in TS 23.040 clause 9.2.3.11); TS 24.011 [92]. Here TP-PI corresponds to “tP_ParamterIndicator”, not tP_ProtocolIdentifier. |
| Summary of change | Correct the template so that expected value of tp_ParamterIndicator corresponds to ‘00000000’B and tp_ProtocolIdentifier is allowed omitted. |
| TTCN module | SMS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (present) SMS_DELIVER_REPORT_Type cr_SMS_DELIVER_REPORT := { /* @status APPROVED (IMS) */ spare1 := '0'B, tP_UserDataHeaderIndicator := '0'B, spare4 := '0000'B, tP_MessageTypeIndicator := tsc_MT_SMS_DELIVER_REPORT, tP_FailureCause := omit, tP_ParameterIndicator := ?, tP_ProtocolIdentifier := cr_TP_ProtocolIdentifier, // @sic R5-205345 sic@ tP_DataCodingScheme_Type := *, tP_UserDataLength := *, tP_UserData := * };

After Change:

template (present) SMS_DELIVER_REPORT_Type cr_SMS_DELIVER_REPORT := { /* @status APPROVED (IMS) */ spare1 := '0'B, tP_UserDataHeaderIndicator := '0'B, spare4 := '0000'B, tP_MessageTypeIndicator := tsc_MT_SMS_DELIVER_REPORT, tP_FailureCause := omit, tP_ParameterIndicator := cs_TP_ParameterIndicator, tP_ProtocolIdentifier := *, // @sic R5-205345 sic@ tP_DataCodingScheme_Type := *, tP_UserDataLength := *, tP_UserData := * };