3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210140

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4584 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_SMS_CP_PDU_DecodeAndMatch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test cases using function f_SMS_CP_PDU_DecodeAndMatch, in case there is a problem during decoding of received CP PDU, will end abruptly with a runtime error as v_Received_CP_PDU is returned and will not contain any value. This runtime error issue can be avoided terminating the test case in case of decoding failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added self.kill to end the test case in case that there is a problem with decoding or decoded PDU does not match expected PDU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runttime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1-11.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Testcases 9.1.8.1.NR5GC, 9.1.8.2.NR5GC and NBIOT 22.1.1M3 are also affected by this |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function f_SMS_CP_PDU_DecodeAndMatch	3

# Corrections required

## Correction to function f_SMS_CP_PDU_DecodeAndMatch

| Function name | Correction to function f_SMS_CP_PDU_DecodeAndMatch |
| --- | --- |
| Reason for change | Test cases using function Correction to function f_SMS_CP_PDU_DecodeAndMatch, in case there is a problem during decoding of received CP PDU, will end abruptly with a runtime error as v_Received_CP_PDU is returned and will not contain any value. |
| Summary of change | Added self.kill to end the test case in case that there is a problem with decoding or decoded PDU does not match expected PDU. |
| TTCN module | Common/NAS/SMS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_SMS_CP_PDU_DecodeAndMatch(bitstring p_EncodedSMS_Message, template CP_PDU_Type p_Expected_CP_PDU, charstring p_Testcase, charstring p_Step) return CP_PDU_Type { var CP_PDU_Type v_Received_CP_PDU; var integer v_DecodeResult := decvalue(p_EncodedSMS_Message, v_Received_CP_PDU); if (v_DecodeResult != 0) { f_SetVerdict(fail,__FILE__, __LINE__, p_Testcase & ": SMS_CP_PDU cannot be decoded in " & p_Step); } if (not match(v_Received_CP_PDU, p_Expected_CP_PDU)) { //* @verdict fail Received decoded template does not match with the expected template f_SetVerdict(fail,__FILE__, __LINE__, p_Testcase & ": SMS_CP_PDU cannot be matched in " & p_Step); } return v_Received_CP_PDU; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_SMS_CP_PDU_DecodeAndMatch(bitstring p_EncodedSMS_Message, template CP_PDU_Type p_Expected_CP_PDU, charstring p_Testcase, charstring p_Step) return CP_PDU_Type { var CP_PDU_Type v_Received_CP_PDU; var integer v_DecodeResult := decvalue(p_EncodedSMS_Message, v_Received_CP_PDU); if (v_DecodeResult != 0) { f_SetVerdict(fail,__FILE__, __LINE__, p_Testcase & ": SMS_CP_PDU cannot be decoded in " & p_Step); self.kill; // => test case will be terminated immediately } if (not match(v_Received_CP_PDU, p_Expected_CP_PDU)) { //* @verdict fail Received decoded template does not match with the expected template f_SetVerdict(fail,__FILE__, __LINE__, p_Testcase & ": SMS_CP_PDU cannot be matched in " & p_Step); self.kill; // => test case will be terminated immediately } return v_Received_CP_PDU; } <<SKIPPED CODE>>