3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210170

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1576 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR PDCP test case 7.1.3.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 10 and step 25, for NR5GC operation, the SS is configured to send only two UL grant (112bits) on the target cell after the HO, for the RRCReconfigurationComplete and PDCP STATUS REPORT. A high failure rate has been observed during testing, where the 1st UL grant, which also serves as contention resolution, can fall outside of the ra-ContentionResolutionTimer window after msg3, and hence the UE could not get enough UL grant for both RRCReconfigurationComplete and PDCP STATUS REPORT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Assign periodic UL grant with 112 bits at step 10 and 25 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

# Change 1

| Function name | f_TC_7_1_3_4_1_NR_Step10to23 |
| --- | --- |
| Reason for change | At step 10, for NR5GC operation, the SS is configured to send only two UL grant (112bits) on the target cell after the HO, for the RRCReconfigurationComplete and PDCP STATUS REPORT. A high failure rate has been observed during testing, where the 1st UL grant, which also serves as contention resolution, can fall outside of the ra-ContentionResolutionTimer window after msg3, and hence the UE could not get enough UL grant for both RRCReconfigurationComplete and PDCP STATUS REPORT |
| Summary of change | Assign periodic UL grant with 112 bits at step 10. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR2 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR1 := -88; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR2 := -91; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell1, v_CellPowerAtT2Cell1_FR1, v_CellPowerAtT2Cell1_FR2), cs_NR_CellPower(nr_Cell2, v_CellPowerAtT2Cell2_FR1, v_CellPowerAtT2Cell2_FR2) }; //@sic R5-195929 sic@ var integer v_Grants:= 1; //@sic R5s201484 sic@ timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU if ( p_WaitforRRCConCmp) { v_Grants:= 2; } //@siclog "Step 10" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3oct BSR or padding = 13 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ //@sic R5s201484 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants); if (p_WaitforRRCConCmp) { interleave {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})){} } } else { //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); ... ... }

After Change:

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR2 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR1 := -88; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR2 := -91; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell1, v_CellPowerAtT2Cell1_FR1, v_CellPowerAtT2Cell1_FR2), cs_NR_CellPower(nr_Cell2, v_CellPowerAtT2Cell2_FR1, v_CellPowerAtT2Cell2_FR2) }; //@sic R5-195929 sic@ //REMOVED var integer v_Grants:= 1 timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU /* REMOVED if ( p_WaitforRRCConCmp) { v_Grants:= 2; } */ //@siclog "Step 10" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3oct BSR or padding = 13 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ //@sic R5s201484 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20); f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); if (p_WaitforRRCConCmp) { interleave {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})){} } } else { //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); ... ... }

## Change 2

| Function name | f_TC_7_1_3_4_1_NR_Step25to30 |
| --- | --- |
| Reason for change | At step 25, for NR5GC operation, the SS is configured to send only two UL grant (112bits) on the target cell after the HO, for the RRCReconfigurationComplete and PDCP STATUS REPORT. A high failure rate has been observed during testing, where the 1st UL grant, which also serves as contention resolution, can fall outside of the ra-ContentionResolutionTimer window after msg3, and hence the UE could not get enough UL grant for both RRCReconfigurationComplete and PDCP STATUS REPORT |
| Summary of change | Assign periodic UL grant with 112 bits at step 25. |
| TTCN module | NR_TC_Common\7_1_3\PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var integer v_Grants:= 1; //@sic R5s201484 sic@ timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU if ( p_WaitforRRCConCmp) { v_Grants:= 2; } //@siclog "Step 24-25" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report. // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, v_Grants); if (p_WaitforRRCConCmp) { interleave {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)) {} //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})){} } } else { //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})); } ... ... }

After Change:

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); //REMOVED var integer v_Grants:= 1; timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU /* REMOVED if ( p_WaitforRRCConCmp) { v_Grants:= 2; } */ //@siclog "Step 24-25" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report. // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); if (p_WaitforRRCConCmp) { interleave {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})){} } } else { //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})); } ... ... }