3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210181

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1376 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR5GC IRAT test case 8.1.4.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - NH associated to NCC=1 is not used to derivate KeNB, so {NH, NCC=2} should be used - Test loop is not closed for steps 8-9, because the current preamble function does not handle TEST_LOOPModeB_ON - AS security key configuration in f_NR5GC_MappedContextFromS1_InitNAS() will also be done later in f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14() |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Set NCC to 2 before handover - Close test loop for TEST_LOOPModeB_ON - Remove AS security key configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_4_2_2_1_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | According to TS 33.401 7.2.8.1.1 NH associated to NCC=1 is not used to derivate KeNB, so {NH, NCC=2} should be used |
| Summary of change | Set NCC to 2 before handover Close test loop for TEST_LOOPModeB_ON |
| TTCN module | IRAT_Handover_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_4_2_2_1_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var octetstring v_NR_HOCmd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); // @sic R5s201078 sic@ as per 33.401 7.2.8.4.3: // the source MME shall increase its locally kept NCC value by one and compute a fresh NH using A.4 v_SecurityParams.NH := f_EUTRA_NB_Authentication_S12(v_SecurityParams.KDF, v_SecurityParams.KASME, v_SecurityParams.NH); v_SecurityParams.NCC := (v_SecurityParams.NCC + 1) mod 4; f_EUTRA_Security_Set (v_SecurityParams); //@siclog "Step 1" siclog@ // Send latest parameters to NR f_EUTRA38_Send_AuthParametersToNR(); f_EUTRA38_SendPduSessionInfoListToNR(); // Wait for NR to send back encoded RRC Reconfiguration message v_NR_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(NR, "HO Cmd"); SRB.send( cas_SRB1_RrcPdu_REQ( eutra_Cell1, cs_TimingInfo_Now, cs_MobilityFromEutraCommand_Handover( tsc_RRC_TI_Def, false, nr, v_NR_HOCmd, omit))); // UE now moved to NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); }

After Change

function fl_TC_8_1_4_2_2_1_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var octetstring v_NR_HOCmd; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); // @sic R5s201078 sic@ as per 33.401 7.2.8.4.3: // the source MME shall increase its locally kept NCC value by one and compute a fresh NH using A.4 v_SecurityParams.NH := f_EUTRA_NB_Authentication_S12(v_SecurityParams.KDF, v_SecurityParams.KASME, v_SecurityParams.NH); v_SecurityParams.NCC := 2; f_EUTRA_Security_Set (v_SecurityParams); //@siclog "Step 1" siclog@ // Send latest parameters to NR f_EUTRA38_Send_AuthParametersToNR(); f_EUTRA38_SendPduSessionInfoListToNR(); // Wait for NR to send back encoded RRC Reconfiguration message v_NR_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(NR, "HO Cmd"); SRB.send( cas_SRB1_RrcPdu_REQ( eutra_Cell1, cs_TimingInfo_Now, cs_MobilityFromEutraCommand_Handover( tsc_RRC_TI_Def, false, nr, v_NR_HOCmd, omit))); // UE now moved to NR f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); }

## Change 2

| Function name | f_EUTRA38_NR5GC_Preamble |
| --- | --- |
| Reason for change | Test loop is not closed for steps 8-9, because the current preamble function does not handle TEST_LOOPModeB_ON |
| Summary of change | Close test loop for TEST_LOOPModeB_ON |
| TTCN module | EUTRA38_NR5GCProcedures |
| MCC160 Comment |  |

Before Change

function f_EUTRA38_NR5GC_Preamble(EUTRA_CellId_Type p_CellId, NR_TESTMODE_STATE_Type p_TestModeState, EUTRA_RRC_STATE_Type p_RRC_State, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (omit) EPS_NetworkFeatureSupport p_NtwkFeatSupport := cs_EPS_NwkFtSup_NR, template (omit) PLMN_List p_EquivalentPlmnList := omit) runs on EUTRA_Multi5GS_PTC { var NR_TESTMODE_STATE_Type v_IdleUpdatedState := p_TestModeState; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel := tsc_ServingCellRS_EPRE; select (p_TestModeState) { // @sic R5-204399 sic@ case (PING_Or_TESTModeB_ON, PING_Or_TEST_LOOPModeB_ON){ if (not pc_IP_Ping) { // don't need to activate test mode if using Ping v_IdleUpdatedState := TESTMode_OFF; } } } // Store the value of NetworkFeatureSupport to be used throughout the test f_EUTRA_MobileInfo_SetNetworkFeatureSupport (p_NtwkFeatSupport); f_EUTRA38_MultiPDN_InitialRegistration(p_CellId, // @sic R5-204399 sic@ v_IdleUpdatedState, p_ReleaseConnection, p_EquivalentPlmnList, v_AbsoluteCellPowerLevel); if (p_RRC_State == RRC_CONNECTED) { f_EUTRA38_MultiPDN_RbEst_Def(p_CellId); if (p_TestModeState == PING_Or_TEST_LOOPModeB_ON and not pc_IP_Ping) { // @sic R%s200887 sic@ f_EUTRA_CloseUE_TestLoopModeB (p_CellId, '00'O); } } }

After Change

function f_EUTRA38_NR5GC_Preamble(EUTRA_CellId_Type p_CellId, NR_TESTMODE_STATE_Type p_TestModeState, EUTRA_RRC_STATE_Type p_RRC_State, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (omit) EPS_NetworkFeatureSupport p_NtwkFeatSupport := cs_EPS_NwkFtSup_NR, template (omit) PLMN_List p_EquivalentPlmnList := omit) runs on EUTRA_Multi5GS_PTC { var NR_TESTMODE_STATE_Type v_IdleUpdatedState := p_TestModeState; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel := tsc_ServingCellRS_EPRE; select (p_TestModeState) { // @sic R5-204399 sic@ case (PING_Or_TESTModeB_ON, PING_Or_TEST_LOOPModeB_ON){ if (not pc_IP_Ping) { // don't need to activate test mode if using Ping v_IdleUpdatedState := TESTMode_OFF; } } } // Store the value of NetworkFeatureSupport to be used throughout the test f_EUTRA_MobileInfo_SetNetworkFeatureSupport (p_NtwkFeatSupport); f_EUTRA38_MultiPDN_InitialRegistration(p_CellId, // @sic R5-204399 sic@ v_IdleUpdatedState, p_ReleaseConnection, p_EquivalentPlmnList, v_AbsoluteCellPowerLevel); if (p_RRC_State == RRC_CONNECTED) { f_EUTRA38_MultiPDN_RbEst_Def(p_CellId); if ((p_TestModeState == PING_Or_TEST_LOOPModeB_ON and not pc_IP_Ping) or (p_TestModeState == TEST_LOOPModeB_ON)) { // @sic R%s200887 sic@ f_EUTRA_CloseUE_TestLoopModeB (p_CellId, '00'O); } } }

## Change 3

| Function name | f_NR5GC_MappedContextFromS1_InitNAS |
| --- | --- |
| Reason for change | AS security key configuration will also be done later in f_NR5GC_508RRC_InterRAT_HO_ToNR_Common_Steps1_14() |
| Summary of change | Remove AS security key configuration Note : This change was previously accepted in R5s201078 but not applied |
| TTCN module | NR5GC_IRAT_CommonFunctions |
| MCC160 Comment |  |

Before Change

function f_NR5GC_MappedContextFromS1_InitNAS(NR_CellId_Type p_CellId, EUTRA_NR_CoOrd_Security_Type p_EUTRA_Auth_Params, boolean p_Handover ) runs on NR5GC_PTC { var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var NR_SecurityParams_Type v_RRC_SecurityParams := f_NR_Security_Get(); if (p_Handover) { v_SecurityParams.KAMF := f_NG_Authentication_A15_HO(p_EUTRA_Auth_Params.NH, // @sic R5s201078 sic@ p_EUTRA_Auth_Params.Key, v_SecurityParams.KDF); } else { v_SecurityParams.KAMF := f_NG_Authentication_A15_IdleMode(p_EUTRA_Auth_Params.NASCountUL, p_EUTRA_Auth_Params.Key, v_SecurityParams.KDF); } // Derives KNASenc key v_SecurityParams.NAS_Ciphering.K_NAS := f_NG_Authentication_A8 (tsc_NAS_Enc_Alg,v_SecurityParams.NAS_Ciphering.Algorithm, v_SecurityParams.KAMF, v_SecurityParams.KDF); // Derives KNASint key v_SecurityParams.NAS_Integrity.K_NAS := f_NG_Authentication_A8 (tsc_NAS_Int_Alg,v_SecurityParams.NAS_Integrity.Algorithm, v_SecurityParams.KAMF, v_SecurityParams.KDF); // Now NAS parameters updated, store updated common values in RRC global variable @sic R5s201078 sic@ v_RRC_SecurityParams.KAMF := v_SecurityParams.KAMF; v_RRC_SecurityParams.KSIamf := v_SecurityParams.KSIamf; // Derive KgNB @sic R5s201078 sic@ v_RRC_SecurityParams.KGNB := f_NR_Authentication_A9(v_RRC_SecurityParams.KDF, v_RRC_SecurityParams.KAMF, 'FFFFFFFF'O); // set NH and NCC as per 33.501 cl.8.4.2 point 3 @sic R5s201078 sic@ v_RRC_SecurityParams.NH:= v_RRC_SecurityParams.KGNB; v_RRC_SecurityParams.NCC := 0; // Now update the SS f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering); f_SS_NG_NAS_CountSet(NASCTRL, '00000000'O, omit); v_RRC_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_CellId, v_RRC_SecurityParams.AS_Integrity, v_RRC_SecurityParams.AS_Ciphering); f_NR_Security_Set(v_RRC_SecurityParams); f_NR5GC_Security_Set (v_SecurityParams); }

After Change

function f_NR5GC_MappedContextFromS1_InitNAS(NR_CellId_Type p_CellId, EUTRA_NR_CoOrd_Security_Type p_EUTRA_Auth_Params, boolean p_Handover ) runs on NR5GC_PTC { var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var NR_SecurityParams_Type v_RRC_SecurityParams := f_NR_Security_Get(); if (p_Handover) { v_SecurityParams.KAMF := f_NG_Authentication_A15_HO(p_EUTRA_Auth_Params.NH, // @sic R5s201078 sic@ p_EUTRA_Auth_Params.Key, v_SecurityParams.KDF); } else { v_SecurityParams.KAMF := f_NG_Authentication_A15_IdleMode(p_EUTRA_Auth_Params.NASCountUL, p_EUTRA_Auth_Params.Key, v_SecurityParams.KDF); } // Derives KNASenc key v_SecurityParams.NAS_Ciphering.K_NAS := f_NG_Authentication_A8 (tsc_NAS_Enc_Alg,v_SecurityParams.NAS_Ciphering.Algorithm, v_SecurityParams.KAMF, v_SecurityParams.KDF); // Derives KNASint key v_SecurityParams.NAS_Integrity.K_NAS := f_NG_Authentication_A8 (tsc_NAS_Int_Alg,v_SecurityParams.NAS_Integrity.Algorithm, v_SecurityParams.KAMF, v_SecurityParams.KDF); // Now NAS parameters updated, store updated common values in RRC global variable @sic R5s201078 sic@ v_RRC_SecurityParams.KAMF := v_SecurityParams.KAMF; v_RRC_SecurityParams.KSIamf := v_SecurityParams.KSIamf; // Derive KgNB @sic R5s201078 sic@ v_RRC_SecurityParams.KGNB := f_NR_Authentication_A9(v_RRC_SecurityParams.KDF, v_RRC_SecurityParams.KAMF, 'FFFFFFFF'O); // set NH and NCC as per 33.501 cl.8.4.2 point 3 @sic R5s201078 sic@ v_RRC_SecurityParams.NH:= v_RRC_SecurityParams.KGNB; v_RRC_SecurityParams.NCC := 0; // Now update the SS f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering); f_SS_NG_NAS_CountSet(NASCTRL, '00000000'O, omit); /* REMOVED v_RRC_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_CellId, false); f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_CellId, v_RRC_SecurityParams.AS_Integrity, v_RRC_SecurityParams.AS_Ciphering); */ f_NR_Security_Set(v_RRC_SecurityParams); f_NR5GC_Security_Set (v_SecurityParams); }