3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210188

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1396 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR RLC test case 7.1.2.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. A duplicated AMD PDU with SN= W-1 is transmitted at step 1A 2. according to Table 7.1.2.3.5.3.2-1, Poll bit should be checked at AMD with SN=W-1, instead of W |  |  |  |  |  |  |  |  |
| Summary of change: |  | - save the AMD PDU with SN = W+1 and transmit it at step 1A2. - correct the condition for poll bit checking to W-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.5.ENDC, 7.1.2.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_7_1_2_3_5and5a |
| --- | --- |
| Reason for change | 1. A duplicated AMD PDU with SN= W-1 is transmitted at step 1A 2. according to Table 7.1.2.3.5.3.2-1, Poll bit should be checked at AMD with SN=W-1, instead of W |
| Summary of change | - save the AMD PDU with SN = W+1 and transmit it at step 1A - correct the condition for poll bit checking to W-1 |
| TTCN module | RLC_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function fl_TC_7_1_2_3_5and5a(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { // @sic R5-203535 sic@ …………………….. // generate 1 more RLC SDU for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); v_NR_RLC_AMD_PDU_TX := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[0]); v_DataPerSlotList[0] := cs_NR_DRB_DataPerSlotList_DL_SingleAmdPdu(v_SlotsNumber * 20 * i, -, v_NR_RLC_AMD_PDU_TX); // EXCEPTION: The SS is configured for step 1 500 ms in advance. The transmissions are performed every second radio frame. // Step 2 is started 100 ms after the first DL AMD PDU has been transmitted in step 1. v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 500); v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // @siclog "Step 2" siclog@ // In the following steps the SS transmits 1 UL grant in every second radio frame to enable the UE to return each received AMD PDU in one looped back AMD PDU. // Step 2 executed before step 1 to ensure the UL grant is configured in time f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); // EXCEPTION: Step 1 shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // @siclog "Step 1" siclog@ // The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. // Each RLC Data PDU contains one RLC SDU. The SS transmits an AMD PDU containing a SDU to the UE. for (j := 0; j < p_OuterLoop; j:= j + 1) { v_DataPerSlotList := v_DataPerSlotArray [j]; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotArray [j])); } // @siclog "Step 1A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotList)); // EXCEPTION: Step 2A shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // EXCEPTION: In Step 2A, SS shall receive a RLC PDU and step 2A is repeated from SN=j*iteration_size to SN=(((j+1)*iteration_ size)-1). // @siclog "Step 2A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. for (j := 0; j < p_OuterLoop; j:= j + 1) { v_DataPerSlotListRx := v_DataPerSlotArray [j]; for (i := 0; i < p_InnerLoop; i:= i + 1) { if (p_RLC_Rec.AM_SN_Size == 18) { v_NR_RLC_AMD_Data := v_DataPerSlotListRx[i].PduSduList.RlcPdu[i].AMD.SN18Bit.Data; } else { v_NR_RLC_AMD_Data := v_DataPerSlotListRx[i].PduSduList.RlcPdu[i].AMD.SN12Bit.Data; } if ((j == p_OuterLoop) and (i == p_InnerLoop)) {// SN = W-1 // @siclog "Step 3" siclog@ // Check: Does the UE transmit the Wth AMD PDU with the Poll bit set and with the contents of the SDU?. f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_AMD_Data, tsc_NR_P_Poll); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); } else {// SN < W-1 f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_AMD_Data, tsc_NR_P_NoPoll); } if ((((j*p_OuterLoop) + (i+1)) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2A" & " SN = " & int2str(((j*p_OuterLoop) + i)))}; } } ……… }

After Change

function fl_TC_7_1_2_3_5and5a(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC { // @sic R5-203535 sic@ ……………….. // generate 1 more RLC SDU for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); v_NR_RLC_AMD_PDU_TX := f_NR_RLC_Get_AMD_FullSDU_TX(p_RLC_Rec, v_Poll, v_NR_RLC_DataList[0]); v_DataPerSlotList[0] := cs_NR_DRB_DataPerSlotList_DL_SingleAmdPdu(v_SlotsNumber * 20 * i, -, v_NR_RLC_AMD_PDU_TX); v_DataPerSlotArray[p_OuterLoop] := v_DataPerSlotList; // EXCEPTION: The SS is configured for step 1 500 ms in advance. The transmissions are performed every second radio frame. // Step 2 is started 100 ms after the first DL AMD PDU has been transmitted in step 1. v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 500); v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // @siclog "Step 2" siclog@ // In the following steps the SS transmits 1 UL grant in every second radio frame to enable the UE to return each received AMD PDU in one looped back AMD PDU. // Step 2 executed before step 1 to ensure the UL grant is configured in time f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); // EXCEPTION: Step 1 shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // @siclog "Step 1" siclog@ // The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. // Each RLC Data PDU contains one RLC SDU. The SS transmits an AMD PDU containing a SDU to the UE. for (j := 0; j < p_OuterLoop; j:= j + 1) { v_DataPerSlotList := v_DataPerSlotArray [j]; v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotArray [j])); } // @siclog "Step 1A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 20*p_InnerLoop); v_DataPerSlotList := v_DataPerSlotArray [p_OuterLoop]; DRB.send (cas_NR_DRB_COMMON_REQ_DataPerSlot(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_DataPerSlotList)); // EXCEPTION: Step 2A shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). // EXCEPTION: In Step 2A, SS shall receive a RLC PDU and step 2A is repeated from SN=j*iteration_size to SN=(((j+1)*iteration_ size)-1). // @siclog "Step 2A" siclog@ // The SS transmits one RLC PDU in the slot following the transmissions of step 1. for (j := 0; j < p_OuterLoop; j:= j + 1) { v_DataPerSlotListRx := v_DataPerSlotArray [j]; for (i := 0; i < p_InnerLoop; i:= i + 1) { if (p_RLC_Rec.AM_SN_Size == 18) { v_NR_RLC_AMD_Data := v_DataPerSlotListRx[i].PduSduList.RlcPdu[i].AMD.SN18Bit.Data; } else { v_NR_RLC_AMD_Data := v_DataPerSlotListRx[i].PduSduList.RlcPdu[i].AMD.SN12Bit.Data; } if ((j == p_OuterLoop-1) and (i == p_InnerLoop-1)) {// SN = W-1 //KS R5sxx change_71235 // @siclog "Step 3" siclog@ // Check: Does the UE transmit the Wth AMD PDU with the Poll bit set and with the contents of the SDU?. f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_AMD_Data, tsc_NR_P_Poll); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); } else {// SN < W-1 f_NR_RxAMD_SDU (p_RLC_Rec, p_NR_DRB_Id, v_NR_RLC_AMD_Data, tsc_NR_P_NoPoll); } if ((((j*p_OuterLoop) + (i+1)) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2A" & " SN = " & int2str(((j*p_OuterLoop) + i)))}; } } …… }