3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210190

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1398 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GMM test case 9.1.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 2-3, “SS transfers rrcReconfiguration to release user-plane resources of UE for the PDU session”. Corresponding SS side resources should also be released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DRB release at SS side at steps 2-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_9_1_7_2_Body |
| --- | --- |
| Reason for change | At step 2-3, “SS transfers rrcReconfiguration to release user-plane resources of UE for the PDU session”. Corresponding SS side resources should also be released. |
| Summary of change | Add DRB release at SS side at steps 2-3 |
| TTCN module | Service_Request_NR5GC |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_7_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB); var template (omit) QosFlow_Identification_Type v_QosFlowId; var DRB_ToReleaseList v_DRBToReleaseList; var template (value) RadioBearerConfig v_RadioBearerConfig; var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var template (value) PDU_SessionStatus v_PDUSessionStatus; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_PSI; var boolean v_Result := false; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; //@sic R5s200716 sic@ var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5s200716 sic@ var NR_RLC_BearerToReleaseList_Type v_RlcToReleaseList; //@sic R5s200716 sic@ ………………… //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCReconfiguration message to release User-plane resources for the PDU session. //@sic R5s201129 sic@ f_NR_SendRRCReconfiguration(v_NGC_NASCellA, -, v_RadioBearerConfig); v_RlcToReleaseList := {f_NR_LogicalChannelId_DRB(f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession())}; //@sic R5s200716 sic@ v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , //@sic R5s200716 sic@ omit, v_RlcToReleaseList, omit, omit, omit); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //@sic R5s200716 sic@ f_NR_SendRRCReconfiguration(v_NGC_NASCellA, -, v_RadioBearerConfig, v_V1530Ext); //@sic R5s200716 sic@ //@siclog "Steps 4" siclog@ ……………. }

After Change:

function fl_TC_9_1_7_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI_SST1eMBB); var template (omit) QosFlow_Identification_Type v_QosFlowId; var DRB_ToReleaseList v_DRBToReleaseList; var template (value) RadioBearerConfig v_RadioBearerConfig; var SDAP_SDUList_Type v_SDAP_Data := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; //Create the IP Data var template (value) PDU_SessionStatus v_PDUSessionStatus; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_PSI; var boolean v_Result := false; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; //@sic R5s200716 sic@ var template (value) CellGroupConfig v_CellGroupConfig; //@sic R5s200716 sic@ var NR_RLC_BearerToReleaseList_Type v_RlcToReleaseList; //@sic R5s200716 sic@ var template (value) NR_RadioBearerList_Type v_RadioBearerDrbList; var integer k; var IntegerList_Type v_DrbList; ………… //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCReconfiguration message to release User-plane resources for the PDU session. //@sic R5s201129 sic@ f_NR_SendRRCReconfiguration(v_NGC_NASCellA, -, v_RadioBearerConfig); v_RlcToReleaseList := {f_NR_LogicalChannelId_DRB(f_NR_GetDefaultDRB_ForFirstNonIMSPDUSession())}; //@sic R5s200716 sic@ v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , //@sic R5s200716 sic@ omit, v_RlcToReleaseList, omit, omit, omit); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //@sic R5s200716 sic@ f_NR_SendRRCReconfiguration(v_NGC_NASCellA, -, v_RadioBearerConfig, v_V1530Ext); //@sic R5s200716 sic@ v_DrbList := f_NR_GetActiveDRBs(v_NGC_NASCellA); for (k := 0; k < lengthof(v_DrbList); k := k+1) { v_RadioBearerDrbList[k] := cs_NR_SS_DRB_Release(v_DrbList[k]); } f_NR_SS_CommonRadioBearerConfig(v_NGC_NASCellA, v_RadioBearerDrbList, cs_TimingInfo_Now); //@siclog "Steps 4" siclog@ ………………… }