3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210215

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4557 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to StaticCellInfo templates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2020-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During LTE test case cell info configuration two issues when tested with Band > 64 can be seen: - cs_UplinkStaticCellInfo: In this template, as first input parameter “ARFCN_ValueEUTRA p_EarfcnUL” is used. As defined in ASN.1: ARFCN-ValueEUTRA ::= INTEGER (0..maxEARFCN) With earfcnUL when Band > 64 being a value comprended between maxEARFCN-Plus1..maxEARFCN2, it can provoke a runtime error as it is out of range. - cs_StaticCellInfo_FDD: In this template, as third input parameter “ARFCN_ValueEUTRA p_EarfcnDL” is used. As defined in ASN.1: ARFCN-ValueEUTRA ::= INTEGER (0..maxEARFCN) With earfcnDL when Band > 64 being a value comprended between maxEARFCN-Plus1..maxEARFCN2, it can provoke a runtime error as it is out of range. In both cases this problem can be solved if type is changed to ARFCN_ValueEUTRA _r9 as values can range from (0..maxEARFCN2) which englobes earfcn values for every band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For both cases, changed input parameter type from ARFCN_ValueEUTRA to ARFCN_ValueEUTRA _r9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may FAIL the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Every LTE test case run with band > 64 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections	4

1.1	Correction to template cs_UplinkStaticCellInfo	4

1.2	Correction to template cs_StaticCellInfo_FDD	5

# Corrections

## Correction to template cs_UplinkStaticCellInfo

| Template name | Correction to template cs_UplinkStaticCellInfo |
| --- | --- |
| Reason for change | cs_UplinkStaticCellInfo: In this template, as first input parameter “ARFCN_ValueEUTRA p_EarfcnUL” is used. As defined in ASN.1: ARFCN-ValueEUTRA ::= INTEGER (0..maxEARFCN) With earfcnUL when Band > 64 being a value comprended between maxEARFCN-Plus1..maxEARFCN2, it can provoke a runtime error as it is out of range. |
| Summary of change | Changed input parameter type from ARFCN_ValueEUTRA to ARFCN_ValueEUTRA _r9. |
| TTCN module | common/EUTRA /EUTRA_CellCfg_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> template (value) UplinkStaticCellInfo_Type cs_UplinkStaticCellInfo(ARFCN_ValueEUTRA p_EarfcnUL, Ul_Bandwidth_Type p_Ul_Bandwidth) := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ Earfcn := p_EarfcnUL, Bandwidth := p_Ul_Bandwidth, CyclicPrefix := normal }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) UplinkStaticCellInfo_Type cs_UplinkStaticCellInfo(ARFCN_ValueEUTRA_r9 p_EarfcnUL_r9, Ul_Bandwidth_Type p_Ul_Bandwidth) := { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ Earfcn := p_EarfcnUL_r9, Bandwidth := p_Ul_Bandwidth, CyclicPrefix := normal }; <<SKIPPED CODE>>

## Correction to template cs_StaticCellInfo_FDD

| Template name | Correction to template cs_StaticCellInfo_FDD |
| --- | --- |
| Reason for change | cs_StaticCellInfo_FDD: In this template, as third input parameter “ARFCN_ValueEUTRA p_EarfcnDL” is used. As defined in ASN.1: ARFCN-ValueEUTRA ::= INTEGER (0..maxEARFCN) With earfcnDL when Band > 64 being a value comprended between maxEARFCN-Plus1..maxEARFCN2, it can provoke a runtime error as it is out of range. |
| Summary of change | Changed input parameter type from ARFCN_ValueEUTRA to ARFCN_ValueEUTRA _r9. |
| TTCN module | common/EUTRA /EUTRA_CellCfg_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> template (value) StaticCellInfo_Type cs_StaticCellInfo_FDD(PhysCellId p_PhysicalCellIdentity, integer p_EutraBand, ARFCN_ValueEUTRA p_EarfcnDL, Dl_Bandwidth_Type p_Dl_Bandwidth, integer p_Tcell, integer p_SfnOffset, integer p_HsfnOffset, // @sic R5w160204r1 sic@ template (omit) UplinkStaticCellInfo_Type p_UplinkStaticCellInfo) := //@sic R5-134786 sic@ { /* @sic R5s100029 change 3: additional parameters p_Tcell, p_SfnOffset sic@ */ /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ //@sic R5-163085 FrequencyBandindicator type changed to integer sic@ Common := { RAT := { FDD := true }, PhysicalCellId := p_PhysicalCellIdentity, eNB_CellId := omit, EutraBand := p_EutraBand, CellTimingInfo := { TcOffset := 0, /* @sic R5w180111 sic@ */ Tcell := p_Tcell, /* @sic R5s100029 sic@ */ SfnOffset := p_SfnOffset, /* @sic R5s100029 sic@ */ HsfnOffset := p_HsfnOffset /* @sic R5w160204r1 sic@ */ } }, Downlink := { Earfcn := p_EarfcnDL, Bandwidth := p_Dl_Bandwidth, RBSize := n_RB_SC_12, /* normal sub-frequency spacing */ CyclicPrefix := normal }, Uplink := p_UplinkStaticCellInfo }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) StaticCellInfo_Type cs_StaticCellInfo_FDD(PhysCellId p_PhysicalCellIdentity, integer p_EutraBand, ARFCN_ValueEUTRA_r9 p_EarfcnDL_r9, Dl_Bandwidth_Type p_Dl_Bandwidth, integer p_Tcell, integer p_SfnOffset, integer p_HsfnOffset, // @sic R5w160204r1 sic@ template (omit) UplinkStaticCellInfo_Type p_UplinkStaticCellInfo) := //@sic R5-134786 sic@ { /* @sic R5s100029 change 3: additional parameters p_Tcell, p_SfnOffset sic@ */ /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NR5GC_IRAT, POS) */ //@sic R5-163085 FrequencyBandindicator type changed to integer sic@ Common := { RAT := { FDD := true }, PhysicalCellId := p_PhysicalCellIdentity, eNB_CellId := omit, EutraBand := p_EutraBand, CellTimingInfo := { TcOffset := 0, /* @sic R5w180111 sic@ */ Tcell := p_Tcell, /* @sic R5s100029 sic@ */ SfnOffset := p_SfnOffset, /* @sic R5s100029 sic@ */ HsfnOffset := p_HsfnOffset /* @sic R5w160204r1 sic@ */ } }, Downlink := { Earfcn := p_EarfcnDL_r9, Bandwidth := p_Dl_Bandwidth, RBSize := n_RB_SC_12, /* normal sub-frequency spacing */ CyclicPrefix := normal }, Uplink := p_UplinkStaticCellInfo }; <<SKIPPED CODE>>