3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210220

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4561 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_CellInfo_SetSysInfo_SIB1v1250_IEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2020-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When initializing SIB1 of cell 10 at the beginning of the TC Body, function f_EUTRA_CellInfo_SetSysInfo_SIB1v1250_IEs is called. And according to Table 6.1.2.23.3.3-1 in the TC prose, some sub-IE fields are left uninitialized (e.g.: tdd_Config_1130 and cellSelectionInfo_1130). As per the below mentioned TTCN-3 Core Language specification this may cause runtime errors. According to "ETSI ES 201 873-1" Clause 15.6.0 "When referencing elements of templates or templates fields with the value or omit restriction, the rules for referencing elements of values are used: Clause "6.2.1.1 Referencing fields of a record type": a) When expanding a value or value field of record type, the subfield referenced in the dot notation shall be set to present and all unreferenced mandatory subfields shall be left uninitialized; when the assignment is used in a scope where the optional attribute is equal to "explicit omit", all unreferenced optional subfields shall be left undefined. When the assignment is used in a scope where the optional attribute is equal to "implicit omit", all unreferenced optional subfields shall be set to omit." |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialized sub-IE fields tdd_Config_v1130 and cellSelectionInfo_v1130. Note: if the assumtion that these sub-IEs can be omitted does not hold true, then an alternative solution must be found. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may FAIL the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections	4

1.1	Correction to f_EUTRA_CellInfo_SetSysInfo_SIB1v1250_IEs	4

# Corrections

## Correction to f_EUTRA_CellInfo_SetSysInfo_SIB1v1250_IEs

| Function name | f_EUTRA_CellInfo_SetSysInfo_SIB1v1250_IEs |
| --- | --- |
| Reason for change | When initializing SIB1 of cell 10 at the beginning of the TC Body, function f_EUTRA_CellInfo_SetSysInfo_SIB1v1250_IEs is called. And according to Table 6.1.2.23.3.3-1 in the TC prose, some sub-IE fields are left uninitialized (e.g.: tdd_Config_1130 and cellSelectionInfo_1130). This may cause runtime errors. See "ETSI ES 201 873-1" Clause 15.6.0. |
| Summary of change | Initialized sub-IE fields tdd_Config_v1130 and cellSelectionInfo_v1130. Note: if the assumtion that these sub-IEs can be omitted does not hold true, then an alternative solution must be found. |
| TTCN module | 6_1/Idle_CellReselection_R12 |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_EUTRA_CellInfo_SetSysInfo_SIB1v1250_IEs(EUTRA_CellId_Type p_CellId, template(omit) SystemInformationBlockType1_v1250_IEs p_SystemInformationBlockType1_v1250) runs on EUTRA_PTC { //@sic R5s170068 sic@ var template (value) EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := p_SystemInformationBlockType1_v1250; f_EUTRA_CellInfo_Set(p_CellId, v_CellInfo); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_EUTRA_CellInfo_SetSysInfo_SIB1v1250_IEs(EUTRA_CellId_Type p_CellId, template(omit) SystemInformationBlockType1_v1250_IEs p_SystemInformationBlockType1_v1250) runs on EUTRA_PTC { //@sic R5s170068 sic@ var template (value) EUTRA_CellInfo_Type v_CellInfo := f_EUTRA_CellInfo_Get(p_CellId); v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.tdd_Config_v1130 := omit; v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.cellSelectionInfo_v1130 := omit; v_CellInfo.Sysinfo.BCCH_Info.SIB1.message_.c1.systemInformationBlockType1.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension := p_SystemInformationBlockType1_v1250; f_EUTRA_CellInfo_Set(p_CellId, v_CellInfo); } <<SKIPPED CODE>>