3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210228

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0657 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FBBA cleanup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test, MTSI_FIXED_ACCESS-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN5 has agreed to stop maintenance and deliveries of IMS_FBBA Test Suite (R5-198137). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cleanup of IMS common code to remove FBBA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unneccessary complexity due to code and branches which are not used anymore |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All test suites with IMS PTC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## Change 1 – IPCAN_RAN_Type, IMS_SecurityScheme_Type

type enumerated IPCAN_RAN_Type { /* radio access network technology used by the IPCAN PTC */ GERAN, UTRAN_FDD, UTRAN_TDD, EUTRA_FDD, EUTRA_TDD, WLAN, /* @sic R5-163201 sic@ */ NR, /* @sic R5-188107 sic@ */ MCX_IP /* @sic R5-203077: MCX IP test model sic@ */ };

type enumerated IMS_SecurityScheme_Type { GIBA, /* 34.229-2 Table A.6a Item 1 */ IMS_Security /* 34.229-2 Table A.6a Item 2 */ };

## Change 2 – Removal of f_IMS_PTC_Security_InitFBBA

function f_IMS_Register_SecurityInit(REGISTER_Request p_RegisterReq, B128_Type p_Rand := tsc_IMS_AuthRAND, boolean p_StartSecurity := true) runs on IMS_PTC { var Common_AuthenticationParams_Type v_AuthenticationParams; var IP_AddrInfo_Type v_UE_Address := f_IMS_PTC_UE_Address_Get(); var IP_AddrInfo_Type v_NW_Address := f_IMS_PTC_NW_Address_Get(); var SipUrl v_ContactUrl := f_MessageHeader_GetContactSipUrl(p_RegisterReq.msgHeader); var template (omit) PortNumber_Type v_UnprotectedPort_us := f_SIP_ContactUrl_GetPort(v_ContactUrl); // may be omit in which case 5060 is chosen by the IP PTC if (ispresent(p_RegisterReq.msgHeader.securityClient)) { /* @sic R5s150873: false for GIBA sic@ */ f_IMS_Register_SecurityInit_ImsSecurity(p_RegisterReq, p_Rand, p_StartSecurity); /* @sic R5s190727 change 1.5: MCX split sic@ */ } else { /* GIBA */ f_IMS_PortsAndSecurityConfig(v_UnprotectedPort_us, cs_IMS_RegistrationInfo(v_NW_Address, v_UE_Address)); } }

## Change 3 – f_IMS_PTC_Init

function f_IMS_PTC_Init(PDN_Index_Type p_PdnIndex := PDN_1, IMS_PTC_Handling_Type p_Handling := testcaseSpecificHandling, template (omit) IMS_SecurityScheme_Type p_SecurityScheme := omit, template (omit) IPCAN_RAN_Type p_RanType := omit, IMS_TestSpec_Type p_TestSpec := TS_34229, NameValueList_Type p_TestsuiteParameterList := {}) runs on IMS_PTC { var charstring v_HomeDomainName; var charstring v_PrivateUserId; var CharStringList_Type v_IMPUList; var IMS_SecurityScheme_Type v_SecurityScheme; var charstring v_Password_FBBA; var IntegerList_Type v_SPI_ValidValueList_PDN1 := fl_IMS_PTC_SPI_ValidValueList_Init(px_IMS_SPI_ValidValueList_PDN1); /* @sic R5-172048 sic@ */ var IntegerList_Type v_SPI_ValidValueList_PDN2 := fl_IMS_PTC_SPI_ValidValueList_Init(px_IMS_SPI_ValidValueList_PDN2); /* @sic R5-172048 sic@ */ var template (omit) integer v_UE_Release := omit; var boolean v_UEwithISIM := px_UEwithISIM; f_IMS_PTC_TestsuiteInfo_Init(p_TestSpec, p_TestsuiteParameterList); v_IsMCX := f_IMS_PTC_TestsuiteInfo_IsMCX(); if (v_IsMCX) { /* @sic R5-196983 sic@ */ v_UEwithISIM := not match(f_IMS_PTC_TestsuiteInfo_GetParameter("MCX_SIP_RegistrationWithTemporaryIdentities"), "true"); if (v_UEwithISIM) { v_HomeDomainName := f_IMS_PTC_TestsuiteInfo_GetParameter("MCX_SIP_HomeDomain_A"); v_PrivateUserId := f_IMS_PTC_TestsuiteInfo_GetParameter("MCX_SIP_PrivateUserId_A"); } else { v_HomeDomainName := f_DomainName_IMS_DerivedFromIMSI(px_IMSI_Def); v_PrivateUserId := f_IMS_PrivateUserIdentityDerivedFromIMSI(px_IMSI_Def); } } else if (px_UEwithISIM ) { /* @sic R5-133628: check of temporary user id sic@ */ v_HomeDomainName := px_IMS_HomeDomainName; v_PrivateUserId := px_IMS_Private_UserId; } else { v_HomeDomainName := f_DomainName_IMS_DerivedFromIMSI(px_IMSI_Def); v_PrivateUserId := f_IMS_PrivateUserIdentityDerivedFromIMSI(px_IMSI_Def); } if (ispresent(p_SecurityScheme)) { v_SecurityScheme := valueof(p_SecurityScheme); } else if (pc_IMS_GIBA_Sec and not pc_IMS_Sec) { v_SecurityScheme := GIBA; } else if (pc_IMS_Sec) { v_SecurityScheme := IMS_Security; } else { FatalError(__FILE__, __LINE__, "Invalid security setting (neither pc_IMS_GIBA_Sec nor pc_IMS_Sec is set)"); } if (v_IsMCX) { /* @sic R5-196983 sic@ */ v_IMPUList := { f_IMS_PTC_TestsuiteInfo_GetParameter("MCX_SIP_PublicUserId_A_1") }; } else { v_IMPUList := {px_IMS_PublicUserIdentity1, px_IMS_PublicUserIdentity2, px_IMS_PublicUserIdentity3}; } vc_IMS_Global.PdnIndex := p_PdnIndex; vc_IMS_Global.Ctrl.TestBody := false; vc_IMS_Global.Ctrl.BlockUlDataInDefault := false; /* @sic R5s140780 - MCC160 Implementation sic@ */ vc_IMS_Global.Ctrl.IgnoreReset := true; /* @sic R5s150031: IMS is already reset after initialisation sic@ */ vc_IMS_Global.Ctrl.Default := null; vc_IMS_Global.Security := valueof(cs_IMS_SecurityInfo_Init); vc_IMS_Global.ImsInfo := valueof(cs_IMS_ImsInfo_Init(p_RanType, v_IMPUList, v_UE_Release, v_HomeDomainName, v_PrivateUserId, v_SecurityScheme, v_SPI_ValidValueList_PDN1, v_SPI_ValidValueList_PDN2, v_UEwithISIM)); f_IMS_PTC_ActivateDefault(p_Handling); }

## Change 4 – f_IMS_MOCallSetup_ReceiveINVITE_Optional

function f_IMS_MOCallSetup_ReceiveINVITE_Optional(template (omit) float p_WaitDuration := omit, template SipUrl p_OtherCalleeUri := omit, template SDP_Message p_SDP_Message := omit, IMS_SDP_AudioCodecList_Type p_AudioCodecList, boolean p_PreconditionSupported := true, boolean p_OIR := false, boolean p_GeolocationInfoAvailable := false) runs on IMS_PTC return template (omit) IMS_InviteRequestWithSdp_Type { var OptionTag_List v_AdditionalOptionTagsForSupported := f_IMS_AdditionalOptionTagsForSupported(p_PreconditionSupported); /* @sic R5s180197 sic@ */ var template (omit) IMS_DATA_REQ v_IMS_DATA_REQ; var template (omit) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp := omit; var template (present) SDP_Message v_SDP_MessageTemplate; var INVITE_A_2_1_Context_Type v_Context := A_2_1_A4; var template (present) SipUrl v_ExpectedCalleeUri := f_IMS_INVITE_CalleeUriRX(p_OtherCalleeUri, v_Context); var template (present) MessageHeader v_MessageHeader_Invite := f_IMS_InviteRequest_MessageHeaderRX(v_ExpectedCalleeUri, v_AdditionalOptionTagsForSupported, v_Context, -, p_GeolocationInfoAvailable); // @sic R5s190882 sic@ var IMS_DATA_REQ v_IMS_DATA_REQ_Value; var SDP_Message v_SDP_Message; var INVITE_Request v_InviteRequest; var boolean v_CheckForbiddenCodecParameters; /* @sic R5-180231 sic@ */ if (ispresent(p_SDP_Message)) { /* specific SDP message content @sic R5s180197 sic@ */ v_SDP_MessageTemplate := p_SDP_Message; v_CheckForbiddenCodecParameters := false; /* no automatic check for forbidden parameters in case of specific (i.e. non C.21(abcd)) SDP messages */ } else { /* SDP message content according to C.21(abcd) */ v_SDP_MessageTemplate := f_IMS_BuildSDP_InitialOfferAudio_RX(p_PreconditionSupported, p_AudioCodecList); /* @sic R5s150704 - additional changes sic@ @sic R5-172931 sic@ */ v_CheckForbiddenCodecParameters := true; /* automatic check for forbidden parameters for Cat-M1 (C.21d), EVS (C.44), WLAN (C.21a) and normal cases (C.21); @sic R5-180231: grace period, R5-192309: grace period for normal cases expired sic@ */ } // Step 2. Receive INVITE Request v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest_A_2_1_Optional(v_Context, v_ExpectedCalleeUri, v_MessageHeader_Invite, p_WaitDuration, -, p_GeolocationInfoAvailable, -, -, p_OIR); if (isvalue(v_IMS_DATA_REQ)) { v_IMS_DATA_REQ_Value := valueof(v_IMS_DATA_REQ); v_InviteRequest := v_IMS_DATA_REQ_Value.Request.Invite; v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_InviteRequest.messageBody, v_SDP_MessageTemplate, p_GeolocationInfoAvailable); // check media description: f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_SDP_Message.media_list[0], p_AudioCodecList, v_CheckForbiddenCodecParameters); /* @sic R5-151968: f_IMS_SDP_CheckMediaDescr_EVS_16000_1, f_IMS_SDP_CheckMediaDescr_AMR_16000_1 replaced by f_IMS_SDP_CheckMediaDescr_AudioCodec sic@ @sic R5-172931: f_IMS_SDP_CheckMediaDescr_AudioCodecs instead of f_IMS_SDP_CheckMediaDescr_AudioCodec sic@ @sic R5-180231: v_CheckForbiddenCodecParameters sic@ */ v_InviteRequestWithSdp := f_IMS_InviteRequestWithSdpMO(v_IMS_DATA_REQ_Value, v_SDP_Message); } return v_InviteRequestWithSdp; }

## Change 5 – f_IMS_MOCallSetup_Video_Step2

function f_IMS_MOCallSetup_Video_Step2(VideoCallTypeMO_Type p_VideoCallType, template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var template (present) SDP_Message v_SDP_Message_Step2; var INVITE_A_2_1_Context_Type v_Context; var IMS_DATA_REQ v_IMS_INVITE_REQ; var SDP_Message v_SDP_Message; var boolean v_A11 := true; /* @sic R5-176371: INVITE for creating a video call sic@ */ var boolean v_PreconditionSupported := true; var OptionTag_List v_AdditionalOptionTagsForSupported := f_IMS_AdditionalOptionTagsForSupported(v_PreconditionSupported); /* @sic R5-170781 sic@ */ var IMS_SDP_AudioCodecList_Type v_AudioCodecList; v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR; select (p_VideoCallType) { case (SetupVideoCall) { v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAudioVideo_RX(v_AudioCodecList); v_Context := A_2_1_A4; } case (AddVideo) { v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAddVideo_RX(); v_Context := A_2_1_A5; } } // Step 2. Receive INVITE Request v_IMS_INVITE_REQ := f_IMS_INVITE_ReceiveRequest(v_Context, v_AdditionalOptionTagsForSupported, p_OtherCalleeUri, -, -, v_A11); /* @sic R5s150180: p_A10 removed sic@ @sic R5-176209: v_A12 removed sic@ */ v_SDP_Message := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_IMS_INVITE_REQ.Request.Invite.messageBody, v_SDP_Message_Step2); // check media description: if (p_VideoCallType == SetupVideoCall) { /* @sic R5-172931: no check of voice codecs for 17.1 sic@ */ f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_SDP_Message.media_list[0], v_AudioCodecList); /* @sic R5-151968: f_IMS_SDP_CheckMediaDescr_AMR_8000_1 replaced by f_IMS_SDP_CheckMediaDescr_AudioCodec sic@ @sic R5-168032, R5-168141: AMR_16000 sic@ @sic R5-172931 sic@ */ } f_IMS_SDP_CheckMediaDescr_H264_90000(v_SDP_Message.media_list[1]); f_IMS_SDP_CheckMediaDescr_TcapPcfg(v_SDP_Message.media_list[1]); return f_IMS_InviteRequestWithSdpMO(v_IMS_INVITE_REQ, v_SDP_Message); }

## Change 6 – f_IMS_BuildSDP_InitialOfferAudio_RX

function f_IMS_BuildSDP_InitialOfferAudio_RX(boolean p_PreconditionsRequired := true, IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR) runs on IMS_PTC return template (present) SDP_Message { var template (present) SDP_attribute_list v_SDP_MediaAttributes := {}; var template (present) integer v_BandwidthRS := ?; /* @sic R5-160906 sic@ */ var template (present) integer v_BandwidthRR := (1..infinity); /* @sic R5-160906 sic@ */ var template (present) SDP_attribute_list v_SDP_PrecondionAttributes := {}; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); /* @sic R5-171476: C.21a sic@ */ var boolean v_IsCatM1 := f_IMS_TestcaseIsCatM1(); /* @sic R5-174483: C.21d sic@ */ v_SDP_MediaAttributes := f_SDP_MediaAttributesAudioCodecListRX(p_AudioCodecList); /* @sic R5-168031, R5-168053, R5-168054, R5-172931 sic@ */ /* @sic R5-150709: cr_SDP_Attribute_inactive removed sic@ */ if (not v_IsWLAN and not v_IsCatM1) { /* Attributes for media security mechanism may be required for C.21 only */ v_SDP_MediaAttributes := f_IMS_SDP_MediaAttributes_AddMediaSecurityAttributesRX(v_SDP_MediaAttributes); /* @sic R5-160293, R5-160915 new TC H.12.3 sic@ */ } if (p_PreconditionsRequired) { /* @sic R5s150704, R5s150751 - Additional changes sic@ */ if (v_IsWLAN) { /* @sic R5-171476 - WLAN Annex C.21a sic@ */ v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_sendrecv, c_none, c_mandatory, c_optional); } else { v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_none, c_none, c_mandatory, c_optional); } } return f_IMS_BuildSDP_RX(cr_SDP_Time_Any, cr_SDP_Media_Audio, v_SDP_MediaAttributes, v_SDP_PrecondionAttributes, -, v_BandwidthRS, v_BandwidthRR); }

## Change 7 – f_IMS_BuildSDP_InitialAnswerAudio_TX

function f_IMS_BuildSDP_InitialAnswerAudio_TX(SDP_Message p_SDP_MessageInvite, boolean p_PreconditionsRequired := true, IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000, integer p_BandwidthAS := 37) runs on IMS_PTC return template (value) SDP_Message { var template (omit) SDP_attribute v_Attribute; var SDP_media_desc v_SDP_Media_Invite := p_SDP_MessageInvite.media_list[0]; var template (present) SDP_attribute_fmtp_format_specific_params v_FmtpParameters := f_SDP_FmtpDefaultParametersAudioCodecRX(p_AudioCodec); /* @sic R5s160379 change 2 sic@ */ var charstring v_FmtAudio := f_SDP_MediaDescr_GetAudioCodec(v_SDP_Media_Invite, p_AudioCodec, v_FmtpParameters); var integer v_BandwidthRS := f_SDP_BandwidthList_GetBandwidth(v_SDP_Media_Invite.bandwidth, "RS"); var integer v_BandwidthRR := f_SDP_BandwidthList_GetBandwidth(v_SDP_Media_Invite.bandwidth, "RR"); var template (value) SDP_bandwidth_list v_Bandwidth_List_Media := cs_SDP_Bandwidth_List_Media(p_BandwidthAS, v_BandwidthRS, v_BandwidthRR); // @sic R5-123505, R5-135020; R5-144700: AS=p_BandwidthAS sic@ var template (omit) SDP_attribute_list v_SDP_MediaAttributes; var template (omit) SDP_attribute_list v_SDP_PrecondionAttributes := omit; var template (omit) SemicolonParam_List v_FmtpParameterList_Answer := omit; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); /* @sic R5-171477 sic@ */ var boolean v_IsCatM1 := f_IMS_TestcaseIsCatM1(); /* @sic R5-174483: C.21d sic@ */ var SemicolonParam_List v_FmtpParameterList_Offer; var CharStringList_Type v_FmtpParametersToBoCopied; var charstring v_BwVal; select (p_AudioCodec) { // @sic R5-151968, R5-153999 sic@ case (EVS_16000){ if (f_IMS_PTC_RanTypeIsNR5GC()) { // @sic R5-206382: implementation of TC_7_4 sic@ v_FmtpParameterList_Offer := f_SDP_MediaDescr_GetFirstEVSCodecParameters(v_SDP_Media_Invite); if (match(v_FmtpParameterList_Offer, cr_EVS_Fmtp_AudioParamsBrBw("13.2", "swb"))) { v_BwVal := "13.2"; // 34.229-5 A.4.1/2 183 Session Progress, NOTE 8 } else { v_BwVal := "5.9-13.2"; // 34.229-5 A.4.1/2 183 Session Progress, NOTE 9 } v_FmtpParameterList_Answer := { cs_EVS_Br(v_BwVal), cs_EVS_Bw("swb"), cs_Fmtp_ModesetParam("0,1,2"), cs_AMR_MaxRed }; } else { // C.44 @sic R5-153999 sic@ v_Attribute := f_SDP_AttributeList_GetAttribute(v_SDP_Media_Invite.attributes, cr_SDP_Attribute_fmtp(v_FmtAudio)); v_BwVal := f_SIP_SemicolonParamList_GetParamValue(v_Attribute.fmtp.params.paramList, "bw"); /* @sic R5s170336/R5-172069 sic@ */ v_FmtpParameterList_Answer := {cs_EVS_Bw(v_BwVal), cs_EVS_BwSend(v_BwVal), cs_EVS_BwRecv(v_BwVal), cs_AMR_MaxRed}; /* @sic R5s160379: Additional Change by MCC160 sic@ @sic R5s170336/R5-172069: v_BwVal sic@ */ v_FmtpParametersToBoCopied := { "br", "br-send", "br-recv" }; v_FmtpParameterList_Offer := f_SDP_MediaDescr_GetCodecParameters(v_SDP_Media_Invite.attributes, v_FmtAudio); v_FmtpParameterList_Answer := f_SIP_SemicolonParamList_CopyParams(v_FmtpParameterList_Offer, v_FmtpParameterList_Answer, v_FmtpParametersToBoCopied); } v_SDP_MediaAttributes := f_SDP_MediaAttributes_EVS_AudioDef(v_FmtAudio, valueof(v_FmtpParameterList_Answer)); // valueof to avoid compiler warning } case (AMR_16000){ // @sic R5-153950 sic@ v_SDP_MediaAttributes := f_SDP_MediaAttributes_AMR_WB_AudioDef(v_FmtAudio); /* @sic R5s160379 change 2 sic@ */ } case (AMR_8000){ v_SDP_MediaAttributes := f_SDP_MediaAttributes_AudioDef(v_FmtAudio); /* @sic R5-144693: new function f_SDP_MediaAttributes_AudioDef sic@ */ } } if (not v_IsWLAN and not v_IsCatM1) { // @sic R5-171476, R5-174483: not required for C.21a and C.21d sic@ v_SDP_MediaAttributes := f_IMS_SDP_MediaAttributes_CopyEcnCapabilityAttributesTX(v_SDP_Media_Invite, v_SDP_MediaAttributes); } if (not v_IsWLAN and not v_IsCatM1) { /* Attributes for media security mechanism may be required for C.21 only */ v_SDP_MediaAttributes := f_IMS_SDP_MediaAttributes_AddMediaSecurityAttributesTX(v_SDP_MediaAttributes); /* @sic R5-160293, R5-160915 new TC H.12.3 sic@ */ } if (p_PreconditionsRequired) { /* @sic R5s150704, R5s150751 - Additional changes sic@ */ if (v_IsWLAN) { /* @sic R5-171477 - WLAN Annex C.21a sic@ */ v_SDP_PrecondionAttributes := cs_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory); // @sic R5s170904 change 3: sendrecv for remote precondition sic@ } else { v_SDP_PrecondionAttributes := cs_SDP_PrecondionAttributes(c_none, c_none, c_mandatory, c_mandatory); v_SDP_PrecondionAttributes := f_SDP_Attributes_Add_TX(v_SDP_PrecondionAttributes, cs_SDP_Attribute_conf_qos); // @sic R5s120530 change 3.5 sic@ } } return f_IMS_BuildSDP_TX(cs_SDP_Media_Audio(v_FmtAudio), v_Bandwidth_List_Media, v_SDP_MediaAttributes, v_SDP_PrecondionAttributes); }

## Change 8 – f_IMS_BuildSDP_InitialOfferAudioVideo_RX

function f_IMS_BuildSDP_InitialOfferAudioVideo_RX(IMS_SDP_AudioCodecList_Type p_AudioCodecList) runs on IMS_PTC return template (present) SDP_Message { var template (present) SDP_attribute_list v_SDP_MediaAttributes_Audio; var template (present) SDP_attribute_list v_SDP_MediaAttributes_Video; var template (present) SDP_attribute_list v_SDP_PrecondionAttributes; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); /* @sic R5-171478 sic@ */ if (v_IsWLAN) { // C.25a v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_sendrecv, c_none, c_mandatory, c_optional); } else { // C.25 v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_none, c_none, c_mandatory, c_optional); } v_SDP_MediaAttributes_Audio := f_SDP_MediaAttributesAudioCodecListRX(p_AudioCodecList); // @sic R5-168032, R5-172931 sic@ v_SDP_MediaAttributes_Video := { // The tcap/pcfg attributes need to be checked later when we know whether we have RTP/AVP or RTP/AVPF cr_SDP_Attribute_rtpmap(-, cr_RTPMAP_H264_90000), cr_SDP_Attribute_fmtp(-, cr_Video_Fmtp_DefaultParametersMO) }; return f_IMS_BuildSDP_AudioVideo_RX(cr_SDP_Time_Any, cr_SDP_Media_Audio, v_SDP_MediaAttributes_Audio, v_SDP_PrecondionAttributes, cr_SDP_Media_Video(-, (c_rtpAvp, c_rtpAvpf)), v_SDP_MediaAttributes_Video, v_SDP_PrecondionAttributes); }

## Change 9 – f_IMS_BuildSDP_InitialOfferAddVideo_RX

function f_IMS_BuildSDP_InitialOfferAddVideo_RX() runs on IMS_PTC return template (present) SDP_Message { var template (present) SDP_attribute_rtpmap_codec v_AudioCodec; var template (present) SDP_attribute_list v_SDP_MediaAttributes_Audio; var template (present) charstring v_StrengthRemote := (c_optional, c_mandatory); /* @sic R5-165081: (c_optional, c_mandatory) sic@ */ var template (present) SDP_attribute_list v_SDP_PrecondionAttributes_Audio; var template (present) SDP_attribute_list v_SDP_PrecondionAttributes_Video; var template (present) SDP_attribute_list v_SDP_MediaAttributes_Video := { // The tcap/pcfg attributes need to be checked later when we know whether we have RTP/AVP or RTP/AVPF cr_SDP_Attribute_rtpmap(-, cr_RTPMAP_H264_90000), cr_SDP_Attribute_fmtp(-, cr_Video_Fmtp_DefaultParametersMO) }; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); v_AudioCodec := cr_RTPMAP_AMR_16000; v_SDP_PrecondionAttributes_Audio := cr_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, v_StrengthRemote); if (v_IsWLAN) { v_SDP_PrecondionAttributes_Video := cr_SDP_PrecondionAttributes(c_sendrecv, c_none, c_mandatory, v_StrengthRemote); } else { v_SDP_PrecondionAttributes_Video := cr_SDP_PrecondionAttributes(c_none, c_none, c_mandatory, v_StrengthRemote); } v_SDP_MediaAttributes_Audio := { cr_SDP_Attribute_rtpmap(-, v_AudioCodec), /* @sic R5-168141 sic@ */ cr_SDP_Attribute_fmtp /* => any parameters for audio */ }; return f_IMS_BuildSDP_AudioVideo_RX(cr_SDP_Time_Any, cr_SDP_Media_Audio, v_SDP_MediaAttributes_Audio, v_SDP_PrecondionAttributes_Audio, cr_SDP_Media_Video(-, (c_rtpAvp, c_rtpAvpf)), v_SDP_MediaAttributes_Video, v_SDP_PrecondionAttributes_Video); }

## Change 10 – f_IMS_BuildSDP_InitialAnswerAudioVideo_TX

function f_IMS_BuildSDP_InitialAnswerAudioVideo_TX(VideoCallTypeMO_Type p_VideoCallType, SDP_Message p_SDP_Message_Step2) runs on IMS_PTC return template (value) SDP_Message { var SDP_media_desc v_SDP_Media_AudioRx := p_SDP_Message_Step2.media_list[0]; var SDP_media_desc v_SDP_Media_VideoRx := p_SDP_Message_Step2.media_list[1]; var template (value) SDP_bandwidth_list v_Bandwidth_List_Audio := v_SDP_Media_AudioRx.bandwidth; var template (value) SDP_bandwidth_list v_Bandwidth_List_Video := v_SDP_Media_VideoRx.bandwidth; var template (omit) SDP_attribute v_FmtpAttributeVideo; var template (value) SemicolonParam_List v_FmtpParamListVideo := {}; var template (omit) SDP_attribute_list v_SDP_MediaAttributes_Audio := {}; var template (omit) SDP_attribute_list v_SDP_MediaAttributes_Video := {}; var template (omit) SDP_attribute_list v_SDP_PrecondionAttributes_Audio := omit; var template (omit) SDP_attribute_list v_SDP_PrecondionAttributes_Video := omit; var charstring v_FmtAudio; var charstring v_FmtVideo; var boolean v_TcapPcfg; var integer v_SessionBandwidthAS; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); /***** media attributes for audio *****/ v_FmtAudio := f_SDP_MediaDescr_GetAMR_WB_16000_1(v_SDP_Media_AudioRx); // @sic R5-168032, R5-168119, R5-168141 sic@ v_SDP_MediaAttributes_Audio := f_SDP_MediaAttributes_AMR_WB_AudioDef(v_FmtAudio); /* @sic R5-144693: new function f_SDP_MediaAttributes_AudioDef sic@ @sic R5-168032, R5-168119, R5-168141 sic@ @sic R5s170427: f_SDP_MediaAttributes_AMR_WB_AudioDef instead of f_SDP_MediaAttributes_AMR_WB_AudioCommon sic@ */ if ((p_VideoCallType == SetupVideoCall) and not v_IsWLAN) { /* NOTE: there is nothing specified in 17.1 or C.25a regarding "a=inactive" */ if (f_SDP_AttributeList_CheckAttribute(v_SDP_Media_AudioRx.attributes, cr_SDP_Attribute_inactive)) { /* @sic R5-151796/R5-151797 sic@ */ v_SDP_MediaAttributes_Audio := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Audio, cs_SDP_Attribute_inactive); /* in case of C.25b this does not make sense, nevertheless it does not harm as long as the UE does not provide a=inactive at step 2 */ } } /***** media attributes for video *****/ v_FmtVideo := f_SDP_MediaDescr_GetH264_90000(v_SDP_Media_VideoRx); v_TcapPcfg := fl_IMS_Video_TcapPcfgRequired(v_SDP_Media_VideoRx); v_FmtpAttributeVideo := f_SDP_AttributeList_GetAttribute(v_SDP_Media_VideoRx.attributes, cr_SDP_Attribute_fmtp(v_FmtVideo)); // @sic R5s141336 change 2.2 - MCC160 implementation sic@ */ if (isvalue(v_FmtpAttributeVideo)) { v_FmtpParamListVideo := valueof(v_FmtpAttributeVideo.fmtp.params.paramList); /* @sic R5-150734, R5-150736 sic@ */ } v_SDP_MediaAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Video, cs_SDP_Attribute_rtpmap(v_FmtVideo, cs_RTPMAP_H264_90000)); v_SDP_MediaAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Video, cs_SDP_Attribute_fmtp(v_FmtVideo, cs_Fmtp_ParamList(v_FmtpParamListVideo))); v_SDP_MediaAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Video, cs_SDP_Attribute_acfg(1, "t=1"), v_TcapPcfg); if ((p_VideoCallType == SetupVideoCall) and not v_IsWLAN) { /* NOTE: there is nothing specified in 17.1 or C.25a or C.25b regarding "a=inactive" */ if (f_SDP_AttributeList_CheckAttribute(v_SDP_Media_VideoRx.attributes, cr_SDP_Attribute_inactive)) { /* @sic R5-150694 sic@ */ v_SDP_MediaAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Video, cs_SDP_Attribute_inactive); } } /***** precondition attributes *****/ if (v_IsWLAN) { // C.25a: all resource are available v_SDP_PrecondionAttributes_Audio := cs_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory); v_SDP_PrecondionAttributes_Video := cs_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory); } else { // C.25 or 17.1 v_SDP_PrecondionAttributes_Video := cs_SDP_PrecondionAttributes(c_none, c_none, c_mandatory, c_mandatory); v_SDP_PrecondionAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_PrecondionAttributes_Video, cs_SDP_Attribute_conf_qos); select (p_VideoCallType) { case (SetupVideoCall) { // C.25 v_SDP_PrecondionAttributes_Audio := v_SDP_PrecondionAttributes_Video; // same preconditions for audio and video } case (AddVideo) { // 17.1 v_SDP_PrecondionAttributes_Audio := cs_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory); } } } if (v_IsWLAN and (p_VideoCallType == SetupVideoCall)) { v_SessionBandwidthAS := f_SDP_BandwidthList_GetBandwidth(p_SDP_Message_Step2.bandwidth, "AS"); /* C.25a: "b=AS: (bandwidth-value) [Note 1]" */ } else { /* @sic R5-174556: b=AS:30 for C.25b (same as for C.25) sic@ */ v_SessionBandwidthAS := 30; } return f_IMS_BuildSDP_AudioVideo_TX(v_SessionBandwidthAS, cs_SDP_Media_Audio(v_FmtAudio), v_Bandwidth_List_Audio, v_SDP_MediaAttributes_Audio, v_SDP_PrecondionAttributes_Audio, cs_SDP_Media_Video(v_FmtVideo), v_Bandwidth_List_Video, v_SDP_MediaAttributes_Video, v_SDP_PrecondionAttributes_Video); }

## Change 11 – f_IMS_BuildSDP_MTCallAudioVideo_TX

function f_IMS_BuildSDP_MTCallAudioVideo_TX(VideoCallTypeMT_Type p_VideoCallType, VideoCallMT_MessageTX_Type p_MessageType, charstring p_DirectionRemote := c_none) runs on IMS_PTC return template (value) SDP_Message { var IMS_SDP_AudioCodecList_Type v_AudioCodecs; var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpList; var template (omit) SDP_attribute_list v_SDP_MediaAttributes_Audio; var template (omit) SDP_attribute_list v_SDP_MediaAttributes_Video; var template (value) SDP_fmt_list v_FmtListAudio; var template (value) SDP_bandwidth_list v_Bandwidth_List_Audio := cs_SDP_Bandwidth_List_Media(37, 0, 2000); // @sic R5-151956, R5s170056/R5-171489: 37 instead of 30 sic@ var template (value) SDP_bandwidth_list v_Bandwidth_List_Video := cs_SDP_Bandwidth_List_Media(315, 0, 2500); var template (omit) SDP_attribute_list v_SDP_PrecondionAttributes_Audio := omit; var template (omit) SDP_attribute_list v_SDP_PrecondionAttributes_Video := omit; var integer v_SessionBandwidthAS := 352; /* @sic R5s170056/R5-171489/R5-174557 sic@ */ var charstring v_FmtVideo := tsc_SDP_FmtVideo; /* @sic R5-197050: tsc_SDP_FmtVideo instead of fl_IMS_BuildSDP_FmtVideo sic@ */ var template (value) SDP_media_field v_MediaVideo := cs_SDP_Media_Video(v_FmtVideo); var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); var boolean v_IsVideoCallSetup := (p_VideoCallType == SetupVideoCall); var boolean v_IsUpdate := (p_MessageType == Update); var boolean v_IsNotRemoveVideo := (p_VideoCallType != RemoveVideo); if (v_IsVideoCallSetup) { /* C.26/C.26a/C.26b */ if (not v_IsUpdate) { /* C.21/C.21a INVITE (Step 1) */ v_AudioCodecs := { AMR_16000, DTMF_16000, AMR_8000, DTMF_8000 }; if (v_IsWLAN) { /* C.21a INVITE (Step 1) */ v_SDP_PrecondionAttributes_Audio := cs_SDP_PrecondionAttributes(c_sendrecv, c_none, c_mandatory, c_optional); } else { /* C.21 INVITE (Step 1) */ v_SDP_PrecondionAttributes_Audio := cs_SDP_PrecondionAttributes(c_none, c_none, c_mandatory, c_optional); } } else { /* C.26 UPDATE (Step 7) */ v_AudioCodecs := { AMR_16000 }; v_SDP_PrecondionAttributes_Audio := cs_SDP_PrecondionAttributes(c_sendrecv, p_DirectionRemote, c_mandatory, c_mandatory); } v_SDP_PrecondionAttributes_Video := v_SDP_PrecondionAttributes_Audio; // same precondions for video and audio } else { /* Add/remove video to/from call */ v_AudioCodecs := { AMR_16000 }; if (v_IsNotRemoveVideo) { if (not v_IsUpdate) { /* 17.2 INVITE (Step 1) */ v_SDP_PrecondionAttributes_Audio := cs_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_optional); v_SDP_PrecondionAttributes_Video := cs_SDP_PrecondionAttributes(c_none, c_none, c_mandatory, c_optional); } else { /* 17.2 UPDATE (Step 6) */ v_SDP_PrecondionAttributes_Audio := cs_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory); v_SDP_PrecondionAttributes_Video := cs_SDP_PrecondionAttributes(c_sendrecv, p_DirectionRemote, c_mandatory, c_mandatory); } } else { /* Remove video: 17.2 step 10, H.17.2 step 9 */ v_SessionBandwidthAS := 37; /* audio only @sic R5-171489 sic@ */ v_MediaVideo.ports.port_number := 0; v_SDP_PrecondionAttributes_Audio := cs_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_optional); v_SDP_PrecondionAttributes_Video := cs_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_optional); } } // ***** Media description audio ***** v_MediaDescriptionRtpList := f_SDP_MediaDescriptionAudioCodecListTX(v_AudioCodecs); /* @sic R5-172930 sic@ */ v_FmtListAudio := v_MediaDescriptionRtpList.FmtList; v_SDP_MediaAttributes_Audio := v_MediaDescriptionRtpList.AttributeList; if (not (v_IsUpdate and (p_VideoCallType == AddVideo))) { // @sic R5-151956: there are no ptime, maxptime for UPDATE of 17.2 sic@ // additional attributes for audio: ptime, maxptime v_SDP_MediaAttributes_Audio := f_SDP_Attributes_Concat_TX(v_SDP_MediaAttributes_Audio, cs_SDP_MediaAttributes_ptime_maxptime); } // ***** Media description video ***** v_SDP_MediaAttributes_Video := { cs_SDP_Attribute_rtpmap(v_FmtVideo, cs_RTPMAP_H264_90000), cs_SDP_Attribute_fmtp(v_FmtVideo, cs_Fmtp_ParamList(cs_Fmtp_VideoParamsDef)) }; v_SDP_MediaAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Video, cs_SDP_Attribute_rtcp_fb("*", "trr-int 5000"), v_IsNotRemoveVideo); /* @sic R5s141329 change 2.2: attribute is for video sic@ */ v_SDP_MediaAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Video, cs_SDP_Attribute_rtcp_fb("*", "nack"), v_IsNotRemoveVideo); /* @sic R5s141329 change 2.2: attribute is for video sic@ */ v_SDP_MediaAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Video, cs_SDP_Attribute_rtcp_fb("*", "nack pli"), v_IsNotRemoveVideo); /* @sic R5s141329 change 2.2: attribute is for video sic@ */ v_SDP_MediaAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Video, cs_SDP_Attribute_rtcp_fb("*", "ccm fir"), v_IsNotRemoveVideo); /* @sic R5s141329 change 2.2: attribute is for video sic@ */ v_SDP_MediaAttributes_Video := f_SDP_Attributes_Add_TX(v_SDP_MediaAttributes_Video, cs_SDP_Attribute_rtcp_fb("*", "ccm tmmbr"), v_IsNotRemoveVideo); /* @sic R5s141329 change 2.2: attribute is for video sic@ */ return f_IMS_BuildSDP_AudioVideo_TX(v_SessionBandwidthAS, cs_SDP_Media_MultiAudio(v_FmtListAudio), v_Bandwidth_List_Audio, v_SDP_MediaAttributes_Audio, v_SDP_PrecondionAttributes_Audio, v_MediaVideo, v_Bandwidth_List_Video, v_SDP_MediaAttributes_Video, v_SDP_PrecondionAttributes_Video); }

## Change 12 – f_IMS_BuildSDP_MTCallAudioVideo_RX

function f_IMS_BuildSDP_MTCallAudioVideo_RX(VideoCallTypeMT_Type p_VideoCallType, template (omit) VideoCallMT_MessageRX_Type p_MessageType := omit) runs on IMS_PTC return template (present) SDP_Message { var template (present) SDP_attribute_rtpmap_codec v_AudioCodec; var template (present) SDP_attribute_list v_SDP_MediaAttributes_Audio; var template (present) SDP_attribute_list v_SDP_MediaAttributes_Video; var template (present) SDP_attribute_list v_SDP_PrecondionAttributes_Audio := {}; // @sic R5-170782 sic@ var template (present) SDP_attribute_list v_SDP_PrecondionAttributes_Video := {}; // @sic R5-170782 sic@ var template (present) SDP_attribute_list v_SDP_PrecondionAttributes_Final := cr_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory); /* preconditions when resources are fully available */ var charstring v_FmtAudio; var charstring v_FmtVideo := tsc_SDP_FmtVideo; /* @sic R5-197050: tsc_SDP_FmtVideo instead of fl_IMS_BuildSDP_FmtVideo sic@ */ var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); v_AudioCodec := cr_RTPMAP_AMR_16000; // @sic R5-168058; R5-168142 sic@ v_FmtAudio := f_SDP_AudioCodec_GetFmt(AMR_16000); /* @sic R5-197050: fix mapping of fmt values sic@ */ if (not v_IsWLAN and match(Response183, p_MessageType)) { // i.e. 183 Session Progress (C.26 step 4 or 17.2 step 3) v_SDP_PrecondionAttributes_Video := cr_SDP_PrecondionAttributes((c_none, c_sendrecv), c_none, c_mandatory, c_mandatory); v_SDP_PrecondionAttributes_Video := f_SDP_Attributes_Add_RX(v_SDP_PrecondionAttributes_Video, cr_SDP_Attribute_conf_qos); if (p_VideoCallType == SetupVideoCall) { // C.26 step 4: same preconditions for video and audio v_SDP_PrecondionAttributes_Audio := v_SDP_PrecondionAttributes_Video; } else { // 17.2 step 3: audio is there already v_SDP_PrecondionAttributes_Audio := v_SDP_PrecondionAttributes_Final; } } else { // WLAN or 200 OK v_SDP_PrecondionAttributes_Audio := v_SDP_PrecondionAttributes_Final; v_SDP_PrecondionAttributes_Video := v_SDP_PrecondionAttributes_Final; } v_SDP_MediaAttributes_Audio := { cr_SDP_Attribute_rtpmap(v_FmtAudio, v_AudioCodec), cr_SDP_Attribute_fmtp(v_FmtAudio) }; v_SDP_MediaAttributes_Video := { cr_SDP_Attribute_rtpmap(v_FmtVideo, cr_RTPMAP_H264_90000), cr_SDP_Attribute_fmtp(v_FmtVideo, cr_Video_Fmtp_DefaultParametersMT) /* @sic R5s141329 - MCC160 implementation sic@ @sic R5-150703: same parameters for C.26 and 17.2 sic@ */ }; return f_IMS_BuildSDP_AudioVideo_RX(-, cr_SDP_Media_Audio(v_FmtAudio), v_SDP_MediaAttributes_Audio, v_SDP_PrecondionAttributes_Audio, cr_SDP_Media_Video(v_FmtVideo), v_SDP_MediaAttributes_Video, v_SDP_PrecondionAttributes_Video); }

## Change 13 – f_IMS_CC_MO_SpeechCall

function f_IMS_CC_MO_SpeechCall(IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR, template (omit) IMS_SDP_AudioCodec_Type p_AudioCodecToBeSelected := omit, boolean p_PreconditionsRequired := true) runs on IMS_PTC return INVITE_Request { var IMS_CC_MO_SpeechCallInfo_Type v_MO_SpeechCallInfo; /* @sic R5-197039, R5-197040: IMS_CC_MO_SpeechCallInfo_Type instead of IMS_InviteRequestWithSdp_Type sic@ */ var INVITE_Request v_InviteRequest; var IMS_SDP_AudioCodec_Type v_AudioCodecToBeSelected; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); if (v_IsWLAN) { v_InviteRequest := f_IMS_MOCallSetup_AnnexC21aC21cC25aC25b(IPCAN_MO_SpeechCall, p_AudioCodecList, p_PreconditionsRequired); } else { if (ispresent(p_AudioCodecToBeSelected)) { v_AudioCodecToBeSelected := valueof(p_AudioCodecToBeSelected); } else { v_AudioCodecToBeSelected := p_AudioCodecList[0]; } v_MO_SpeechCallInfo := f_IMS_MOCallSetup_AnnexC21C21b(p_AudioCodecList, v_AudioCodecToBeSelected); v_InviteRequest := v_MO_SpeechCallInfo.InviteRequestWithSdp.Invite; } return v_InviteRequest; }

## Change 14 – f_IMS_MTCallwithTIR

function f_IMS_MTCallwithTIR(IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR) runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_Response_180_183_WithSdp_Type v_Response_180_183_WithSdp; // @sic R5s180436: IMS_Response_180_183_WithSdp_Type instead of v_OptionalResponseWithSdp sic@ var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var IMS_Response_Type v_SessionProgress183; var template (present) SDP_Message v_SDP_Message; var MessageHeader v_MessageHeader_SessionProgress; var template (omit) MessageHeader v_MessageHeader_Ringing := omit; var MessageHeader v_MessageHeader_200OK; var SipUrl v_ContactUrl; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); var boolean v_PreconditionsRequired := true; var IMS_SDP_AudioCodec_Type v_AudioCodecToBeSelected := p_AudioCodecList[0]; timer t_Dummy; // according to the test case prose of G.15.4 and H.15.4 the 180 Ringing seems to be optional f_IMS_TestBody_Set(true); f_UT_MMI_ActivateIMS_TIR(MMI); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(p_AudioCodecList, v_PreconditionsRequired); if (v_IsWLAN) { // @siclog "Step 1" siclog@ IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite)); // @siclog "Step 2 - 5" siclog@ v_SDP_Message := f_IMS_BuildSDP_AnswerAudio_RX(FinalAnswer, v_AudioCodecToBeSelected, v_PreconditionsRequired); v_Response_180_183_WithSdp := f_IMS_MTCallSetup_AnnexC11aC11cC26aC26b_Steps2_5(t_Dummy, v_InviteRequestWithSdp, v_SDP_Message); f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MT_SpeechCall); /* trigger IPCAN to do the routing for RTP/RTCP loopback */ if (ispresent(v_Response_180_183_WithSdp.Response180)) { v_MessageHeader_Ringing := v_Response_180_183_WithSdp.Response180.msgHeader; } if(ispresent (v_Response_180_183_WithSdp.Response183)){ // @sic R5-184056 sic@ v_MessageHeader_SessionProgress := v_Response_180_183_WithSdp.Response183.msgHeader; fl_MessageHeader_CheckPrivacy(v_MessageHeader_SessionProgress); } } else { // @siclog "Step 1 - 4" siclog@ v_ResponseWithSdp := f_IMS_MTCallSetup_AnnexC11_Steps1_4(v_InviteRequestWithSdp, v_AudioCodecToBeSelected, v_PreconditionsRequired); v_SessionProgress183 := v_ResponseWithSdp.Response; v_MessageHeader_SessionProgress := v_SessionProgress183.msgHeader; fl_MessageHeader_CheckPrivacy(v_MessageHeader_SessionProgress); // Step 5 - 6. Send PRACK, Receive 200 OK f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequestWithSdp.Invite, v_SessionProgress183.msgHeader); /* @sic R5-181141: steps 5-6 before sending the trigger sic@ */ // Establish Dedicated EPS bearer // See TS. 36.508 EUTRA/EPS signalling for IMS MT speech call Table 4.5A.7.3-1 Step 9-12 f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MT_SpeechCall); // Step 7 - 11. Send UPDATE, Receive 200 OK, Receive optional 180 Ringing and send PRACK if requested v_MessageHeader_Ringing := f_IMS_MTCallSetup_AnnexC11_Steps7_11(v_InviteRequestWithSdp, v_ResponseWithSdp, v_AudioCodecToBeSelected); } if (ispresent(v_MessageHeader_Ringing)) { fl_MessageHeader_CheckPrivacy(valueof(v_MessageHeader_Ringing)); } // Step 11A - Make UE accept the speech AMR offer f_UT_AnswerCall(MMI); // Step 12. Receive 200 OK, Send ACK v_ResponseWithSdp := f_IMS_MTCallSetup_Receive200OK_SendACK(v_InviteRequestWithSdp); v_MessageHeader_200OK := v_ResponseWithSdp.Response.msgHeader; fl_MessageHeader_CheckPrivacy(v_MessageHeader_200OK); // MT Call is now established f_Delay(3.0); /* @sic R5s130510 change 1; draft prose CR sic@ */ // Step 14-15. SS sends BYE to release the call; UE sends 200 OK for the BYE request and ends the call // @siclog "Step 14 -15" siclog@ v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); //@sic R5s180433 sic@ f_IMS_CallReleaseMT(v_ContactUrl); f_UT_MMI_DeactivateIMS_TIR(MMI); f_IMS_TestBody_Set(false); }

## Change 15 – fl_IMS_BuildSDP_MOCallAudioVideo_ReleaseVideo

function fl_IMS_BuildSDP_MOCallAudioVideo_ReleaseVideo(SDP_Message p_OriginalSdpOffer) runs on IMS_PTC return template (present) SDP_Message { var charstring v_FmtAudio; var charstring v_FmtVideo; var template (present) SDP_attribute_rtpmap_codec v_RtpmapAttrAudio; var template (present) SDP_attribute_list v_SDP_MediaAttributes_Audio; var template (present) SDP_attribute_list v_SDP_MediaAttributes_Video; var template (present) SDP_attribute_list v_SDP_PrecondionAttributes; v_FmtAudio := f_SDP_MediaDescr_GetAMR_WB_16000_1(p_OriginalSdpOffer.media_list[0]); /* @sic R5s170012/R5-171474: f_SDP_MediaDescr_GetAMR_WB_16000_1 sic@ */ v_RtpmapAttrAudio := cr_RTPMAP_AMR_16000; /* @sic R5s170012/R5-171474: cr_RTPMAP_AMR_16000 sic@ */ v_SDP_PrecondionAttributes := cr_SDP_PrecondionAttributes(c_sendrecv, c_sendrecv, c_mandatory, (c_optional, c_mandatory)); /* @sic R5s160604 change 3 sic@ */ v_FmtVideo := f_SDP_MediaDescr_GetH264_90000(p_OriginalSdpOffer.media_list[1]); v_SDP_MediaAttributes_Audio := { cr_SDP_Attribute_rtpmap(v_FmtAudio, v_RtpmapAttrAudio), cr_SDP_Attribute_fmtp(v_FmtAudio) }; v_SDP_MediaAttributes_Video := { cr_SDP_Attribute_rtpmap(v_FmtVideo, cr_RTPMAP_H264_90000), cr_SDP_Attribute_fmtp(v_FmtVideo) }; return f_IMS_BuildSDP_AudioVideo_RX(cr_SDP_Time_Any, cr_SDP_Media_Audio, v_SDP_MediaAttributes_Audio, v_SDP_PrecondionAttributes, cr_SDP_Media(c_video, 0, c_rtpAvpf, cr_SDP_FmtList_AtLeastOne(v_FmtVideo)), /* port number == 0 => release video stream @sic R5s160604: RTP/AVPF sic@ */ v_SDP_MediaAttributes_Video, v_SDP_PrecondionAttributes); }

## Change 16 – fl_IMS_MOCall_ReleaseVideo_Invite

function fl_IMS_MOCall_ReleaseVideo_Invite(SDP_Message p_OriginalSdpOffer) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { var IMS_DATA_REQ v_IMS_INVITE_REQ; var SDP_Message v_SDP_Message; var boolean v_A11 := false; /* @sic R5-176371: A11 - INVITE for creating a video call => false in case of call release sic@ */ var boolean v_PreconditionSupported := true; var OptionTag_List v_AdditionalOptionTagsForSupported := f_IMS_AdditionalOptionTagsForSupported(v_PreconditionSupported); var template (present) SDP_Message v_SDP_MessageTemplate := fl_IMS_BuildSDP_MOCallAudioVideo_ReleaseVideo(p_OriginalSdpOffer); v_IMS_INVITE_REQ := f_IMS_INVITE_ReceiveRequest(A_2_1_A5, v_AdditionalOptionTagsForSupported, -, -, -, v_A11); /* @sic R5-176209: v_A12 removed sic@ */ v_SDP_Message := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_IMS_INVITE_REQ.Request.Invite.messageBody, v_SDP_MessageTemplate); return f_IMS_InviteRequestWithSdpMO(v_IMS_INVITE_REQ, v_SDP_Message); }

## Change 17 – f_IMS_MOSpeechAddVideoRemoveVideo

function f_IMS_MOSpeechAddVideoRemoveVideo() runs on IMS_PTC { var template (present) SipUrl v_CalleeContactUri := f_SIP_BuildSipUri_RX(px_IMS_CalleeContactUri); var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) SDP_Message v_SdpAnswer; var template (value) MessageBody v_MessageBody; var template (value) MessageHeader v_MessageHeader_Response; var template (present) MessageHeader v_MessageHeader_Request; var charstring v_CallIdentification; var IMS_SDP_AudioCodecList_Type v_AudioCodecList; var boolean v_PreconditionsRequired; var IMS_PreconditionIndication_Type v_PreconditionIndication; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR; v_PreconditionIndication := PreconditionRequired; v_PreconditionsRequired := true; // Initial conditions - Setup MO speech call f_IMS_CC_StartCall(IPCAN_MO_SpeechCall, -, normalCall_ReturnId); // @sic R5s160855 sic@ f_IMS_CC_MO_SpeechCall(v_AudioCodecList, -, v_PreconditionsRequired); // MO voice call according v_CallIdentification := f_UT_ReceivedIMSCallId(MMI); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); // @sic R5s160855 sic@ // @siclog "Step 2" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_Video_Step2(AddVideo); v_InviteRequest := v_InviteRequestWithSdp.Invite; if (v_IsWLAN) { // G.17.1, H.17.1 // @siclog "Step 3" siclog@ f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp); // Send 100 Trying f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_AddVideo); // @siclog "Step 4 - 6 (H.17.1): Void" siclog@ // @siclog "Step 4 (G.17.1)" siclog@ // @siclog "Step 7 (H.17.1)" siclog@ v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudioVideo_TX(AddVideo, v_InviteRequestWithSdp.SdpOffer); // same as for C.25a/C.25b v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, cs_ContentTypeSDP, -, -, v_PreconditionIndication); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader_Response, cs_MessageBody_SDP(v_SdpAnswer)))); // @siclog "Step 5 (G.17.1)" siclog@ // @siclog "Step 8 (H.17.1)" siclog@ v_MessageHeader_Request := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine200); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeContactUri, v_MessageHeader_Request))); } else { // 17.1 // @siclog "Step 3 - 4" siclog@ v_MessageHeader_Response := f_IMS_MOCallSetup_Video_Step3_4(AddVideo, v_InviteRequestWithSdp); f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_AddVideo); // @siclog "Step 5 - 8" siclog@ f_IMS_MOCallSetup_Video_Steps5_8(AddVideo, v_InviteRequestWithSdp, v_MessageHeader_Response); // @siclog "Step 9 - 10" siclog@ f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); } f_IMS_IPCAN_WaitForTrigger(IPCAN); // Wait for trigger from IPCAN to ensure that radio resources have really been established // @siclog "Step 10a (17.1)" siclog@ // @siclog "Step 6 (G.17.1)" siclog@ // @siclog "Step 9 (H.17.1)" siclog@ f_UT_IMS_RemoveVideoMedia(MMI, v_CallIdentification); // @siclog "Step 11 (17.1)" siclog@ // @siclog "Step 7 (G.17.1)" siclog@ // @siclog "Step 10 (H.17.1)" siclog@ v_InviteRequestWithSdp := fl_IMS_MOCall_ReleaseVideo_Invite(v_InviteRequestWithSdp.SdpOffer); v_InviteRequest := v_InviteRequestWithSdp.Invite; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); // @siclog "Step 12 (17.1)" siclog@ // @siclog "Step 8 (G.17.1)" siclog@ // @siclog "Step 11 (H.17.1)" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine100, f_IMS_InviteResponse_100_MessageHeaderTX(v_InviteRequest)))); // @siclog "Step 12a (17.1)" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_ReleaseVideo); // @siclog "Step 13 (17.1)" siclog@ // @siclog "Step 9 (G.17.1)" siclog@ // @siclog "Step 12 (H.17.1)" siclog@ v_MessageBody := cs_MessageBody_SDP(f_IMS_BuildSDP_CommonResponseToOfferTX(v_InviteRequestWithSdp.SdpOffer)); v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, cs_ContentTypeSDP, -, -, v_PreconditionIndication); // @sic R5s170344: cs_ContentTypeSDP, PreconditionRequired sic@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader_Response, v_MessageBody))); // @sic R5s170344: v_MessageBody sic@ // @siclog "Step 14 (17.1)" siclog@ // @siclog "Step 10 (G.17.1)" siclog@ // @siclog "Step 13 (H.17.1)" siclog@ v_MessageHeader_Request := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine200); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeContactUri, v_MessageHeader_Request))); f_IMS_IPCAN_WaitForTrigger(IPCAN); // Wait for trigger from IPCAN to ensure that radio resources have been released // @siclog "Step 15 - 16 (17.1)" siclog@ // @siclog "Step 11 - 12 (G.17.1)" siclog@ // @siclog "Step 14 - 15 (H.17.1)" siclog@ f_IMS_CallReleaseMO(v_InviteRequest); f_IMS_TestBody_Set(false); }

## Change 18 – f_IMS_MTSpeechAddVideoRemoveVideo

function f_IMS_MTSpeechAddVideoRemoveVideo() runs on IMS_PTC { var IMS_CC_MT_SpeechCallInfo_Type v_SpeechCallInfo; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var IMS_Response_Type v_SessionProgress183; var template (present) IMS_Response_Type v_ExpectedSessionProgress183; var template (value) INVITE_Request v_InviteRequest; var template (value) SDP_Message v_SdpOffer; var template (present) SDP_Message v_SdpAnswer; var template SDP_Message v_SdpAnswer200OK; var template (value) MessageHeader v_MessageHeader_Invite; var template (value) MessageHeader v_MessageHeader_Update; var template (value) MessageHeader v_MessageHeader_Ack; var template (present) MessageHeader v_MessageHeader_Response; var template (omit) SDP_attribute v_SDP_Attribute; var template (omit) IMS_DATA_RSP v_IMS_DATA_RSP_Optional; var SipUrl v_ContactUrl; var InternetProtocol_Type v_Protocol; var SDP_media_desc v_MediaDesc; var IMS_SDP_AudioCodecList_Type v_AudioCodecList; var boolean v_PreconditionsRequired; var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN(); timer t_Timer := 5.0; f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); // @sic R5s190003, R5s190006 sic@ if (v_IsWLAN) { v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR; v_PreconditionsRequired := true; v_SpeechCallInfo := f_IMS_MTCallSetup_AnnexC11aC11c(v_AudioCodecList, v_PreconditionsRequired); } else { v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR; v_SpeechCallInfo := f_IMS_MTCallSetup_AnnexC11C11b(v_AudioCodecList); } v_ContactUrl := v_SpeechCallInfo.ContactUrl; f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ // Step 1 SS sends re-INVITE with second SDP offer to add video. v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(AddVideo, Invite, v_ContactUrl); /* @sic R5-172930: v_FmtAudio, v_FmtVideo removed sic@ */ v_InviteRequest := v_InviteRequestWithSdp.Invite; if (v_IsWLAN) { // @siclog "Step 2" siclog@ v_Protocol := v_InviteRequestWithSdp.RoutingInfo.Protocol; v_SdpAnswer200OK := f_IMS_BuildSDP_MTCallAudioVideo_RX(AddVideo); //@siclog "Step 2A" siclog@ v_ExpectedSessionProgress183 := cr_Response(c_statusLine183, f_IMS_InviteResponse_183_MessageHeaderRX(v_InviteRequest), cr_MessageBody_SDP); //Start timer t_Timer.start; //not in Prose!! v_IMS_DATA_RSP_Optional := f_IMS_ReceiveResponse_OptionalTrying_Timeout(t_Timer, v_InviteRequest, v_Protocol, v_ExpectedSessionProgress183); t_Timer.stop; //@siclog "Step 2B-2C" siclog@ if (isvalue(v_IMS_DATA_RSP_Optional)) { // @sic R5s190003 sic@ v_SessionProgress183 := valueof(v_IMS_DATA_RSP_Optional.Response); f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_SessionProgress183.messageBody, v_SdpAnswer200OK); f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequest, v_SessionProgress183.msgHeader); v_SdpAnswer200OK := omit; } f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Step 3 - 5: Void (H.17.2)" siclog@ } else { // @siclog "Step 2: Void" siclog@ // @siclog "Step 3" siclog@ v_ResponseWithSdp := f_IMS_MTCallSetup_Video_Receive183(AddVideo, v_InviteRequestWithSdp); /* @sic R5-172930: v_FmtAudio, v_FmtVideo removed sic@ */ v_SessionProgress183 := v_ResponseWithSdp.Response; // @siclog "Step 4 - 5" siclog@ f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequest, v_SessionProgress183.msgHeader); f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); /* @sic R5-181141: trigger moved after step 4/5 sic@ */ f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 6" siclog@ v_MediaDesc := valueof(f_IMS_SDP_MediaList_GetMediaStream(v_ResponseWithSdp.SdpMessage.media_list, c_video)); /* @sic R5s180304: f_IMS_SDP_MediaList_GetMediaStream sic@ */ v_SDP_Attribute := f_SDP_AttributeList_GetAttribute(v_MediaDesc.attributes, cr_SDP_Attribute_curr_qos(c_local, ?)); v_SdpOffer := f_IMS_BuildSDP_MTCallAudioVideo_TX(AddVideo, Update, valueof(v_SDP_Attribute.curr.direction)); /* @sic R5-172930: v_FmtAudio, v_FmtVideo removed sic@ */ v_MessageHeader_Update := f_IMS_UpdateRequest_MessageHeaderTX(v_InviteRequest, PreconditionSupported); /* @sic R5-160795: PreconditionSupported sic@ */ v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); /* @sic R5s160540 change 1.5 sic@ */ IMS_Client.send(cas_IMS_Update_Request(cs_IMS_RoutingInfo(v_Protocol), cs_UPDATE_Request(v_ContactUrl, v_MessageHeader_Update, cs_MessageBody_SDP(v_SdpOffer)))); // @siclog "Step 7" siclog@ v_SdpAnswer := f_IMS_BuildSDP_MTCallAudioVideo_RX(AddVideo, Response200); /* @sic R5-172930: v_FmtAudio, v_FmtVideo removed sic@ */ v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Update, cr_ContentTypeSDP); /* @sic R5-160795, R5-162930: precondition required sic@ */ v_IMS_DATA_RSP_Optional := f_IMS_ReceiveResponse(v_MessageHeader_Update, v_Protocol, cr_Response(c_statusLine200, v_MessageHeader_Response, cr_MessageBody_SDP)); f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_IMS_DATA_RSP_Optional.Response.messageBody, v_SdpAnswer); v_SdpAnswer200OK := omit; } // @siclog "Step 7A (17.2)" siclog@ // @siclog "Step 2D (G.17.2)" siclog@ // @siclog "Step 6 (17.2)" siclog@ f_UT_AnswerCall(MMI); // @siclog "Step 8 - 9 (17.2)" siclog@ // @siclog "Step 3 - 4 (G.17.2)" siclog@ // @siclog "Step 7 - 8 (H.17.2)" siclog@ // The UE responds to the re-INVITE with a 200 OK final response. // The SS acknowledges the receipt of 200 OK for the re-INVITE. f_IMS_MTCallSetup_Receive200OK_SendACK(v_InviteRequestWithSdp, v_SdpAnswer200OK); // @siclog "Step 10 (17.2)" siclog@ // @siclog "Step 5 (G.17.2)" siclog@ // @siclog "Step 9 (H.17.2)" siclog@ // SS sends a re-INVITE with a SDP offer indicating that the video component is removed from the call v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(RemoveVideo, Invite, v_ContactUrl); /* @sic R5-172930: v_FmtAudio, v_FmtVideo removed sic@ */ // @siclog "Step 10A - 11 (17.2)" siclog@ // @siclog "Step 6 - 7 (G.17.2)" siclog@ // @siclog "Step 10 - 11 (H.17.2)" siclog@ v_MessageHeader_Invite := v_InviteRequestWithSdp.Invite.msgHeader; v_MessageHeader_Response := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeader_Invite, cr_ContentTypeSDP); /* @sic R5-160795, R5-162930: cr_ContentTypeSDP, precondition required sic@ */ f_IMS_ReceiveResponse_OptionalTrying(v_InviteRequestWithSdp.Invite, v_InviteRequestWithSdp.RoutingInfo.Protocol, cr_Response(c_statusLine200, v_MessageHeader_Response, cr_MessageBody_SDP)); /* @sic R5s160842, R5s170013 sic@ */ // @siclog "Step 11A (17.2)" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_ReleaseVideo); // @siclog "Step 12 (17.2)" siclog@ // @siclog "Step 8 (G.17.2)" siclog@ // @siclog "Step 12 (H.17.2)" siclog@ v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite); IMS_Client.send(cas_IMS_Ack_Request(cs_IMS_RoutingInfo(v_InviteRequestWithSdp.RoutingInfo.Protocol), cs_ACK_Request(v_ContactUrl, v_MessageHeader_Ack))); f_IMS_IPCAN_WaitForTrigger(IPCAN); // Wait for trigger from IPCAN to ensure that radio resources have been released // @siclog "Step 13: Void (17.2)" siclog@ // @siclog "Step 14 - 16 (17.2)" siclog@ // @siclog "Step 9 - 10 (G.17.2)" siclog@ // @siclog "Step 13 - 14 (H.17.2)" siclog@ f_IMS_CallReleaseMT(v_ContactUrl); f_IMS_TestBody_Set(false); }

## Change 19 – f_IMS_PTC_Security_GetHashAlgorithm

function f_IMS_PTC_Security_GetHashAlgorithm() runs on IMS_PTC return charstring { var charstring v_Algorithm; select (vc_IMS_Global.ImsInfo.SecurityScheme) { case (IMS_Security) { v_Algorithm := tsc_AKAv1_MD5; } case else { v_Algorithm := ""; // dummy } } return v_Algorithm; }

## Change 20 – f_IMS_MOCallSetup_AnnexC21C25_Steps5_8

function f_IMS_MOCallSetup_AnnexC21C25_Steps5_8(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, template (value) MessageHeader p_MessageHeader_Prev183Response, boolean p_GeolocationInfoAvailable := false, template SDP_Message p_SDP_Message_Step5or7 := omit, template (omit) SDP_Message p_PrevSDP_Message := omit, IMS_CallType_Type p_CallType := NORMAL_CALL, boolean p_ResourcesEstablished := false, IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000, IMS_PreconditionIndication_Type p_PreconditionIndicationStep5and7 := PreconditionRequired) runs on IMS_PTC return SDP_Message { var SipUrl v_ContactUrl := f_MessageHeader_GetContactSipUrl(p_MessageHeader_Prev183Response); /* @sic R5s130333 change 8.2 - acc. to A.2.4, A.2.5 same as in previous response sic@ */ var INVITE_Request v_InviteRequest := p_InviteRequestWithSdp.Invite; var SDP_Message v_SDP_Message := p_InviteRequestWithSdp.SdpOffer; /* @sic R5w140112 sic@ */ var template (present) SDP_Message v_SDP_Message_Step5or7; var boolean v_MediaConfigured := false; var IMS_DATA_REQ v_IMS_DATA_REQ; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) MessageHeader v_MessageHeader_Prack; var template (omit) MessageBody v_MessageBodyTx := omit; var template (omit) ContentType v_ContentType := omit; var PRACK_Request v_PrackRequest; var SDP_Message v_SDP_LatestOffer; if (isvalue(p_PrevSDP_Message)) { /* e.g. in case of C.27 the INVITE's SDP message is not the latest SDP sent by the UE */ v_SDP_Message := valueof(p_PrevSDP_Message); } if (not ispresent(p_SDP_Message_Step5or7)) { v_SDP_Message_Step5or7 := f_IMS_BuildSDP_UpdatedOfferAudio_RX(v_SDP_Message, p_ResourcesEstablished, p_AudioCodec); /* => use speech per default */ } else { v_SDP_Message_Step5or7 := p_SDP_Message_Step5or7; /* In case of video the SDP message needs to be handed over NOTE: compiler warning cannot be avoided here */ } // Step 5. Receive PRACK v_MessageHeader_Prack := f_IMS_PrackRequest_MessageHeaderRX(v_InviteRequest, p_MessageHeader_Prev183Response); v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Prack_Request(cr_PRACK_Request(v_ContactUrl, v_MessageHeader_Prack, cr_MessageBody_SDP ifpresent))); v_PrackRequest := v_IMS_DATA_REQ.Request.Prack; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); // fl_IMS_Params_CheckSigComp(v_PrackRequest.msgHeader.route.routeBody[0].rrParam); // acc. to 34.229-1 cl. 13.2.4 step 4 (PRACK) "comp=sigcomp" is optional for Via and Route if (ispresent(v_PrackRequest.messageBody)) { f_IMS_MessageHeader_CheckPreconditionIndicationForOffer(v_PrackRequest.msgHeader, p_PreconditionIndicationStep5and7); /* @sic R5-160794 sic@ */ v_SDP_LatestOffer := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_PrackRequest.messageBody, v_SDP_Message_Step5or7); // @sic R5w140112 sic@ // check session version increased @sic R5-150702: done in f_IMS_SIP_DecodeMatchAndCheckSDP sic@ */ v_ContentType := cs_ContentTypeSDP; v_MessageBodyTx := cs_MessageBody_SDP(f_IMS_BuildSDP_UpdatedAnswer_TX(v_SDP_LatestOffer)); /* @sic R5w140112 sic@ */ v_MediaConfigured := true; } // Step 6. Send 200 OK IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, f_IMS_OtherResponse_200_MessageHeaderTX(v_PrackRequest.msgHeader, v_ContentType, -, -, PreconditionRequired, p_CallType), /* @sic R5s150237 change 2 sic@ @sic R5s150692 change 16 sic@ @sic R5-160794, R5-162925, R5-162927, R5-162928, R5-162929: PreconditionRequired sic@ */ v_MessageBodyTx))); // Step 7 Optional. Receive Update, send 200 OK // Note: UE will send Update if Media is not yet configured if (not v_MediaConfigured) { v_SDP_LatestOffer := f_IMS_MOCallSetup_ReceiveUpdateSend200OK(v_InviteRequest, v_ContactUrl, v_SDP_Message_Step5or7, p_GeolocationInfoAvailable, p_CallType, p_PreconditionIndicationStep5and7); /* @sic R5s201620 sic@ */ } return v_SDP_LatestOffer; }

## Change 21 – f_IMS_InviteRequest_MessageHeaderRX

function f_IMS_InviteRequest_MessageHeaderRX(template (present) SipUrl p_ExpectedCalleeUri, OptionTag_List p_AdditionalOptionTagsForSupported := tsc_OptionTagList_Empty, INVITE_A_2_1_Context_Type p_Context := A_2_1_A4, boolean p_A3 := pc_MultimediaTelephonyService, /* @sic R5s130510 additional change 1: tsc_IMS_MTSI replaced by pc_MultimediaTelephonyService sic@ */ boolean p_A8 := false, boolean p_A11 := false) runs on IMS_PTC return template (present) MessageHeader { var boolean v_IsGiba := f_IMS_PTC_SecurityScheme_IsGiba(); // => A1/A2 var template (present) MessageHeader v_MessageHeader_Invite := cr_MessageHeader_Dummy; var template (present) RouteBody_List v_RouteBodyList; var IMS_ProtectedPorts_Type v_Protected; var template integer v_Port_us_Contact := *; var template integer v_Port_ps_Route := *; var template DisplayName v_DisplayName := *; var template (present) SipUrl v_SipUrlFrom := ?; /* per default any SIP URI is accepted in the From header in the first place; further checks may be applied in f_IMS_A_2_1_Invite_CommonChecks (depending on the conditions acc. to A.2.1 */ var template (present) charstring v_Pcscf; var charstring v_MediaType := c_sdpApplication; var template (present) CallId v_CallId; var template (present) integer v_CSeqValue := ?; var template (present) From v_From; var template (present) To v_To; var template AcceptBody_List v_AcceptBodyList := {}; var OptionTag_List v_OptionTag_List := tsc_OptionTagList_100rel & p_AdditionalOptionTagsForSupported; var boolean v_GRUUsInSIP := pc_IMS_GRUUsInSIP; var boolean v_A4 := (p_Context == A_2_1_A4); // INVITE creating a dialog @sic R5-166280 sic@ var boolean v_A5 := (p_Context == A_2_1_A5); // re-INVITE within a dialog @sic R5-166280 sic@ var boolean v_A6 := (p_Context == A_2_1_A6); // emergency session in case of no registration @sic R5-166280 sic@ var boolean v_A19 := (p_Context == A_2_1_A19); // emergency session in case of no registration (GIBA) @sic R5-174545 sic@ var boolean v_A26 := pc_IMS_Session_Timer; // UE supports Session Timer @sic R5-192321 sic@ if (pc_IMS_CS_PS_SRVCCMidCall or pc_IMS_CS_PS_SRVCCAlert) { // @sic R5-145746 sic@ v_OptionTag_List := v_OptionTag_List & tsc_OptionTagList_norefersub; } if (v_A4 and v_A26) { /* @sic R5-192321 sic@ NOTE: A4, A6, A7 and A19 are mutual exclusive; use of A20, A21 (eCall) is unclear */ v_OptionTag_List := v_OptionTag_List & tsc_OptionTagList_timer; /* @sic R5-173024: support of session timer acc. RFC 4028 sic@ @sic R5-180230: end of grace period sic@ */ } if (v_A6 or v_A19) { // INVITE for creating an emergency session in case of no registration // Route, Contact: ports to be used v_Port_us_Contact := ?; v_Port_ps_Route := tsc_IMS_PortNumber_5060; if (not v_A19) { /* @sic R5-191118 sic@ */ // From: Display v_DisplayName := cr_DisplayName("Anonymous"); /* @sic R5s140005, R5-140901, R5w140111 sic@ */ } v_GRUUsInSIP := false; /* @sic R5s160366: no GRUU in case of no registration sic@ */ } else { // Route, Contact: ports to be used if (not v_IsGiba or (p_Context == A_2_1_A7)) { v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); v_Port_us_Contact := v_Protected.Port_us; v_Port_ps_Route := v_Protected.Port_ps; } else { /* @sic R5s160366: GIBA sic@ */ v_Port_us_Contact := ?; v_Port_ps_Route := tsc_IMS_PortNumber_5060 ifpresent; } } if (v_A5) { /* re-invite @sic R5s140017 MCC160 implementation sic@ */ v_RouteBodyList := f_IMS_RouteSet_RouteBodyList_RX(); v_CallId := f_IMS_PTC_ImsInfo_DialogGetCallId(); v_From := f_IMS_RequestInDialog_FromHeaderRX(); v_To := f_IMS_RequestInDialog_ToHeaderRX(); v_CSeqValue := f_IMS_RequestInDialog_CSeqValueRX(); /* @sic R5s140235, R5s140385 change 3 MCC160 implementation sic@ */ } else { /* normal case: no re-invite */ // route body acc. to conditions in A.2.1 v_Pcscf := ?; /* @sic R5s140934 change 6: P-CSCF address is checked later on (f_IMS_A_2_1_Invite_CommonChecks -> f_IMS_MessageHeader_Request_CheckRoute) sic@ */ v_RouteBodyList[0] := cr_RouteBody(*, cr_SipUri_HostPort_lr(v_Pcscf, v_Port_ps_Route)); /* @sic R5s140873: v_Pcscf; R5s140934 change 6 sic@ @sic R5-155174: "loose routing" parameter mandatory for any kind of call (i.e. emergency call too, in contrast to R5s150817) sic@ */ if (not match(p_Context, (A_2_1_A6, A_2_1_A7, A_2_1_A19))) { // => no emergency call v_RouteBodyList[1] := cr_RouteBody(*, cr_SipUri_HostPort_lr(tsc_IMS_Scscf)); } if (p_A8) { v_MessageHeader_Invite.geolocation := cr_GeolocationAny; v_MessageHeader_Invite.geolocationRouting := cr_GeolocationRoutingSupported; // @sic R5-134798 sic@ v_MediaType := tsc_MimeType_MultipartyMixed; } v_CallId := cr_CallId(?); v_From := cr_From(v_DisplayName, v_SipUrlFrom); v_To := cr_ToDef(p_ExpectedCalleeUri); } v_MessageHeader_Invite.via := f_IMS_SipRequest_ViaRX(); /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of FQDN or IP address @sic R5-133358: rport is optional for tcp sic@ */ v_MessageHeader_Invite.route := cr_Route(v_RouteBodyList); v_MessageHeader_Invite.fromField := v_From; /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of FQDN or IP address, check against P-Preferred-Identity header */ v_MessageHeader_Invite.toField := v_To; /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of tag */ v_MessageHeader_Invite.callId := v_CallId; /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: value acc. to A4 or A5 */ v_MessageHeader_Invite.cSeq := cr_CseqDef(v_CSeqValue, "INVITE"); /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: A5 */ v_MessageHeader_Invite.supported := cr_Supported(v_OptionTag_List); v_MessageHeader_Invite.contact := cr_Contact(f_Contact_SipUri_HostPortRX(?, v_Port_us_Contact, v_GRUUsInSIP)); /* additional checks in f_IMS_A_2_1_Invite_CommonChecks: check of FQDN or IP address, check of feature-param for A3, A6, A10, A12 @sic R5s150721, R5-153760: cr_SipUri_HostPort replaced by f_Contact_SipUri_HostPortRX sic@ @sic R5s160366: v_GRUUsInSIP sic@ */ v_MessageHeader_Invite.contentType := cr_ContentType(v_MediaType); v_MessageHeader_Invite.maxForwards := cr_MaxForwardsDef; // additional headers for specific conditions: if (v_IsGiba or v_A6 or v_A19) { v_MessageHeader_Invite.securityVerify := omit; if (v_A6 or v_A19) { v_MessageHeader_Invite.require := omit; v_MessageHeader_Invite.proxyRequire := omit; } } else { // -> A1 or A7 v_MessageHeader_Invite.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); if (not v_A5) { /* @sic R5s140017 change 9.3 sic@ */ v_MessageHeader_Invite.require := cr_RequireSecAgree; v_MessageHeader_Invite.proxyRequire := cr_ProxyRequireSecAgree; } } if (not v_IsGiba) { v_MessageHeader_Invite.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); } if (v_A4) { /* @sic R5s140017 change 9.3 sic@ */ v_AcceptBodyList := {cr_AcceptBody(c_sdpApplication), cr_AcceptBody(c_ims3gppApplication)}; } if (pc_IMS_CS_PS_SRVCCMidCall) { // @sic R5-145746 sic@ v_AcceptBodyList[lengthof(v_AcceptBodyList)] := cr_AcceptBody("application/vnd.3gpp.mid-call+xml"); // add another element in the array } if (pc_IMS_CS_PS_SRVCCAlert) { // @sic R5-145746 sic@ v_AcceptBodyList[lengthof(v_AcceptBodyList)] := cr_AcceptBody("application/vnd.3gpp.state-and-event-info+xml"); // add another element in the array v_MessageHeader_Invite.recvInfo := cr_RecvInfo_Def(tsc_IMS_InfoPackageType_3gpp); // @sic R5-145746, R5s150704, sic@ } if (lengthof(v_AcceptBodyList) > 0) { if (v_A5) { // @sic R5s150971 header optional if A5 sic@ v_MessageHeader_Invite.accept := cr_Accept(superset(all from v_AcceptBodyList)) ifpresent; } else { v_MessageHeader_Invite.accept := cr_Accept(superset(all from v_AcceptBodyList)); } } if (p_A8) { v_MessageHeader_Invite.geolocation := cr_GeolocationAny; // Checked in f_IMS_A_2_1_Invite_CommonChecks v_MessageHeader_Invite.geolocationRouting := cr_GeolocationRoutingAny; // Checked in f_IMS_A_2_1_Invite_CommonChecks @sic R5-133151 sic@ } if (p_Context == A_2_1_A7) { v_MessageHeader_Invite.pPreferredID := cr_PPreferredID(?); // Checked in f_IMS_A_2_1_Invite_CommonChecks } if (p_A3 and v_A4) { /* @sic R5s140263 change 5, modified by R5-142949 (-> A4 only) sic@ */ v_MessageHeader_Invite.pPreferredService := cr_PPreferredService(tsc_IMS_FeatureParamValue_3gpp_icsi_ref); /* @sic R5s140565 change 2 sic@ */ } if ((p_A3 and v_A4) or p_A11) { /* @sic R5-142949 sic@ */ v_MessageHeader_Invite.acceptContact := cr_AcceptContact; // Checked in f_IMS_A_2_1_Invite_CommonChecks (depending on conditions) } if (pc_EarlyMedia and not v_A5) { /* @sic R5-162934: condition A16 for A.2.1 sic@ @sic R5-166280: "not A5" sic@ */ v_MessageHeader_Invite.pEarlyMedia := cr_PEarlyMedia({"supported"}); } return v_MessageHeader_Invite; }

## Change 22 – f_IMS_ByeRequest_MessageHeaderRX

function f_IMS_ByeRequest_MessageHeaderRX(template (value) INVITE_Request p_InviteRequest, IMS_CallType_Type p_CallType := NORMAL_CALL, template (omit) IMS_PTC_DialogIndex_Type p_DialogIndex := omit, boolean p_IsEmergencyCall := false) runs on IMS_PTC return template (present) MessageHeader { var boolean v_A1 := f_IMS_PTC_SecurityScheme_IsImsSecurity(); var boolean v_A2 := f_IMS_PTC_SecurityScheme_IsGiba(); var boolean v_A6 := p_IsEmergencyCall and not f_IMS_PTC_ImsInfo_IsRegistered(); /* @sic R5-172934 sic@ */ var template (value) MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader; var template (present) MessageHeader v_MessageHeader_Bye := cr_MessageHeader_Dummy; var template (present) integer v_CSeqValue := f_IMS_RequestInDialog_CSeqValueRX(dialogRemote, p_DialogIndex); /* @sic R5s200320: f_IMS_RequestInDialog_CSeqValueRX instead of f_IMS_PTC_ImsInfo_CseqIncr sic@ */ var template (present) RouteBody_List v_RouteBodyList; var template (present) Via v_Via; var template (present) PAccessNetworkInfo v_PAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); if (not fl_IMS_InviteRequestIsMO(p_InviteRequest)) { /* @sic R5-134116: sic@ in case of MT call establishment the INVITE sent to the UE contains a Record-Route header with a valid route set */ v_Via := f_IMS_SipRequest_ViaRX(); /* as defined in A.2.1 */ } else { /* in case of MO call establishment the route set as for the 183 response shall be used (in reverse order) */ v_Via := f_IMS_Via_AsInPrevInvite(v_MessageHeader_Invite.via) } v_RouteBodyList := f_IMS_RouteSet_RouteBodyList_RX(p_CallType, p_DialogIndex); /* @sic R5s140017 MCC160 implementation: re-INVITE - Route is different for MO and MT calls sic@ @sic R5s150692 change 19 sic@ */ v_MessageHeader_Bye.via := v_Via; v_MessageHeader_Bye.route := cr_Route(v_RouteBodyList); v_MessageHeader_Bye.toField := f_IMS_RequestInDialog_ToHeaderRX(p_DialogIndex); /* @sic R5s130497 change 1 - MCC160 implementation; R5s140017 MCC160 implementation sic@ */ v_MessageHeader_Bye.fromField := f_IMS_RequestInDialog_FromHeaderRX(p_DialogIndex); /* @sic R5s130497 change 1 - MCC160 implementation; R5s140017 MCC160 implementation sic@ */ v_MessageHeader_Bye.callId := v_MessageHeader_Invite.callId; v_MessageHeader_Bye.cSeq := cr_CseqDef(v_CSeqValue, "BYE"); v_MessageHeader_Bye.maxForwards := cr_MaxForwardsDef; if (v_A6) { /* @sic R5-172934 sic@ */ v_MessageHeader_Bye.require := omit; v_MessageHeader_Bye.proxyRequire := omit; v_MessageHeader_Bye.securityVerify := omit; } else { if (v_A1) { v_MessageHeader_Bye.require := cr_RequireSecAgree; v_MessageHeader_Bye.proxyRequire := cr_ProxyRequireSecAgree; v_MessageHeader_Bye.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); /* @sic R5-196983 sic@ */ } else if (v_A2) { v_MessageHeader_Bye.securityVerify := omit; } } if (v_A1) { v_MessageHeader_Bye.pAccessNetworkInfo := v_PAccessNetworkInfo; } else if (v_A2) { v_MessageHeader_Bye.pAccessNetworkInfo := v_PAccessNetworkInfo ifpresent; } return v_MessageHeader_Bye; }

## Change 23 – f_IMS_Register_NonGIBA

function f_IMS_Register_NonGIBA(IMS_IpcanPortType_Type p_IpcanType := IPCAN_E, template (omit) IMS_DATA_REQ p_IMS_DATA_REQ := omit, charstring p_RegisterExpiration := tsc_IMS_RegisterExpiration, boolean p_SigCompIsActive := false, boolean p_IsEmergency := false) runs on IMS_PTC return IMS_DATA_REQ { var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterReq; var default v_RetransmissionHandler := null; v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(p_IpcanType, p_IMS_DATA_REQ, -, -, p_SigCompIsActive, p_IsEmergency); // check REGISTER and init GlobalInfo v_RegisterReq := v_IMS_DATA_REQ.Request.Register; if (fl_IMS_RetransmissionsAllowedDuringRegistration(v_IMS_DATA_REQ)) { v_RetransmissionHandler := activate(a_IMS_RetransmissionHandler(v_IMS_DATA_REQ)); } f_IMS_Register_SecurityInit(v_RegisterReq); IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, p_IsEmergency); v_RegisterReq := v_IMS_DATA_REQ.Request.Register; IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, p_IsEmergency, p_RegisterExpiration)))); // @sic R5s130561 sic@ deactivate(v_RetransmissionHandler); return v_IMS_DATA_REQ; }

## Change 24 – f_IMS_Registration

function f_IMS_Registration(IMS_IpcanPortType_Type p_IpcanType := IPCAN_E, template (omit) IMS_DATA_REQ p_IMS_DATA_REQ := omit, boolean p_SigCompIsActive := false, charstring p_RegisterExpiration := tsc_IMS_RegisterExpiration) runs on IMS_PTC return IMS_DATA_REQ { var IMS_DATA_REQ v_IMS_DATA_REQ; var boolean v_WaitForPublish := true; v_IMS_DATA_REQ := f_IMS_Register(p_IpcanType, p_IMS_DATA_REQ, p_SigCompIsActive, p_RegisterExpiration); f_IMS_Register_SubscribeNotify(v_IMS_DATA_REQ, v_WaitForPublish); return v_IMS_DATA_REQ; }

## Change 25 – f_IMS_SubscribeRequest_MessageHeaderRX_C30

function f_IMS_SubscribeRequest_MessageHeaderRX_C30(IMS_PTC_DialogIndex_Type p_DialogIndex) runs on IMS_PTC return template (present) MessageHeader { var template (present) MessageHeader v_MessageHeader_Subscribe := cr_MessageHeader_Dummy; var EventType v_EventType := f_SIP_SUBSCRIBE_DialogIndex2EventType(p_DialogIndex); /* @sic R5s200743: f_SIP_SUBSCRIBE_DialogIndex2EventType sic@ */ var template (present) PAccessNetworkInfo v_PAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); v_MessageHeader_Subscribe.cSeq := cr_CseqDef(?, "SUBSCRIBE"); /* @sic R5s160045: any CSeqValue; CSeqValue to be check after the SUBSCRIBE has been effectively matched an appropriate receive statement sic@ */ v_MessageHeader_Subscribe.fromField := f_IMS_RequestInDialog_FromHeaderRX(p_DialogIndex); // @sic R5s150843: p_DialogIndex sic@ v_MessageHeader_Subscribe.toField := f_IMS_RequestInDialog_ToHeaderRX(p_DialogIndex); // @sic R5s150843: p_DialogIndex sic@ v_MessageHeader_Subscribe.callId := f_IMS_PTC_ImsInfo_DialogGetCallId(p_DialogIndex); // @sic R5s150843: p_DialogIndex sic@ v_MessageHeader_Subscribe.expires := cr_Expires("0"); v_MessageHeader_Subscribe.event := cr_EventDef(v_EventType); return v_MessageHeader_Subscribe; }

## Change 26 – f_IMS_PublishRequest_MessageHeaderRX

function f_IMS_PublishRequest_MessageHeaderRX(template (present) SipUrl p_SipUrlTo := ?, template (present) EventType p_EventType := ?) runs on IMS_PTC return template (present) MessageHeader { /* @sic R5-196983: p_SipUrlTo tocope with specific To-URI for MCX sic@ */ var template (present) MessageHeader v_MessageHeader_Publish := cr_MessageHeader_Dummy; var IMS_ProtectedPorts_Type v_Protected; var template (present) PAccessNetworkInfo v_PAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); var template (present) Expires v_Expires := cr_Expires(tsc_IMS_ExpiresDef); var template integer v_Port_ps := f_IMS_PTC_GetPort_ps(); var template integer v_Port_us_Via := *; var template (present) SipUrl v_SipUrlFrom := ?; /* "any IMPU within the set of IMPUs on ISIM" */ v_MessageHeader_Publish.cSeq := cr_CseqDef(?, "PUBLISH"); v_MessageHeader_Publish.fromField := cr_FromWithTag(v_SipUrlFrom); // checked by f_IMS_PublishRequest_CheckPublicUserIdentity v_MessageHeader_Publish.toField := cr_ToDef(p_SipUrlTo); // NOTE: absence of the To tag needs to be checked separately (f_IMS_MessageHeader_CheckTo) v_MessageHeader_Publish.callId := cr_CallId(?); v_MessageHeader_Publish.maxForwards := cr_MaxForwardsDef; if (f_IMS_PTC_SecurityScheme_IsGiba()) { v_MessageHeader_Publish.require := omit; v_MessageHeader_Publish.proxyRequire := omit; v_MessageHeader_Publish.securityVerify := omit; v_MessageHeader_Publish.pAccessNetworkInfo := v_PAccessNetworkInfo ifpresent; v_MessageHeader_Publish.expires := v_Expires ifpresent; // "same as registration timer" acc. to A.4.3 } else { v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); v_Port_us_Via := v_Protected.Port_us; v_MessageHeader_Publish.pAccessNetworkInfo := v_PAccessNetworkInfo; v_MessageHeader_Publish.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); v_MessageHeader_Publish.expires := v_Expires ifpresent; // "same as registration timer" acc. to A.4.3 } v_MessageHeader_Publish.route := cr_Route(cr_RouteBodyList_InitialReq(v_Port_ps)); /* P-SCCF needs to be checked after receiving (f_IMS_MessageHeader_Request_CheckRoute) */ v_MessageHeader_Publish.via := cr_ViaDef(-, cr_HostPort(?, v_Port_us_Via)); /* host address needs to be checked after receiving (f_IMS_MessageHeader_Request_CheckVia) */ v_MessageHeader_Publish.event := cr_EventDef(p_EventType); return v_MessageHeader_Publish; }

## Change 27 – fl_IMS_Register_BuildAuthorizationHeader

function fl_IMS_Register_BuildAuthorizationHeader(RegistrationType_Type p_RegistrationType, template (present) charstring p_Nc := "00000001") runs on IMS_PTC return template Authorization { /* @sic R5-160290: FBBA registration - new function sic@ */ var charstring v_Username := f_IMS_PTC_ImsInfo_GetPrivateUserId(); /* => FBBA: px_IMS_Private_UserId (see f_IMS_PTC_Init) */ var charstring v_Realm := f_IMS_PTC_ImsInfo_GetHomeDomainName(); /* => FBBA: px_IMS_HomeDomainName (see f_IMS_PTC_Init) */ var charstring v_Uri := "sip:" & v_Realm; var template charstring v_Opaque := tsc_IMS_Opaque; var charstring v_Nonce; var charstring v_Algorithm; var template Authorization v_Authorization; select (p_RegistrationType) { case (initial) { v_Authorization := f_AuthorizationCommon(v_Username, v_Realm, v_Uri); } case (subsequent) { v_Nonce := f_IMS_PTC_Security_GetNonce(); v_Algorithm := f_IMS_PTC_Security_GetHashAlgorithm(); v_Authorization := f_AuthorizationSubsequent(v_Username, v_Realm, v_Nonce, v_Uri, v_Algorithm, p_Nc, v_Opaque); } } return v_Authorization; }

## Change 28 – f_IMS_RegisterResponse_401_MessageHeaderTX

function f_IMS_RegisterResponse_401_MessageHeaderTX(REGISTER_Request p_RegisterRequest, template (omit) charstring p_Nonce := omit) runs on IMS_PTC return template (value) MessageHeader { var charstring v_Nonce; var template (value) SecurityMechanism_List v_SecurityMech; var template (value) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderTX(p_RegisterRequest.msgHeader); if (not isvalue(p_Nonce)) { v_Nonce := f_IMS_PTC_Security_GetNonce(); } else { v_Nonce := valueof(p_Nonce); } v_MessageHeader_Response.wwwAuthenticate := cs_WwwAuthChallenge(f_IMS_PTC_ImsInfo_GetHomeDomainName(), v_Nonce, f_IMS_PTC_Security_GetHashAlgorithm()); /* @sic R5s120727 change 2 sic@ @sic R5-160290 FBBA registration: f_IMS_PTC_Security_GetHashAlgorithm sic@ */ v_SecurityMech := f_IMS_PTC_BuildSecurityServerParam(); v_MessageHeader_Response.securityServer := cs_SecServerDef(v_SecurityMech); return v_MessageHeader_Response; }

## Change 29 – f_IMS_RegisterRequest_MessageHeaderRX_Initial

function f_IMS_RegisterRequest_MessageHeaderRX_Initial(InternetProtocol_Type p_Protocol, template (present) integer p_CseqValue := ?, template (present) SemicolonParam_List p_MandatoryContactParams := {}) runs on IMS_PTC return template (present) MessageHeader { var boolean v_CondA3 := f_IMS_PTC_SecurityScheme_IsGiba(); var boolean v_IsMCX := f_IMS_PTC_TestsuiteInfo_IsMCX(); /* MCX */ var template (present) SipUrl v_SipUrlPublicUserId := ?; /* Acc. to 24.229 cl. 5.1.1.2.1 for initial registration the UE shall populate 'To' and 'From' with the public user identity to be registered; Normally this is one of the IMPUs stored on the ISIM; The public user identity is explicitly checked after matching the REGISTER message */ var template (present) MessageHeader v_MessageHeader_Register := fl_IMS_RegisterRequest_MessageHeaderRX_Base(v_CondA3, p_CseqValue); var template (present) PAccessNetworkInfo v_PAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); var template GenericParam v_ViaRportParam := omit; if ((p_Protocol == tcp) or v_IsMCX) { /* @sic R5-196983: rport parameter is not checked for MCX sic@ */ v_ViaRportParam := omit; /* @sic R5s120858 change 2; R5s130369; R5-133358: "rport" is optional for tcp (independent from A3) sic@ */ } else { v_ViaRportParam := cr_GenericParam_NoValue("rport"); // @sic R5s120907 change 8 - rport has no value sic@ } v_MessageHeader_Register.via := cr_ViaDef(-, -, v_ViaRportParam); v_MessageHeader_Register.fromField := cr_FromWithTag(v_SipUrlPublicUserId); // checked in f_IMS_Register_MessageHeader_CheckPublicUserIdentity v_MessageHeader_Register.toField := cr_ToDef(v_SipUrlPublicUserId); // checked in f_IMS_Register_MessageHeader_CheckPublicUserIdentity v_MessageHeader_Register.contact := cr_Contact(cr_SipUri_HostPort(?, *), p_MandatoryContactParams); // additionally checked in fl_IMS_Register_ContactHeader_CheckParams, f_IMS_Register_MessageHeader_CommonChecks if (v_IsMCX) { /* @sic R5-196983: pAccessNetworkInfo mandatory for MCX sic@ */ v_MessageHeader_Register.pAccessNetworkInfo := v_PAccessNetworkInfo; } else { v_MessageHeader_Register.pAccessNetworkInfo := v_PAccessNetworkInfo ifpresent; // optional for A1, A3; cellId is not checked } if (not v_CondA3) { // => 34.229-1 A.1.1 condition A1 v_MessageHeader_Register.securityVerify := omit; v_MessageHeader_Register.authorization := fl_IMS_Register_BuildAuthorizationHeader(initial); /* @sic R5-160290 FBBA registration - new function fl_IMS_Register_BuildAuthorizationHeader sic@ */ } return v_MessageHeader_Register; }

## Change 30 – f_IMS_RegisterRequest_MessageHeaderRX_Subsequent

function f_IMS_RegisterRequest_MessageHeaderRX_Subsequent(integer p_NonceCount, InternetProtocol_Type p_Protocol, integer p_CseqValue, template (present) SemicolonParam_List p_MandatoryContactParams := {}) runs on IMS_PTC return template (present) MessageHeader { var template (present) MessageHeader v_MessageHeader_Register; var template (present) SipUrl v_SipUrlPublicUserId; var charstring v_NonceCountStr := "0000000" & int2str(p_NonceCount); // assuming p_NonceCount < 9 var IMS_ProtectedPorts_Type v_Protected; var boolean v_IsGiba := false; var template integer v_ViaPort := *; var template integer v_ContactPort; v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); v_ContactPort := v_Protected.Port_us; if (p_Protocol == udp) { // @sic R5-145727 sic@ v_ViaPort := v_Protected.Port_us; } v_SipUrlPublicUserId := f_IMS_PTC_ImsInfo_GetSipUriRX(registrationId); /* public user identity is checked against what the UE has sent in initial REGISTER (in case the UE does not have an ISIM this is the temporary user id) => we don't need to consider A7 (emergency registration) @sic R5s160451: use of f_IMS_PTC_ImsInfo_GetSipUriRX sic@ */ v_MessageHeader_Register := fl_IMS_RegisterRequest_MessageHeaderRX_Base(v_IsGiba, p_CseqValue); v_MessageHeader_Register.via := cr_ViaDef(-, cr_HostPort(?, v_ViaPort)); // @sic R5s120727 change 7; R5-145727 sic@ v_MessageHeader_Register.fromField := cr_FromWithTag(v_SipUrlPublicUserId); v_MessageHeader_Register.toField := cr_ToDef(v_SipUrlPublicUserId); v_MessageHeader_Register.contact := cr_Contact(cr_SipUri_HostPort(?, v_ContactPort), p_MandatoryContactParams); // additionally checked in f_IMS_Register_MessageHeader_CommonChecks; @sic R5s120727 change 7 sic@ v_MessageHeader_Register.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); // cell id only if applicable v_MessageHeader_Register.authorization := fl_IMS_Register_BuildAuthorizationHeader(subsequent, v_NonceCountStr); /* @sic R5-160290 FBBA registration - new function fl_IMS_Register_BuildAuthorizationHeader sic@ */ v_MessageHeader_Register.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); return v_MessageHeader_Register; }

## Change 31 – f_IMS_RegisterRequest_MessageHeaderRX_Derregister

function f_IMS_RegisterRequest_MessageHeaderRX_Derregister(integer p_CseqValue) runs on IMS_PTC return template (present) MessageHeader { /* @sic R5-176758: p_CseqValue, to be consistent with f_IMS_RegisterRequest_MessageHeaderRX_Subsequent sic@ */ var boolean v_CondA3 := f_IMS_PTC_SecurityScheme_IsGiba(); /* condition A3 acc. to 34.229-1 Annex A.1.1 (REGISTER for the case UE supports GIBA) */ var boolean v_CondA2 := not v_CondA3; var template (present) MessageHeader v_MessageHeader_Register; var template (present) SipUrl v_SipUrlPublicUserId := f_IMS_PTC_ImsInfo_GetSipUriRX(registrationId); /* public user identity is checked against what the UE has sent in initial REGISTER @sic R5-173026: acc. to IR.92 V10.0 the UE has to use the same URI as in the initial REGISTER sic@ @sic R5s160451: use of f_IMS_PTC_ImsInfo_GetSipUriRX sic@ */ var IMS_ProtectedPorts_Type v_Protected; var template integer v_Port_us := *; if (v_CondA2) { // @sic R5-160290 FBBA registration sic@ v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); v_Port_us := v_Protected.Port_us; } v_MessageHeader_Register := fl_IMS_RegisterRequest_MessageHeaderRX_Base(v_CondA3, p_CseqValue); // common parameters for A2, A3 and A15: v_MessageHeader_Register.fromField := cr_FromWithTag(v_SipUrlPublicUserId); v_MessageHeader_Register.toField := cr_ToDef(v_SipUrlPublicUserId); v_MessageHeader_Register.supported := *; // Supported: header may be missing or it may contain any value v_MessageHeader_Register.via := cr_ViaDef(-, cr_HostPort(?, v_Port_us)); // address will be checked in f_IMS_Register_MessageHeader_CommonChecks (@sic R5s120727 sic@) v_MessageHeader_Register.contact := (cr_Contact_Wildcard, cr_Contact(cr_SipUri_HostPort(?, v_Port_us))); // address will be checked in f_IMS_Register_MessageHeader_CommonChecks (@sic R5s120727 sic@) if (not v_CondA3) { v_MessageHeader_Register.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); // cell id only if applicable v_MessageHeader_Register.authorization := fl_IMS_Register_BuildAuthorizationHeader(subsequent, ?); /* @sic R5-160290 FBBA registration - new function fl_IMS_Register_BuildAuthorizationHeader sic@ */ v_MessageHeader_Register.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); } return v_MessageHeader_Register; }

## Change 32 – f_IMS_SubscribeRequest_MessageHeaderRX

function f_IMS_SubscribeRequest_MessageHeaderRX(EventType p_EventType := "reg", template SipUrl p_SipUrlTo := omit, template (present) charstring p_ExpireTime := "600000", template (present) SemicolonParam_List p_MandatoryContactParams := {}) runs on IMS_PTC return template (present) MessageHeader { var template (present) MessageHeader v_MessageHeader_Subscribe := cr_MessageHeader_Dummy; var IMS_ProtectedPorts_Type v_Protected; var template (present) PAccessNetworkInfo v_PAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); var template (present) SipUrl v_SipUrlFrom := f_IMS_PTC_ImsInfo_GetSipUriSubscriptionRX(); var template (present) SipUrl v_SipUrlTo; var template integer v_Port_ps := *; var template integer v_Port_us_Via := *; var template integer v_Port_us_Contact := ?; var boolean v_CondA2 := f_IMS_PTC_SecurityScheme_IsGiba(); // GIBA if (ispresent(p_SipUrlTo)) { v_SipUrlTo := p_SipUrlTo; // NOTE: compiler warning cannot be avoided here } else { v_SipUrlTo := v_SipUrlFrom; } v_MessageHeader_Subscribe.cSeq := cr_CseqDef(?, "SUBSCRIBE"); v_MessageHeader_Subscribe.fromField := cr_FromWithTag(v_SipUrlFrom); v_MessageHeader_Subscribe.toField := cr_ToDef(v_SipUrlTo); v_MessageHeader_Subscribe.expires := cr_Expires(p_ExpireTime); // @sic R5-130548 sic@ v_MessageHeader_Subscribe.callId := cr_CallId(?); v_MessageHeader_Subscribe.maxForwards := cr_MaxForwardsDef; v_MessageHeader_Subscribe.event := cr_EventDef(p_EventType); if (v_CondA2 ) { // GIBA or FBBA @sic R5-160290 FBBA registration sic@ v_MessageHeader_Subscribe.require := omit; v_MessageHeader_Subscribe.proxyRequire := omit; v_MessageHeader_Subscribe.securityVerify := omit; if (v_CondA2) { v_MessageHeader_Subscribe.pAccessNetworkInfo := v_PAccessNetworkInfo ifpresent; } else { v_MessageHeader_Subscribe.pAccessNetworkInfo := v_PAccessNetworkInfo; } } else { // IMS security v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); v_Port_ps := v_Protected.Port_ps; v_Port_us_Via := v_Protected.Port_us; v_Port_us_Contact := v_Protected.Port_us; v_MessageHeader_Subscribe.require := cr_RequireSecAgree; v_MessageHeader_Subscribe.proxyRequire := cr_ProxyRequireSecAgree; v_MessageHeader_Subscribe.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); v_MessageHeader_Subscribe.pAccessNetworkInfo := v_PAccessNetworkInfo; } v_MessageHeader_Subscribe.route := cr_Route(cr_RouteBodyList_InitialReq(v_Port_ps)); /* @sic R5s140934 change 1 sic@ P-CSCF address is checked by f_IMS_MessageHeader_Request_CheckRoute */ v_MessageHeader_Subscribe.via := cr_ViaDef(-, cr_HostPort(?, v_Port_us_Via)); /* host address needs to be checked after receiving (f_IMS_MessageHeader_Request_CheckVia) @sic R5s120727 change 6.1 sic@ */ v_MessageHeader_Subscribe.contact := cr_Contact(f_Contact_SipUri_HostPortRX(?, v_Port_us_Contact), p_MandatoryContactParams); /* host address needs to be checked after receiving (f_IMS_MessageHeader_CheckContactAddr) @sic R5s120727 change 6.1 sic@ @sic R5s130925 change 1: UE may use any port as server port sic@ @sic R5s150721, R5-153760: cr_SipUri_HostPort replaced by f_Contact_SipUri_HostPortRX sic@ */ select (p_EventType) { case ("reg") { // normal case v_MessageHeader_Subscribe.accept := cr_AcceptDef(c_xmlreginfoApplication) ifpresent; } case ("conference") { // "SUBSCRIBE for conference event package" acc. annex A.5.1 v_MessageHeader_Subscribe.accept := cr_AcceptDef("application/conference-info+xml"); v_MessageHeader_Subscribe.expires := cr_Expires(?); // !!!! should be removed !!!! } case ("message-summary") { // "SUBSCRIBE for Message Waiting Indication package" acc. annex A.6.1 v_MessageHeader_Subscribe.accept := cr_AcceptDef("application/simple-message-summary"); } case ("presence") { // SUBSCRIBE for presence event package MCX TS 36.579-1 Table 5.5.2.14-1 v_MessageHeader_Subscribe.accept := cr_AcceptDef(tsc_MimeType_Pidf); } case ("xcap-diff") { // SUBSCRIBE for xcap-diff event package, MCX TS 24.379 clause 6.3.5 v_MessageHeader_Subscribe.accept := cr_AcceptDef(tsc_MimeType_XcapDiff); } case ("poc-settings") { // SUBSCRIBE for poc-settings event package, MCX TS 24.379 clause 7.3.6 v_MessageHeader_Subscribe.accept := cr_AcceptDef(tsc_MimeType_pocSettings); } } return v_MessageHeader_Subscribe; }

## Change 33 – f_IMS_ReferRequest_MessageHeaderRX

function f_IMS_ReferRequest_MessageHeaderRX(template (present) SipUrl p_RequestUri, template (present) SipUrl p_ReferToUri, boolean p_ReferCreatingDialog, template (present) integer p_CSeqValue := ?, IMS_CallType_Type p_CallType := CONFERENCE_CALL) runs on IMS_PTC return template (present) MessageHeader { var boolean v_IsGiba := f_IMS_PTC_SecurityScheme_IsGiba(); // => A1/A2 var template (present) MessageHeader v_MessageHeader_Refer := cr_MessageHeader_Dummy; var template (present) RouteBody_List v_RouteBodyList; var template (present) SipUrl v_SipUrlFrom := ?; var template (present) From v_From; var template (present) To v_To; var template (present) CallId v_CallId; var IMS_ProtectedPorts_Type v_Protected; var template integer v_Port_us := ?; // Acc. A.2.10 mandatory even for GIBA var template integer v_Port_ps := *; if (not v_IsGiba) { v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); v_Port_us := v_Protected.Port_us; v_Port_ps := v_Protected.Port_ps; } if (p_ReferCreatingDialog) { // FFS: currently not used (p_ReferCreatingDialog is always false) v_RouteBodyList := { // same as in INVITE cr_RouteBody(*, cr_SipUri_HostPort_lr(f_IMS_PTC_Pcscf_Get(), v_Port_ps)), cr_RouteBody(*, cr_SipUri_HostPort_lr(tsc_IMS_Scscf)) }; v_From := cr_From(*, v_SipUrlFrom); v_To := cr_ToDef(p_RequestUri); v_CallId := cr_CallId(?); } else { v_RouteBodyList := f_IMS_RouteSet_RouteBodyList_RX(p_CallType); /* @sic R5s160011: p_CallType sic@ */ v_From := f_IMS_RequestInDialog_FromHeaderRX(); v_To := f_IMS_RequestInDialog_ToHeaderRX(); v_CallId := f_IMS_PTC_ImsInfo_DialogGetCallId(); /* @sic R5-151954 sic@ */ } v_MessageHeader_Refer.via := f_IMS_SipRequest_ViaRX(?); // same as for INVITE; but p_Port_us is mandatory v_MessageHeader_Refer.route := cr_Route(v_RouteBodyList); v_MessageHeader_Refer.toField := v_To; v_MessageHeader_Refer.fromField := v_From; v_MessageHeader_Refer.callId := v_CallId; v_MessageHeader_Refer.cSeq := cr_CseqDef(p_CSeqValue, "REFER"); v_MessageHeader_Refer.maxForwards := cr_MaxForwardsDef; v_MessageHeader_Refer.contact := cr_Contact(f_Contact_SipUri_HostPortRX(?, v_Port_us)); /* additional checks required after message has been received @sic R5s150721, R5-153760: cr_SipUri_HostPort replaced by f_Contact_SipUri_HostPortRX sic@ */ v_MessageHeader_Refer.referTo := cr_ReferTo(p_ReferToUri); if (not v_IsGiba) { v_MessageHeader_Refer.require := cr_RequireSecAgree; v_MessageHeader_Refer.proxyRequire := cr_ProxyRequireSecAgree; v_MessageHeader_Refer.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); v_MessageHeader_Refer.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); } else { v_MessageHeader_Refer.securityVerify := omit; } return v_MessageHeader_Refer; }

## Change 34 – fl_ConferenceEventPackage_SubscribeRequest_MessageHeaderRX

function fl_ConferenceEventPackage_SubscribeRequest_MessageHeaderRX(template (present) SipUrl p_RequestUri) runs on IMS_PTC return template (present) MessageHeader { var boolean v_IsGiba := f_IMS_PTC_SecurityScheme_IsGiba(); // => A1/A2 var template (present) MessageHeader v_MessageHeader_Subscribe := cr_MessageHeader_Dummy; var template (present) PAccessNetworkInfo v_PAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); var template (present) SipUrl v_SipUrlFrom := ?; var template (present) RouteBody_List v_RouteBodyList; var template (present) From v_From; var template (present) To v_To; var template (present) CallId v_CallId; var template (present) integer v_Port_us := ?; var template integer v_Port_ps := *; var IMS_ProtectedPorts_Type v_Protected; if (v_IsGiba) { v_MessageHeader_Subscribe.require := omit; v_MessageHeader_Subscribe.proxyRequire := omit; v_MessageHeader_Subscribe.securityVerify := omit; v_MessageHeader_Subscribe.pAccessNetworkInfo := v_PAccessNetworkInfo ifpresent; } else { v_Protected := f_IMS_PTC_Security_GetProtectedPorts(); v_Port_us := v_Protected.Port_us; v_Port_ps := v_Protected.Port_ps; v_MessageHeader_Subscribe.require := cr_RequireSecAgree; v_MessageHeader_Subscribe.proxyRequire := cr_ProxyRequireSecAgree; v_MessageHeader_Subscribe.securityVerify := f_IMS_PTC_BuildSecurityVerifyHeader(); v_MessageHeader_Subscribe.pAccessNetworkInfo := v_PAccessNetworkInfo } v_RouteBodyList := { // same as in REFER cr_RouteBody(*, cr_SipUri_HostPort_lr(f_IMS_PTC_Pcscf_Get(), v_Port_ps)), cr_RouteBody(*, cr_SipUri_HostPort_lr(tsc_IMS_Scscf)) }; v_From := cr_From(*, v_SipUrlFrom); v_To := cr_ToDef(p_RequestUri); v_CallId := cr_CallId(?); v_MessageHeader_Subscribe.via := f_IMS_SipRequest_ViaRX(?); /* same as for INVITE; but p_Port_us is mandatory @sic R5-153986: common review of implementation sic@ */ v_MessageHeader_Subscribe.route := cr_Route(v_RouteBodyList); v_MessageHeader_Subscribe.toField := v_To; v_MessageHeader_Subscribe.fromField := v_From; v_MessageHeader_Subscribe.callId := v_CallId; v_MessageHeader_Subscribe.cSeq := cr_CseqDef(?, "SUBSCRIBE"); v_MessageHeader_Subscribe.expires := cr_Expires(?); v_MessageHeader_Subscribe.maxForwards := cr_MaxForwardsDef; v_MessageHeader_Subscribe.event := cr_EventDef("conference"); v_MessageHeader_Subscribe.accept := cr_AcceptDef("application/conference-info+xml"); v_MessageHeader_Subscribe.contact := cr_Contact(f_Contact_SipUri_HostPortRX(?, v_Port_us)); /* host address needs to be checked after receiving @sic R5s120727 change 6.1 sic@ @sic R5s150721, R5-153760: cr_SipUri_HostPort replaced by f_Contact_SipUri_HostPortRX sic@ */ return v_MessageHeader_Subscribe; }

## Change 35 – f_IMS_PrackRequest_MessageHeaderRX

function f_IMS_PrackRequest_MessageHeaderRX(template (value) INVITE_Request p_InviteRequest, template (value) MessageHeader p_MessageHeader_PrevReliableResponse, template ContentType p_ContentType := * ) runs on IMS_PTC return template (present) MessageHeader { var MessageHeader v_MessageHeader_PrevReliableResponse := valueof(p_MessageHeader_PrevReliableResponse); var template (value) MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader; var template (present) MessageHeader v_MessageHeader_Prack := cr_MessageHeader_Dummy; var CSeq v_CSeq := v_MessageHeader_PrevReliableResponse.cSeq; var integer v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogRemote); /* @sic R5s130497 change 3; R5-133188 sic@ */ var integer v_ResponseNumber := v_MessageHeader_PrevReliableResponse.rSeq.responseNum; /* @sic R5s130333 change 5.2 sic@ */ v_MessageHeader_Prack.via := f_IMS_Via_AsInPrevInvite(v_MessageHeader_Invite.via); // @sic R5s120530 change 3.9 sic@ v_MessageHeader_Prack.route := fl_IMS_Route_ReverseOrderOfRecordRouteRX(v_MessageHeader_PrevReliableResponse.recordRoute); v_MessageHeader_Prack.toField := f_IMS_RequestInDialog_ToHeaderRX(); /* @sic R5s130497 change 1 - MCC160 implementation; R5s140017 MCC160 implementation sic@ */ v_MessageHeader_Prack.fromField := f_IMS_RequestInDialog_FromHeaderRX(); /* @sic R5s130497 change 1 - MCC160 implementation; R5s140017 MCC160 implementation sic@ */ v_MessageHeader_Prack.callId := v_MessageHeader_Invite.callId; v_MessageHeader_Prack.cSeq := cr_CseqDef(v_CSeqValue, "PRACK"); v_MessageHeader_Prack.maxForwards := cr_MaxForwardsDef; v_MessageHeader_Prack.rAck := cr_RAckDef(v_ResponseNumber, v_CSeq.seqNumber, v_CSeq.method); v_MessageHeader_Prack.contentType := p_ContentType; if (not f_IMS_PTC_SecurityScheme_IsGiba()) { /* @sic R5s170099 sic@ */ v_MessageHeader_Prack.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()); } else { v_MessageHeader_Prack.pAccessNetworkInfo := cr_PAccessNetworkInfo(f_IMS_PTC_GetRanType()) ifpresent; } return v_MessageHeader_Prack; }

## Change 36 – f_IMS_REGISTER_InitialRequest

function f_IMS_REGISTER_InitialRequest(IMS_IpcanPortType_Type p_IpcanType := IPCAN_E, template (omit) IMS_DATA_REQ p_IMS_DATA_REQ := omit, template (present) SemicolonParam_List p_MandatoryContactParams := {}, template (omit) charstring p_ExplicitExpiry := omit, boolean p_SigCompIsActive := false, boolean p_CondA7 := false, boolean p_CondA3 := false ) runs on IMS_PTC return IMS_DATA_REQ { var IPCAN_INFO_Type v_IpcanInfo; var IMS_DATA_REQ v_IMS_DATA_REQ; var IMS_RoutingInfo_Type v_RoutingInfo_UL; var InternetProtocol_Type v_Protocol; var IP_AddrInfo_Type v_UE_Address; var IP_AddrInfo_Type v_NW_Address; var MessageHeader v_MessageHeader; var template (present) integer v_CseqValue := ?; var boolean v_Match := false; v_IMS_DATA_REQ := fl_IMS_ReceiveOrGetRegisterRequest(p_IMS_DATA_REQ, v_CseqValue); /* @sic R5-176758: v_CseqValue sic@ */ v_MessageHeader := v_IMS_DATA_REQ.Request.Register.msgHeader; v_Protocol := v_IMS_DATA_REQ.RoutingInfo.Protocol; select (p_IpcanType) { case (IPCAN_E) { v_IpcanInfo := f_IMS_IPCAN_Query(IPCAN); } case else { v_IpcanInfo := f_IMS_IPCAN_Query(OtherIPCAN); } } f_IMS_PTC_SetIpcanInfo(v_IpcanInfo); v_Match := match(v_MessageHeader, f_IMS_RegisterRequest_MessageHeaderRX_Initial(v_Protocol, v_CseqValue, p_MandatoryContactParams)); /* @sic R5s130369 sic@ @sic R5-176758: v_CseqValue sic@ @sic R5s200000: p_MandatoryContactParams sic@*/ if (not v_Match) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "invalid REGISTER Request"); } v_RoutingInfo_UL := v_IMS_DATA_REQ.RoutingInfo; v_UE_Address := v_RoutingInfo_UL.UE_Address; v_NW_Address := v_RoutingInfo_UL.NW_Address; v_Protocol := v_RoutingInfo_UL.Protocol; f_IMS_PTC_ImsInfo_Init(v_UE_Address, v_NW_Address, v_IMS_DATA_REQ.Request.Register, p_SigCompIsActive); // additional checks f_IMS_Register_MessageHeader_CommonChecks(v_MessageHeader, v_Protocol, p_CondA7, v_UE_Address, p_ExplicitExpiry); f_IMS_Register_MessageHeader_CheckPublicUserIdentity(v_MessageHeader, p_CondA7, p_CondA3); // check 'To' and 'From' field (needed for initial registration only); @sic R5s140356: p_CondA3 sic@ if (p_SigCompIsActive) { f_IMS_MessageHeader_CheckSigComp(v_MessageHeader); } return v_IMS_DATA_REQ; }

## Change 37 – fl_IMS_RegisterRequest_MessageHeaderRX_Base

function fl_IMS_RegisterRequest_MessageHeaderRX_Base(boolean p_CondA3 := false, template (present) integer p_CseqValue ) runs on IMS_PTC return template (present) MessageHeader { var template (present) MessageHeader v_MessageHeader_Register := cr_MessageHeader_Dummy; var OptionTag_List v_SupportedOptions := { "path" }; /* @sic R5s150721 - Additional change: "path" shall always be included sic@ */ var boolean v_IsMCX := f_IMS_PTC_TestsuiteInfo_IsMCX(); /* MCX */ if (v_IsMCX) { /* @sic R5-196983: MCX sic@ */ v_SupportedOptions[1] := "timer"; } else if (pc_IMS_GRUUsInSIP) { v_SupportedOptions[1] := "gruu"; } v_MessageHeader_Register.route := omit; // @sic R5-134659 sic@ v_MessageHeader_Register.cSeq := cr_CseqDef(p_CseqValue, "REGISTER"); v_MessageHeader_Register.expires := *; // as the 'expires' information can be included in 'expires' as well as in 'contact' (or in both) it needs to be checked seperately v_MessageHeader_Register.supported := cr_Supported(v_SupportedOptions); // @sic R5-196983: cr_Supported instead of cr_Supported_Register sic@ v_MessageHeader_Register.callId := cr_CallId(?); // per default call id itself is not checked at all v_MessageHeader_Register.maxForwards := cr_MaxForwardsDef; if (p_CondA3 ) { // GIBA or FBBA v_MessageHeader_Register.require := omit; v_MessageHeader_Register.proxyRequire := omit; v_MessageHeader_Register.securityClient := omit; v_MessageHeader_Register.securityVerify := omit; v_MessageHeader_Register.authorization := omit; /* A14/A15: gets overwritten with expected header */ } else { v_MessageHeader_Register.require := cr_RequireSecAgree; v_MessageHeader_Register.proxyRequire := cr_ProxyRequireSecAgree; v_MessageHeader_Register.securityClient := cr_SecClientMandatoryFields; // further checks in fl_IMS_SecurityClient_Check } return v_MessageHeader_Register; }

## Change 38 – f_IMS_A_2_1_Invite_CommonChecks

function f_IMS_A_2_1_Invite_CommonChecks(IMS_DATA_REQ p_IMS_INVITE_REQ, INVITE_A_2_1_Context_Type p_Context := A_2_1_A4, boolean p_A3 := false, boolean p_A8 := false, boolean p_A10 := pc_IMS_Video_FeatureTag, boolean p_A11 := false, boolean p_SrvccAlertingSupported := pc_IMS_SRVCCAlert, boolean p_OriginatingIdentificationRestriction := false, boolean p_A18 := pc_BSRVCC) runs on IMS_PTC { var MessageHeader v_MessageHeader := p_IMS_INVITE_REQ.Request.Invite.msgHeader; var SemicolonParam_List v_ContactParams := {}; var SemicolonParam_List v_AcceptContactParams := {}; var InternetProtocol_Type v_Protocol := p_IMS_INVITE_REQ.RoutingInfo.Protocol; var SipUrl v_SipUrlFrom := f_Addr_Union_GetSipUrl(v_MessageHeader.fromField.addressField); var SipUrl v_SipUrlPreferred; var template SipUrl v_ExpectedEmergencyUrl := f_IMS_PTC_ImsInfo_GetSipUriRX(emergencyId, *); var PPreferredIDValue_List v_PreferredIDValueList; var CharStringList_Type v_PreferredPublicUserIdList; var boolean v_EmergencyCallWithoutRegistration := (p_Context == A_2_1_A6); var boolean v_AllowFQDN := not v_EmergencyCallWithoutRegistration; // for A6 there is no FQDN for the UE var boolean v_CheckRport := v_EmergencyCallWithoutRegistration; var boolean v_A12 := p_SrvccAlertingSupported and (p_Context != A_2_1_A5); /* @sic R5s140370 change 7 sic@ */ var integer i; var template (present) GenValue v_SipInstanceValue; var template integer v_Port_us_Via := *; if ((v_EmergencyCallWithoutRegistration and (v_Protocol==udp))) { /* @sic R5-172412 sic@ */ v_Port_us_Via := tsc_IMS_PortNumber_5060; } if (ispresent(v_MessageHeader.contact.contactBody.contactAddresses[0].contactParams)) { v_ContactParams := v_MessageHeader.contact.contactBody.contactAddresses[0].contactParams; } if (ispresent(v_MessageHeader.acceptContact.acValues[0].acRcParams)) { v_AcceptContactParams := v_MessageHeader.acceptContact.acValues[0].acRcParams; } // **** Common checks **** f_IMS_MessageHeader_Request_CheckVia(v_MessageHeader, v_Protocol, v_AllowFQDN, v_CheckRport, v_Port_us_Via); /* check whether via contains "SIP URI with IP address or FQDN" (see @sic R5s120727 sic@) @sic R5-172412: v_Port_us_Via sic@ */ f_IMS_MessageHeader_CheckContactAddr(v_MessageHeader, v_EmergencyCallWithoutRegistration); /* check whether contact contains "SIP URI with IP address or FQDN" (see @sic R5s120727 sic@) @sic R5s160366: v_EmergencyCallWithoutRegistration sic@ */ if (p_Context != A_2_1_A5) { f_IMS_MessageHeader_CheckTo(v_MessageHeader, noToTag); /* check that there is no To tag */ f_IMS_MessageHeader_Request_CheckRoute(v_MessageHeader); /* @sic R5s140934 change 7 sic@ */ } // **** Check URIs in From header, P-Preferred-Identity header and further headers **** select (p_Context) { case (A_2_1_A4) { // Check Call-Id if (match(v_MessageHeader.callId.callid, f_IMS_PTC_ImsInfo_GetCallIdRegistration())) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A4: invalid 'CallId' in INVITE (has to be different than in previous REGISTER)"); } // Additional checks regarding SIP URI in the From header @sic R5-130750 change 3 sic@ if (not p_OriginatingIdentificationRestriction) { /* @sic R5-160975, R5s160687: p_OriginatingIdentificationRestriction sic@ */ if (not f_IMS_PublicUserId_Match(f_IMS_PTC_ImsInfo_GetAllIMPUs(), v_SipUrlFrom)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "invalid 'From'"); } if (ispresent(v_MessageHeader.pPreferredID.pPreferredIDValueList)) { v_PreferredIDValueList := v_MessageHeader.pPreferredID.pPreferredIDValueList; for (i:=0; i < lengthof(v_PreferredIDValueList); i := i+1) { v_PreferredPublicUserIdList[i] := f_SIP_SipUrl_GetPublicUserIdentity(f_Addr_Union_GetSipUrl(v_PreferredIDValueList[i])); } if (not f_IMS_PublicUserId_Match(v_PreferredPublicUserIdList, v_SipUrlFrom)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A4: invalid 'From': does not match with P-Preferred-Identity"); } } } } case (A_2_1_A5) { /* @sic R5s140017 MCC160 implementation: no further checks for A5 are needed in this function anymore sic@ */ } case (A_2_1_A6) { v_SipInstanceValue := cr_GenValueQuoted(f_SIP_InstanceId()); /* @sic R5s130756 change 11; R5w140111: GenValue; R5s141212 sic@ */ if (not match(v_ContactParams, cr_SemicolonParam_List_OneSpecificParam("+sip.instance", v_SipInstanceValue))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "IMEI code of the UE, coded as specified in draft-montemurro-gsma-imei-urn, not matching"); } } case (A_2_1_A7) { if (not match(v_SipUrlFrom, v_ExpectedEmergencyUrl)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A7: invalid 'From'"); } v_SipUrlPreferred := f_Addr_Union_GetSipUrl(v_MessageHeader.pPreferredID.pPreferredIDValueList[0]); if (not match(v_SipUrlPreferred, v_ExpectedEmergencyUrl)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A7: invalid 'P-Preferred-Identity'"); } } case (A_2_1_A19) { // @sic R5-174545, Change 2 R5s180463 sic@ // Via, Route and Contact header are checked in function f_IMS_InviteRequest_MessageHeaderRX if (not match(v_SipUrlFrom, v_ExpectedEmergencyUrl)) { // @sic R5-191118 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A19: invalid 'From'"); } } case else { FatalError(__FILE__, __LINE__, "unknown context"); } } // **** Check further Contact feature-param acc. to further conditions of A.2.1: **** // if (p_A3) { // A.1.2 condition A3: MTSI if (not match(v_ContactParams, cr_FeatureParamList_3gpp_icsi_ref)) { // @sic R5s130738: string needs to be quoted; R5-142247 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A3: Missing feature-param in Contact header"); } if (p_Context == A_2_1_A4) { // @sic R5-142949 sic@ if (not match(v_AcceptContactParams, cr_FeatureParamList_3gpp_icsi_ref)) { // @sic R5s130738: string needs to be quoted; R5-142247 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A3: Missing feature-param in AcceptContact header"); } } } if (p_A8) { // A.1.2 condition A8: check geolocation and geolocationRouting @sic R5-133151 sic@ /* NOTE: v_MessageHeader.geolocation and geolocationRouting are checked already by template returned by f_IMS_InviteRequest_MessageHeaderRX: * geolocation and geolocationRouting to be present. @sic R5-133151 sic@ */ if (not match(v_MessageHeader.geolocationRouting.state, GEOLOCATION_ROUTING_YES_E)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A8: Wrong geolocation-routing"); } } if (p_A10) { /* A.1.2 condition A10: UE supports video feature tag A.1.2 condition A11: INVITE for creating a video call @sic R5-153766: "p_A10 and p_A11 and (p_Context != A_2_1_A6) and (p_Context != A_2_1_A7)" sic@ @sic R5-160553: "p_A10 and p_A11" sic@ @sic R5-176371: "p_A10" for ContactParams, "p_A10 and p_A11" for AcceptContactParams sic@ */ if (not match(v_ContactParams, cr_VideoInContactParams)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A10, A11: Missing feature-param in Contact header"); } if (p_A11 and not match(v_AcceptContactParams, cr_VideoInContactParams)) { /* @sic R5-176371 sic@ */ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A10, A11: Missing feature-param in Accept-Contact header"); } } if (f_IMS_PTC_AudioFeatureTagIsRequired() and not match(v_ContactParams, cr_AudioInContactParams)) { // check 'feature-param' @sic R5-165943, R5-170564 sic@ @sic R5-173021 sic@ @sic R5-177127 sic@ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "missing audio feature-param in Contact header"); } if (v_A12) { // A.1.2 condition A12: INVITE for creating a voice or video call and UE supports g.3gpp.srvcc-alerting media feature tag if (not match(v_ContactParams, cr_SemicolonParam_List_OneSpecificParamNoValue(tsc_IMS_FeatureCap_SrvccAlerting))) { /* @sic R5s140370 change 7: tsc_IMS_FeatureCap_SrvccAlerting sic@ */ f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A12: Missing feature-param"); } } if (p_A18 and (p_Context != A_2_1_A5)) { // A.1.2 condition A18: UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request @sic R5-171486 sic@ @sic R5-182793 sic@ if (not match(v_ContactParams, cr_SemicolonParam_List_OneSpecificParamNoValue(tsc_IMS_FeatureCap_SrvccPreAlerting))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "A.2.1 condition A18: Missing feature-param"); } } }

## Change 39 – fl_PublicUserIdentity_Check

function fl_PublicUserIdentity_Check(SipUrl p_SipUrl, boolean p_CondA7 := false, boolean p_CondA3 := false, boolean p_EmergencyRegistrationWithoutPriorRegularRegistration := false) runs on IMS_PTC return boolean { var boolean v_Result := true; var IMS_UriInfo_UriType_Type v_UriType; if (p_CondA7) { /* emergency registration */ if (f_IMS_PTC_ImsInfo_GetUEwithISIM()) { /* UE has ISIM @sic R5-203077: f_IMS_PTC_ImsInfo_GetUEwithISIM instead of px_UEwithISIM sic@ */ v_UriType := emergencyId; /* A.1.1 Note 4: UE is using ISIM */ } else { /* UE has no ISIM */ if (not p_EmergencyRegistrationWithoutPriorRegularRegistration) { /* @sic R5-176207 sic@ */ v_UriType := defaultId; /* A.1.1 Note 4 UE has no ISIM but successfully performed IMS registration with the IM CN subsystem before */ } else { v_UriType := temporaryId; } } v_Result := f_SIP_MatchURI(f_IMS_PTC_ImsInfo_GetUriString(v_UriType), p_SipUrl); } else { /* no emergency registration */ if (not p_CondA3 and f_IMS_PTC_ImsInfo_GetUEwithISIM()) { /* A1 and UE has ISIM @sic R5-203077: f_IMS_PTC_ImsInfo_GetUEwithISIM instead of px_UEwithISIM sic@ */ v_Result := f_IMS_PublicUserId_Match(f_IMS_PTC_ImsInfo_GetAllIMPUs(), p_SipUrl); /* any IMPU within the set of IMPUs on ISIM NOTE: for MCX there is only one IMPU (px_MCPTT_Client_A_ID, see f_IMS_PTC_Init) as according to 36.579-1 Table 5.5.2.13-1 */ } else { /* A3 or no ISIM */ v_Result := f_SIP_MatchURI(f_IMS_PTC_ImsInfo_GetUriString(temporaryId), p_SipUrl); /* public user identity derived from IMSI */ } } return v_Result; }

## Change 40 – f_IMS_Register_MessageHeader_CheckPublicUserIdentity

function f_IMS_Register_MessageHeader_CheckPublicUserIdentity(MessageHeader p_MessageHeader, boolean p_CondA7 := false, boolean p_CondA3 := false, boolean p_CondA14 := false) runs on IMS_PTC { var SipUrl v_SipUrlFrom := f_Addr_Union_GetSipUrl(p_MessageHeader.fromField.addressField); /* @sic R5s130235 additional changes sic@ */ var SipUrl v_SipUrlTo := f_Addr_Union_GetSipUrl(p_MessageHeader.toField.addressField); /* @sic R5s130235 additional changes sic@ */ if (not fl_PublicUserIdentity_Check(v_SipUrlTo, p_CondA7, p_CondA3, p_CondA14)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid public user identity in 'toField' of the REGISTER request"); } if (f_SIP_SipUrl2Str(v_SipUrlFrom) != f_SIP_SipUrl2Str(v_SipUrlTo)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "public user identity is different in From and To header of the REGISTER request"); } }

## Change 41 – f_MTC_MainLoop_IMS

function f_MTC_MainLoop_IMS(timer p_GuardTimer) runs on MTC_IMS { var boolean v_IMS1_Done := (vc_Components.IMS1 == null); // @sic R5s130011 sic@ var boolean v_IMS2_Done := (vc_Components.IMS2 == null); // @sic R5s130011 sic@ f_MTC_BASE_Init(); while (not (v_IMS1_Done and v_IMS2_Done)) { // @sic R5s130011 sic@ alt { [not v_IMS1_Done] vc_Components.IMS1.done { // terminate the loop when the IMS PTC(s) have done its work v_IMS1_Done := true; } [not v_IMS2_Done] vc_Components.IMS2.done { // terminate the loop when the IMS PTC(s) have done its work v_IMS2_Done := true; } [] a_MTC_ReceiveFromPTC_SendToUT(tsc_MTC_PortIndex_EUTRA); // handle upper tester for PTC 0 (EUTRA) [] a_MTC_ReceiveFromPTC_SendToUT(tsc_MTC_PortIndex_UTRAN); // handle upper tester for PTC 1 (UTRAN) [] a_MTC_ReceiveFromPTC_SendToUT(tsc_MTC_PortIndex_IMS1); // handle upper tester for PTC 4 (IMS1) [] a_MTC_ReceiveFromPTC_SendToUT(tsc_MTC_PortIndex_IMS2); // handle upper tester for PTC 5 (IMS2) [] a_MTC_ReceiveFromPTC_SendToUT(tsc_MTC_PortIndex_WLAN); // handle upper tester for PTC 6 (WLAN @sic R5w160212 sic@) [] a_MTC_ReceiveFromPTC_SendToUT(tsc_MTC_PortIndex_NR); // handle upper tester for PTC 8 (NR5GC @sic R5s201327 change 6 sic@) [] a_MTC_ReceiveFromUT_SendToPTC(); [] p_GuardTimer.timeout {f_MTC_Stop(fail);} /* acc. to the assumption that preamble and postamble are short compared to the testbody a FAIL verdict is assigned (rather than an INCONC); @sic R5s140243 - 2nd timeout removed sic@ */ [] any component.killed { f_MTC_Stop(inconc); } } } f_MTC_Stop(pass); /* Note: "pass" overrules "none" only */ }

## Change 42 – module MTC_Base

const integer tsc_NoOfCordPorts := 8;

const integer tsc_MTC_PortIndex_NR := 7;

## Change 43 – fl_IMS_GetDrbRoutingInfo

function fl_IMS_GetDrbRoutingInfo() runs on IMS_PTC return template (value) IP_DrbInfo_Type { var template (omit) IPCAN_RAN_Type v_RanType := f_IMS_PTC_GetRanType(); /* @sic R5-163201: IPCAN_RAN_Type instead of charstring sic@ */ var IP_DrbId_Type v_DrbId; var template (value) IP_DrbInfo_Type v_DrbRoutingInfo; var WLAN_CellId_Type v_CellId; if (isvalue(v_RanType)) { select(valueof(v_RanType)) { case (EUTRA_FDD, EUTRA_TDD) { v_DrbId := f_EUTRA_NB_EpsBearerAssociatedDRB(tsc_EpsDefaultBearerId2ndPDN); /* DRB Id for 2nd PDN; @sic R5s141351 change 4 sic@ */ v_DrbRoutingInfo := cs_DrbInfo_EUTRA(eutra_Cell1, v_DrbId); } case (WLAN) { v_CellId := wlan_Cell27; v_DrbRoutingInfo:= cs_DrbInfo_WLAN(v_CellId); } case else { FatalError(__FILE__, __LINE__, "RAN type not supported by current implementation"); } } } else { FatalError(__FILE__, __LINE__, "Unknown RAN type"); } return v_DrbRoutingInfo; }