3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210255

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1460 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_GetPDN_DNNTypeFromAPN in 20wk50 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the 20wk50 TTCN implementation of the function f_GetPDN_DNNTypeFromAPN is not in-line with 23.003 cl 9.1 and RFC 1035cl 4.1.2 The current implementation does not take into account the Octets for label. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Re-worked the whole function. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | All SA testcases will fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All SA tetscases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	function f_GetPDN_DNNTypeFromAPN	3

# Corrections required

## function f_GetPDN_DNNTypeFromAPN

| Function name | f_GetPDN_DNNTypeFromAPN |
| --- | --- |
| Reason for change | In the 20wk50 TTCN implementation of the function f_GetPDN_DNNTypeFromAPN is not in-line with 23.003 cl 9.1 and RFC 1035cl 4.1.2 The current implementation does not take into account the Octets for label. This needs to be corrected. |
| Summary of change | Re-worked the whole function. Please see below. |
| TTCN module | Common/NAS/NAS_AuxiliaryDefsAndFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_GetPDN_DNNTypeFromAPN(octetstring p_APN) return PDU_PDN_DNN_Type { var PDU_PDN_DNN_Type v_PDNType := None; if (match (oct2str(p_APN), pc_APN_ID_Internet)) { v_PDNType := Internet_DNN; } else if (match (oct2str(p_APN), pc_APN_ID_IMS)) { v_PDNType := IMS_DNN; } else if (f_IsDNNForEmergency(p_APN)) { // this isn't specified in table 4.8.4-1, but is a valid PDU type v_PDNType := Emergency_PDN; } return v_PDNType; }

After Change:

function f_GetPDN_DNNTypeFromAPN(octetstring p_APN) return PDU_PDN_DNN_Type { var PDU_PDN_DNN_Type v_PDNType := None; var octetstring v_APN_Id_Internet := (int2oct(lengthof(char2oct(pc_APN_ID_Internet)), 1)) & (char2oct(pc_APN_ID_Internet)); //WA#20wk50 var octetstring v_APN_Id_IMS := (int2oct(lengthof(char2oct(pc_APN_ID_IMS)), 1)) & (char2oct(pc_APN_ID_IMS)); //WA#20wk50; if (match (p_APN, v_APN_Id_Internet /*WA#20wk50 pc_APN_ID_Internet*/)) { v_PDNType := Internet_DNN; } else if (match (p_APN, v_APN_Id_IMS /*WA#20wk50 pc_APN_ID_IMS*/)) { v_PDNType := IMS_DNN; } else if (f_IsDNNForEmergency(p_APN)) { // this isn't specified in table 4.8.4-1, but is a valid PDU type v_PDNType := Emergency_PDN; } return v_PDNType; }