3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210343

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4601 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT testcases 22.3.1.6 and 22.3.1.6a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current implementation of ttcn-3 NPDSCH_ImcsValue_Type as well as NPUSCH_ImcsValue_Type have a value between (0..12) where in TS 36.213 Tables 16.4.1.5.1-1 and 16.5.1.2-2 can be seen that this value should range from (0..13). A runtime runtime error can be provoked as index 13 can be used but is out of array defined range. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increased range of NPDSCH_ImcsValue_Type and NPUSCH_ImcsValue_Type from (0..12) to (0..13) according to TS 36.213. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runtime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.6 and 22.3.1.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to module NBIOT_ASP_TypeDefs	3

# Corrections required

## Correction to module NBIOT_ASP_TypeDefs

| Module name | NBIOT_ASP_TypeDefs |
| --- | --- |
| Reason for change | A runtime runtime error can be provoked as index 13 can be used but is out of array defined range. |
| Summary of change | Increased range of NPDSCH_ImcsValue_Type and NPUSCH_ImcsValue_Type from (0..12) to (0..13) according to TS 36.213. |
| TTCN module | NBIOT/Common/NBIOT_ASP_TypeDefs.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> // common types type integer NPDSCH_ImcsValue_Type (0..12); /* Modulation and coding scheme index coding; TS 36.213 Table 16.4.1.5.1-1 */ type integer NPUSCH_ImcsValue_Type (0..12); /* Modulation and coding scheme index coding; TS 36.213 Table 16.5.1.2-2 */ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> // common types type integer NPDSCH_ImcsValue_Type (0..13); /* Modulation and coding scheme index coding; TS 36.213 Table 16.4.1.5.1-1 */ type integer NPUSCH_ImcsValue_Type (0..13); /* Modulation and coding scheme index coding; TS 36.213 Table 16.5.1.2-2 */ <<SKIPPED CODE>>