3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210377

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3660 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UTRAN testcase 12.2.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2021-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the currecnt TTCN implemenation at Step 3 in the Attach REQ message, the value for the GPRS ciphering key sequence number (tcv_PS_KeySeq) is checked with variable tcv_PS_KeySeq. After a successful preamble, the CKSN is set to a value other than “111”B. However, when the test case is executed in USIM removal mode, the UE deletes the stored CKSI/KSI which results in UE sending subsequent Attach Request in step 3 with CKSN set to “111”B. This causes testcase failure. In order to avoid this situation, the value of information field tcv_PS_KeySeq is replaced with ‘?’. Besides, te Prose does not mandate to check the value for CKSN at Step 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In local tree lt_Steps_3To5a, replaced tcv_PS_KeySeq with ? in Attach REQ at Step 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4243

1.	Change 1	4244

# 1.	Change 1

| Object name | lt_Steps_3To5a |
| --- | --- |
| Reason for change | In the currecnt TTCN implemenation at Step 3 in the Attach REQ message, the value for the GPRS ciphering key sequence number (tcv_PS_KeySeq) is checked with variable tcv_PS_KeySeq. After a successful preamble, the CKSN is set to a value other than “111”B. However, when the test case is executed in USIM removal mode, the UE deletes the stored CKSI/KSI which results in UE sending subsequent Attach Request in step 3 with CKSN set to “111”B. This causes testcase failure. In order to avoid this situation, the value of information field tcv_PS_KeySeq is replaced with ‘?’. Besides, te Prose does not mandate to check the value for CKSN at Step 3. |
| Summary of change | In local tree lt_Steps_3To5a, replaced tcv_PS_KeySeq with ? in Attach REQ at Step 3. |
| Module |  |
| Label | WA# |

Before:

After: