3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210395

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1648 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_NR_PDCP_SDU_GetList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation for the test cases 7.1.3.1.1 , 7.1.3.1.2. f_NR_PDCP_SDU_GetList function is in place to receive the loop back DRB packets , this function is expecting PDCP SDU , instead as per the Prose receive should be for PDCP PDU |  |  |  |  |  |  |  |  |
| Summary of change: |  | Renamed the function f_NR_PDCP_SDU_GetList to f_NR_PDCP_PDU_GetList and changed the implementation to receive PDCP PDUs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case due to unexpected data reception error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.1.1.ENDC, 7.1.3.1.2.ENDC, 7.1.3.1.1.NR5GC and 7.1.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function fl_TC_7_1_3_1_1and2	2

1.2	Correction to function f_NR_PDCP_SDU_GetList	4

# Corrections required

## Correction to function fl_TC_7_1_3_1_1and2

| Function name | Function fl_TC_7_1_3_1_1and2 |
| --- | --- |
| Reason for change | In the current TTCN implementation for the test cases 7.1.3.1.1 , 7.1.3.1.2. f_NR_PDCP_SDU_GetList function is in place to receive the loop back DRB packets , this function is expecting PDCP SDU , instead as per the Prose receive should be for PDCP PDU |
| Summary of change | Renamed the function f_NR_PDCP_SDU_GetList to f_NR_PDCP_PDU_GetList and changed the implementation to receive PDCP PDUs |
| TTCN module | ttcn-3\ttcn\develop\NR_TC_Common\7_1_3\PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_3_1_1and2(NR_PDCP_SS_State_Type p_NR_PDCP_State, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC {//@sic R5-211387 sic@ <<< SKIPPED CODE >>>> // @siclog "Step 2" siclog@ // CHECK: Does UE transmit PDCP PDUs with SN=0 for the first iteration and all PDCP PDUs for each iteration?(Note 2) for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-211387 sic@ // Get next list of PDCP PDUs received v_NR_L2DataList := f_NR_PDCP_SDU_GetList(p_NR_PDCP_State); v_LenPDUList := lengthof(v_NR_L2DataList.PdcpPdu); for (k := 0; k < v_LenPDUList; k := k + 1) { // set i to account for k i := i + k; if (i == p_InnerLoop) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, too many PDCP PDUs received");}; // check ok ??? // extract the PDCP PDU to be handled v_NR_PDCP_PDU_Received := v_NR_L2DataList.PdcpPdu[k]; // build the PDCP PDU expected (template) v_NR_PDCP_PDU_Expected := v_PdcpPduList[i]; // ith element in the currently used v_PdcpPduList // match the PDCP PDU to be handled against the PDCP PDU expected if (not match (v_NR_PDCP_PDU_Received, v_NR_PDCP_PDU_Expected)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, mismatch of received and expected PDUs"); }; if ((i == 0) and (j == 0)) { // SN = 0 f_NR_PreliminaryPass(__FILE__, __LINE__, " SN=0 "); } else { // SN = 1 .. if ((((j * p_InnerLoop) + i + 1) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, " SN = " & int2str((j * p_InnerLoop) + i) )}; // @sic R5-185083, R5-187106, R5s200315, R5s200957 sic@ } } } } <<< SKIPPED CODE >>>

After Change:

function fl_TC_7_1_3_1_1and2(NR_PDCP_SS_State_Type p_NR_PDCP_State, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC {//@sic R5-211387 sic@ <<< SKIPPED CODE >>> // @siclog "Step 2" siclog@ // CHECK: Does UE transmit PDCP PDUs with SN=0 for the first iteration and all PDCP PDUs for each iteration?(Note 2) for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-211387 sic@ // Get next list of PDCP PDUs received v_NR_L2DataList := f_NR_PDCP_PDU_GetList(p_NR_PDCP_State); v_LenPDUList := lengthof(v_NR_L2DataList.PdcpPdu); for (k := 0; k < v_LenPDUList; k := k + 1) { // set i to account for k i := i + k; if (i == p_InnerLoop) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, too many PDCP PDUs received");}; // check ok ??? // extract the PDCP PDU to be handled v_NR_PDCP_PDU_Received := v_NR_L2DataList.PdcpPdu[k]; // build the PDCP PDU expected (template) v_NR_PDCP_PDU_Expected := v_PdcpPduList[i]; // ith element in the currently used v_PdcpPduList // match the PDCP PDU to be handled against the PDCP PDU expected if (not match (v_NR_PDCP_PDU_Received, v_NR_PDCP_PDU_Expected)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, mismatch of received and expected PDUs"); }; if ((i == 0) and (j == 0)) { // SN = 0 f_NR_PreliminaryPass(__FILE__, __LINE__, " SN=0 "); } else { // SN = 1 .. if ((((j * p_InnerLoop) + i + 1) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, " SN = " & int2str((j * p_InnerLoop) + i) )}; // @sic R5-185083, R5-187106, R5s200315, R5s200957 sic@ } } } } <<< SKIPPED CODE >>>

## Correction to function f_NR_PDCP_SDU_GetList

| Function name | Function f_NR_PDCP_SDU_GetList |
| --- | --- |
| Reason for change | In the current TTCN implementation for the test cases 7.1.3.1.1 , 7.1.3.1.2. f_NR_PDCP_SDU_GetList function is in place to receive the loop back DRB packets , this function is expecting PDCP SDU , instead as per the Prose receive should be for PDCP PDU |
| Summary of change | Renamed the function f_NR_PDCP_SDU_GetList to f_NR_PDCP_PDU_GetList and changed the implementation to receive PDCP PDUs |
| TTCN module | ttcn-3\ttcn\develop\NR_TC_Common\7_1_3\PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_PDCP_SDU_GetList(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC return NR_L2DataList_Type { // @sic R5-211387 sic@ var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var NR_L2DataList_Type v_NR_L2DataList; DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_PDCP_State.DRB_Id)) -> value v_NR_DRB_COMMON_IND; v_NR_L2DataList := v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList; return v_NR_L2DataList; }

After Change:

function f_NR_PDCP_PDU_GetList(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC return NR_L2DataList_Type { // @sic R5-211387 sic@ var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var NR_L2DataList_Type v_NR_L2DataList; DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell1, p_NR_PDCP_State.DRB_Id)) -> value v_NR_DRB_COMMON_IND; v_NR_L2DataList := v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList; return v_NR_L2DataList; }