3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210406

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1654 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.1.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following implementation of R5-211388, postamble for this test case is performed in in end state E2_T3440. For this state the RRC connection is released and there is a 5s delay before the UE is switched off. In this time the UE may reselect back to the NR cell which is still at serving power level. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Power down NR cell at the end of the test case Note : An associated prose CR on 38.523-1 will be raised at RAN5#91. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case in postamble |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_1_3_2_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | Following implementation of R5-211388, postamble for this test case is performed in in end state E2_T3440. For this state the RRC connection is released and there is a 5s delay before the UE is switched off. In this time the UE may reselect back to the NR cell which is still at serving power level. |
| Summary of change | Send trigger to NR PTC at the end of test case so NE cell can be powered down. |
| TTCN module | RRC_ConnRel_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_1_3_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Step 2" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def, -, -, noRrcConnectionRelease); // @sic R5-204435 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }

After Change

function fl_TC_8_1_1_3_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // Send EUTRA frequency to NR f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // @siclog "Step 2" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1 (eutra_Cell1, tsc_RRC_TI_Def, -, -, noRrcConnectionRelease); // @sic R5-204435 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2"); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); }

<Indicate the exact TTCN changes performed >

## Change 2

| Function name | fl_TC_8_1_1_3_2_TestBody |
| --- | --- |
| Reason for change | Following implementation of R5-211388, postamble for this test case is performed in in end state E2_T3440. For this state the RRC connection is released and there is a 5s delay before the UE is switched off. In this time the UE may reselect back to the NR cell which is still at serving power level. |
| Summary of change | Power down NR cell at the end of the test case Note : An associated prose CR on 38.523-1 will be raised at RAN5#91. |
| TTCN module | RRC_ConnRel_NR5GC_NR |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_1_3_2_TestBody() runs on NR5GC_PTC { var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); f_NR_SetCellPower (nr_Cell1, -85, -82); //@siclog "Step 1" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo ))); //@siclog "Step 2" siclog@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }

After Change

function fl_TC_8_1_1_3_2_TestBody() runs on NR5GC_PTC { var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); f_NR_SetCellPower (nr_Cell1, -85, -82); //@siclog "Step 1" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cell). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease (tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo ))); //@siclog "Step 2" siclog@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_SetCellPower (nr_Cell1, -110, -110); f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Postamble"); }