3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210422

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1664 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to a_ApplicationCtrl |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In IRAT test case 8.1.1.3.2, function “f_IP_PTC_PDN_GetDrbInfoForDefaultDrb()” is called with the below flow f_EUTRA_UE_Detach_SwitchOffUe_Trigger f_IP_ChangeDrbMappingEUTRA(IP, p_CellId, p_DRB, PDN_1); IP_ConfigReq::ChangeDrbInfo f_IP_PTC_PDN_GetDrbInfoForDefaultDrb In this test case PDN_3 ( internet PDN ) is used in association with DRB ID 2 . hence during PDN establishment procedure , IP CONFIG REQ : StartPDN is setting vc_IP_PTC_PDN_MappingTable[PDN_3].DefaultDrb. vc_IP_PTC_PDN_MappingTable[PDN_1].DefaultDrb and vc_IP_PTC_PDN_MappingTable[PDN_2].DefaultDrb are omit. With the function f_IP_ChangeDrbMappingEUTRA , the IP CONFIG REQ:ChangeDrbInfo is received for PDN_1 ; f_IP_PTC_PDN_GetDrbInfoForDefaultDrb is called with PDN_1 passed as paramter, which returns Omit. This leads to CompulsoryValueOmittedException run time Issue |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a check condition to confirm whether vc_IP_PTC_PDN_MappingTable[PDN_Index] is having the values , before calling the function f_IP_PTC_PDN_GetDrbInfoForDefaultDrb with the PDN_Index |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Runtime error may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.2.NR5GC, 8.1.4.2.1.1.NR5GC, other IRAT testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to altstep a_ApplicationCtrl	3

# Corrections required

## Correction to altstep a_ApplicationCtrl

| Altstep name | a_ApplicationCtrl |
| --- | --- |
| Reason for change | In IRAT test case 8.1.1.3.2, function “f_IP_PTC_PDN_GetDrbInfoForDefaultDrb()” is called with the below flow f_EUTRA_UE_Detach_SwitchOffUe_Trigger f_IP_ChangeDrbMappingEUTRA(IP, p_CellId, p_DRB, PDN_1); IP_ConfigReq::ChangeDrbInfo f_IP_PTC_PDN_GetDrbInfoForDefaultDrb In this test case PDN_3 ( internet PDN ) is used in association with DRB ID 2 . hence during PDN establishment procedure , IP CONFIG REQ : StartPDN , vc_IP_PTC_PDN_MappingTable[PDN_3].DefaultDrb is set ; vc_IP_PTC_PDN_MappingTable[PDN_1].DefaultDrb and vc_IP_PTC_PDN_MappingTable[PDN_2].DefaultDrb are omit As part of the above mentioned flow , the IP CONFIG REQ:ChangeDrbInfo is received for PDN_1 ; f_IP_PTC_PDN_GetDrbInfoForDefaultDrb is called with PDN_1 passed as paramter, which returns Omit. This leads to CompulsoryValueOmittedException run time Issue |
| Summary of change | Adding a check whether vc_IP_PTC_PDN_MappingTable[PDN_Index] is having the values , before calling the function f_IP_PTC_PDN_GetDrbInfoForDefaultDrb with the PDN_Index, |
| TTCN module | Common/IP_PTC/ IP_PTC_Main.ttcn |
| MCC160 Comment |  |

Before Change:

altstep a_ApplicationCtrl(IP_PTC_CTRL_PORT p_CtrlPort) runs on IP_PTC { var IP_ConfigReq_Type v_IP_ConfigReq; var PDN_Index_Type v_PdnIndex; var IP_DrbInfo_Type v_DrbInfo; var IP_DrbInfo_Type v_CurrentDrbInfo; var EUTRA_CellId_Type v_EutraCellId; var integer i; [] p_CtrlPort.receive(cr_IP_CtrlAnyReq) -> value v_IP_ConfigReq { if (ischosen(v_IP_ConfigReq.StartPDN)) { v_PdnIndex := v_IP_ConfigReq.StartPDN.PdnId; v_DrbInfo := v_IP_ConfigReq.StartPDN.DrbInfo; f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); select (v_PdnIndex) { case (PDN_1, PDN_2, PDN_2a) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); fl_IP_ImsServer_Start(v_PdnIndex); } case (PDN_Internet, nrPDN_Internet, PDN_3) { // @sic R5s191074 sic@ fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); } case (ePDG_IMS1, ePDG_IMS2) { fl_IP_ImsServer_Start(v_PdnIndex); } case (ePDG_XCAP) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo, dnsServerOnly); } case (ePDG_Internet) { // nothing to be done } } } else if (ischosen(v_IP_ConfigReq.StopPDN)) { if (ischosen(v_IP_ConfigReq.StopPDN.PdnId)) { /* @sic R5s131000: stop particular PDN sic@ */ v_PdnIndex := v_IP_ConfigReq.StopPDN.PdnId; /* @sic R5s201177 sic@ */ fl_StopPDNs(v_PdnIndex); /* stop particular PDN */ } else { fl_StopPDNs(); /* => stop all PDNs and IMS servers @sic R5s201177 sic@ */ } } else if (ischosen(v_IP_ConfigReq.StartWLAN)) { fl_WLAN_Start(v_IP_ConfigReq.StartWLAN.AddressInfo, v_IP_ConfigReq.StartWLAN.DrbInfo, v_IP_ConfigReq.StartWLAN.ExplicitDnsHandling); } else if (ischosen(v_IP_ConfigReq.StopWLAN)) { fl_WLAN_Stop(); } else if (ischosen(v_IP_ConfigReq.ChangeEutraCell)) { /* @sic R5s150648 Change 6 sic@ */ v_EutraCellId := v_IP_ConfigReq.ChangeEutraCell; if (f_RoutingTable_ChangeEutraCell(v_EutraCellId)) { for (i:=0; i<tsc_NoOfPDNs; i:=i+1) { if (isvalue(vc_IP_PTC_PDN_MappingTable[i].DefaultDrb)) { /* @sic R5s201501 sic@ */ f_IP_DrbInfo_ChangeEutraCell(vc_IP_PTC_PDN_MappingTable[i].DefaultDrb, v_EutraCellId); } } } } else if (ischosen(v_IP_ConfigReq.ChangeDrbInfo)) { /* @sic R5s150648 Change 6 sic@ */ v_PdnIndex := v_IP_ConfigReq.ChangeDrbInfo.PdnId; v_DrbInfo := v_IP_ConfigReq.ChangeDrbInfo.DrbInfo; v_CurrentDrbInfo := f_IP_PTC_PDN_GetDrbInfoForDefaultDrb(v_PdnIndex); if (f_RoutingTable_ChangeDrbInfo(v_CurrentDrbInfo, v_DrbInfo)) { f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); } } else if (ischosen(v_IP_ConfigReq.SendRouterAdvertisement)) { fl_ICMPv6_SendRouterAdvertisement(v_IP_ConfigReq.SendRouterAdvertisement.PdnId); } else if (ischosen(v_IP_ConfigReq.ExplicitRouting)) { if (ischosen(v_IP_ConfigReq.ExplicitRouting.Add)) { f_IP_PTC_RoutingTable_AddEntry(v_IP_ConfigReq.ExplicitRouting.Add.IpInfo, v_IP_ConfigReq.ExplicitRouting.Add.DRB, v_IP_ConfigReq.ExplicitRouting.Add.DataMode); } else if (ischosen(v_IP_ConfigReq.ExplicitRouting.Remove)) { /* @sic R5s160131 sic@ */ f_IP_PTC_RoutingTable_RemoveEntry(v_IP_ConfigReq.ExplicitRouting.Remove); } } else if (ischosen(v_IP_ConfigReq.ChangeToRawmode)) { /* @sic R5s150074 - Additional change sic@ */ f_RoutingInfo_ChangeMode(true); // @sic R5s200044 change 4, R5s200490 sic@ } else if (ischosen(v_IP_ConfigReq.RestoreRoutingTable)) { // @sic R5s200044 change 4 sic@ f_RoutingInfo_ChangeMode(false); // @sic R5s200490 sic@ } p_CtrlPort.send(cs_IP_CommonCnf); } }

After Change:

altstep a_ApplicationCtrl(IP_PTC_CTRL_PORT p_CtrlPort) runs on IP_PTC { var IP_ConfigReq_Type v_IP_ConfigReq; var PDN_Index_Type v_PdnIndex; var IP_DrbInfo_Type v_DrbInfo; var IP_DrbInfo_Type v_CurrentDrbInfo; var EUTRA_CellId_Type v_EutraCellId; var integer i; [] p_CtrlPort.receive(cr_IP_CtrlAnyReq) -> value v_IP_ConfigReq { if (ischosen(v_IP_ConfigReq.StartPDN)) { v_PdnIndex := v_IP_ConfigReq.StartPDN.PdnId; v_DrbInfo := v_IP_ConfigReq.StartPDN.DrbInfo; f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); select (v_PdnIndex) { case (PDN_1, PDN_2, PDN_2a) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); fl_IP_ImsServer_Start(v_PdnIndex); } case (PDN_Internet, nrPDN_Internet, PDN_3) { // @sic R5s191074 sic@ fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); } case (ePDG_IMS1, ePDG_IMS2) { fl_IP_ImsServer_Start(v_PdnIndex); } case (ePDG_XCAP) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo, dnsServerOnly); } case (ePDG_Internet) { // nothing to be done } } } else if (ischosen(v_IP_ConfigReq.StopPDN)) { if (ischosen(v_IP_ConfigReq.StopPDN.PdnId)) { /* @sic R5s131000: stop particular PDN sic@ */ v_PdnIndex := v_IP_ConfigReq.StopPDN.PdnId; /* @sic R5s201177 sic@ */ fl_StopPDNs(v_PdnIndex); /* stop particular PDN */ } else { fl_StopPDNs(); /* => stop all PDNs and IMS servers @sic R5s201177 sic@ */ } } else if (ischosen(v_IP_ConfigReq.StartWLAN)) { fl_WLAN_Start(v_IP_ConfigReq.StartWLAN.AddressInfo, v_IP_ConfigReq.StartWLAN.DrbInfo, v_IP_ConfigReq.StartWLAN.ExplicitDnsHandling); } else if (ischosen(v_IP_ConfigReq.StopWLAN)) { fl_WLAN_Stop(); } else if (ischosen(v_IP_ConfigReq.ChangeEutraCell)) { /* @sic R5s150648 Change 6 sic@ */ v_EutraCellId := v_IP_ConfigReq.ChangeEutraCell; if (f_RoutingTable_ChangeEutraCell(v_EutraCellId)) { for (i:=0; i<tsc_NoOfPDNs; i:=i+1) { if (isvalue(vc_IP_PTC_PDN_MappingTable[i].DefaultDrb)) { /* @sic R5s201501 sic@ */ f_IP_DrbInfo_ChangeEutraCell(vc_IP_PTC_PDN_MappingTable[i].DefaultDrb, v_EutraCellId); } } } } else if (ischosen(v_IP_ConfigReq.ChangeDrbInfo)) { /* @sic R5s150648 Change 6 sic@ */ v_PdnIndex := v_IP_ConfigReq.ChangeDrbInfo.PdnId; v_DrbInfo := v_IP_ConfigReq.ChangeDrbInfo.DrbInfo; if(ispresent(vc_IP_PTC_PDN_MappingTable[f_PDN_PdnIndex2Integer(v_PdnIndex)].DefaultDrb)) //WA_21wk12_Start { v_CurrentDrbInfo := f_IP_PTC_PDN_GetDrbInfoForDefaultDrb(v_PdnIndex); if (f_RoutingTable_ChangeDrbInfo(v_CurrentDrbInfo, v_DrbInfo)) { f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); } else { f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); } } //WA_21wk12_End } else if (ischosen(v_IP_ConfigReq.SendRouterAdvertisement)) { fl_ICMPv6_SendRouterAdvertisement(v_IP_ConfigReq.SendRouterAdvertisement.PdnId); } else if (ischosen(v_IP_ConfigReq.ExplicitRouting)) { if (ischosen(v_IP_ConfigReq.ExplicitRouting.Add)) { f_IP_PTC_RoutingTable_AddEntry(v_IP_ConfigReq.ExplicitRouting.Add.IpInfo, v_IP_ConfigReq.ExplicitRouting.Add.DRB, v_IP_ConfigReq.ExplicitRouting.Add.DataMode); } else if (ischosen(v_IP_ConfigReq.ExplicitRouting.Remove)) { /* @sic R5s160131 sic@ */ f_IP_PTC_RoutingTable_RemoveEntry(v_IP_ConfigReq.ExplicitRouting.Remove); } } else if (ischosen(v_IP_ConfigReq.ChangeToRawmode)) { /* @sic R5s150074 - Additional change sic@ */ f_RoutingInfo_ChangeMode(true); // @sic R5s200044 change 4, R5s200490 sic@ } else if (ischosen(v_IP_ConfigReq.RestoreRoutingTable)) { // @sic R5s200044 change 4 sic@ f_RoutingInfo_ChangeMode(false); // @sic R5s200490 sic@ } p_CtrlPort.send(cs_IP_CommonCnf); } }