3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210472

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1695 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GSM test case 10.1.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - During PDU session release procedure, RRCReconfigurationComplete and PDU SESSION RELEASE COMPLETE may arrive in either order. - At step 26 of 10.1.3.2, a PDU session is established using DNN name “ABCC” (=tsc_APN1_String). In the current implementation no PDU type is stored for this PDU session, which causes a problem at the next step which calls f_GetLastConfiguredInternetPDUSessionID() which relies on PDU type being set to “internet” Note : This problem exists because the function f_GetPDN_DNNTypeFromAPN(), as called from f_Check_NG_PDUSessionEstablishmentReq() via f_GetPDNTypeFromDNN(), will only set PDNType if the DNN received from the UE matches pc_APN_ID_Internet or pc_APN_ID_IMS |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add interleave handling for RRCReconfigurationComlete and PDU SESSION RELEASE COMPLETE - For cases where specific DNN name is passed into this function, PDU type should be set to “internet”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1 Change 1	4

. 2.2 Change 2	6

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_NR5GC_PDUSessionRelease() |
| --- | --- |
| Reason for change | During PDU session release procedure, RRCReconfigurationComplete and PDU SESSION RELEASE COMPLETE may arrive in either order. |
| Summary of change | Add interleave handling for RRCReconfigurationComlete and PDU SESSION RELEASE COMPLETE |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before change

function f_NR5GC_PDUSessionRelease (NR_CellId_Type p_NR_CellId,

O1_Type p_PDU_SessionId,

template (value) GMM_GSM_Cause p_Cause := cs_GMM_GSM_Cause (omit, '00011010'B),

template (omit) GPRS_Timer3 p_BackOff := omit,

template (omit) NGSM_CongestionReattemptInd p_NGSM_CongestionReattemptInd := omit, // @sic R5s190543 sic@,

template (omit) ExtdProtocolConfigOptions p_ExtdPCO := omit,

template (omit) EAP_Message p_EAP := omit,

ProcedureTransactionIdentifier p_PTI := '00'O,

boolean p_ReceiveComplete := true)  runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(p_PDU_SessionId));

var NR_DRBInfo_Type v_DRBInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList;

var template (value) DRB_ToReleaseList v_DRB_ToReleaseList;

var template (value) NR_RadioBearerList_Type v_SS_Config;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

var integer i;

…………………

if (p_ReceiveComplete) {

// @sic R5-206266 sic@

// The UE transmit an RRCReconfigurationComplete message

SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete));

// The UE transmits an ULInformationTransfer message and a UL NAS TRANSPORT containing PDU SESSION RELEASE COMPLETE.

SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,

tsc_NR_RbId_SRB2,

cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, p_PDU_SessionId)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed");

}

}

f_NR5GC_MobileInfo_SetSessionInfoList (v_PDUSessionInfoList); // @sic R5-206266 sic@

}

After change

f unction f_NR5GC_PDUSessionRelease (NR_CellId_Type p_NR_CellId,

O1_Type p_PDU_SessionId,

template (value) GMM_GSM_Cause p_Cause := cs_GMM_GSM_Cause (omit, '00011010'B),

template (omit) GPRS_Timer3 p_BackOff := omit,

template (omit) NGSM_CongestionReattemptInd p_NGSM_CongestionReattemptInd := omit, // @sic R5s190543 sic@,

template (omit) ExtdProtocolConfigOptions p_ExtdPCO := omit,

template (omit) EAP_Message p_EAP := omit,

ProcedureTransactionIdentifier p_PTI := '00'O,

boolean p_ReceiveComplete := true)  runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForGivenPDUSession(oct2int(p_PDU_SessionId));

var NR_DRBInfo_Type v_DRBInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList;

var template (value) DRB_ToReleaseList v_DRB_ToReleaseList;

var template (value) NR_RadioBearerList_Type v_SS_Config;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

var integer i;

……………..

if (p_ReceiveComplete) {

// @sic R5-206266 sic@

interleave{

// The UE transmit an RRCReconfigurationComplete message

[]SRB.receive(car_NR_SRB_RrcPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete));

// The UE transmits an ULInformationTransfer message and a UL NAS TRANSPORT containing PDU SESSION RELEASE COMPLETE.

[]SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId,

tsc_NR_RbId_SRB2,

cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp;

}

if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, p_PDU_SessionId)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed");

}

}

f_NR5GC_MobileInfo_SetSessionInfoList (v_PDUSessionInfoList); // @sic R5-206266 sic@

}

## . 2.2 Change 2

| Function name | function f_NR5GC_PDUSessionEstablishment_InnerLoop() |
| --- | --- |
| Reason for change | At step 26 of 10.1.3.2, a PDU session is established using DNN name “ABCC” (=tsc_APN1_String). In the current implementation of f_NR5GC_PDUSessionEstablishment_InnerLoop(), no PDU type is stored for this PDU session, which causes issue at the next step which calls f_GetLastConfiguredInternetPDUSessionID() which relies on PDU type being set to “internet” Note : This change is needed because the function f_GetPDN_DNNTypeFromAPN(), as called from f_Check_NG_PDUSessionEstablishmentReq() via f_GetPDNTypeFromDNN(), will only set PDNType if the DNN received from the UE matches pc_APN_ID_Internet or pc_APN_ID_IMS |
| Summary of change | For cases where specific DNN name is passed into this function, PDU type should be set to “internet”. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before change

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId,

NR_SRB_COMMON_IND p_ReceivedAsp,

SRBs_AlreadyConfigured p_SRB := SRB1_2,

template (omit) SDAP_Config p_SDAP_Config,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := ?,

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_BuildQosFlow := omit,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@

{ // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@

…………………..

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

p_ReceivedAsp.Signalling.Nas[0].Pdu,

p_PDU_SessionId,

p_PTI,

p_IntegrityProtMaxDataRate,

p_PDU_SessionType,

p_ExpectedSSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_OldPDU_SessionId,

p_RequestType,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_AdditionalInfo)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@

v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@

} else {

if (v_GSM_MobilityInfo.PDUType == IMS_DNN) {   // @sic R5-206296 sic@

v_SDAP_Config := cs_SDAP_Config(-, -, {2});

} else {

v_SDAP_Config := cs_SDAP_Config;

}

}

…………………

}

After change

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId,

NR_SRB_COMMON_IND p_ReceivedAsp,

SRBs_AlreadyConfigured p_SRB := SRB1_2,

template (omit) SDAP_Config p_SDAP_Config,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := ?,

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_BuildQosFlow := omit,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@

{ // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@

……….

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

p_ReceivedAsp.Signalling.Nas[0].Pdu,

p_PDU_SessionId,

p_PTI,

p_IntegrityProtMaxDataRate,

p_PDU_SessionType,

p_ExpectedSSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_OldPDU_SessionId,

p_RequestType,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_AdditionalInfo)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

………….

if (isvalue(p_DNN)){

v_GSM_MobilityInfo.PDUType := Internet_DNN;

}

if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@

v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@

} else {

if (v_GSM_MobilityInfo.PDUType == IMS_DNN) {   // @sic R5-206296 sic@

v_SDAP_Config := cs_SDAP_Config(-, -, {2});

} else {

v_SDAP_Config := cs_SDAP_Config;

}

}

…………….

}