3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210536

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1525 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Due to race conditions in the SS, the indication of PRACH preamble and msg3 reception can come to the TTCN ports in any order. - Receipt of RRCSetupComplete in parallel with step 10 should fail the test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add interleave blocks ar steps 2-4 and 6-8 into interleave, and add guard checks at steps 10-12 - set fail veridict if RRCSetupComplete is received at steps 10-12 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case and a non-conformant UE may pass the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_1_6_NR_TestBody() |
| --- | --- |
| Reason for change | - Due to race conditions in the SS, the indication of PRACH preamble and msg3 reception can come to the TTCN ports in any order. - Receipt of RRCSetupComplete in parallel with step 10 should fail the test case |
| Summary of change | - Add interleave blocks ar steps 2-4 and 6-8 into interleave, and add guard checks at steps 10-12 - set fail veridict if RRCSetupComplete is received at steps 10-12 |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_1_6_NR_TestBody() runs on NR_BASE_PTC return NG_NAS_MSG_Indication_Type { var NG_NAS_MSG_Indication_Type v_NasInd; timer t_Watchdog := 10.0; //@sic r5s190764 sic@ t_Watchdog.start; // @siclog "Step 2" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ //Preconfigured at SS // @siclog "Step 4" siclog@ SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)); // @siclog "Step 5" siclog@ //Preconfigured at SS // @siclog "Step 6" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ //Preconfigured at SS // @siclog "Step 8" siclog@ SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)); // @siclog "Step 9" siclog@ //Preconfigured at SS // @siclog "Step 10" siclog@ alt { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } [] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def))) { } }; // @siclog "Step 11" siclog@ //Preconfigured at SS // @siclog "Step 12" siclog@ SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)); // @siclog "Step 13" siclog@ //Preconfigured at SS // @siclog "Step 14" siclog@ v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); // disable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); return v_NasInd; };

After Change

function f_TC_7_1_1_1_6_NR_TestBody() runs on NR_BASE_PTC return NG_NAS_MSG_Indication_Type { var NG_NAS_MSG_Indication_Type v_NasInd; timer t_Watchdog := 10.0; //@sic r5s190764 sic@ var boolean v_rach_flag := false; var boolean v_msg3_flag := false; t_Watchdog.start; // @siclog "Step 2" siclog@ // @siclog "Step 3" siclog@ //Preconfigured at SS // @siclog "Step 4" siclog@ interleave { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)){} } // @siclog "Step 5" siclog@ //Preconfigured at SS // @siclog "Step 6" siclog@ // @siclog "Step 7" siclog@ //Preconfigured at SS // @siclog "Step 8" siclog@ interleave { [] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)){} } // @siclog "Step 9" siclog@ //Preconfigured at SS // @siclog "Step 10" siclog@ alt { [not v_rach_flag] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, -, (0..31))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); v_rach_flag := true; if (not v_msg3_flag) { repeat; } } [not v_rach_flag] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Parallel Step 10, Receive unexpected RRCSetupComplete"); } [not v_msg3_flag] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest)) { v_msg3_flag := true; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); if(not v_rach_flag) { repeat; } } }; // @siclog "Step 11" siclog@ //Preconfigured at SS // @siclog "Step 12" siclog@ // @siclog "Step 13" siclog@ //Preconfigured at SS // @siclog "Step 14" siclog@ v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); // disable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); return v_NasInd; };