3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210556

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1619 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 7.1.1.3.8.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of NR5GC test case 7.1.1.3.8.1 in FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 7.1.1.3.8.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.8.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

2 Corrections required	3

2.1 Change 1	3

2.2 Change 2	7

2.3 Change 3	8

3 Branches executed	11

4 Execution Log Files	11

4.1 HiSilicon Balong 5000	11

5 References	12

# Overview

This document lists all the changes needed to correct issues in the ATS ATS NR5GC_IWD_20wk50 related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_3_8_NR_Common |
| --- | --- |
| Reason for change | 1 If we pass true to function f_NR5GC_Init_CA,it goes to f_NR_InitialiseMultiplePHR →f_NR_CellInfo_Set_MultiplePHR, only multiplePHR is initialized which will lead to test case failure when use the configuration later: phr_Config := { setup := { phr_PeriodicTimer := <undefined>, phr_ProhibitTimer := <undefined>, phr_Tx_PowerFactorChange := <undefined>, multiplePHR := true, dummy := <undefined>, phr_Type2OtherCell := <undefined>, phr_ModeOtherCG := <undefined>, mpe_Reporting_FR2_r16 := <undefined> } } 2 According to RAN5#90 agreed cr R5-211385, there is no need to close the UE test loop. 3 According to R5-211385,phr-Tx-PowerFactorChange=inifinity is configured in the RRCReconfiguration message, UE will not report PHR when pathloss change. TP2/3 can not be test.Test Purpose (TP) have changed the PHR type from Type2 to Type1, but Test Procedure does not change. |
| Summary of change | 1 Delete true and use default value. 2 Use default value TESTMode_OFF in f_NR5GC_RRC_ConnectedState3N_Def. 3 Add extra RRCReconfiguration message to set phr-Tx-PowerFactorChange=3dB, when pathloss change higher than 3dB and phr-ProhibitTimer expired, PHR could be triggered. |
| TTCN module | MAC_CA_NR5GC.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_7_1_1_3_8_NR_Common (NR_CellId_Type    p_PCell,

NR_CellId_Type    p_SCell,

NR_CA_Tested_Type p_NR_CA_Tested

) runs on NR5GC_PTC

{//UE power headroom reporting / SCell activation / DL pathloss change reporting

var template (value) CellGroupConfig v_SCGConfig;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var boolean v_RRC_MsgRcvd;

var boolean v_PHR_Rcvd;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_UplinkBWP_Type v_NR_UplinkBWP ;

f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested, true);

// Create and configure cells

f_NR_CellConfig_PCell_Def(p_PCell);

f_NR_CellConfig_SCell(p_SCell,Scell_Active);

// Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

//           Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (p_PCell, STATE_IDLE_1A, TESTModeA_ON);

//Set null cipher Algorithm

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR5GC_RRC_ConnectedState3N_Def(p_PCell,

TEST_LOOPModeA_ON,

cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

-,  //p_IpPduDelayTime

-,

-,  //MAC_CellGroupConfig

-,

-

);

f_NR_TestBody_Set(true);

f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

//@siclog Steps 1-2 siclog@

//The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell.

//The UE transmits the RRCReconfigurationComplete message?

f_NR5GC_AddMod_1SCell(p_PCell, p_SCell,-, true); // NR Cell1, NR Cell3/10

v_RRC_MsgRcvd:= false;

v_PHR_Rcvd := false;

v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_PCell, tsc_NR_BWP_Id);

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP);

//@siclog "Steps 4 preconfig" siclog@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PCell);

v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_71138);

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG,

omit,

omit,

v_MAC_CellGroupConfig);

v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig)));

//@siclog "Steps 3" siclog@

//The SS transmits UL grant to the UE at every 10ms in PDCCH occasion.

f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PCell, cs_TimingInfo_Now, v_NR_ResourceAllocation, 10);

//@siclog "Steps 4" siclog@

f_NR_SS_SystemIndCtrlConfig(p_PCell, cds_NR_SystemIndCtrl_MAC_PHR(enable));

f_NR_SendRRCReconfiguration (p_PCell, -,-, v_V1530Ext,-,-,-,false);

//@siclog "Steps 5" siclog@

alt {

[not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

v_RRC_MsgRcvd := true;

if(not v_PHR_Rcvd){ repeat;}

}

[not v_PHR_Rcvd] SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, cr_NR_MAC_CE_MultiEntryPHR_1Entry(?)))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 Parallel Behaviour");

v_PHR_Rcvd := true;

if(not v_RRC_MsgRcvd){ repeat;}

}

};

//@siclog "Steps 6-20" siclog@

f_TC_7_1_1_3_8_NR_TestBody(p_PCell, p_SCell,cr_NR_MAC_CE_MultiEntryPHR(?),tsc_SCellIndex_1);

...

} //end of f_TC_7_1_1_3_8_NR_Common

After change

function f_TC_7_1_1_3_8_NR_Common (NR_CellId_Type    p_PCell,

NR_CellId_Type    p_SCell,

NR_CA_Tested_Type p_NR_CA_Tested

) runs on NR5GC_PTC

{//UE power headroom reporting / SCell activation / DL pathloss change reporting

var template (value) CellGroupConfig v_SCGConfig;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var boolean v_RRC_MsgRcvd;

var boolean v_PHR_Rcvd;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_UplinkBWP_Type v_NR_UplinkBWP ;

f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested    );

// Create and configure cells

f_NR_CellConfig_Pcell_Def(p_PCell);

f_NR_CellConfig_Scell(p_SCell,Scell_Active);

// Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

//           Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (p_PCell, STATE_IDLE_1A, TESTModeA_ON);

//Set null cipher Algorithm

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR5GC_RRC_ConnectedState3N_Def(p_PCell,

-,

cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

-,  //p_IpPduDelayTime

-,

-,  //MAC_CellGroupConfig

-,

-

);

f_NR_TestBody_Set(true);

f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

//@siclog Steps 1-2 siclog@

//The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell.

//The UE transmits the RRCReconfigurationComplete message?

f_NR5GC_AddMod_1SCell(p_PCell, p_SCell,-, true); // NR Cell1, NR Cell3/10

v_RRC_MsgRcvd:= false;

v_PHR_Rcvd := false;

v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_PCell, tsc_NR_BWP_Id);

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP);

//@siclog "Steps 4 preconfig" siclog@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PCell);

v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_71138(-,dB3));

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG,

omit,

omit,

v_MAC_CellGroupConfig);

v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig)));

//@siclog "Steps 3" siclog@

//The SS transmits UL grant to the UE at every 10ms in PDCCH occasion.

f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PCell, cs_TimingInfo_Now, v_NR_ResourceAllocation, 10);

//@siclog "Steps 4" siclog@

f_NR_SS_SystemIndCtrlConfig(p_PCell, cds_NR_SystemIndCtrl_MAC_PHR(enable));

f_NR_SendRRCReconfiguration (p_PCell, -,-, v_V1530Ext,-,-,-,false);

//@siclog "Steps 5" siclog@

alt {

[not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

v_RRC_MsgRcvd := true;

if(not v_PHR_Rcvd){ repeat;}

}

[not v_PHR_Rcvd] SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PCell, cr_NR_MAC_CE_MultiEntryPHR_1Entry(?)))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5 Parallel Behaviour");

v_PHR_Rcvd := true;

if(not v_RRC_MsgRcvd){ repeat;}

}

};

//@siclog "Steps 6-20" siclog@

f_TC_7_1_1_3_8_NR_TestBody(p_PCell, p_SCell,cr_NR_MAC_CE_MultiEntryPHR(?),tsc_SCellIndex_1);

...

} //end of f_TC_7_1_1_3_8_NR_Common

## Change 2

| Function name | template (value) PHR_Config cs_PHR_Config_71138 |
| --- | --- |
| Reason for change | For change 1.3. |
| Summary of change | Add new parameters for cs_PHR_Config_71138 |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before change

template (value) PHR_Config cs_PHR_Config_71138  :=

{ //As per 38.523-1 Table 7.1.1.3.8.1.3.3-5: PHR-Config

/* @status     */

phr_PeriodicTimer         := sf500,

phr_ProhibitTimer         := sf1000,

phr_Tx_PowerFactorChange  := infinity_,

multiplePHR               := true,

dummy                     := true,

phr_Type2OtherCell        := true,

phr_ModeOtherCG           := real,

mpe_Reporting_FR2_r16     := omit           /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

};

After change

template (value) PHR_Config cs_PHR_Config_71138 (

PHR_Config.phr_PeriodicTimer p_Phr_PeriodicTimer:= infinity_,

PHR_Config.phr_Tx_PowerFactorChange p_Phr_Tx_PowerFactorChange := infinity_

) :=

{ //As per 38.523-1 Table 7.1.1.3.7.3.3-2&6: PHR-Config

/* @status    APPROVED (ENDC, NR5GC) */

//@sic R5-197026 sic@

phr_PeriodicTimer         := p_Phr_PeriodicTimer,

phr_ProhibitTimer         := sf1000,

phr_Tx_PowerFactorChange  := p_Phr_Tx_PowerFactorChange,

multiplePHR               := true,

dummy                     := true,

phr_Type2OtherCell        := false,

phr_ModeOtherCG           := real,

mpe_Reporting_FR2_r16     := omit          /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

};

## Change 3

| Function name | f_TC_7_1_1_3_8_NR_TestBody |
| --- | --- |
| Reason for change | 1 The order of the two input parameters in function f_SubFrameTimingDuration_Float is reversed. 2 The step number is not correct. |
| Summary of change | 1 Exchange input parameter sequence. 2 Correct the step number. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_7_1_1_3_8_NR_TestBody(NR_CellId_Type    p_SPCell,

NR_CellId_Type    p_SCell,

template (present) NR_MAC_CE_MultiEntryPHR_Type   p_NR_MAC_CE_MultiEntryPHR,

SCellIndex p_SCellIndex := tsc_SCellIndex_1) runs on NR_BASE_PTC

{ //UE power headroom reporting / SCell activation / DL pathloss change reporting/

var NR_SYSTEM_IND v_NR_SYSTEM_IND;

var SubFrameTiming_Type v_TimingInfo1;

var SubFrameTiming_Type v_TimingInfo2;

var SubFrameTiming_Type v_TimingInfo3;

var SubFrameTiming_Type v_TimingInfo4;

var SubFrameTiming_Type v_TimingInfo5;

...

//@siclog "Steps 10-11" siclog@

SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_SPCell, p_NR_MAC_CE_MultiEntryPHR))  -> value v_NR_SYSTEM_IND;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11");

v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo2, v_TimingInfo1);

if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11");

}

else  {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11: TP3 failed, UE didn't send PHR after periodic PHR expiry");

};

//@siclog "Step 12" siclog@

f_NR_SetCellPowerList(v_CellPowerList_AtT2_T4);

//@siclog "Steps 13-14" siclog@

SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_SPCell, p_NR_MAC_CE_MultiEntryPHR))  -> value v_NR_SYSTEM_IND;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11");

v_TimingInfo3 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo3, v_TimingInfo2);

if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

}

else  {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14: TP3 failed, UE didn't send PHR after periodic PHR expiry");

}

//@siclog "Step 15" siclog@

f_NR_SetCellPowerList(v_CellPowerList_AtT3);

//@siclog "Steps 16-17" siclog@

SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_SPCell, p_NR_MAC_CE_MultiEntryPHR))  -> value v_NR_SYSTEM_IND;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17");

v_TimingInfo4 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo4, v_TimingInfo3);

if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17");

}

else  {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17: TP3 failed, UE didn't send PHR after periodic PHR expiry");

}

//@siclog "Step 18" siclog@

f_NR_SetCellPowerList(v_CellPowerList_AtT2_T4);

//@siclog "Steps 19-20" siclog@

SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_SPCell, p_NR_MAC_CE_MultiEntryPHR))  -> value v_NR_SYSTEM_IND;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20");

v_TimingInfo5 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo5, v_TimingInfo4);

if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20");

}

else  {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20: TP3 failed, UE didn't send PHR after periodic PHR expiry");

}

};

After change

function f_TC_7_1_1_3_8_NR_TestBody(NR_CellId_Type    p_SPCell,

NR_CellId_Type    p_SCell,

template (present) NR_MAC_CE_MultiEntryPHR_Type   p_NR_MAC_CE_MultiEntryPHR,

SCellIndex p_SCellIndex := tsc_SCellIndex_1) runs on NR_BASE_PTC

{ //UE power headroom reporting / SCell activation / DL pathloss change reporting/

var NR_SYSTEM_IND v_NR_SYSTEM_IND;

var SubFrameTiming_Type v_TimingInfo1;

var SubFrameTiming_Type v_TimingInfo2;

var SubFrameTiming_Type v_TimingInfo3;

var SubFrameTiming_Type v_TimingInfo4;

var SubFrameTiming_Type v_TimingInfo5;

...

//@siclog "Steps 10-11" siclog@

SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_SPCell, p_NR_MAC_CE_MultiEntryPHR))  -> value v_NR_SYSTEM_IND;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11");

v_TimingInfo2 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo1, v_TimingInfo2);

if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11");

}

else  {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11: TP3 failed, UE didn't send PHR after periodic PHR expiry");

};

//@siclog "Step 12" siclog@

f_NR_SetCellPowerList(v_CellPowerList_AtT2_T4);

//@siclog "Steps 13-14" siclog@

SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_SPCell, p_NR_MAC_CE_MultiEntryPHR))  -> value v_NR_SYSTEM_IND;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

v_TimingInfo3 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo2, v_TimingInfo3);

if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

}

else  {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14: TP3 failed, UE didn't send PHR after periodic PHR expiry");

}

//@siclog "Step 15" siclog@

f_NR_SetCellPowerList(v_CellPowerList_AtT3);

//@siclog "Steps 16-17" siclog@

SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_SPCell, p_NR_MAC_CE_MultiEntryPHR))  -> value v_NR_SYSTEM_IND;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17");

v_TimingInfo4 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo3, v_TimingInfo4);

if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17");

}

else  {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17: TP3 failed, UE didn't send PHR after periodic PHR expiry");

}

//@siclog "Step 18" siclog@

f_NR_SetCellPowerList(v_CellPowerList_AtT2_T4);

//@siclog "Steps 19-20" siclog@

SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_SPCell, p_NR_MAC_CE_MultiEntryPHR))  -> value v_NR_SYSTEM_IND;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20");

v_TimingInfo5 := v_NR_SYSTEM_IND.Common.TimingInfo.SubFrame;

v_TimerDuration := f_SubFrameTimingDuration_Float(v_TimingInfo4, v_TimingInfo5);

if (( v_TimerDuration< v_TimerMax) and ( v_TimerDuration > v_TimerMin) ) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20");

}

else  {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 20: TP3 failed, UE didn't send PHR after periodic PHR expiry");

}

};

# Branches executed

The test case was executed on NR band CA_n41C.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000  passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:  
\ TC_7_1_1_3_8_1_NR5GC_CA_n41C_PASS.spm

- PIXCIT settings used:

\ PCT_PICS_PIXIT.sppar

# References

| [1] | R5s210337: Supporting information for agreement of NR5GC test case 7.1.1.3.8.1 in FR1. |
| --- | --- |