3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210672

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1774 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the function f_NR5GC_IRAT_S1ToN1_SessionModification, the PDU Session Modification is transmitted on SRB1 in case the DRB is already configured. In case the DRB is not configured, then the PDU Session Modification Command is transmitted as a part of function f_NR_RRCReconfigNewDRBWithSSConfig. In this function, the SRB2 is configured as a part of rrcReconfiguration procedure, and hence the PDU Session Modification Complete would be expected on SRB2. The current TTCN does not handle this. Also, in the function function f_NR5GC_IRAT_S1ToN1_Steps14 after calling the funciton f_NR5GC_IRAT_S1ToN1_SessionModification v_SRB needs to be set as SRB2 since the function f_NR5GC_IRAT_S1ToN1_SessionModification configures SRB2. This also needs to be corrected. Later in the implementation of function f_NR5GC_IRAT_S1ToN1_Steps14, the TTCN implements the PU Session Establishment based on condition : if (v_MsgRxd and ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)). This will always evaluate to false when only a PDU Sesision Modification is received. Thus, the TTCN will always set a fail or inconclusive verdict with message “Wrong NAS Message received”. This also needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_NR5GC_IRAT_S1ToN1_SessionModification, assigned p_SRB := tsc_NR_RbId_SRB2 after calling function f_NR_RRCReconfigNewDRBWithSSConfig. In the function f_NR5GC_IRAT_S1ToN1_Steps14, assigned v_SRB := tsc_NR_RbId_SRB2; after call to function f_NR5GC_IRAT_S1ToN1_SessionModification. In the function f_NR5GC_IRAT_S1ToN1_Steps14, re-worked the TTCN implementation that handles the PDU Session Establishment. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase(s) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.1.NR5GC, 6.2.3.2.NR5GC, 6.2.3.3.NR5GC, 6.2.3.4.NR5GC, 6.2.3.8.NR5GC, 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to function f_NR5GC_IRAT_S1ToN1_SessionModification	4

1.2	Correction to function f_NR5GC_IRAT_S1ToN1_Steps14	6

# Corrections required

## Correction to function f_NR5GC_IRAT_S1ToN1_SessionModification

| Function name | f_NR5GC_IRAT_S1ToN1_SessionModification |
| --- | --- |
| Reason for change | In the function f_NR5GC_IRAT_S1ToN1_SessionModification, the PDU Session Modification is transmitted on SRB1 in case the DRB is already configured. In case the DRB is not configured, then the PDU Session Modification Command is transmitted as a part of function f_NR_RRCReconfigNewDRBWithSSConfig. In this function, the SRB2 is configured as a part of rrcReconfiguration procedure, and hence the PDU Session Modification Complete would be expected on SRB2. The current TTCN does not handle this. |
| Summary of change | In the function f_NR5GC_IRAT_S1ToN1_SessionModification, assigned p_SRB := tsc_NR_RbId_SRB2 after calling function f_NR_RRCReconfigNewDRBWithSSConfig. Please see below. |
| TTCN module | NR5GC_IRAT_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> if (p_DRBsAlreadyConfigured) { // @sic R5-213442 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu))); // @sic R5s201413 sic@ } else { // @sic R5-213442 sic@ // Send with a RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_RadioBearerList, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu) }); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> if (p_DRBsAlreadyConfigured) { // @sic R5-213442 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, p_SRB, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu))); // @sic R5s201413 sic@ } else { // @sic R5-213442 sic@ // Send with a RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_RadioBearerList, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu) }); p_SRB := tsc_NR_RbId_SRB2; //WA#R&S : SRB2 is configured in the rrcReconfiguration message that is used to send PDU Session Modification Command } <<SKIPPED CODE>>

## Correction to function f_NR5GC_IRAT_S1ToN1_Steps14

| Function name | f_NR5GC_IRAT_S1ToN1_Steps14 |
| --- | --- |
| Reason for change | In the function function f_NR5GC_IRAT_S1ToN1_Steps14 after calling the funciton f_NR5GC_IRAT_S1ToN1_SessionModification v_SRB needs to be set as SRB2 since the function f_NR5GC_IRAT_S1ToN1_SessionModification configures SRB2. This needs to be corrected. Later in the implementation of function f_NR5GC_IRAT_S1ToN1_Steps14, the TTCN implements the PU Session Establishment based on condition : if (v_MsgRxd and ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)). This will always evaluate to false when only a PDU Sesision Modification is received. Thus, the TTCN will always set a fail or inconclusive verdict with message “Wrong NAS Message received”. This also needs to be corrected. |
| Summary of change | In the function f_NR5GC_IRAT_S1ToN1_Steps14, assigned v_SRB := tsc_NR_RbId_SRB2; after call to function f_NR5GC_IRAT_S1ToN1_SessionModification. In the function f_NR5GC_IRAT_S1ToN1_Steps14, re-worked the TTCN implementation that handles the PDU Session Establishment. Please see below. |
| TTCN module | NR5GC_IRAT_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_IRAT_S1ToN1_Steps14(NR_CellId_Type p_NR_CellId, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, boolean p_FirstTimeS1ToN1 := true, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC return NR_RadioBearerId_Type { var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; //@sic R5s201060 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var boolean v_FailWhenK_LessN := false; var template NG_Request_Type v_RequestType := cr_NG_Request_Type('001'B); var boolean v_MsgRxd := false; var integer i; var default v_MyDefaultVar := null; timer t_Wait := 8.0; // @sic R5-204399, R5s210077 sic@ if (p_IwkN26 == SUPPORTED) { // @sic R5-204401 sic@ v_FailWhenK_LessN := true; // The only difference between 14a1 and 14b1 branch v_RequestType := cr_NG_Request_Type('010'B); // @sic R5-206290 sic@ } // always wait for PDU info, even if not 1st time v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); // @sic R5s210520 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfPDUSessionEstReq(); // Then activate store for any new msgs received - this works for Modification Reqs too v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); t_Wait.start; // @sic R5-204399 sic@ // make sure all the PDNs configured on EUTRA are changed in routing table @sic R5s210149, R5s210520 sic@ for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { f_IP_ChangeDrbMappingNR(IP, p_NR_CellId, cs_IP_QosFlowId(v_PduSessionInfoList[i].PDU_SessionId, v_PduSessionInfoList[i].QFI), v_PduSessionInfoList[i].PdnIndex); } // @sic R5s210520 sic@ // has a NAS msg already been received? if so, stop waiting if (f_NR5GC_MsgInDefault_GetNumOfPDUSessionEstReq() > 0) { v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOnePDUSessionEstReq(1); v_MsgRxd := true; t_Wait.stop; } else { alt { // @sic R5-204401 sic@ [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp // @sic R5s201413 sic@ { v_MsgRxd := true; t_Wait.stop; } [] t_Wait.timeout {} } } if (v_MsgRxd and ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request)) { // @sic R5s210520 sic@ if (p_FirstTimeS1ToN1 and p_IwkN26 == NOT_SUPPORTED) { f_NR5GC_IRAT_S1ToN1_SessionModification(p_NR_CellId, v_PduSessionInfoList, false, v_SRB); // @sic R5s201413 sic@ } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not expecting PDU Session Modification message"); } // reset flag as 1st msg now been processed v_MsgRxd := false; t_Wait.start; // restart timer in case there are now PDU Establishment messages alt { // @sic R5-204401 sic@ [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp // @sic R5s201413 sic@ { // update v_ReceivedAsp and msg store if received again @sic R5s210520 sic@ f_NR5GC_MsgInDefault_StorePDUSessionEstReq (v_ReceivedAsp); v_MsgRxd := true; t_Wait.stop; } [] t_Wait.timeout { // Do nothing as no (more) PDU Modification/Establishment messages received so carry on } } } // don't need this anymore so deactivate it before it needs reactivating deactivate(v_MyDefaultVar); // @sic R5s210520 sic@ if (v_MsgRxd and ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)) { // @sic R5s210520 sic@ f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsSameConnection, tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, "", v_FailWhenK_LessN, -, v_PduSessionInfoList, -, 1); // @sic R5-206290, R5s210149, R5s210520 sic@ v_SRB := tsc_NR_RbId_SRB2; // @sic R5-204401 sic@ // @sic R5-204401 sic@ if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(p_NR_CellId); f_NR5GC_RRC_Idle_Extension(p_NR_CellId, TESTMode_OFF, noRrcConnectionRelease); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong NAS Message Received"); } // in case there's no PDU messages at all, don't release the connection in the above functions, but do it here instead if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } return v_SRB; // @sic R5-204401 sic@ }

After Change:

function f_NR5GC_IRAT_S1ToN1_Steps14(NR_CellId_Type p_NR_CellId, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, boolean p_FirstTimeS1ToN1 := true, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC return NR_RadioBearerId_Type { var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; //@sic R5s201060 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var boolean v_FailWhenK_LessN := false; var template NG_Request_Type v_RequestType := cr_NG_Request_Type('001'B); var boolean v_MsgRxd := false; var integer i; var default v_MyDefaultVar := null; timer t_Wait := 8.0; // @sic R5-204399, R5s210077 sic@ if (p_IwkN26 == SUPPORTED) { // @sic R5-204401 sic@ v_FailWhenK_LessN := true; // The only difference between 14a1 and 14b1 branch v_RequestType := cr_NG_Request_Type('010'B); // @sic R5-206290 sic@ } // always wait for PDU info, even if not 1st time v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA); // @sic R5s210520 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfPDUSessionEstReq(); // Then activate store for any new msgs received - this works for Modification Reqs too v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); t_Wait.start; // @sic R5-204399 sic@ // make sure all the PDNs configured on EUTRA are changed in routing table @sic R5s210149, R5s210520 sic@ for (i := 0; i < lengthof (v_PduSessionInfoList); i := i + 1) { f_IP_ChangeDrbMappingNR(IP, p_NR_CellId, cs_IP_QosFlowId(v_PduSessionInfoList[i].PDU_SessionId, v_PduSessionInfoList[i].QFI), v_PduSessionInfoList[i].PdnIndex); } // @sic R5s210520 sic@ // has a NAS msg already been received? if so, stop waiting if (f_NR5GC_MsgInDefault_GetNumOfPDUSessionEstReq() > 0) { v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOnePDUSessionEstReq(1); v_MsgRxd := true; t_Wait.stop; } else { alt { // @sic R5-204401 sic@ [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp // @sic R5s201413 sic@ { v_MsgRxd := true; t_Wait.stop; } [] t_Wait.timeout {} } } if (v_MsgRxd and ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request)) { // @sic R5s210520 sic@ if (p_FirstTimeS1ToN1 and p_IwkN26 == NOT_SUPPORTED) { f_NR5GC_IRAT_S1ToN1_SessionModification(p_NR_CellId, v_PduSessionInfoList, false, v_SRB); // @sic R5s201413 sic@ v_SRB := tsc_NR_RbId_SRB2; //WA#R&S SRB2 is configured during PDU Session Modification so all NAS messages will come on SRB2 now } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not expecting PDU Session Modification message"); } // reset flag as 1st msg now been processed v_MsgRxd := false; t_Wait.start; // restart timer in case there are now PDU Establishment messages alt { // @sic R5-204401 sic@ [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp // @sic R5s201413 sic@ { // update v_ReceivedAsp and msg store if received again @sic R5s210520 sic@ f_NR5GC_MsgInDefault_StorePDUSessionEstReq (v_ReceivedAsp); v_MsgRxd := true; t_Wait.stop; } [] t_Wait.timeout { // Do nothing as no (more) PDU Modification/Establishment messages received so carry on } } } // don't need this anymore so deactivate it before it needs reactivating deactivate(v_MyDefaultVar); // @sic R5s210520 sic@ /*COMMENTED OUT if (v_MsgRxd and ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)) { // @sic R5s210520 sic@ f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsSameConnection, tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, "", v_FailWhenK_LessN, -, v_PduSessionInfoList, -, 1); // @sic R5-206290, R5s210149, R5s210520 sic@ v_SRB := tsc_NR_RbId_SRB2; // @sic R5-204401 sic@ // @sic R5-204401 sic@ if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(p_NR_CellId); f_NR5GC_RRC_Idle_Extension(p_NR_CellId, TESTMode_OFF, noRrcConnectionRelease); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong NAS Message Received"); }*/ //WA#R&S : R&S Change Start if (v_MsgRxd){ if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)){ f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsSameConnection, tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestType, -, -, -, -, -, -, "", v_FailWhenK_LessN, -, v_PduSessionInfoList, -, 1); // @sic R5-206290, R5s210149, R5s210520 sic@ v_SRB := tsc_NR_RbId_SRB2; // @sic R5-204401 sic@ // @sic R5-204401 sic@ if (pc_noOf_PDUsNewConnection >0 ) { f_NR_RRCRelease(p_NR_CellId); f_NR5GC_RRC_Idle_Extension(p_NR_CellId, TESTMode_OFF, noRrcConnectionRelease); } }else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong NAS Message Received"); } } //WA#R&S : R&S Change stop // in case there's no PDU messages at all, don't release the connection in the above functions, but do it here instead if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } return v_SRB; // @sic R5-204401 sic@ }