3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210785

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1829 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_INVITE_ReceiveRequest_Optional |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementaiton of this function, the timer t_Timer is started based on value of v_TimerControl. However, in the alt statement, this timer is stopped regardless whether it is started or not. This needs to be corrected |  |  |  |  |  |  |  |  |
| Summary of change: |  | Depenfing on whether the timer was started, called the statement t_Timer.stop; Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent TTCN implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.6.NR5GC, and other testcases that need Emergency PDU Session |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_IMS_INVITE_ReceiveRequest_Optional	2

# Corrections required

## Correction to function f_IMS_INVITE_ReceiveRequest_Optional

| Function name | f_IMS_INVITE_ReceiveRequest_Optional |
| --- | --- |
| Reason for change | In the current TTCN implementaiton of this function, the timer t_Timer is started based on value of v_TimerControl. However, in the alt statement, this timer is stopped regardless whether it is started or not. This needs to be corrected |
| Summary of change | Depenfing on whether the timer was started, called the statement t_Timer.stop; Please see below. |
| TTCN module | IMS_Procedures_CallControlCommon.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_INVITE_ReceiveRequest_Optional(template (present) SipUrl p_ExpectedCalleeUri, template (present) MessageHeader p_MessageHeader_Invite, template (present) MessageBody p_MessageBody, template (omit) float p_WaitDuration := omit, boolean p_IsReInvite := false, boolean p_IsEmergencyCallWithoutRegistration := false) runs on IMS_PTC return template (omit) IMS_DATA_REQ { var IMS_DATA_REQ v_IMS_DATA_REQ_ByRef; var INVITE_Request v_InviteRequest; var SipUrl v_ContactUrl; var boolean v_TimerControl := ispresent(p_WaitDuration); timer t_Timer; if (v_TimerControl) { t_Timer.start(valueof(p_WaitDuration)); } alt { [] a_IMS_ReceiveRequest(car_IMS_Invite_Request(cr_INVITE_Request(p_ExpectedCalleeUri, p_MessageHeader_Invite, p_MessageBody)), v_IMS_DATA_REQ_ByRef) { t_Timer.stop; v_InviteRequest := v_IMS_DATA_REQ_ByRef.Request.Invite; if (p_IsEmergencyCallWithoutRegistration) { // emergency call without registration @sic R5s130756 change 14 sic@ v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); /* @sic R5s150268 change 1 sic@ */ f_IMS_PTC_ImsInfo_Init(v_IMS_DATA_REQ_ByRef.RoutingInfo.UE_Address, v_IMS_DATA_REQ_ByRef.RoutingInfo.NW_Address); f_IMS_PortsAndSecurityConfig(f_SIP_ContactUrl_GetPort(v_ContactUrl)); /* @sic R5s150268 change 1 sic@ */ } if (not p_IsReInvite) { /* @sic R5s140020 change 8.2 sic@ */ f_IMS_PTC_ImsInfo_DialogInit(dialogRemote, v_InviteRequest.msgHeader); } else { if (not isvalue(f_IMS_PTC_ImsInfo_CseqGet(dialogRemote))) { /* @sic R5s140235 sic@ */ f_IMS_PTC_ImsInfo_CseqSet(dialogRemote, v_InviteRequest.msgHeader.cSeq.seqNumber); } } return v_IMS_DATA_REQ_ByRef; } [v_TimerControl] t_Timer.timeout {} } return omit; }

After Change:

function f_IMS_INVITE_ReceiveRequest_Optional(template (present) SipUrl p_ExpectedCalleeUri, template (present) MessageHeader p_MessageHeader_Invite, template (present) MessageBody p_MessageBody, template (omit) float p_WaitDuration := omit, boolean p_IsReInvite := false, boolean p_IsEmergencyCallWithoutRegistration := false) runs on IMS_PTC return template (omit) IMS_DATA_REQ { var IMS_DATA_REQ v_IMS_DATA_REQ_ByRef; var INVITE_Request v_InviteRequest; var SipUrl v_ContactUrl; var boolean v_TimerControl := ispresent(p_WaitDuration); timer t_Timer; if (v_TimerControl) { t_Timer.start(valueof(p_WaitDuration)); } alt { [] a_IMS_ReceiveRequest(car_IMS_Invite_Request(cr_INVITE_Request(p_ExpectedCalleeUri, p_MessageHeader_Invite, p_MessageBody)), v_IMS_DATA_REQ_ByRef) { if (v_TimerControl) {//WA#WI862307 t_Timer.stop; } v_InviteRequest := v_IMS_DATA_REQ_ByRef.Request.Invite; if (p_IsEmergencyCallWithoutRegistration) { // emergency call without registration @sic R5s130756 change 14 sic@ v_ContactUrl := f_MessageHeader_GetContactSipUrl(v_InviteRequest.msgHeader); /* @sic R5s150268 change 1 sic@ */ f_IMS_PTC_ImsInfo_Init(v_IMS_DATA_REQ_ByRef.RoutingInfo.UE_Address, v_IMS_DATA_REQ_ByRef.RoutingInfo.NW_Address); f_IMS_PortsAndSecurityConfig(f_SIP_ContactUrl_GetPort(v_ContactUrl)); /* @sic R5s150268 change 1 sic@ */ } if (not p_IsReInvite) { /* @sic R5s140020 change 8.2 sic@ */ f_IMS_PTC_ImsInfo_DialogInit(dialogRemote, v_InviteRequest.msgHeader); } else { if (not isvalue(f_IMS_PTC_ImsInfo_CseqGet(dialogRemote))) { /* @sic R5s140235 sic@ */ f_IMS_PTC_ImsInfo_CseqSet(dialogRemote, v_InviteRequest.msgHeader.cSeq.seqNumber); } } return v_IMS_DATA_REQ_ByRef; } [v_TimerControl] t_Timer.timeout {} } return omit; }