3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210887

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0144 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Positioning Functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current implementation of the functions: f_POS_NR_CheckSignalsGps f_POS_NR_CheckSignalsGalileo f_POS_NR_CheckSignalsGlonass f_POS_NR_CheckSignalsQzss throws a FAIL verdict even if the GNSS signals reported in the LPP Provide location information message match the pics parameters. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The implementation of the functions f_POS_NR_CheckSignalsGps f_POS_NR_CheckSignalsGalileo f_POS_NR_CheckSignalsGlonass f_POS_NR_CheckSignalsQzss has to be modified to compare the GNSS reported signals with UE capabilities once all individual signals are evaluated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.2.15s.NR5GC, 9.3.4.4.15s.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

Change 1	5

Change 2	6

Change 3	7

Change 4	8

## Change 1

| Function name | f_POS_NR_CheckSignalsGps |
| --- | --- |
| Reason for change | Test case fails even if the GNSS signals are correctly reported |
| Summary of change | Final condition has to be evaluated after all reported GNSS signals are firstly checked |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before change:

function f_POS_NR_CheckSignalsGps(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_A_GPS_L1C_A_Reported := false;

var boolean v_A_GPS_L1C_Reported := false;

var boolean v_A_GPS_L2C_Reported := false;

var boolean v_A_GPS_L5_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_A_GPS_L1C_A_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_A_GPS_L1C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_A_GPS_L2C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_A_GPS_L5_Reported := true;

}

if (not((v_A_GPS_L1C_A_Reported == pc_A_GPS_L1C_A) and (v_A_GPS_L1C_Reported == pc_A_GPS_L1C) and (v_A_GPS_L2C_Reported == pc_A_GPS_L2C) and (v_A_GPS_L5_Reported == pc_A_GPS_L5)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GPS signals not reported correctly");

}

}

}

After change:

function f_POS_NR_CheckSignalsGps(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_A_GPS_L1C_A_Reported := false;

var boolean v_A_GPS_L1C_Reported := false;

var boolean v_A_GPS_L2C_Reported := false;

var boolean v_A_GPS_L5_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_A_GPS_L1C_A_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_A_GPS_L1C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_A_GPS_L2C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_A_GPS_L5_Reported := true;

}

}

if (not((v_A_GPS_L1C_A_Reported == pc_A_GPS_L1C_A) and (v_A_GPS_L1C_Reported == pc_A_GPS_L1C) and (v_A_GPS_L2C_Reported == pc_A_GPS_L2C) and (v_A_GPS_L5_Reported == pc_A_GPS_L5)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GPS signals not reported correctly");

}

}

## Change 2

| Function name | f_POS_NR_CheckSignalsGlonass |
| --- | --- |
| Reason for change | Test case fails even if the GNSS signals are correctly reported |
| Summary of change | Final condition has to be evaluated after all reported GNSS signals are firstly checked |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before change:

function f_POS_NR_CheckSignalsGlonass(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GLONASS_G1_Reported := false;

var boolean v_GLONASS_G2_Reported := false;

var boolean v_GLONASS_G3_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GLONASS_G1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GLONASS_G2_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GLONASS_G3_Reported := true;

}

if (not((v_GLONASS_G1_Reported == pc_GLONASS_G1) and (v_GLONASS_G2_Reported == pc_GLONASS_G2) and (v_GLONASS_G3_Reported == pc_GLONASS_G3)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

}

After change:

function f_POS_NR_CheckSignalsGlonass(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GLONASS_G1_Reported := false;

var boolean v_GLONASS_G2_Reported := false;

var boolean v_GLONASS_G3_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GLONASS_G1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GLONASS_G2_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GLONASS_G3_Reported := true;

}

}

if (not((v_GLONASS_G1_Reported == pc_GLONASS_G1) and (v_GLONASS_G2_Reported == pc_GLONASS_G2) and (v_GLONASS_G3_Reported == pc_GLONASS_G3)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

## Change 3

| Function name | f_POS_NR_CheckSignalsGalileo |
| --- | --- |
| Reason for change | Test case fails even if the GNSS signals are correctly reported |
| Summary of change | Final condition has to be evaluated after all reported GNSS signals are firstly checked |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before change:

function f_POS_NR_CheckSignalsGalileo(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GALILEO_E1_Reported := false;

var boolean v_GALILEO_E5a_Reported := false;

var boolean v_GALILEO_E5b_Reported := false;

var boolean v_GALILEO_E6_Reported := false;

var boolean v_GALILEO_E5aE5b_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GALILEO_E1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GALILEO_E5a_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GALILEO_E5b_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_GALILEO_E6_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(4))) {

v_GALILEO_E5aE5b_Reported := true;

}

if (not((v_GALILEO_E1_Reported == pc_GALILEO_E1) and (v_GALILEO_E5a_Reported == pc_GALILEO_E5a) and (v_GALILEO_E5b_Reported == pc_GALILEO_E5b) and

(v_GALILEO_E6_Reported == pc_GALILEO_E6) and (v_GALILEO_E5aE5b_Reported == pc_GALILEO_E5aE5b)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

}

After change:

function f_POS_NR_CheckSignalsGalileo(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GALILEO_E1_Reported := false;

var boolean v_GALILEO_E5a_Reported := false;

var boolean v_GALILEO_E5b_Reported := false;

var boolean v_GALILEO_E6_Reported := false;

var boolean v_GALILEO_E5aE5b_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GALILEO_E1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GALILEO_E5a_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GALILEO_E5b_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_GALILEO_E6_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(4))) {

v_GALILEO_E5aE5b_Reported := true;

}

}

if (not((v_GALILEO_E1_Reported == pc_GALILEO_E1) and (v_GALILEO_E5a_Reported == pc_GALILEO_E5a) and (v_GALILEO_E5b_Reported == pc_GALILEO_E5b) and

(v_GALILEO_E6_Reported == pc_GALILEO_E6) and (v_GALILEO_E5aE5b_Reported == pc_GALILEO_E5aE5b)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GALILEO signals not reported correctly");

}

}

## Change 4

| Function name | f_POS_NR_CheckSignalsQzss |
| --- | --- |
| Reason for change | Test case fails even if the GNSS signals are correctly reported |
| Summary of change | Final condition has to be evaluated after all reported GNSS signals are firstly checked |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before change:

function f_POS_NR_CheckSignalsQzss(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_QZSS_QZS_L1_Reported := false;

var boolean v_QZSS_QZS_L1C_Reported := false;

var boolean v_QZSS_QZS_L2C_Reported := false;

var boolean v_QZSS_QZS_L5_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_QZSS_QZS_L1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_QZSS_QZS_L1C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_QZSS_QZS_L2C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_QZSS_QZS_L5_Reported := true;

}

if (not ((v_QZSS_QZS_L1_Reported == pc_QZSS_QZS_L1) and (v_QZSS_QZS_L1C_Reported == pc_QZSS_QZS_L1C) and

(v_QZSS_QZS_L2C_Reported == pc_QZSS_QZS_L2C) and (v_QZSS_QZS_L5_Reported == pc_QZSS_QZS_L5)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "QZSS signals not reported correctly");

}

}

}

After change:

function f_POS_NR_CheckSignalsQzss(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_QZSS_QZS_L1_Reported := false;

var boolean v_QZSS_QZS_L1C_Reported := false;

var boolean v_QZSS_QZS_L2C_Reported := false;

var boolean v_QZSS_QZS_L5_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_QZSS_QZS_L1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_QZSS_QZS_L1C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_QZSS_QZS_L2C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_QZSS_QZS_L5_Reported := true;

}

}

if (not ((v_QZSS_QZS_L1_Reported == pc_QZSS_QZS_L1) and (v_QZSS_QZS_L1C_Reported == pc_QZSS_QZS_L1C) and

(v_QZSS_QZS_L2C_Reported == pc_QZSS_QZS_L2C) and (v_QZSS_QZS_L5_Reported == pc_QZSS_QZS_L5)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "QZSS signals not reported correctly");

}

}