3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210956

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0712 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS over 5GS test case 7.33 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS call control test case 7.33 to the IMS/NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS call control test case 7.33 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.33.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Change 2 needs a prose change. Draft prose CR attached. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

1	Branches executed	6

2	Execution Log Files	6

2.1	Samsung SM-G986U	6

3	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.33.NR5GC which is part of the iWD-TTCN3-B2020-09_D21wk24 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Hans Rohnert
	hans.rohnert@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.33.NR5GC

ATS Version:	iWD-TTCN3-B2020-09_D21wk24

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Samsung SM-G986-U

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_7_33_NR5GC_IMS1 |
| --- | --- |
| Reason for change | In step 19, 200 OK, the timer tag needs to be in the Supported rather than in the Require header, see TS 34.229-5 resp agreed prose CR R5-213545. Note that the UE is allowed to also add it to the Require header but this is not to be checked. |
| Summary of change | Replaced Require by Supported header in the UL message. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_7_33_NR5GC_IMS1() runs on IMS_PTC

{ // Session Timer / MT Call / Remote end chooses UE as refresher / 5GS

var Method_List v_MethodList := {"INVITE", "ACK", "OPTIONS", "CANCEL", "BYE"};

var template (value) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var SipUrl v_ContactUrlUE;

var SipUrl v_ContactUrlSS;

var integer v_DeltaSec := 1800;

var integer i;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

f_IMS_TestBody_Set(true);

f_IMS_CC_StartCall(IPCAN_MT_SpeechCall);

// @siclog "Step 1 - 8" IPCAN siclog@

// @siclog "Step 9" siclog@  @sic R5-213545: A.5.2 instead of A.5.1 sic@

v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_SDP_WithoutPreconditions);

v_InviteRequestWithSdp.Invite.msgHeader.sessionExpires := cs_SessionExpires(v_DeltaSec, "uas");

v_InviteRequestWithSdp.Invite.msgHeader.allow := cs_Allow(v_MethodList);

IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite));

v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader);

// @siclog "Step 10 - 16" IPCAN siclog@

f_IMS_MTCallSetup_NR5GC_A_5_2_Step2_8(valueof(v_InviteRequestWithSdp));    // @sic R5-213545: A.5.2 instead of A.5.1 sic@

// @siclog "Steps 17" siclog@

f_UT_AnswerCall(MMI);

// @siclog "Steps 18" Void siclog@

// @siclog "Steps 19 - 20" siclog@

f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(valueof(v_InviteRequestWithSdp), cr_Require_timer, -, v_DeltaSec, "uas");

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19");

// @siclog "Steps 21 - 26" siclog@

for (i := 0; i < 2; i := i + 1) {

f_IMS_CallWithSessionTimer_MORefresh(v_ContactUrlSS, v_DeltaSec, "uac");  /* @sic R5-210124: common function sic@ */

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21/24");

}

// @siclog "Step 20 - 23" siclog@

v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);

f_IMS_CallReleaseMT(v_ContactUrlUE);

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MT_SpeechCall);}

After change:

function f_TC_7_33_NR5GC_IMS1() runs on IMS_PTC

{ // Session Timer / MT Call / Remote end chooses UE as refresher / 5GS

var Method_List v_MethodList := {"INVITE", "ACK", "OPTIONS", "CANCEL", "BYE"};

var template (value) IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var SipUrl v_ContactUrlUE;

var SipUrl v_ContactUrlSS;

var integer v_DeltaSec := 1800;

var integer i;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

f_IMS_TestBody_Set(true);

f_IMS_CC_StartCall(IPCAN_MT_SpeechCall);

// @siclog "Step 1 - 8" IPCAN siclog@

// @siclog "Step 9" siclog@  @sic R5-213545: A.5.2 instead of A.5.1 sic@

v_InviteRequestWithSdp := f_IMS_MTCallSetup_BuildInviteRequestWithSdp(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_SDP_WithoutPreconditions);

v_InviteRequestWithSdp.Invite.msgHeader.sessionExpires := cs_SessionExpires(v_DeltaSec, "uas");

v_InviteRequestWithSdp.Invite.msgHeader.allow := cs_Allow(v_MethodList);

IMS_Client.send(cas_IMS_Invite_Request(v_InviteRequestWithSdp.RoutingInfo, v_InviteRequestWithSdp.Invite));

v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_InviteRequestWithSdp.Invite.msgHeader);

// @siclog "Step 10 - 16" IPCAN siclog@

f_IMS_MTCallSetup_NR5GC_A_5_2_Step2_8(valueof(v_InviteRequestWithSdp));    // @sic R5-213545: A.5.2 instead of A.5.1 sic@

// @siclog "Steps 17" siclog@

f_UT_AnswerCall(MMI);

// @siclog "Steps 18" Void siclog@

// @siclog "Steps 19 - 20" siclog@

f_IMS_MTCallSetupWithSessionTimer_Receive200OK_SendACK(valueof(v_InviteRequestWithSdp), -, cr_Supported_timer, v_DeltaSec, "uas");

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 19");

// @siclog "Steps 21 - 26" siclog@

for (i := 0; i < 2; i := i + 1) {

f_IMS_CallWithSessionTimer_MORefresh(v_ContactUrlSS, v_DeltaSec, "uac");  /* @sic R5-210124: common function sic@ */

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21/24");

}

// @siclog "Step 20 - 23" siclog@

v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);

f_IMS_CallReleaseMT(v_ContactUrlUE);

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MT_SpeechCall);

}

## Change 2

| Function name | f_IMS_CallWithSessionTimer_MORefresh |
| --- | --- |
| Reason for change | For steps 22 and 25, TS 34.229-5 wrongly puts the timer tag into the Supported rather than into the Require header (draft prose CR attached) |
| Summary of change | Replaced Supported by Require header in DL. |
| TTCN module | IMS_CC_SessionTimerCommon.ttcn |
| MCC160 Comment |  |

Before change:

function f_IMS_CallWithSessionTimer_MORefresh(SipUrl p_ContactUrl,

integer p_DeltaSecValue,

charstring p_RefresherValue) runs on IMS_PTC

{ /* @sic R5-210124: 200 OK and ACK added to this function sic@ */

var IMS_DATA_REQ v_IMS_DATA_REQ;

var IMS_RoutingInfo_Type v_RoutingInfo_DL;

var template (present) MessageHeader v_MessageHeaderExpected;

var INVITE_Request v_InviteRequest;

v_MessageHeaderExpected := f_IMS_InviteRequest_MessageHeaderRX(p_ContactUrl, -, A_2_1_A5);

v_MessageHeaderExpected.sessionExpires := cr_SessionExpires;

// Receive INVITE

v_IMS_DATA_REQ := f_IMS_CallWithSessionTimer_ReceiveRefreshRequest(p_DeltaSecValue, car_IMS_Invite_Request(cr_INVITE_Request(p_ContactUrl, v_MessageHeaderExpected, cr_MessageBody_SDP)));

v_RoutingInfo_DL := v_IMS_DATA_REQ.RoutingInfo;

f_IMS_A_2_1_Invite_CommonChecks(v_IMS_DATA_REQ, A_2_1_A5);

f_IMS_SIP_DecodeMatchAndCheckSDP(v_IMS_DATA_REQ.Request.Invite.messageBody.sdpMessageBody, cr_SDP_Message_Common, sameOriginAsPreviousSDP);

v_InviteRequest := v_IMS_DATA_REQ.Request.Invite;

f_IMS_CallWithSessionTimer_CheckSessionExpires(v_InviteRequest.msgHeader.sessionExpires, p_DeltaSecValue, p_RefresherValue);

// Send 200 OK

f_IMS_CallWithSessionTimer_Send200OK(v_InviteRequest.msgHeader, v_RoutingInfo_DL, -, -, cs_Supported_timer, cs_SessionExpires(p_DeltaSecValue, "uac"), -, f_SIP_SipUrl2Str(p_ContactUrl));

// Receive ACK

f_IMS_CallWithSessionTimer_ReceiveACK(v_InviteRequest, p_ContactUrl);

}

After change:

function f_IMS_CallWithSessionTimer_MORefresh(SipUrl p_ContactUrl,

integer p_DeltaSecValue,

charstring p_RefresherValue) runs on IMS_PTC

{ /* @sic R5-210124: 200 OK and ACK added to this function sic@ */

var IMS_DATA_REQ v_IMS_DATA_REQ;

var IMS_RoutingInfo_Type v_RoutingInfo_DL;

var template (present) MessageHeader v_MessageHeaderExpected;

var INVITE_Request v_InviteRequest;

v_MessageHeaderExpected := f_IMS_InviteRequest_MessageHeaderRX(p_ContactUrl, -, A_2_1_A5);

v_MessageHeaderExpected.sessionExpires := cr_SessionExpires;

// Receive INVITE

v_IMS_DATA_REQ := f_IMS_CallWithSessionTimer_ReceiveRefreshRequest(p_DeltaSecValue, car_IMS_Invite_Request(cr_INVITE_Request(p_ContactUrl, v_MessageHeaderExpected, cr_MessageBody_SDP)));

v_RoutingInfo_DL := v_IMS_DATA_REQ.RoutingInfo;

f_IMS_A_2_1_Invite_CommonChecks(v_IMS_DATA_REQ, A_2_1_A5);

f_IMS_SIP_DecodeMatchAndCheckSDP(v_IMS_DATA_REQ.Request.Invite.messageBody.sdpMessageBody, cr_SDP_Message_Common, sameOriginAsPreviousSDP);

v_InviteRequest := v_IMS_DATA_REQ.Request.Invite;

f_IMS_CallWithSessionTimer_CheckSessionExpires(v_InviteRequest.msgHeader.sessionExpires, p_DeltaSecValue, p_RefresherValue);

// Send 200 OK

f_IMS_CallWithSessionTimer_Send200OK(v_InviteRequest.msgHeader, v_RoutingInfo_DL, -, cs_Require_timer, -, cs_SessionExpires(p_DeltaSecValue, "uac"), -, f_SIP_SipUrl2Str(p_ContactUrl));

// Receive ACK

f_IMS_CallWithSessionTimer_ReceiveACK(v_InviteRequest, p_ContactUrl);

}

# Branches executed

n/a

# Execution Log Files

## Samsung SM-G986U

The Samsung SM-G986U UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Rohde & Schwarz: 7_33_RS_Samsung_ttcn_tli

# References

| [1] | R5s210679: Supporting information for agreement of TC 7.33 |
| --- | --- |