3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211281

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2043 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 8.1.6.1.4.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In test case 8.1.6.1.4.4 operation valueof is applied to a template which have not defined values “*” causing a runtime error. This occurs with 5th input parameter of f_NR5GC_RRCReestablishment_Steps3To7 which actually is of type template, valueof operation is not required in this case, it can be removed. From ETSI ES 201 873-1: 15.10 Valueof Operation … Restrictions In addition to the general static rules of TTCN-3 given in clause 5, the following restrictions apply: a) The template shall be completely initialized and resolve to a specific value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed operation valueof as it is not required being input parameter of type template as well as causing a runtime error as template is not resolving to specific values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation may FAIL a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1. Correction to fl_TC_8_1_6_1_4_4_TestBody ()	3

## 1. Correction to fl_TC_8_1_6_1_4_4_TestBody ()

| Function name | Correction to fl_TC_8_1_6_1_4_4_TestBody () |
| --- | --- |
| Reason for change | In test case 8.1.6.1.4.4 operation valueof is applied to a template which have not defined values “*” causing a runtime error. |
| Summary of change | Removed operation valueof as it is not required being input parameter of type template as well as causing a runtime error as template is not resolving to specific values. |
| TTCN module | Removed operation valueof as it is not required being input parameter of type template and causing a runtime error as values are not resolving to specific values. |
| MCC160 Comment |  |

Before Change:

<<< SKIPPED CODE >>> //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReestablishment message on Cell 2. //Check: Does the UE send an RRCReestablishmentComplete message with connEstFailInfoAvailable on Cell 2? v_RRCReestablishmentComplete_v1610_IEs := { ue_MeasurementsAvailable_r16 := cr_UE_MeasurementsAvailable_r16(-, -, -, true_, true_), // @sic R5-215681 sic@ nonCriticalExtension := * }; f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell1, nr_Cell2, -, -, valueof(v_RRCReestablishmentComplete_v1610_IEs)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); <<< SKIPPED CODE >>>

After Change:

<<< SKIPPED CODE >>> //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReestablishment message on Cell 2. //Check: Does the UE send an RRCReestablishmentComplete message with connEstFailInfoAvailable on Cell 2? v_RRCReestablishmentComplete_v1610_IEs := { ue_MeasurementsAvailable_r16 := cr_UE_MeasurementsAvailable_r16(-, -, -, true_, true_), // @sic R5-215681 sic@ nonCriticalExtension := * }; f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell1, nr_Cell2, -, -, v_RRCReestablishmentComplete_v1610_IEs); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); <<< SKIPPED CODE >>>