3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211329

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2082 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.1.5.1.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to Rel-16 TS 24.501 cl. 5.5.1.2.5, after receving a REGISTRATION REJECT with cause #73 the UE shall delete the ngKSI. Therefore the following REGISTRATION REQUEST message should be transmited without integrity protection in Step 14 for a Rel-16 UE. #73 (Serving network not authorized). This cause value received from a cell belonging to an SNPN is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.5.1.2.7. The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete any 5G-GUTI, last visited registered TAI, TAI list and ngKSI. Currently the check at step 14 is incorrect as it sends the message with integrity protection for a Rel16 UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the UE release check at step 14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_5_1_8_TestBody |
| --- | --- |
| Reason for change | According to Rel-16 TS 24.501 cl. 5.5.1.2.5, after receving a REGISTRATION REJECT with cause #73 the UE shall delete the ngKSI. Therefore the following REGISTRATION REQUEST message should be transmited without integrity protection in Step 14 for a Rel-16 UE. #73 (Serving network not authorized). This cause value received from a cell belonging to an SNPN is considered as an abnormal case and the behaviour of the UE is specified in subclause 5.5.1.2.7. The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete any 5G-GUTI, last visited registered TAI, TAI list and ngKSI. Currently the check at step 14 is incorrect as it sends the message with integrity protection for a Rel16 UE |
| Summary of change | Correct the UE release check at step 14 |
| TTCN module | Initial_Registration_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_9_1_5_1_8_TestBody() runs on NR5GC_PTC { ……………. //@siclog "Step 13" siclog@ //The SS configures-NGC Cell A as "Serving Cell"? f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 14" siclog@ //Check: Does the UE perform Registration procedure on NGC Cell A as specified in TS 38.508-1 [4] subclause 4.5.2? v_AccessStratumRelease:= f_NR_MobileInfo_NRCap_Get_AccessStratumRelease();//@sic R5s210768 sic@ if (v_AccessStratumRelease == rel16){//@sic R5s210768 sic@ v_SecurityHeader:= tsc_SHT_IntegrityProtected;//@sic R5s210768 sic@ v_RegType:= Initial_Secure;//@sic R5s210768 sic@ } else { v_SecurityHeader:= tsc_SHT_NoSecurityProtection;//@sic R5s210768 sic@ v_RegType:= Initial_NoSecurity;//@sic R5s210768 sic@ } v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, v_SecurityHeader);//@sic R5 200154 sic@//@sic R5s210768 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_RegType)) {//@sic R5s210768 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); …… }

After Change

function fl_TC_9_1_5_1_8_TestBody() runs on NR5GC_PTC { …………. //@siclog "Step 13" siclog@ //The SS configures-NGC Cell A as "Serving Cell"? f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 14" siclog@ //Check: Does the UE perform Registration procedure on NGC Cell A as specified in TS 38.508-1 [4] subclause 4.5.2? v_AccessStratumRelease:= f_NR_MobileInfo_NRCap_Get_AccessStratumRelease();//@sic R5s210768 sic@ if (v_AccessStratumRelease == rel15){//@sic R5s210768 sic@ //KS R5s21xxx v_SecurityHeader:= tsc_SHT_IntegrityProtected;//@sic R5s210768 sic@ v_RegType:= Initial_Secure;//@sic R5s210768 sic@ } else { v_SecurityHeader:= tsc_SHT_NoSecurityProtection;//@sic R5s210768 sic@ v_RegType:= Initial_NoSecurity;//@sic R5s210768 sic@ } v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, v_SecurityHeader);//@sic R5 200154 sic@//@sic R5s210768 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_RegType)) {//@sic R5s210768 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); …… }