3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211393

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1858 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The R5s201052 introduced 100ms delay in steps 9-10 in order to combat the situation where UL grant is disabled with explicit timing which gets activated after enabling the UL grant on SR with timing NOW. Thus leading to situation where UL grant is stopped when new RRC connection is expected. The solution works, however we propose a more robust enhancement that will avoid artificial delay (f_Delay(0.100)) introduced by R5s201052, will increase readability of the step 9-10 implementation and will make the usability of f_NR_RRCRelease_Local more universal by extending it of the return parameter carrying the timing used for the essential SS configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change 1 extends the f_NR_RRCRelease_Local of return parameter to provide timing of SS configurations. - Change 2 removes the artificial delay of 100ms in steps 9-10 and ensures that UL grant is started on SR after all the SS configuration performed by f_NR_RRCRelease_Local is completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Less robust and readable implementation of steps 9-10 in test 9.1.7.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.7.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR_RRCRelease_Local |
| --- | --- |
| Reason for change | f_NR_RRCRelease_Local needs to be extended of return parameter for providing the timing information of the last SS configuration. |
| Summary of change | f_NR_RRCRelease_Local has been extended of return parameter for providing the timing information of the last SS configuration. |
| TTCN module | 9_1_7\Service_Request_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR_RRCRelease_Local(NR_CellId_Type p_NR_CellId, SubFrameTiming_Type p_TimingAtT, template (omit) boolean p_CnfFlag := omit ) runs on NR_BASE_PTC { /* NOTE: the function schedules all steps to release and reconfigure the SRBs and (default) DRB * => this will in fact happen in the future */ /// @sic R5s190984: added p_CnfFlag sic@ var SubFrameTiming_Type v_Timing := p_TimingAtT; var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_CellId); /* Step 1 At T: Send RRCRelease(done already), stop UL grants */ f_NR_ULGrantConfiguration_Stop(p_NR_CellId, cs_TimingInfo_NR(v_Timing)); /* Step 2 at T+5ms: release security */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 5); f_NR_SS_RRC_ReleaseSecurity(p_NR_CellId, cs_TimingInfo_NR(v_Timing)); /* Step 3 at T+10ms: Release DRX configuration at the SS. */ //@sic R5-197222 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 10); f_NR_SS_DrxCtrlConfig(p_NR_CellId, cs_NR_DrxCtrl_None, cs_TimingInfo_NR(v_Timing)); /* Step 4: At T + 15ms: Release measurement gap configuration at the SS. */ //@sic R5-197222 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 15); f_NR_SS_MeasGapCtrlConfig(p_NR_CellId, cs_NR_MeasGapCtrl_None, cs_TimingInfo_NR(v_Timing)); /* Step 5 at T+55ms: Release SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 55); f_NR_SS_SRBs_DRBs_Release(p_NR_CellId, cs_TimingInfo_NR(v_Timing), v_DrbIdList); /* Step 6 at T+60ms: Configure SRBs */ // DRBs to be reconfigure during RRCReconfiguration v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 60); f_NR_SS_SRBs_DRBs_Config(p_NR_CellId, cs_TimingInfo_NR(v_Timing), omit, p_CnfFlag); //@sic R5s190109 change 23 sic@ }

After Change

function f_NR_RRCRelease_Local(NR_CellId_Type p_NR_CellId, SubFrameTiming_Type p_TimingAtT, template (omit) boolean p_CnfFlag := omit ) runs on NR_BASE_PTC return SubFrameTiming_Type { /* NOTE: the function schedules all steps to release and reconfigure the SRBs and (default) DRB * => this will in fact happen in the future */ /// @sic R5s190984: added p_CnfFlag sic@ var SubFrameTiming_Type v_Timing var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_CellId); /* Step 1 At T: Send RRCRelease(done already), stop UL grants */ f_NR_ULGrantConfiguration_Stop(p_NR_CellId, cs_TimingInfo_NR(v_Timing)); /* Step 2 at T+5ms: release security */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 5); f_NR_SS_RRC_ReleaseSecurity(p_NR_CellId, cs_TimingInfo_NR(v_Timing)); /* Step 3 at T+10ms: Release DRX configuration at the SS. */ //@sic R5-197222 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 10); f_NR_SS_DrxCtrlConfig(p_NR_CellId, cs_NR_DrxCtrl_None, cs_TimingInfo_NR(v_Timing)); /* Step 4: At T + 15ms: Release measurement gap configuration at the SS. */ //@sic R5-197222 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 15); f_NR_SS_MeasGapCtrlConfig(p_NR_CellId, cs_NR_MeasGapCtrl_None, cs_TimingInfo_NR(v_Timing)); /* Step 5 at T+55ms: Release SRBs and DRBs */ v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 55); f_NR_SS_SRBs_DRBs_Release(p_NR_CellId, cs_TimingInfo_NR(v_Timing), v_DrbIdList); /* Step 6 at T+60ms: Configure SRBs */ // DRBs to be reconfigure during RRCReconfiguration v_Timing := f_SubFrameTiming_AddMilliSeconds(p_TimingAtT, 60); f_NR_SS_SRBs_DRBs_Config(p_NR_CellId, cs_TimingInfo_NR(v_Timing), omit, p_CnfFlag); //@sic R5s190109 change 23 sic@ return v_Timing; }

## Change 2

| Function name | fl_TC_9_1_7_1_Body |
| --- | --- |
| Reason for change | In order to avoid artificial delay of 100ms used in step 9-10 the timing information provided by f_NR_RRCRelease_Local is used to start the UL grant on SR after all the SS configuration from f_NR_RRCRelease_Local is completed. |
| Summary of change | Removed f_Delay(0.100) from step 9-10 implementation and started UL grant on SR after f_NR_RRCRelease_Local is completed. |
| TTCN module | 9_1_7\Service_Request_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

//@siclog "Step 8F - 8H" siclog@ //@sic R5-199099, R5-201241 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B)) { //@sic R5s200087 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } //@siclog "Step 9 - 10" siclog@ //@sic R5-199099 sic@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0)); alt{ []SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected, cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@ cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@ omit,omit,*)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry"); } []t_T3517.timeout { f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA), tsc_CnfReq); //@sic R5s200087, R5s200662 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); f_Delay(0.100); //@sic R5s201052 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); //@sic R5s200087 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else //@sic R5s200087 sic@ { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } } } } //For loop //@siclog "Step 11" siclog@

After Change

//@siclog "Step 8F - 8H" siclog@ //@sic R5-199099, R5-201241 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B)) { //@sic R5s200087 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } //@siclog "Step 9 - 10" siclog@ //@sic R5-199099 sic@ for (v_Count := 1; v_Count < 5; v_Count := v_Count + 1) { t_T3517.start(f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 15.0)); alt{ []SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected, cr_NG_SERVICE_REQUEST('0001'B, //@sic R5-197076 sic@ cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, cr_ULDataStatus(-,-), //@sic R5-199099, R5s200087 sic@ omit,omit,*)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service REQ received before T3517 expiry"); } []t_T3517.timeout { v_TimingDL := f_NR_RRCRelease_Local(v_NGC_NASCellA, f_NR_GetNextSendOccasion_DL(v_NGC_NASCellA), tsc_CnfReq); //@sic R5s200087, R5s200662 sic@ f_NR_ULGrantConfiguration_Start(v_NGC_NASCellA, cs_TimingInfo_NR(v_TimingDL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //REMOVED f_Delay(0.100); //@sic R5s201052 sic@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); //@sic R5s200087 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI_A, '0001'B, -, -)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } v_Result := false; //@sic R5s200087 sic@ if(v_PSI <8){ //@sic R5s200087 sic@ v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi0_7, 7-v_PSI); } else //@sic R5s200087 sic@ { v_Result := f_Bitstring_BitIsSet(v_ReceivedMsg.Pdu.PiggybackedPduList[0].Msg.service_Request.ulDataStatus.psi8_15, 15-v_PSI); // @sic R5s200757 sic@ } if(not v_Result) //@sic R5s200087 sic@ { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "UL Data status in Service Request Message Failed"); } } } } //For loop //@siclog "Step 11" siclog@