3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211524

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2146 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.4.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation after Step 11 function f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2)); is called, because the IP handling associated with this needs to be restarted. However, in the FOR loop at Step 12, not only the emergency PDU session is released, the IMS PDU session is always released which is inline with Prose. But the TTCN is not releasing the IP handling associated with IMS PDN. This needs to be corrected. In the current TTCN implementation, at Step 15 the function fl_EmergencyCallAndRelease is called. Within this function when the emergency call is released, the UE may transmit a PDU Session Release REQ as there is no reason for the UE to keep this PDU Session active (This was solely established for emergency call). This also needs to be handled. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2));, called f_IP_Handling_StopPDN(IP, cs_IP_StopAllPDNs); Instead of calling the fucntion fl_EmergencyCallAndRelease, called individual functions in the main test body. Created a default handler function to handle the incoming PDU Session Release REQ. and activated it before the call to function f_NR5GC_EmergencyCallRelease. Please see below for detailed change description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Corrections required	4

1.1	Correction to function fl_TC_11_4_4_NR5GC_TestBody	4

1.2	function a_PDU_Session_Release_REQ	9

# Corrections required

## Correction to function fl_TC_11_4_4_NR5GC_TestBody

| Function name | fl_TC_11_4_4_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation after Step 11 function f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2)); is called, because the IP handling associated with this needs to be restarted. However, in the FOR loop at Step 12, not only the emergency PDU session is released, the IMS PDU session is always released which is inline with Prose. But the TTCN is not releasing the IP handling associated with IMS PDN. This needs to be corrected. In the current TTCN implementation, at Step 15 the function fl_EmergencyCallAndRelease is called. Within this function when the emergency call is released, the UE may transmit a PDU Session Release REQ as there is no reason for the UE to keep this PDU Session active (This was solely established for emergency call). This also needs to be handled. A Prose CR will be raised for this. |
| Summary of change | Instead of calling f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2));, called f_IP_Handling_StopPDN(IP, cs_IP_StopAllPDNs); Instead of calling the fucntion fl_EmergencyCallAndRelease, called individual functions in the main test body. Created a default handler function to handle the incoming PDU Session Release REQ. and activated it before the call to function f_NR5GC_EmergencyCallRelease. Please see below for detailed change description |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

After Change:

function fl_TC_11_4_4_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var O1_Type v_PDUSessionIdToRelease; var PDUSessionInfoList_Type v_PDUSessionInfo; var integer i; var integer v_NoOfSessions; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var default v_MyDefaultVar; //WA#11_4_4 // @siclog "Step 1" siclog@ // The SS configures: // - NR Cell 11 as "Serving cell" // - NR Cell 1 as "Non-Suitable "Off" cell". f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // @siclog "Steps 2 - 4" siclog@ // The UE transmits an RRCSetupRequest message. // SS transmit an RRCSetup message. // The UE transmits an RRCSetupComplete message to confirm the successful completion of the connection establishment and // a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?, tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 5" siclog@ // SS sends a REGISTRATION REJECT message containing 5GMM cause value = #22 (Congestion). SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_NoSecurityProtection, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 6" siclog@ // The SS transmits an RRCConnectionRelease message. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 7" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) f_UT_RequestIMSEmergencyCall(UT, px_EmergencyCallNumber); //@siclog "Step 8" siclog@ //@sic R5-213508 sic@ //Check: Does the UE perform steps 1-3 from the Registration procedure for mobility registration update as specified in TS 38.508-1 [4] subclause 4.9.5 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell11, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { //@sic R5-213508 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@sic R5-213508 sic@ //@siclog "Step 8AA - 8AB" siclog@ //The SS transmits a SecurityModeCommand message. //The UE transmits a SecurityModeComplete message. f_NR_RRC_ActivateSecurity (nr_Cell11, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); // @siclog "Step 8A - 9D" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with 'Request type' set to 'initial emergency request', and, a PDU SESSION ESTABLISHMENT REQUEST for establishing an emergency PDU? // Steps 9-13 from Generic Test Procedure for IMS Emergency call establishment with IMS Emergency registration as specified in TS 38.508-1 [4], subclause 4.9.11, including the parallel behaviour specified in table 4.9.11.2.2-2 are performed. //f_NR5GC_EmergencyCallWithRegistration(nr_Cell11); f_NR5GC_EmergencyCallPDUSession(nr_Cell11, tsc_NR_RbId_SRB1); //@sic R5s210515 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8A"); // @siclog "Step 10 - 11" siclog@ // Make the UE release the emergency call. (NOTE 1) f_NR5GC_EmergencyCallRelease(nr_Cell11); // Release IP handling as this will be (re)started when this PDU is (re)established in the next loop //WA#11_4_4 COMMENTED OUT f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2)); f_IP_Handling_StopPDN(IP, cs_IP_StopAllPDNs); //WA#11_4_4 v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @siclog "Step 12" siclog@ for (i := 0; i<v_NoOfSessions; i:=i+1) { // @sic R5s210515 R5-216200 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfo[i].SessionId; v_MsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011010'B), //5GSM cause Insufficient resources cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00101'B)); //T3396 = 5 minutes // p_NGSM_CongestionReattemptInd, SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB2, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));//@sic R5s210808sic@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell11, tsc_NR_RbId_SRB2, //@sic R5s210808sic@ cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } } // @siclog "Step 13" siclog@ // Void // @siclog "Step 14" siclog@ // The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 15" siclog@ //Make the UE attempt an IMS emergency call dialling an emergency number e.g. 112 or 911). (NOTE 1) // Prose CR RRC Release //WA#11_4_4 COMMENTED OUT fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber, "Step 16"); //WA#11_4_4 : R&s change starts f_UT_RequestIMSEmergencyCall (UT, px_EmergencyCallNumber); f_NR5GC_EmergencyCallWithRegistration(nr_Cell11); v_MyDefaultVar := activate (a_PDU_Session_Release_REQ (nr_Cell11, f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN))); f_NR5GC_EmergencyCallRelease(nr_Cell11); deactivate (v_MyDefaultVar); f_NR_RRCRelease(nr_Cell11); // Release IP handling as this will be (re)started when this PDU is (re)established in the next loop f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2)); //WA#11_4_4 : Change stop. f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16 "); }

## function a_PDU_Session_Release_REQ

| Function name | a_PDU_Session_Release_REQ |
| --- | --- |
| Reason for change | Please see change 1.1 above |
| Summary of change | New function |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

altstep a_PDU_Session_Release_REQ(NR_CellId_Type p_NR_CellId, template GSM_MobilityInfo_Type p_GSM_MobilityInfo) runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { if (not f_Check_NG_PDUSessionReleaseReq (p_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } }; };