3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211537

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0750 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_MTCallSetup_BuildInviteRequestWithSdp |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current implementation does not pass the parameter p_PreconditionsRequiredAndSupported once the return function f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp is called. Hence the constructed INVITE always contains preconditions regardless of the requirement of the original calling function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass p_PreconditionsRequiredAndSupported to the return function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.NR5GC, 7.8.NR5GC, 7.10.NR5GC, 7.23.NR5GC, 7.33.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_IMS_MTCallSetup_BuildInviteRequestWithSdp |
| --- | --- |
| Reason for change | Current implementation does not pass the parameter p_PreconditionsRequiredAndSupported once the return function f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp is called. Hence the constructed INVITE always contains preconditions regardless of the requirement of the original calling function. |
| Summary of change | Pass p_PreconditionsRequiredAndSupported to the return function |
| TTCN module | IMS_Procedures_CallControl4G5G.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_MTCallSetup_BuildInviteRequestWithSdp(IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR, boolean p_PreconditionsRequiredAndSupported := true) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5w140112: return IMS_InviteRequestWithSdp_Type instead of IMS_DATA_REQ; initialisation of routing info sic@ */ /* @sic R5-153798: p_AdditionalAudioCodecs sic@ */ /* @sic R5-169060: allow no preconditions sic@ */ /* @sic R5-173535: p_AudioCodecList instead of p_AdditionalAudioCodecs sic@ */ var template (omit) SDP_Message v_SdpOffer := omit; if (lengthof(p_AudioCodecList) > 0) { // @sic R5-206381: TC_7_10, TC_7_25 INVITE with no SDP message sic@ v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudio_TX(p_AudioCodecList, p_PreconditionsRequiredAndSupported); } return f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp(v_SdpOffer); /* @sic R5-213069: f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp sic@ */ }

After Change

function f_IMS_MTCallSetup_BuildInviteRequestWithSdp(IMS_SDP_AudioCodecList_Type p_AudioCodecList := tsc_IMS_SDP_AudioCodecList_WBAMR_AMR, boolean p_PreconditionsRequiredAndSupported := true) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */ /* @sic R5w140112: return IMS_InviteRequestWithSdp_Type instead of IMS_DATA_REQ; initialisation of routing info sic@ */ /* @sic R5-153798: p_AdditionalAudioCodecs sic@ */ /* @sic R5-169060: allow no preconditions sic@ */ /* @sic R5-173535: p_AudioCodecList instead of p_AdditionalAudioCodecs sic@ */ var template (omit) SDP_Message v_SdpOffer := omit; if (lengthof(p_AudioCodecList) > 0) { // @sic R5-206381: TC_7_10, TC_7_25 INVITE with no SDP message sic@ v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudio_TX(p_AudioCodecList, p_PreconditionsRequiredAndSupported); } return f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp(v_SdpOffer, p_PreconditionsRequiredAndSupported); /* @sic R5-213069: f_IMS_MTCallSetup_BuildInviteRequestWithSpecificSdp sic@ */ }