3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211658

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2203 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 11.3.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-16 NR5GC test case 11.3.1 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 11.3.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

3	Branches executed	9

4	Execution Log Files	9

4.1	HiSilicon Balong 5000	9

5	References	9

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.3.1 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Pengchong Shen
	shenpengchong@datangmobile.cn

  - Verification Test Summary

Test Case: 	11.3.1

ATS Version:	iwd-TTCN3-B2020-09_D21wk37

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | f_TC_11_3_1_NR5GC |
| --- | --- |
| Reason for change | - According to 38.523-1, the UE is switched on and brought to state 1N-A not connected. The current TTCN is wrong. |
| Summary of change | - Modify STATE_CONNECTED_3A to STATE_IDLE_1A. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_3_1_NR5GC() runs on NR5GC_PTC { // UAC / Access Identity 0 / 0% access probability / MTSI MO speech call/SMSoIP/Uplink User data transfer f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A, TESTMode_OFF); // @sic R5-216192 sic@ f_NR_TestBody_Set(true); fl_TC_11_3_1_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change

function f_TC_11_3_1_NR5GC() runs on NR5GC_PTC { // UAC / Access Identity 0 / 0% access probability / MTSI MO speech call/SMSoIP/Uplink User data transfer f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTMode_OFF); // @sic R5-216192 sic@ f_NR_TestBody_Set(true); fl_TC_11_3_1_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

  - Change 2

| Function name | fl_TC_11_3_1_TestBody |
| --- | --- |
| Reason for change | - For step 14, it missed the configuration of barring information. Note: it will be a prose CR to support. - According to 38.508-1 Table 4.9.19.3-1, at step36-43, establishment cause should be mo-SMS not mo-data. - Trigger to start step42-49 at IMS PTC should be moved before f_NR5GC_RRC_ConnectedState3N_Steps5_8. |
| Summary of change | - Add barring information for cell1before step13. - Modify mo-data to mo-SMS. - Move f_NR5GC_RRC_ConnectedState3N_Steps5_8 after f_IMS_IPCAN_SendCoOrdMsg. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var float v_TimerValue := 6.0; timer t_WaitTime; // @sic R5-216192 sic@ //@siclog "Step 13" siclog@ //Make the UE attempt a MMTel voice call. f_UT_RequestIMSCall(UT); //@siclog "Steps 14" siclog@ //Check: Does the UE transmit RRCSetupRequest message including mo-VoiceCall with within 6 s? t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mo_VoiceCall,?))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } [] t_WaitTime.timeout { } } ...... //@siclog "Steps 34" siclog@ //SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1) f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); // @sic R5-216192 sic@ // f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 35" siclog@ //AT command to make the UE attempt to send another SMS over IP f_UT_IMS_TriggerSMS(UT); //@siclog "Steps 36-43" siclog@ //Steps 2-15 from generic procedure IMS MO SMS in 5GS as described in TS 38.508-1 [4] Table 4.9.19.2.2-1 take place. //f_NR5GC_IPCAN_MO_IMS_Signalling(nr_Cell1); v_ReceivedMsg:= f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_Data); // @sic R5s201327 sic@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); }

After Change

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; var float v_TimerValue := 6.0; timer t_WaitTime; //@siclog "Step 12C" siclog@ f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(4,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B))); f_NR_ModifySysinfo(nr_Cell1); f_NR_WaitModificationPeriods(nr_Cell1); // @sic R5-216192 sic@ //@siclog "Step 13" siclog@ //Make the UE attempt a MMTel voice call. f_UT_RequestIMSCall(UT); //@siclog "Steps 14" siclog@ //Check: Does the UE transmit RRCSetupRequest message including mo-VoiceCall with within 6 s? t_WaitTime.start(v_TimerValue); alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mo_VoiceCall,?))) { t_WaitTime.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } [] t_WaitTime.timeout { } } ...... //@siclog "Steps 34" siclog@ //SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1) f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); // @sic R5-216192 sic@ // f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Steps 35" siclog@ //AT command to make the UE attempt to send another SMS over IP f_UT_IMS_TriggerSMS(UT); //@siclog "Steps 36-43" siclog@ //Steps 2-15 from generic procedure IMS MO SMS in 5GS as described in TS 38.508-1 [4] Table 4.9.19.2.2-1 take place. //f_NR5GC_IPCAN_MO_IMS_Signalling(nr_Cell1); v_ReceivedMsg:= f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_SMS); // @sic R5s201327 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); }

  - Change 3

| Function name | f_TC_11_3_1_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - It missed a trigger to start add voice QoS for step 18 on NR5GC PTC. |
| Summary of change | - Add f_IMS_IPCAN_SendCoOrdMsg after Step 3 – 6 on IMS PTC. |
| TTCN module | UAC_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_3_1_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //@siclog "Steps 18-28" siclog@ // Step3 to step13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place, if(not pc_Preconditions) { //IF the UE was configured to not use preconditions THEN steps 2-5 of Annex A.4.2 of TS 34.229-5 [47] take place. v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(); // @siclog "Step 3 - 6" siclog@ 100 Trying, 183 Session Progress, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_2_Step2_5(v_InviteRequestWithSdp); } else{ //ELSE steps 2-5 of Annex A.4.1 of TS 34.229-5 [47] take place. //Steps 1 subclause A.4.1 v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1(); //Steps 2 to 5 of subclause A.4.1 f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_5(v_InviteRequestWithSdp); } f_IMS_IPCAN_WaitForTrigger(IPCAN); if(not pc_Preconditions) { f_IMS_MOCallSetup_NR5GC_A_4_2_Step6(v_InviteRequestWithSdp); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); } else{ //v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_MessageHeader183); f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_12(v_InviteRequestWithSdp); } f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Steps 29-30" siclog@ //Steps 1-2 from test procedure for test procedure for IMS MO speech call release as described in TS 38.508-1 [4] Table 4.9.17.2.2-1 take place, f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Steps 42-49" siclog@ //Steps 9-16 of generic procedure for MO SMS as per 38.508-1 steps 9-16, corresponding to (steps 1-8) 34.229-5 cl. A.13 f_IMS_NR5GC_MO_SMS_Steps9_16 (); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_11_3_1_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //@siclog "Steps 18-28" siclog@ // Step3 to step13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place, if(not pc_Preconditions) { //IF the UE was configured to not use preconditions THEN steps 2-5 of Annex A.4.2 of TS 34.229-5 [47] take place. v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(); // @siclog "Step 3 - 6" siclog@ 100 Trying, 183 Session Progress, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_2_Step2_5(v_InviteRequestWithSdp); } else{ //ELSE steps 2-5 of Annex A.4.1 of TS 34.229-5 [47] take place. //Steps 1 subclause A.4.1 v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1(); //Steps 2 to 5 of subclause A.4.1 f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_5(v_InviteRequestWithSdp); } f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); if(not pc_Preconditions) { f_IMS_MOCallSetup_NR5GC_A_4_2_Step6(v_InviteRequestWithSdp); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); } else{ //v_ContactUrlSS := f_MessageHeader_GetContactSipUrl(v_MessageHeader183); f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_12(v_InviteRequestWithSdp); } f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Steps 29-30" siclog@ //Steps 1-2 from test procedure for test procedure for IMS MO speech call release as described in TS 38.508-1 [4] Table 4.9.17.2.2-1 take place, f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Steps 42-49" siclog@ //Steps 9-16 of generic procedure for MO SMS as per 38.508-1 steps 9-16, corresponding to (steps 1-8) 34.229-5 cl. A.13 f_IMS_NR5GC_MO_SMS_Steps9_16 (); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

- Branches executed

NR Band 41, nea2, Nia2.

- Execution Log Files

  - HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_11_3_1_NR5GC_N41.log

- PIXCIT settings used:
\ Pics(Hisilicon-IRAT).xml

\ Pixit(TC_11_3_1_NR5GC).xml

- References

| [1] | R5s211659: Supporting information for agreement of NR5GC test case 11.3.1 in FR1 |
| --- | --- |