3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211665

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2207 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR MAC test cases 7.1.1.5.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of constant LCID, added conditional LCID value as per DRB id. Note : Similar change was implemented for 7.1.1.2.1 under R5s211241 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant IMS-enabled UE can fail these test cases |  |  |  |  |  |  |  |  |
|  |  | 8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.1.NR5GC, 7.1.1.5.2.NR5GC, 7.1.1.5.3.NR5GC, 7.1.1.5.4.NR5GC, 7.1.1.5.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	7

2. 3 Change 3	8

2.4	Change 4	10

2.5	Change 5	11

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | function f_TC_7_1_1_5_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) |
| --- | --- |
| Reason for change | LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. |
| Summary of change | Instead constant LCID, added conditional LCID value as per DRB id. |
| TTCN module | DRX_MAC_TC_Common_NR.TTCN |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_5_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { const UInt_Type tsc_DrxStartOffset := 7; // in ms const UInt_Type tsc_OnDurationTimer := 20; // in ms /* Skipping code */ // The other things in Step 3 automatically taken care by SS t_Watchdog.start; v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; /* Skipping Code*/ //@siclog "Step 5" siclog@ v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); // Last PDCCH occasion while On Duration Timer is running. v_Timing1 := f_NR_SubFrameTiming_AddSlots(v_Timing1, (tsc_OnDurationTimer *v_SlotsInSubframe) - 1); // - the last slot during onDurationTimer v_Timing1 := f_NR_LastDL_Slot(nr_Cell1,v_Timing1); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL)); //@siclog "Step 7" siclog@ // preconfig SS v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_InactivityTimer * v_SlotsInSubframe ); //@sic R5s200711 R5s200833 sic@ v_Timing2 := f_NR_LastDL_Slot(nr_Cell1,v_Timing2); //v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_InactivityTimer * f_NR_DRX_LastDL_Slot(nr_Cell1)); // The other things in Step 7 automatically taken care by SS v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing2), v_MAC_PDUListDL)); //@sic R5s201481 sic@ t_Watchdog.start; /*Skipping Code */ // step 9 and 11 should be on one HARQ process v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(3, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(3, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment));} /*skipping code */ // step 13 and 15 should be on one HARQ process v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(4, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(4, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); /*Skipping Code */ //@siclog "Step 19" siclog@ // (tsc_NR_SlotOffsetK2 = )4 slots + (drx-HARQ-RTT-TimerUL ) 4 slots + (drx-RetransmissionTimerUL ) 12 slots =20 slots, a multiple of 5 slots //v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, 20); v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_NR_SlotOffsetK2 + tsc_DRX_HARQ_RTT_TimerUL + tsc_DRXRetransTimer_UL ); //@sic R5s200711 R5s200833 sic@ v_Timing2 := f_NR_LastDL_Slot(nr_Cell1,v_Timing2); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(5, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(5, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL:= { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing2), v_MAC_PDUListDL)); /*Skipping Code */

After Change:

function f_TC_7_1_1_5_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { const UInt_Type tsc_DrxStartOffset := 7; // in ms const UInt_Type tsc_OnDurationTimer := 20; // in ms /* Skipping code */ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//KS@21xxxx /* Skipping code */ // The other things in Step 3 automatically taken care by SS t_Watchdog.start; v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; // v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx /* Skipping Code*/ //@siclog "Step 5" siclog@ v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); // Last PDCCH occasion while On Duration Timer is running. v_Timing1 := f_NR_SubFrameTiming_AddSlots(v_Timing1, (tsc_OnDurationTimer *v_SlotsInSubframe) - 1); // - the last slot during onDurationTimer v_Timing1 := f_NR_LastDL_Slot(nr_Cell1,v_Timing1); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); // v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL)); //@siclog "Step 7" siclog@ // preconfig SS v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_InactivityTimer * v_SlotsInSubframe ); //@sic R5s200711 R5s200833 sic@ v_Timing2 := f_NR_LastDL_Slot(nr_Cell1,v_Timing2); //v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_InactivityTimer * f_NR_DRX_LastDL_Slot(nr_Cell1)); // The other things in Step 7 automatically taken care by SS v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B))); //v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing2), v_MAC_PDUListDL)); //@sic R5s201481 sic@ t_Watchdog.start; /*Skipping Code */ // step 9 and 11 should be on one HARQ process v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(3, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(3, crs_NR_PDCP_SDU_36B))); //v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment));} /*skipping code */ // step 13 and 15 should be on one HARQ process v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(4, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(4, crs_NR_PDCP_SDU_36B))); //v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); /*Skipping Code */ //@siclog "Step 19" siclog@ // (tsc_NR_SlotOffsetK2 = )4 slots + (drx-HARQ-RTT-TimerUL ) 4 slots + (drx-RetransmissionTimerUL ) 12 slots =20 slots, a multiple of 5 slots //v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, 20); v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, tsc_NR_SlotOffsetK2 + tsc_DRX_HARQ_RTT_TimerUL + tsc_DRXRetransTimer_UL ); //@sic R5s200711 R5s200833 sic@ v_Timing2 := f_NR_LastDL_Slot(nr_Cell1,v_Timing2); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(5, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(5, crs_NR_PDCP_SDU_36B))); //v_SDU_SubPDUListDL:= { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL:= { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing2), v_MAC_PDUListDL)); /*Skipping Code */ }

## 2.2 Change 2

| Function name | function f_TC_7_1_1_5_2_NR_TestBody(DRB_Identity p_NR_DRB_Id) |
| --- | --- |
| Reason for change | LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. |
| Summary of change | Instead constant LCID, added conditional LCID value as per DRB id. |
| TTCN module | DRX_MAC_TC_Common_NR.TTCN |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_5_2_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.2 : DRX operation / Short cycle not configured / Long DRX command MAC control element reception */ const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ const UInt_Type tsc_HarqProcessId :=0; //Arbitrarily selected const UInt_Type tsc_DRXRetransTimer_DL := 80;// in slots const UInt_Type tsc_DRX_HARQ_RTT_TimerDL := 4;// in slots 56/14 /*Skipping code */ // step 3 and 7 should be on one HARQ process v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId))); /*Skipping Code */ }

After Change:

function f_TC_7_1_1_5_2_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.2 : DRX operation / Short cycle not configured / Long DRX command MAC control element reception */ const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ const UInt_Type tsc_HarqProcessId :=0; //Arbitrarily selected const UInt_Type tsc_DRXRetransTimer_DL := 80;// in slots const UInt_Type tsc_DRX_HARQ_RTT_TimerDL := 4;// in slots 56/14 /*Skipping code */ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//KS@21xxxx /*Skipping Code*/ // step 3 and 7 should be on one HARQ process v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); //v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId))); /*Skipping Code */ }

## 2. 3 Change 3

| Function name | function f_TC_7_1_1_5_3_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) |
| --- | --- |
| Reason for change | LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. |
| Summary of change | Instead constant LCID, added conditional LCID value as per DRB id. |
| TTCN module | DRX_MAC_TC_Common_NR.TTCN |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_5_3_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ const UInt_Type tsc_OnDurationTimer := 20; // in ms // As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120) var UInt_Type v_LongDRXCycle := 640; // in ms var UInt_Type v_ShortDRXCycle := 80; // in ms @sic R5s202609 sic@ var SubFrameTiming_Type v_Timing := valueof(cs_SubFrameTiming(0, 0, 0)); var SubFrameTiming_Type v_Timing1 := v_Timing; var integer v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); var integer v_SlotsInframe := v_SlotsInSubframe * 10; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; /*Skipping Code */ //@siclog "Step 3" siclog@ t_Watchdog.start; v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; // Choose the next subframe when the onDurationTimer is started. /Skipping Code */ /@siclog "Step 5" siclog@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_ShortDRXCycle, tsc_DrxStartOffset); /*Skipping Code */ /@siclog "Step 7-8" siclog@ // The f_NR_GetNextSendOccasion_7_1_1_5_X selects time 2 long cycles in advance hence drx-ShortCycleTimer should have expired v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); /Skipping Code */ }

After Change:

function f_TC_7_1_1_5_3_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) runs on NR_BASE_PTC { const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ const UInt_Type tsc_OnDurationTimer := 20; // in ms // As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120) var UInt_Type v_LongDRXCycle := 640; // in ms var UInt_Type v_ShortDRXCycle := 80; // in ms @sic R5s202609 sic@ var SubFrameTiming_Type v_Timing := valueof(cs_SubFrameTiming(0, 0, 0)); var SubFrameTiming_Type v_Timing1 := v_Timing; var integer v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); var integer v_SlotsInframe := v_SlotsInSubframe * 10; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//KS@21xxxx /*Skipping Code */ //@siclog "Step 3" siclog@ t_Watchdog.start; v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); //v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; // Choose the next subframe when the onDurationTimer is started. /Skipping Code */ /@siclog "Step 5" siclog@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); // v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_ShortDRXCycle, tsc_DrxStartOffset); /*Skipping Code */ /@siclog "Step 7-8" siclog@ // The f_NR_GetNextSendOccasion_7_1_1_5_X selects time 2 long cycles in advance hence drx-ShortCycleTimer should have expired v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_36B))); //v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_LongDRXCycle, tsc_DrxStartOffset); /Skipping Code */ }

## Change 4

| Function name | function f_TC_7_1_1_5_4_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) |
| --- | --- |
| Reason for change | LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. |
| Summary of change | Instead constant LCID, added conditional LCID value as per DRB id. |
| TTCN module | DRX_MAC_TC_Common_NR.TTCN |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_5_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.4 : DRX Operation / Short cycle configured / DRX command MAC control element reception */ const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ /*Skipping Code */ var integer v_NR_K1:= 5; timer t_T7115 := 0.05; timer t_Watchdog := 5.0; /Skipping Code */ //@siclog "Step 3" siclog@ //@sic R5s201437 R5-206123 sic@ //f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); //@sic R5s200867 sic@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; /Skipping Code */ }

After Change:

function f_TC_7_1_1_5_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.4 : DRX Operation / Short cycle configured / DRX command MAC control element reception */ const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ /*Skipping Code */ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//KS@21xxxx var integer v_NR_K1:= 5; timer t_T7115 := 0.05; timer t_Watchdog := 5.0; /Skipping Code */ //@siclog "Step 3" siclog@ //@sic R5s201437 R5-206123 sic@ //f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); //@sic R5s200867 sic@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); // v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; /Skipping Code */ }

## Change 5

| Function name | function f_TC_7_1_1_5_5_NR_TestBody(DRB_Identity p_NR_DRB_Id, float p_WatchDog := 5.0) |
| --- | --- |
| Reason for change | LCID value ‘000101’ is fixed for RLC SDUs always on this case. For internet PDN TTCN is assigning DRB id 2 and LCID 5. For ims PDN, TTCN is assigning DRB id 1 and LCID 4. But, LCID 5(tsc_LCID_SCG_Bearer) is only used in construction of MAC PDU in case of DRBid=1 or DRBid=2. Hence, it will fail the TC if this TC will be executed with IMS as default dnn. |
| Summary of change | Instead constant LCID, added conditional LCID value as per DRB id. |
| TTCN module | DRX_MAC_TC_Common_NR.TTCN |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_5_5_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.5 : DRX Operation / Short cycle configured / Long DRX command MAC control element reception */ const UInt_Type tsc_DrxStartOffset := 7; // in ms // As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120) //@sic R5s201495 sic@ /*Skipping Code */ //@siclog "Step 3" siclog@ //@sic R5s201495 R5-206319 sic@ //f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; /Skipping Code */ //@siclog "Step 9" siclog@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; /Skipping Code */ }

After Change:

function f_TC_7_1_1_5_5_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { /* L2/MAC * 7.1.1.5.5 : DRX Operation / Short cycle configured / Long DRX command MAC control element reception */ const UInt_Type tsc_DrxStartOffset := 7; // in ms // As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120) //@sic R5s201495 sic@ /*Skipping Code */ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//KS@21xxxx /*Skipping Code */ //@siclog "Step 3" siclog@ //@sic R5s201495 R5-206319 sic@ //f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); //v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; /Skipping Code */ //@siclog "Step 9" siclog@ v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); //v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//KS@21xxxx v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; /Skipping Code */ }