3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211667

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.123-3 | CR | 3663 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to GMM test case 12.2.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2021-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the currecnt TTCN implemenation in step 5 when the SS transmits the ATTACH REJ with causes “GPRS and non-GPRS services not allowed” the UE delete the CKSN (which is inline with 24.008, clause 4.7.3.14 cause #8 ) Thus, at step 22 when UE tries to ATTACH it sets the ciphering key sequnece number (CKSN) to “111”B which is same as no CKSN. However the SS expects CKSN to be set to a value other than ‘111’B which causes the test case to fail. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Assign tcv_PS_KeySeq = ‘111’B before calling the localtree lt_Steps_22To24a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4243

1.	Change 1	4244

# 1.	Change 1

| Object name | lt_Testbody |
| --- | --- |
| Reason for change | In the currecnt TTCN implemenation in step 5 when the SS transmits the ATTACH REJ with causes “GPRS and non-GPRS services not allowed” the UE delete the CKSN (which is inline with 24.008, clause 4.7.3.14 cause #8 ) Thus, at step 22 when UE tries to ATTACH it sets the ciphering key sequnece number (CKSN) to “111”B which is same as no CKSN. However the SS expects CKSN to be set to a value other than ‘111’B which causes the test case to fail. This needs to be addressed. |
| Summary of change | Assign tcv_PS_KeySeq = ‘111’B before calling the localtree lt_Steps_22To24a |
| Module |  |
| Label | WA# |

Before:

After: