3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220066

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4658 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to DRX testcase 7.1.6.2 for CAT-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-UEConTest, TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-12-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - With current ttcn implementation, the reception of NACK (Step2) and PDSCH transmission of Step 1, are expected to happen in consecutive subframes of a SFN. This cannot be achieved in case of FDD_TypeB_HalfDuplex operation due to guard period restriction (Cl. 6.2.5 TS 36.211). - The UL grant configuration (SF4) can conflict with Scheduling request (SF5) in case of FDD_TypeB_HalfDuplex. Proposed change modifies the UL grant configuration to SF8 to include guard subframe between UL and DL. Reference CR R5s201658 - With current ttcn implementation, the reception of ACK (Step4) and scheduling of MAC PDU (Step5) are expected to happen in consecutive subframes of a SFN. This cannot be achieved in case of FDD_TypeB_HalfDuplex operation due to guard period restriction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed the value of tsc_SubFrameY=3 to 1 and tsc_SubFrameZ=4 to 2. Note: The values are chosen such that the scheduling of PDUs and reception of ACK/NACK happens without any conflict throughout the testcase. - Changed UL grant configuration from SF4 to SF8 - 1 subframe is added to overcome guard period restriction in CAT-M1 during scheduling of MAC PDU at Step5 and reception of ACK at Step4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_7_1_6_2_EUTRA |
| --- | --- |
| Reason for change | - With current ttcn implementation, the reception of NACK (Step2) and PDSCH transmission of Step 1, are expected to happen in consecutive subframes of a SFN. This cannot be achieved in case of FDD_TypeB_HalfDuplex operation due to guard period restriction. - The UL grant configuration (SF4) can conflict with Scheduling request (SF5) in case of FDD_TypeB_HalfDuplex. Proposed change modifies the UL grant configuration to SF8 to include guard subframe between UL and DL. Reference CR R5s201658 - With current ttcn implementation, the reception of ACK (Step4) and scheduling of MAC PDU (Step5) are expected to happen in consecutive subframes of a SFN. This cannot be achieved in case of FDD_TypeB_HalfDuplex operation due to guard period restriction. |
| Summary of change | - Changed the value of tsc_SubFrameY=3 to 1 and tsc_SubFrameZ=4 to 2 Note: The values are chosen such that the scheduling of PDUs and reception of ACK/NACK happens without any conflict throughout the testcase. - Changed UL grant configuration from SF4 to SF8 - 1 subframe is added to overcome guard period restriction in CAT-M1 during scheduling of MAC PDU at Step5 and reception of ACK at Step4 |
| TTCN module | LTE\7_1\MAC_716.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC

{ /* L2/MAC

* 7.1.6.2 : DRX Operation / Parameters (short cycle not configured) / DRX command MAC control element reception

*/

//@sic R5s110173 R5-113679 sic@

const integer tsc_LongDRXCycle := 640;

const integer tsc_DrxStartOffset := 4; //@sic R5s110692 sic@

const integer tsc_OnDurationTimer := 20;

const integer tsc_DRXRetranTimer := 6;

const integer tsc_SubFrameX := 6;

const integer tsc_SubFrameY := 3; //@sic R5s210585 sic@

const integer tsc_SubFrameZ := 4;

var integer v_NormalSF := 0;

var SubFrameTiming_Type v_Timing  := valueof(cs_SubFrameTiming(0, 0, 0));

var SubFrameTiming_Type v_Timing1 := v_Timing;

var SubFrameTiming_Type v_Timing2 := v_Timing;

var SubFrameTiming_Type v_Timing3 := v_Timing;

var SubFrameTiming_Type v_Timing4 := v_Timing;

var SubFrameTiming_Type v_Timing5 := v_Timing;

var integer v_HARQRTTTimer := 0;

var octetstring v_EncodedRlcPdu;

var EUTRA_FDD_TDD_Mode_Type  v_EUTRA_FDD_TDD_Mode;

timer t_Watchdog := 5.0;

///////////////////////////////////////////////////////////////////////////////////

//      PREAMBLE

//////////////////////////////////////////////////////////////////////////////////

f_EUTRA_Init(c1);

v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1);

f_EUTRA_CellConfig_Def (eutra_Cell1);

f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeA);

f_EUTRA_AS_CipheringAlgorithm_Set(eea0); // set null AS ciphering algorithm

//f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1);

// creates cell and performs registration

//@sic R5-104796 R5-106675DRX support on SS mandated sic@

//f_EUTRA_GenericRbEst_Common_716( eutra_Cell1, 0, 0, omit, cs_MAC_MainConfig_Explicit_RBC_DrxL(0), omit, omit, omit, 5); //@sic R5-112665 sic@

// Period is set as 4 = 80% of 6ms the Inactivity Timer

//@sic R5s100466 change of period for TA from 16 to 4 sic@

//f_EUTRA_CloseUE_TestLoopModeA( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) );

//f_Delay(0.66); //@sic R5s100466 add timer for TA MCE to be sent once sic@

//f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def);

// Go to state 4 to return no data in UL

// step to configure SS RLC/PDCP and MAC in No header manipulation mode on SS Side for DRB

//f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly);

// to configure SS to report reception of HARQ ACK or NACK

//f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1, tsc_L1Mac_IndicationMode_Enable );

//@sic R5-104796 R5-106675 DRX support on SS mandated sic@

f_EUTRA_LoopBackActivation_State4_Def( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) );

///////////////////////////////////////////////////////////////////////////////////

//      Test Body

//////////////////////////////////////////////////////////////////////////////////

f_EUTRA_TestBody_Set(true);

f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1,

cs_TimingInfo_SubFrame(4),

cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4), // Imcs = Itbs =4, Nprb Results in TB size of 328 selected to be neither too big nor too small

cs_UplinkTimeAlignment_Synch_Auto_Def,                      //@sic R5s110618 sic@

cs_UL_GrantConfig_ContPeriodic(5));       // periodic grant every 5ms (80% of 6ms the Inactivity Timer)

// Reconfigure RRC Connection: Step 0A and 0B

f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig (eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_DrxL(tsc_DrxStartOffset)); //@sic R5s110698 sic@

f_Delay(0.66); //

// stop periodic UL grants and restart automatic grant allocation instead; periodic TA is kept stopped:

f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def);

// suppress number of DL retransmissions:

f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1);

f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly);

// to configure SS to report reception of HARQ ACK or NACK

f_SS_ConfigUL_HARQ_IndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Enable);

//@siclog "Step 1" siclog@

// set CRC error as error mode

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,cs_DL_SCH_CRC_C_RNTI_Error(Erroneous));

// Choose the next subframe when the onDurationTimer is started.

v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_LongDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 1 - tsc_SubFrameX);

v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(0, tsc_NoPoll, f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN0, crs_PDCP_SDU_17B))));//@sic R5s110552 sic@

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU (eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing1),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(1)));

v_HARQRTTTimer := f_GetHARQRTTTimer(v_Timing1.Subframe.Number, v_EUTRA_FDD_TDD_Mode);

//@siclog "Step 3" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing1, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Normal),

cs_TimingInfo_EUTRA_NB(v_Timing));

v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, tsc_SubFrameY, v_EUTRA_FDD_TDD_Mode);

v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing2),

cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@

cs_HarqProcessAssignment_Specific(2)));

//@siclog "Step 5" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing2, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Erroneous),

cs_TimingInfo_EUTRA_NB(v_Timing));

/* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll,

int2bit (1, 10),

f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN1, crs_PDCP_SDU_17B))))); */

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing1, v_HARQRTTTimer);

if (v_EUTRA_FDD_TDD_Mode == FDD) //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF( v_Timing3.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);

}

else

{

v_NormalSF := f_DRX_NormalSF( v_Timing3.Subframe.Number, 1, v_EUTRA_FDD_TDD_Mode);

}

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing3, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing3),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(3)));

v_HARQRTTTimer := f_GetHARQRTTTimer(v_Timing3.Subframe.Number, v_EUTRA_FDD_TDD_Mode);

//@siclog "Step 7" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing3, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Normal),

cs_TimingInfo_EUTRA_NB(v_Timing));

v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer);

if (v_EUTRA_FDD_TDD_Mode == FDD)      //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, tsc_DRXRetranTimer - tsc_SubFrameZ, v_EUTRA_FDD_TDD_Mode);

}

else if (v_EUTRA_FDD_TDD_Mode == TDD)

{

v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);

}

v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing4),

cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@

cs_HarqProcessAssignment_Specific(4)));

//@siclog "Step 9" siclog@

/* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll,

int2bit (2,10),

f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN2, crs_PDCP_SDU_17B))))); */

if (v_EUTRA_FDD_TDD_Mode == FDD)    //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF(v_Timing4.Subframe.Number, tsc_SubFrameZ - 1, v_EUTRA_FDD_TDD_Mode);

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF);

}

else if (v_EUTRA_FDD_TDD_Mode == TDD)

{

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer);

v_NormalSF := f_DRX_NormalSF(v_Timing5.Subframe.Number, 5, v_EUTRA_FDD_TDD_Mode);

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing5, v_NormalSF);

}

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing5),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(5) ));

// Preconfig SS for preventing UE to enter DRX

// @sic R5s100520 sic@

//@sic R5-104796 DRX support on SS mandated sic@

// Period is set as 16 = 80% of 20ms the Inactivity Timer

t_Watchdog.start;

v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode);

//@siclog "Step 2" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 1?

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), nack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

//@siclog "Step 4" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 3?

v_Timing2 := f_AckNack_TimingInfo(v_Timing2, v_EUTRA_FDD_TDD_Mode);

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing2.SFN.Number, v_Timing2.Subframe.Number), ack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

//@siclog "Step 6" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 5?

v_Timing3 := f_AckNack_TimingInfo(v_Timing3, v_EUTRA_FDD_TDD_Mode);

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing3.SFN.Number, v_Timing3.Subframe.Number), nack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

//@siclog "Step 8" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 7?

v_Timing4 := f_AckNack_TimingInfo(v_Timing4, v_EUTRA_FDD_TDD_Mode);

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing4.SFN.Number, v_Timing4.Subframe.Number), ack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

//@siclog "Step 10" siclog@

// Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 9?

v_Timing5 := f_AckNack_TimingInfo(v_Timing5, v_EUTRA_FDD_TDD_Mode);

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing5.SFN.Number, v_Timing5.Subframe.Number), ack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 10");

t_Watchdog.stop;

//test body ends

f_EUTRA_TestBody_Set(false);

///////////////////////////////////////////////////////////////////////////////////

//      Post Amble

//////////////////////////////////////////////////////////////////////////////////

// to configure SS to stop report reception of HARQ ACK or NACK

f_SS_ConfigUL_HARQ_IndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Disable);

f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NormalMode); //@sic R5s100354 sic@

// leaving UE test state 4

//@sic R5-103847 Deactivate test loop and open test loop is removed sic@

v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_LongDRXCycle, tsc_DrxStartOffset, 0);

f_EUTRA_RRC_ConnectionRelease(eutra_Cell1, tsc_RRC_TI_Def, other, cs_TimingInfo_EUTRA_NB(v_Timing1));

// Release RRC Connection and Switch off

f_EUTRA_Postamble(eutra_Cell1, E1_IDLE);//@sic R5-103875 sic@

};

After change:

function f_TC_7_1_6_2_EUTRA ( ) runs on EUTRA_PTC

{ /* L2/MAC

* 7.1.6.2 : DRX Operation / Parameters (short cycle not configured) / DRX command MAC control element reception

*/

//@sic R5s110173 R5-113679 sic@

const integer tsc_LongDRXCycle := 640;

const integer tsc_DrxStartOffset := 4; //@sic R5s110692 sic@

const integer tsc_OnDurationTimer := 20;

const integer tsc_DRXRetranTimer := 6;

const integer tsc_SubFrameX := 6;

const integer tsc_SubFrameY := 1; //@sic R5s210585 sic@

const integer tsc_SubFrameZ := 2;

var integer v_NormalSF := 0;

var SubFrameTiming_Type v_Timing  := valueof(cs_SubFrameTiming(0, 0, 0));

var SubFrameTiming_Type v_Timing1 := v_Timing;

var SubFrameTiming_Type v_Timing2 := v_Timing;

var SubFrameTiming_Type v_Timing3 := v_Timing;

var SubFrameTiming_Type v_Timing4 := v_Timing;

var SubFrameTiming_Type v_Timing5 := v_Timing;

var integer v_HARQRTTTimer := 0;

var octetstring v_EncodedRlcPdu;

var EUTRA_FDD_TDD_Mode_Type  v_EUTRA_FDD_TDD_Mode;

timer t_Watchdog := 5.0;

///////////////////////////////////////////////////////////////////////////////////

//      PREAMBLE

//////////////////////////////////////////////////////////////////////////////////

f_EUTRA_Init(c1);

v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1);

f_EUTRA_CellConfig_Def (eutra_Cell1);

f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeA);

f_EUTRA_AS_CipheringAlgorithm_Set(eea0); // set null AS ciphering algorithm

//f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1);

// creates cell and performs registration

//@sic R5-104796 R5-106675DRX support on SS mandated sic@

//f_EUTRA_GenericRbEst_Common_716( eutra_Cell1, 0, 0, omit, cs_MAC_MainConfig_Explicit_RBC_DrxL(0), omit, omit, omit, 5); //@sic R5-112665 sic@

// Period is set as 4 = 80% of 6ms the Inactivity Timer

//@sic R5s100466 change of period for TA from 16 to 4 sic@

//f_EUTRA_CloseUE_TestLoopModeA( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) );

//f_Delay(0.66); //@sic R5s100466 add timer for TA MCE to be sent once sic@

//f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def);

// Go to state 4 to return no data in UL

// step to configure SS RLC/PDCP and MAC in No header manipulation mode on SS Side for DRB

//f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly);

// to configure SS to report reception of HARQ ACK or NACK

//f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1, tsc_L1Mac_IndicationMode_Enable );

//@sic R5-104796 R5-106675 DRX support on SS mandated sic@

f_EUTRA_LoopBackActivation_State4_Def( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) );

///////////////////////////////////////////////////////////////////////////////////

//      Test Body

//////////////////////////////////////////////////////////////////////////////////

f_EUTRA_TestBody_Set(true);

f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1,

cs_TimingInfo_SubFrame(8),//@sic R5s170542 R5s180475 sic@

cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4), // Imcs = Itbs =4, Nprb Results in TB size of 328 selected to be neither too big nor too small

cs_UplinkTimeAlignment_Synch_Auto_Def,                      //@sic R5s110618 sic@

cs_UL_GrantConfig_ContPeriodic(5));       // periodic grant every 5ms (80% of 6ms the Inactivity Timer)

// Reconfigure RRC Connection: Step 0A and 0B

f_RRC_ConnectionReconfiguration_MAC_MainConfig_PhysicalConfig (eutra_Cell1, cs_MAC_MainConfig_Explicit_RBC_DrxL(tsc_DrxStartOffset)); //@sic R5s110698 sic@

f_Delay(0.66); //

// stop periodic UL grants and restart automatic grant allocation instead; periodic TA is kept stopped:

f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1, cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def);

// suppress number of DL retransmissions:

f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1);

f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly);

// to configure SS to report reception of HARQ ACK or NACK

f_SS_ConfigUL_HARQ_IndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Enable);

//@siclog "Step 1" siclog@

// set CRC error as error mode

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,cs_DL_SCH_CRC_C_RNTI_Error(Erroneous));

// Choose the next subframe when the onDurationTimer is started.

v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_LongDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 1 - tsc_SubFrameX);

v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(0, tsc_NoPoll, f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN0, crs_PDCP_SDU_17B))));//@sic R5s110552 sic@

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU (eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing1),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(1)));

v_HARQRTTTimer := f_GetHARQRTTTimer(v_Timing1.Subframe.Number, v_EUTRA_FDD_TDD_Mode);

//@siclog "Step 3" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing1, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Normal),

cs_TimingInfo_EUTRA_NB(v_Timing));

v_NormalSF := f_DRX_NormalSF(v_Timing1.Subframe.Number, tsc_SubFrameY, v_EUTRA_FDD_TDD_Mode);

v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing1, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing2),

cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@

cs_HarqProcessAssignment_Specific(2)));

//@siclog "Step 5" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing2, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Erroneous),

cs_TimingInfo_EUTRA_NB(v_Timing));

/* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll,

int2bit (1, 10),

f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN1, crs_PDCP_SDU_17B))))); */

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing1, v_HARQRTTTimer);

if (v_EUTRA_FDD_TDD_Mode == FDD) //@sic R5s120658 sic@

{

if (not pc_ue_CategoryDL_M1) {

v_NormalSF := f_DRX_NormalSF( v_Timing3.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);

} else {

v_NormalSF := f_DRX_NormalSF( v_Timing3.Subframe.Number, 1, v_EUTRA_FDD_TDD_Mode);

}

}

else

{

v_NormalSF := f_DRX_NormalSF( v_Timing3.Subframe.Number, 1, v_EUTRA_FDD_TDD_Mode);

}

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing3, v_NormalSF+1);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing3),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(3)));

v_HARQRTTTimer := f_GetHARQRTTTimer(v_Timing3.Subframe.Number, v_EUTRA_FDD_TDD_Mode);

//@siclog "Step 7" siclog@

v_Timing := f_EUTRA_TimingInfoAdd(v_Timing3, 1);

f_SS_ConfigCRC_ErrorMode(eutra_Cell1,

cs_DL_SCH_CRC_C_RNTI_Error(Normal),

cs_TimingInfo_EUTRA_NB(v_Timing));

v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer);

if (v_EUTRA_FDD_TDD_Mode == FDD)      //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, tsc_DRXRetranTimer - tsc_SubFrameZ, v_EUTRA_FDD_TDD_Mode);

}

else if (v_EUTRA_FDD_TDD_Mode == TDD)

{

v_NormalSF := f_DRX_NormalSF( v_Timing4.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);

}

v_Timing4 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF);

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing4),

cs_MAC_PDUList_1 ( cs_MAC_PDU_DRX_Command_10Padding ), //@sic R5s110618 sic@

cs_HarqProcessAssignment_Specific(4)));

//@siclog "Step 9" siclog@

/* v_EncodedRlcPdu := f_RLC_PDU_Encvalue (cs_RLC_AMD_1PDU(tsc_NoPoll,

int2bit (2,10),

f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN2, crs_PDCP_SDU_17B))))); */

if (v_EUTRA_FDD_TDD_Mode == FDD)    //@sic R5s120658 sic@

{

v_NormalSF := f_DRX_NormalSF(v_Timing4.Subframe.Number, tsc_SubFrameZ - 1, v_EUTRA_FDD_TDD_Mode);

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing4, v_NormalSF);

}

else if (v_EUTRA_FDD_TDD_Mode == TDD)

{

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing3, v_HARQRTTTimer);

v_NormalSF := f_DRX_NormalSF(v_Timing5.Subframe.Number, 5, v_EUTRA_FDD_TDD_Mode);

v_Timing5 := f_EUTRA_TimingInfoAdd(v_Timing5, v_NormalSF);

}

DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU ( eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing5),

cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Specific(5) ));

// Preconfig SS for preventing UE to enter DRX

// @sic R5s100520 sic@

//@sic R5-104796 DRX support on SS mandated sic@

// Period is set as 16 = 80% of 20ms the Inactivity Timer

t_Watchdog.start;

v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode);

//@siclog "Step 2" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 1?

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), nack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

//@siclog "Step 4" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 3?

v_Timing2 := f_AckNack_TimingInfo(v_Timing2, v_EUTRA_FDD_TDD_Mode);

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing2.SFN.Number, v_Timing2.Subframe.Number), ack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

//@siclog "Step 6" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 5?

v_Timing3 := f_AckNack_TimingInfo(v_Timing3, v_EUTRA_FDD_TDD_Mode);

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing3.SFN.Number, v_Timing3.Subframe.Number), nack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

//@siclog "Step 8" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 7?

v_Timing4 := f_AckNack_TimingInfo(v_Timing4, v_EUTRA_FDD_TDD_Mode);

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing4.SFN.Number, v_Timing4.Subframe.Number), ack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

//@siclog "Step 10" siclog@

// Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 9?

v_Timing5 := f_AckNack_TimingInfo(v_Timing5, v_EUTRA_FDD_TDD_Mode);

SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, cr_TimingInfo(v_Timing5.SFN.Number, v_Timing5.Subframe.Number), ack ));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 10");

t_Watchdog.stop;

//test body ends

f_EUTRA_TestBody_Set(false);

///////////////////////////////////////////////////////////////////////////////////

//      Post Amble

//////////////////////////////////////////////////////////////////////////////////

// to configure SS to stop report reception of HARQ ACK or NACK

f_SS_ConfigUL_HARQ_IndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Disable);

f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NormalMode); //@sic R5s100354 sic@

// leaving UE test state 4

//@sic R5-103847 Deactivate test loop and open test loop is removed sic@

v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, tsc_LongDRXCycle, tsc_DrxStartOffset, 0);

f_EUTRA_RRC_ConnectionRelease(eutra_Cell1, tsc_RRC_TI_Def, other, cs_TimingInfo_EUTRA_NB(v_Timing1));

// Release RRC Connection and Switch off

f_EUTRA_Postamble(eutra_Cell1, E1_IDLE);//@sic R5-103875 sic@

};