3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220067

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2283 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EN-DC CA test case 8.2.4.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add EN-DC CA test case 8.2.4.2.1.1 to the IWD_21wk49 ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.2.4.2.1.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1 Overview	2

1.1 Verification Test Summary	2

2 Corrections required	3

3 Branches executed	3

4 Execution Log Files	3

4.1 HiSilicon Balong 5000 UE	3

5 References	3

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of EN-DC test case 8.2.4.2.1.1 which is part of the EN-DC test suite in iWD_TTCN3-B2020-09_D21wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Ting Gao

gaoting@starpointcomm.com

  - Verification Test Summary

Test Case: 	8.2.4.2.1.1.ENDC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	Starpoint SP9500 Test System

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

- Corrections required

None.

- Branches executed

## This EN-DC TC was executed in Band combination “DC_3A_n78C” using NEA2/NIA2 algorithms.

- Execution Log Files

  - HiSilicon Balong 5000 UE

The HiSi Balong 5000 UE passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file: 
\TC_8_2_4_2_1_1_ENDC_DC_3A_n78C_PASS.spm

- PIXCIT settings used:

\PCT_PICS_PIXIT.sppar

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s220068: Supporting information for agreement of EN-DC CA test case 8.2.4.2.1.1 in FR1 |
| --- | --- |