3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220094

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2296 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1: Addition of NR5GC Logged MDT FR1 test case 8.1.6.1.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - SS is expecting not to include IE logMeasAvailable in the RRCSetupComplete at step 7. Hence UE will not populate nonCriticalEextension for UE_MeasurementsAvailable_r16.. - UE uses SRB2 when it has included logMeasReport_r16 as per 38.331 clause 5.7.10.3 at step 26. 1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; - else: 2> submit the UEInformationResponse message to lower layers for transmission via SRB1.” - To evaluate relativeTimeStamp last entry, v_FieldCnt count reduce by one at step 26, for detailed info please see below. - SS is expecting not to include measResultNeighCells-r16 at step 26 on UEInformationResponse. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - nonCriticalExtension omitted from RRCSetupComplete. - UEInformationResponse reception modified for SRB2. - Updated v_Fieldcnt to evaluate relativeTimeStamp last entry. - measResultNeighCells_r16 ommited while evaluating UEInformationResponse. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will fail conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR …CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

1.1	Verification Test Summary	4

2	Corrections required	4

2.1	Change 1	4

3	Branches executed	15

4	Execution Log Files	15

4.1	Mediatek MT6983	15

5	References	15

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.2.6 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.6.1.2.6

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Mediatek MT6983

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_2_6_TestBody () |
| --- | --- |
| Reason for change | - SS is expecting not to include IE logMeasAvailable in the RRCSetupComplete at step 7. Hence UE will not populate nonCriticalEextension for UE_MeasurementsAvailable_r16. - UE uses SRB2 when it has included logMeasReport_r16 as per 38.331 clause 5.7.10.3 at step 26. 1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; - else: 2> submit the UEInformationResponse message to lower layers for transmission via SRB1.” - To evaluate relativeTimeStamp last entry, v_FieldCnt count reduce by one at step 26, for detailed info please see below. - SS is expecting not to include measResultNeighCells-r16 at step 26 on UEInformationResponse. |
| Summary of change | - nonCriicalExtension omitted from RRCSetupComplete. - UEInformationResponse reception modified for SRB2. - Updated v_Fieldcnt to evaluate relativeTimeStamp last entry. - measResultNeighCells_r16 omitted while evaluating UEInformationResponse. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_2_6_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PLMN_Identity; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity; var PhysCellId v_PhysCellId; var TrackingAreaCode v_TAC; var ARFCN_ValueNR v_CarrierFreq; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_RelativeTimeStamp1; var integer v_RelativeTimeStamp2; var integer v_FieldCnt; var integer i; //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to outOfCoverage, to configure the UE to perform logging of measurement results v_PLMN_Identity := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PLMN_Identity, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_OutOfCoverage))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508 Table 4.5.4.2-3 are executed //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16_AllOmit)); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 12" siclog@ //The SS transmits a UEInformationRequest message with logMeasReportReq present SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 13" siclog@ //Check: Does the UE transmit a UEInformationResponse message including logMeasReport SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, omit, omit, omit, omit, omit))); //@siclog "Steps 14" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 15" siclog@ //The SS changes Cell 1 levels according to the row "T1" in table 8.1.6.1.2.6.3.2-1/2 f_NR_SetCellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @sic R5-216302 sic@ //@siclog "Steps 16" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 17" siclog@ //The SS changes Cell 1 levels according to the row "T2" in table 8.1.6.1.2.6.3.2-1/2 f_NR_SetCellPower(nr_Cell1, -85, -82); // @sic R5-216302 sic@ //@siclog "Steps 18" siclog@ //Wait 10s to allow UE to go to camp normaly state and activate logging f_Delay(10.0); //@siclog "Steps 19" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Steps 20" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_, omit, omit, omit, omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 21-24" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 25" siclog@ //The SS transmits a UEInformationRequest message SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 26" siclog@ //Check: Does the UE transmit a UEInformationResponse message SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, cr_LogMeasReport_r16(v_AbsoluteTimeInfo, cr_TraceReference_r16(v_PLMN_Identity), '001A'O, '05'O, ?, omit,omit,omit), omit, omit, omit, omit))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26: Not enough LogMeasurementInfo entries"); } v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_CarrierFreq := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); v_RelativeTimeStamp1 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[0].relativeTimeStamp_r16; v_RelativeTimeStamp2 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[v_FieldCnt].relativeTimeStamp_r16; if ((v_RelativeTimeStamp2 - v_RelativeTimeStamp1) < 10){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26: The relative TimeStamp of first entry is less than 10s before the last one") } for (i:=0; i<v_FieldCnt; i:=i+1) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(valueof(v_LogMeasInfo_r16), cr_LogMeasInfo_r16(-, cr_CGI_Info_Logging_r16(v_PLMN_Identity, v_CellIdentity, v_TAC), cr_MeasResultServCell_r16(v_MeasQuantityResults), {cr_MeasResultLogging2NR_r16(v_CarrierFreq, {cr_MeasResultLoggingNR_r16(v_PhysCellId, v_MeasQuantityResults)}) }, omit))){ if (i == v_FieldCnt-1) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26: None of the LogMeasurementInfo matched the template")} } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "step 26"); break; } } } //End of f_TC_8_1_6_1_2_6_TestBody

After Change

function f_TC_8_1_6_1_2_6_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PLMN_Identity; var B48_Type v_AbsoluteTimeInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams ; var template(present) NG_MobileIdentity v_S_TMSI; var template(present) LogMeasInfo_r16 v_LogMeasInfo_r16; var CellIdentity v_CellIdentity; var PhysCellId v_PhysCellId; var TrackingAreaCode v_TAC; var ARFCN_ValueNR v_CarrierFreq; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (present) MeasQuantityResults v_MeasQuantityResults; var integer v_RelativeTimeStamp1; var integer v_RelativeTimeStamp2; var integer v_FieldCnt; var integer i; //@siclog "Steps 1" siclog@ //SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to outOfCoverage, to configure the UE to perform logging of measurement results v_PLMN_Identity := f_NR_CellInfo_GetPLMN(nr_Cell1); v_AbsoluteTimeInfo := f_MDT_GetAbsolute_TimeStamp(); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PLMN_Identity, v_AbsoluteTimeInfo, cs_LoggedMeasurementConfiguration_reportType_OutOfCoverage))); //@siclog "Steps 2" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 4-6" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508 Table 4.5.4.2-3 are executed //@siclog "Steps 7" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? f_NR_Paging_Def(nr_Cell1, RRC_IDLE); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas(nr_Cell1,tsc_NR_RRC_TI_Def,?,-,-,-,-, -); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 8-11" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 12" siclog@ //The SS transmits a UEInformationRequest message with logMeasReportReq present SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 13" siclog@ //Check: Does the UE transmit a UEInformationResponse message including logMeasReport SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, omit, omit, omit, omit, omit))); //@siclog "Steps 14" siclog@ //The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell1); //@siclog "Steps 15" siclog@ //The SS changes Cell 1 levels according to the row "T1" in table 8.1.6.1.2.6.3.2-1/2 f_NR_SetCellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); // @sic R5-216302 sic@ //@siclog "Steps 16" siclog@ //Wait 30s to allow UE to activate logging f_Delay(30.0); //@siclog "Steps 17" siclog@ //The SS changes Cell 1 levels according to the row "T2" in table 8.1.6.1.2.6.3.2-1/2 f_NR_SetCellPower(nr_Cell1, -85, -82); // @sic R5-216302 sic@ //@siclog "Steps 18" siclog@ //Wait 10s to allow UE to go to camp normaly state and activate logging f_Delay(10.0); //@siclog "Steps 19" siclog@ //Steps 1 to 3 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed f_NR_Paging_Def(nr_Cell1, RRC_IDLE); f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_RRCSetup_Def(nr_Cell1); //@siclog "Steps 20" siclog@ //Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def,-,-,-, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(true_, omit, omit, omit, omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20"); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0010'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } //@siclog "Steps 21-24" siclog@ //Steps 5 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Steps 25" siclog@ //The SS transmits a UEInformationRequest message SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Steps 26" siclog@ //Check: Does the UE transmit a UEInformationResponse message SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1,tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, omit, omit, cr_LogMeasReport_r16(v_AbsoluteTimeInfo, cr_TraceReference_r16(v_PLMN_Identity), '001A'O, '05'O, ?, omit,omit,omit), omit, omit, omit, omit))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26: Not enough LogMeasurementInfo entries"); } v_CellIdentity := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_CarrierFreq := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); v_RelativeTimeStamp1 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[0].relativeTimeStamp_r16; v_RelativeTimeStamp2 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[v_FieldCnt-1].relativeTimeStamp_r16; if ((v_RelativeTimeStamp2 - v_RelativeTimeStamp1) < 10){ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26: The relative TimeStamp of first entry is less than 10s before the last one") } for (i:=0; i<v_FieldCnt; i:=i+1) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(valueof(v_LogMeasInfo_r16), cr_LogMeasInfo_r16(-, cr_CGI_Info_Logging_r16(v_PLMN_Identity, v_CellIdentity, v_TAC), cr_MeasResultServCell_r16(v_MeasQuantityResults), omit, omit))) { if (i == v_FieldCnt-1) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26: None of the LogMeasurementInfo matched the template")} } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "step 26"); break; } } } //End of f_TC_8_1_6_1_2_6_TestBody

# Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Mediatek MT6983

The Mediatek MT6983 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_6_1_2_6_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_6_1_2_6_PIXIT.txt

# References

| [1] | R5s220093 – NR5GC FR1 : Supporting information for addition of NR5GC Logged MDT test case 8.1.6.1.2.6 |
| --- | --- |