3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220098

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2300 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.4.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following implementation of R5-216170, the initialization for these test cases uses function f_NR_CellInfo_Pdsch_ConfigCommon() which uses template cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x, so the configured TDRA list is not correct for 7.1.1.1.4.2.x test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Create new function for PDSCH_CommonConfig for 7.1.1.4.2.x test cases using correct TDRA list |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.5.ENDC, 7.1.1.4.2.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR_Init_7_1_1_4_2_x() |
| --- | --- |
| Reason for change | Following implementation of R5-216170, the initialization for these test cases uses function f_NR_CellInfo_Pdsch_ConfigCommon() which uses template cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x. The configured TDRA list is not correct for 7.1.1.1.4.2.x test cases |
| Summary of change | Use a new function for PDSCH_ConfigCommon for 7.1.1.4.2.x test cases |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_NR_Init_7_1_1_4_2_x(NR_CellId_Type p_NR_CellId, NR_SearchSpace_SearchSpaceType_Type.ue_Specific.dci_Formats p_DciFormats, template (omit) PUSCH_Config.mcs_Table p_Mcs_Table, template (omit) PUSCH_Config.transformPrecoder p_TransformPrecoder,//@sic R5s201144 sic@ template (omit) RACH_ConfigCommon.msg3_transformPrecoder p_Msg_TransformPrecoder := omit) runs on NR_BASE_PTC { var BWP_Id v_BWP_Id := tsc_NR_BWP_Id; var template (value) NR_UplinkBWP_Type v_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, v_BWP_Id); var template (value) NR_DownlinkBWP_Type v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_NR_CellId, v_BWP_Id); var integer v_SearchSpaceIndex := f_NR_DownlinkBWP_GetSearchSpace_Index(v_DownlinkBWP, tsc_NR_SearchSpaceId_USS); //@sic R5s190927 sic@ var integer v_K2 := 4; // FR2 and Fr1 var PUSCH_TimeDomainResourceAllocation.startSymbolAndLength v_StartSymbolAndLength := 42; var NR_CellInfo_Type v_NR_CellInfo :=f_NR_CellInfo_Get(p_NR_CellId); //@sic R5s201144 sic@ var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_StartSymbolAndLength := 52;} v_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_71142x(v_K2, v_StartSymbolAndLength); v_UplinkBWP.Dedicated.R15.pusch_Config.setup.mcs_Table := p_Mcs_Table; //qam64 if omit v_UplinkBWP.Dedicated.R15.pusch_Config.setup.transformPrecoder :=p_TransformPrecoder; v_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_71142x(v_SubcarrierSpacing)); //@sic R5-210321 sic@ v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; if (match(enabled, p_TransformPrecoder)) { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.mcs_TableTransformPrecoder := omit; } if (ispresent (p_Msg_TransformPrecoder)){//@sic R5s201144 sic@ v_UplinkBWP.Common.R15.rach_ConfigCommon.setup.msg3_transformPrecoder := p_Msg_TransformPrecoder; if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.servingCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.msg3_transformPrecoder := enabled; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } } v_DownlinkBWP.Pdcch.SearchSpaceArray[v_SearchSpaceIndex].SearchSpaceConfigAtUE.R15.searchSpaceType.ue_Specific.dci_Formats := p_DciFormats; if(p_DciFormats == formats0_1_And_1_1) { if (pc_dynamicSwitchRA_Type0_1_PUSCH) //@sic R5s202610 sic@ { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.resourceAllocation := dynamicSwitch; } else { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.resourceAllocation :=resourceAllocationType1 } } //@sic R5-216170 sic@ v_DownlinkBWP.Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x; f_NR_CellInfo_SetUplinkBWP(p_NR_CellId, v_UplinkBWP, v_BWP_Id); f_NR_CellInfo_SetDownlinkBWP(p_NR_CellId, v_DownlinkBWP, v_BWP_Id); f_NR_CellInfo_Pdsch_ConfigCommon(p_NR_CellId); };

After Change

function f_NR_Init_7_1_1_4_2_x(NR_CellId_Type p_NR_CellId, NR_SearchSpace_SearchSpaceType_Type.ue_Specific.dci_Formats p_DciFormats, template (omit) PUSCH_Config.mcs_Table p_Mcs_Table, template (omit) PUSCH_Config.transformPrecoder p_TransformPrecoder,//@sic R5s201144 sic@ template (omit) RACH_ConfigCommon.msg3_transformPrecoder p_Msg_TransformPrecoder := omit) runs on NR_BASE_PTC { var BWP_Id v_BWP_Id := tsc_NR_BWP_Id; var template (value) NR_UplinkBWP_Type v_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, v_BWP_Id); var template (value) NR_DownlinkBWP_Type v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_NR_CellId, v_BWP_Id); var integer v_SearchSpaceIndex := f_NR_DownlinkBWP_GetSearchSpace_Index(v_DownlinkBWP, tsc_NR_SearchSpaceId_USS); //@sic R5s190927 sic@ var integer v_K2 := 4; // FR2 and Fr1 var PUSCH_TimeDomainResourceAllocation.startSymbolAndLength v_StartSymbolAndLength := 42; var NR_CellInfo_Type v_NR_CellInfo :=f_NR_CellInfo_Get(p_NR_CellId); //@sic R5s201144 sic@ var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_StartSymbolAndLength := 52;} v_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_71142x(v_K2, v_StartSymbolAndLength); v_UplinkBWP.Dedicated.R15.pusch_Config.setup.mcs_Table := p_Mcs_Table; //qam64 if omit v_UplinkBWP.Dedicated.R15.pusch_Config.setup.transformPrecoder :=p_TransformPrecoder; v_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_71142x(v_SubcarrierSpacing)); //@sic R5-210321 sic@ v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_UplinkBWP.Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; if (match(enabled, p_TransformPrecoder)) { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.mcs_TableTransformPrecoder := omit; } if (ispresent (p_Msg_TransformPrecoder)){//@sic R5s201144 sic@ v_UplinkBWP.Common.R15.rach_ConfigCommon.setup.msg3_transformPrecoder := p_Msg_TransformPrecoder; if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.servingCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.msg3_transformPrecoder := enabled; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } } v_DownlinkBWP.Pdcch.SearchSpaceArray[v_SearchSpaceIndex].SearchSpaceConfigAtUE.R15.searchSpaceType.ue_Specific.dci_Formats := p_DciFormats; if(p_DciFormats == formats0_1_And_1_1) { if (pc_dynamicSwitchRA_Type0_1_PUSCH) //@sic R5s202610 sic@ { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.resourceAllocation := dynamicSwitch; } else { v_UplinkBWP.Dedicated.R15.pusch_Config.setup.resourceAllocation :=resourceAllocationType1 } } //@sic R5-216170 sic@ v_DownlinkBWP.Pdsch.ConfigCommon.R15.pdsch_TimeDomainAllocationList := cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x; f_NR_CellInfo_SetUplinkBWP(p_NR_CellId, v_UplinkBWP, v_BWP_Id); f_NR_CellInfo_SetDownlinkBWP(p_NR_CellId, v_DownlinkBWP, v_BWP_Id); f_NR_CellInfo_Pdsch_ConfigCommon_71142x(p_NR_CellId); };

## Change 2

| Function name | New function f_NR_CellInfo_Pdsch_ConfigCommon_71142x() |
| --- | --- |
| Reason for change | Following implementation of R5-216170, the initialization for these test cases uses function f_NR_CellInfo_Pdsch_ConfigCommon() which uses template cs_NR_PDSCH_TimeDomainResourceAllocationList_71141x. The configured TDRA list is not correct for 7.1.1.1.4.2.x test cases |
| Summary of change | New function to configure PDCH_ConfigCommon for 7.1.1.4.2.x, using template cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

New function

function f_NR_CellInfo_Pdsch_ConfigCommon_71142x(NR_CellId_Type p_NR_CellId ) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ //@sic R5s190892 R5s190826 sic@ v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.servingCellConfigCommon.downlinkConfigCommon.initialDownlinkBWP.pdsch_ConfigCommon.setup.pdsch_TimeDomainAllocationList := valueof (cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x); } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }