3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220129

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0062 | rev | - | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 5.4 to the MCX ATS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2022-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add MCPTT TC 5.4 to the MCX ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX IPCAN test model. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to the MCX ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Logs in R5s220130 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

2.4	Change 4	6

3	Branches executed	7

4	Execution Log Files	7

4.1	Nemergent Solutions Client v7.0	7

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 5.4.MCPTT which is part of the ATS iwd-TTCN3-B2020-09_D21wk49 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Fidel Liberal
	fidel.liberal@ehu.eus

## Verification Test Summary

Test Case: 	5.4.MCPTT

ATS Version:	ATS iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	MCSTaaSting Tester

UE used:	Nemergent Client v7.0 on COTS LTE UE

Verification Status:	PASS

# Corrections required

Following changes are related to the need of a global entry in MCX_Global_Type to hold Floor Control mc_priority value.  This is needed at step 7 to 9 of TC 5.4:  A new mc-priority is set via UPDATE from the SS at step 7 which shall match the value in the 200 OK from the UE at step 8. Also at step 9 again a new value is used in the INVITE from the SS which shall match the mc-priority in the following 200 OK from the UE.

## Change 1

| Record name | MCX_Global_Type |
| --- | --- |
| Reason for change | Need of a global entry in MCX_Global_Type to hold Floor Control mc_priority value. This is needed at step 7 to 9 of TC 5.4: A new mc-priority is set via UPDATE from the SS at step 7 which shall match the value in the 200 OK from the UE at step 8. Also at step 9 again a new value is used in the INVITE from the SS which shall match the mc-priority in the following 200 OK from the UE |
| Summary of change | New entry added in record MCX_Global_Type |
| TTCN module | MCX_IMS_Component.ttcn |
| MCC160 Comment |  |

Before Change:

type record MCX_Global_Type { /* @status APPROVED (MCX) */ MCX_SecurityParameters_Type SecurityParameters optional, MCX_GroupInfoList_Type GroupInfoList, charstring ClientId optional, PortNumber_Type MediaControlPortUE, integer FloorControlSeqNums[2], O4_Type SSRC_UE optional, };

After Change:

type record MCX_Global_Type { /* @status APPROVED (MCX) */ MCX_SecurityParameters_Type SecurityParameters optional, MCX_GroupInfoList_Type GroupInfoList, charstring ClientId optional, PortNumber_Type MediaControlPortUE, integer FloorControlSeqNums[2], O4_Type SSRC_UE optional, integer FloorControl_mc_priority optional };

## Change 2

| Function name | fl_MCX_SDP_MediaDescription_CheckFloorControl |
| --- | --- |
| Reason for change | See Change 1 |
| Summary of change | Mc-priority of v_ExpectedFloorControlParameters depends on the value set in global record vc_Global.FloorControl_mc_priority |
| TTCN module | MCX_SDP_Messages.ttcn |
| MCC160 Comment |  |

Before Change:

fl_MCX_SDP_MediaDescription_CheckFloorControl (…) { … // Check floor control parameters select (p_OfferAnswer) { case (SDP_OFFER) { if (p_IsInitialOffer) { v_InitialOffer := true; } v_ExpectedFloorControlParameters := cr_MCPTT_SDP_FloorControlParameters_OFFER(v_Queuing, v_InitialOffer, p_ImplicitGrantRequest); } case (SDP_ANSWER) { v_ExpectedFloorControlParameters := cr_MCPTT_SDP_FloorControlParameters_ANSWER(v_Queuing); } } ...

After Change:

fl_MCX_SDP_MediaDescription_CheckFloorControl (…) { … // Check floor control parameters select (p_OfferAnswer) case (SDP_OFFER) { if (p_IsInitialOffer) { v_InitialOffer := true; } v_ExpectedFloorControlParameters := cr_MCPTT_SDP_FloorControlParameters_OFFER(v_Queuing, v_InitialOffer, p_ImplicitGrantRequest); } case (SDP_ANSWER) { if (isvalue(vc_Global.FloorControl_mc_priority)) { v_ExpectedFloorControlParameters := cr_MCPTT_SDP_FloorControlParameters_ANSWER(v_Queuing, vc_Global.FloorControl_mc_priority); } else { v_ExpectedFloorControlParameters := cr_MCPTT_SDP_FloorControlParameters_ANSWER(v_Queuing); } } }

## Change 3

| Template name | cs_MCX_IMS_PTC_Global_Init |
| --- | --- |
| Reason for change | See Change 1 |
| Summary of change | Entry FloorControl_mc_priority set to omit in initialization template |
| TTCN module | MCX_IMS_Component.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) MCX_Global_Type cs_MCX_IMS_PTC_Global_Init := { /* @status APPROVED (MCX) */ SecurityParameters := omit, GroupInfoList := {}, ClientId := omit, MediaControlPortUE := 0, FloorControlSeqNums := { 0, 0 }, SSRC_UE := omit };

After Change:

template (value) MCX_Global_Type cs_MCX_IMS_PTC_Global_Init := { /* @status APPROVED (MCX) */ SecurityParameters := omit, GroupInfoList := {}, ClientId := omit, MediaControlPortUE := 0, FloorControlSeqNums := { 0, 0 }, SSRC_UE := omit, FloorControl_mc_priority := omit };

## Change 4

| Function name | f_TC_5_4_MCPTT_MCX_IMS |
| --- | --- |
| Reason for change | See Change 1 |
| Summary of change | - Mc-priority value is set in global record in order to use the correct value when matching SDP/FloorControl received contents. - ‘GROUP_CALL’ must be passed as parameter to function f_MCX_InviteMT_SendINVITE |
| TTCN module | MCPTT_ClientConfiguration_MCX_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_5_4_MCPTT_MCX_IMS(...) { … // @siclog "Steps 7 - 8" siclog@ v_MC_Priority := 2; v_SdpMessageTX := f_MCPTT_SdpOffer_TX(-, v_FmtAudio, -, v_MC_Priority); v_SdpInfoRX := f_MCPTT_SdpInfo_RX(-, SDP_ANSWER); v_ContactSS := f_MCX_CallSetupMO_ContactSS(MCPTT, v_SessionID); // same Contact as in the response for the INVITE creating the dialog (200 OK) f_MCX_SendUPDATE_Receive200OK(v_InviteRequestWithSdpMO, v_ContactSS, v_SdpMessageTX, v_SdpInfoRX); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); v_MC_Priority := 3; v_SdpMessageTX := f_MCPTT_SdpOffer_TX(-, v_FmtAudio, -, v_MC_Priority); // @siclog "Step 9" siclog@ v_InviteRequestWithSdpMT := f_MCX_InviteMT_SendINVITE(MCPTT, -, -, -, v_ContactSS, -, v_SdpMessageTX, -, tsc_MCX_ReInvite); f_MCPTT_InviteMT_Receive200OK_SendACK(-, v_InviteRequestWithSdpMT, -, PRE_EST_SESSION_ESTAB_MOD, v_SdpInfoRX); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 Void" siclog@ ... }

After Change:

function f_TC_5_4_MCPTT_MCX_IMS(...) { … // @siclog "Steps 7 - 8" siclog@ v_MC_Priority := 2; vc_Global.FloorControl_mc_priority := v_MC_Priority; v_SdpMessageTX := f_MCPTT_SdpOffer_TX(-, v_FmtAudio, -, v_MC_Priority); v_SdpInfoRX := f_MCPTT_SdpInfo_RX(-, SDP_ANSWER); v_ContactSS := f_MCX_CallSetupMO_ContactSS(MCPTT, v_SessionID); // same Contact as in the response for the INVITE creating the dialog (200 OK) f_MCX_SendUPDATE_Receive200OK(v_InviteRequestWithSdpMO, v_ContactSS, v_SdpMessageTX, v_SdpInfoRX); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); v_MC_Priority := 3; vc_Global.FloorControl_mc_priority := v_MC_Priority; v_SdpMessageTX := f_MCPTT_SdpOffer_TX(-, v_FmtAudio, -, v_MC_Priority); // @siclog "Step 9" siclog@ v_InviteRequestWithSdpMT := f_MCX_InviteMT_SendINVITE(MCPTT, GROUP_CALL, -, -, v_ContactSS, -, v_SdpMessageTX, -, tsc_MCX_ReInvite); f_MCPTT_InviteMT_Receive200OK_SendACK(-, v_InviteRequestWithSdpMT, -, PRE_EST_SESSION_ESTAB_MOD, v_SdpInfoRX); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 Void" siclog@ ... }

# Branches executed

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE Band 1

- EEA2/EIA2 RRC/NAS ciphering/integrity algorithms

# Execution Log Files

## Nemergent Solutions Client v7.0

The Nemergent Solutions Client v7.0 passed this test case against the MCSTaaSting Tester. The documentation below is enclosed as evidence of the successful test case run [1]:

- SSLog_Coversheet.txt

- Signalling-TC5.4.MCPTT.html (RRC/NAS signalling, IP flow decrypted/decoded)

- MCX-PIXIT.cfg (Titan configuration and PIXITs)

- TLI Interface log

# References

| [1] | R5s220130: Supporting information for the addition of MCPTT test case 5.4 to the MCX ATS |
| --- | --- |