3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220156

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2182 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC R16 SON-MDT test case 8.1.6.1.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC RRC test case 8.1.6.1.3.3 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC RRC test case 8.1.6.1.3.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 R5-216824 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3.	Branches executed	7

4.	Execution Log Files	8

4.1	Qualcomm SM8450+SDX65	8

5.	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.3.3 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Narendra Kalahasti

narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.6.1.3.3.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8450+SDX65

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | fl_TC_8_1_6_1_3_3_TestBody |
| --- | --- |
| Reason for change | - The function f_NR5GC_RRCReestablishment_Step1To2() changes the RNTI value of any target cell to ‘tsc_C_RNTI_Value4’ if p_RNTI_Value is left as default. This affects the RNTI value later reported in the RLFReport. - At step 3 minimum tolerance should be applied to the timer T301 - Need to reconfigure SRB2 + DRBs and AS security at SS side ready for RRC connection re-establishment after step 8. - At step 16 failedPCellID IE in the RLF_Report can either include CGI or PCI-ARFCN |
| Summary of change | - Used RNTI value of current serving cell as input parameter value to function f_NR5GC_RRCReestablishment_Steps1To2() - Apply minimum timer tolerance at step 3 - Add step 8A to reconfigure SRB2 + DRBs - At step 16 allow any valid value for failedPCellID Note : The associated prose CR R5-216824 is raised at RAN5’#93. |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_6_1_3_3_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var RLF_Report_r16 v_Nr_Rlf_Report_r16; var RNTI_Value v_RNTI_Value_Cell3; // @sic R5-216290 sic@ var PhysCellId v_PhysCellId_Cell3 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell3); var ARFCN_ValueNR v_SSB_Freq_Cell3 := f_NR_CellInfo_GetFrequencySSB(nr_Cell3); var CellIdentity v_CellIdentity_Cell6 := f_NR_CellInfo_GetCellIdentity(nr_Cell6); var PLMN_Identity v_PLMN_Identity_Cell6 := f_NR_CellInfo_GetPLMN (nr_Cell6); var TrackingAreaCode v_TAC_Cell6 := f_NR_CellInfo_GetTAC (nr_Cell6); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell6, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; //@siclog "Step 1" siclog@ //The SS changes NR Cell 1 and Cell 3 parameters according to the row "T1" in Table 8.1.6.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The UE transmits an RRCReestablishmentRequest message on NR Cell 3. f_NR5GC_RRCReestablishment_Steps1To2(nr_Cell1, nr_Cell3, otherFailure); v_RNTI_Value_Cell3 := f_NR_CellInfo_GetRNTI(nr_Cell3); // @sic R5-216290 sic@ //@siclog "Step 3" siclog@ //The SS does not respond to any RRCReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to NR RRC_IDLE state on NR Cell 3. f_Delay(1.0); //@siclog "Steps 4-5" siclog@ //Steps 1-2 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. f_NR_RRC_SetupRequest_Def(nr_Cell3); f_NR_RRCSetup_Def(nr_Cell3); //@siclog "Step 6" siclog@ //Check: Does the UE transmit an RRCSetupComplete message with rlf-InfoAvailable included and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell3, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(-, -, -, -, true_)))); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 7-8" siclog@ //Steps 4-5 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); //@siclog "Step 9" siclog@ //The SS changes NR Cell 3 and Cell 6 parameters according to the row "T2" in Table 8.1.6.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Steps 10-14" siclog@ //The UE transmits an RRCReestablishmentRequest message on NR Cell 6. //The SS transmits an RRCReestablishment message. //Check: Does the UE transmit an RRCReestablishmentComplete message with rlf-InfoAvailable included? //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. //The UE transmits an RRCReconfigurationtComplete message. f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell3, nr_Cell6, rlf_InfoAvailable); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 15" siclog@ //The SS transmits a UEInformationRequest message with rlf-ReportReq set to true. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell6, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Step 16" siclog@ //Check: Does the UE transmit a UEInformationResponse message on Cell 6 with rlf-Report included? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell6, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-,?))) -> value v_ReceivedAsp; v_Nr_Rlf_Report_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_RNTI_Value_Cell3, -, // @sic R5-216290 sic@ ?, ?, omit, omit, cr_NR_RLF_Report_failedPCellId_PciArfcn_r16(v_PhysCellId_Cell3, v_SSB_Freq_Cell3), omit, omit, cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell6, v_CellIdentity_Cell6, v_TAC_Cell6), omit, -, rlf, t310_Expiry, -, omit))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16"); } }

After Change

function fl_TC_8_1_6_1_3_3_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var RLF_Report_r16 v_Nr_Rlf_Report_r16; var RNTI_Value v_RNTI_Value_Cell3; // @sic R5-216290 sic@ var PhysCellId v_PhysCellId_Cell3 := f_NR_CellInfo_GetPhysicalCellId(nr_Cell3); var ARFCN_ValueNR v_SSB_Freq_Cell3 := f_NR_CellInfo_GetFrequencySSB(nr_Cell3); var CellIdentity v_CellIdentity_Cell6 := f_NR_CellInfo_GetCellIdentity(nr_Cell6); var PLMN_Identity v_PLMN_Identity_Cell6 := f_NR_CellInfo_GetPLMN (nr_Cell6); var TrackingAreaCode v_TAC_Cell6 := f_NR_CellInfo_GetTAC (nr_Cell6); var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell3, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell6, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; var float v_T301Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //@siclog "Step 1" siclog@ //The SS changes NR Cell 1 and Cell 3 parameters according to the row "T1" in Table 8.1.6.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The UE transmits an RRCReestablishmentRequest message on NR Cell 3. f_NR5GC_RRCReestablishment_Steps1To2(nr_Cell1, nr_Cell3, otherFailure, f_NR_CellInfo_GetRNTI(nr_Cell3)); v_RNTI_Value_Cell3 := f_NR_CellInfo_GetRNTI(nr_Cell3); // @sic R5-216290 sic@ //@siclog "Step 3" siclog@ //The SS does not respond to any RRCReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to NR RRC_IDLE state on NR Cell 3. f_Delay(v_T301Min); //@siclog "Steps 4-5" siclog@ //Steps 1-2 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. f_NR_RRC_SetupRequest_Def(nr_Cell3); f_NR_RRCSetup_Def(nr_Cell3); //@siclog "Step 6" siclog@ //Check: Does the UE transmit an RRCSetupComplete message with rlf-InfoAvailable included and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area? v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell3, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, -, -, -, cr_RRCSetupComplete_v1610_IEs(*,*,cr_UE_MeasurementsAvailable_r16(-, -, -, -, true_)))); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Steps 7-8" siclog@ //Steps 4-5 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); //KS@8.1.6.1.x.x //@siclog "Step 8A" siclog@ //SS will perform reconfiguration of SRB2 and DRBs as per Steps 5 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR_RRC_ActivateSecurity (nr_Cell3, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR_RRCReconfigExistingDRBs (nr_Cell3, omit); //@siclog "Step 9" siclog@ //The SS changes NR Cell 3 and Cell 6 parameters according to the row "T2" in Table 8.1.6.1.3.3.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Steps 10-14" siclog@ //The UE transmits an RRCReestablishmentRequest message on NR Cell 6. //The SS transmits an RRCReestablishment message. //Check: Does the UE transmit an RRCReestablishmentComplete message with rlf-InfoAvailable included? //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. //The UE transmits an RRCReconfigurationtComplete message. f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell3, nr_Cell6, rlf_InfoAvailable); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 15" siclog@ //The SS transmits a UEInformationRequest message with rlf-ReportReq set to true. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell6, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Step 16" siclog@ //Check: Does the UE transmit a UEInformationResponse message on Cell 6 with rlf-Report included? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell6, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-,?))) -> value v_ReceivedAsp; v_Nr_Rlf_Report_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; //if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_RNTI_Value_Cell3, -, // @sic R5-216290 sic@ if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_RNTI_Value_Cell3, -,//KS@8.1.6.1.x.x ?, ?, omit, omit, ?), //KS@8.1.6.1.x.x omit, omit, cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell6, v_CellIdentity_Cell6, v_TAC_Cell6), omit, -, rlf, t310_Expiry, -, omit))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16"); } }

# Branches executed

The test case was executed on NR band n78 uing NR ciphering nea1 and integrity algorithm nia1

# Execution Log Files

## Qualcomm SM8450+SDX65

The Qualcomm SM8450+SDX65 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_8_1_6_1_3_3_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_8_1_6_1_3_3_PIXIT.xml

The Qualcomm SM8450+SDX65 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_8_1_6_1_3_3_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_8_1_6_1_3_3_PICS.txt

# References

| [1] | R5s211601 : NR5GC FR1 : Supporting information for addition of NR5GC R16 SON-MDT test case 8.1.6.1.3.3 |
| --- | --- |