3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220170

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0152 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC_Positioning_Functions for capability check |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | After including R5-214181 and R5-213638 in the TTCN implementation, the functions: f_POS_OTDOA_CheckEutraBand f_POS_NR_CheckCapabilities_NR_UL_SRS f_POS_NR_CheckCapabilities_Srs_CapabilityBandList include capability checks that are not properly implemented, leading into fail verdicts if UE does not report optional IEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the capability checks avoiding that the UE throw fail verdicts if not reporting optional IEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.1.NR5GC, 9.3.4.1.NR5GC, 9.3.4.2.NR5GC, 9.3.4.3.NR5GC, 9.3.4.4.NR5GC, 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1.1 Correction to f_POS_NR_OTDOA_CheckEutraBand	2

1.2 Correction to f_POS_NR_CheckCapabilities_NR_UL_SRS	6

1.3 Correction to f_POS_NR_CheckCapabilities_Srs_CapabilityBandList	7

## 1.1 Correction to f_POS_NR_OTDOA_CheckEutraBand

| Function name | f_POS_NR_OTDOA_CheckEutraBand |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function f_POS_OTDOA_CheckEutraBand checks for the value of: v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14 v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14 v_OtdoaProvideCapabilities.numberOfRXantennas_r14 without checking first that such fields are actually present. This leads into fail verdicts if UE does not report any of the 3 aforementioned fields. |
| Summary of change | Implement a presence check before checking the value of the fields aforementioned. |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_POS_NR_OTDOA_CheckEutraBand(OTDOA_ProvideCapabilities p_OtdoaProvideCapabilities) runs on NR_BASE_PTC

{

var OTDOA_ProvideCapabilities v_OtdoaProvideCapabilities;

var integer i;

var integer v_BandEUTRA;

var integer v_BitStringIndex;

var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics();

var bitstring v_SuppEutraBandsAccToPICS_v9a0 := f_DeriveSuppEutraBandsFromPics_v9a0();

var bitstring v_SuppEutraBandsAccToPICS_Chunk;

v_OtdoaProvideCapabilities := p_OtdoaProvideCapabilities;

if (not isvalue(v_OtdoaProvideCapabilities.supportedBandListEUTRA)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "SupportedBandListEUTRA not included in OTDOA capabilities");

}

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

// Check only the bands for which there are PICS; do not check other bands which may or may not be reported

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 0, 14); // bands 1 - 14

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 0, 14);

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 16, 30); // bands 17 - 46

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 16, 30);

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 47, 1); // band 48

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 47, 1);

if (lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA) == maxFBI) {

//Check supportedBandListEUTRA-v9a0

if (isvalue (v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0)) {

// @sic R5-150483 sic@

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

for (i := 0; i < lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0); i := i + 1) {

v_BandEUTRA := v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0[i].bandEUTRA_v9a0;

v_BitStringIndex := v_BandEUTRA - 1;

if (not f_Bitstring_BitIsSet(v_SuppEutraBandsAccToPICS_v9a0, v_BitStringIndex)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for band nr. " & int2str(v_BandEUTRA));

}

}

}

}

//Checking of interFreqRSTDmeasurement-r10.

if (pc_InterFreq_RSTD_WithGaps) {

if (not(v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10 == supported)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "interFreqRSTDmeasurement_r10 not reported")

}

}

//Checking of OTDOA measurements

//@sic R5-201624 sic@

if (pc_OTDOA_interFreqRSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interFreqRSTDmeasurement_r10")

}

if (pc_OTDOA_additionalNeighbourCellInfoList != ispresent (v_OtdoaProvideCapabilities.additionalNeighbourCellInfoList_r10)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalNeighbourCellInfoList_r10")

}

if (pc_OTDOA_prs_id != ispresent (v_OtdoaProvideCapabilities.prs_id_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_id_r14")

}

if (pc_OTDOA_tp_separation_via_muting != ispresent (v_OtdoaProvideCapabilities.tp_separation_via_muting_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for tp_separation_via_muting_r14")

}

if (pc_OTDOA_additional_prs_config != ispresent (v_OtdoaProvideCapabilities.additional_prs_config_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additional_prs_config_r14")

}

if (pc_OTDOA_prs_based_tbs != ispresent (v_OtdoaProvideCapabilities.prs_based_tbs_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_based_tbs_r14")

}

if (pc_OTDOA_additionalPathsReport != ispresent (v_OtdoaProvideCapabilities.additionalPathsReport_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalPathsReport_r14")

}

if (pc_OTDOA_densePrsConfig != ispresent (v_OtdoaProvideCapabilities.densePrsConfig_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for densePrsConfig_r14")

}

if (not (ispresent (v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14) and (v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14 == pc_OTDOA_maxSupportedPrsBandwidth_r14))){ //@sic R5-214181 sic@

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsBandwidth_r14")

}

if (pc_OTDOA_prsOccGroup != ispresent (v_OtdoaProvideCapabilities.prsOccGroup_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsOccGroup_r14")

}

if (pc_OTDOA_prsFrequencyHopping != ispresent (v_OtdoaProvideCapabilities.prsFrequencyHopping_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsFrequencyHopping_r14")

}

if (not (ispresent (v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14) and (v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14 == pc_OTDOA_maxSupportedPrsConfigs_r14))){ //@sic R5-214181 sic@

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsConfigs_r14")

}

if (pc_OTDOA_periodicalReporting != ispresent (v_OtdoaProvideCapabilities.periodicalReporting_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for periodicalReporting_r14")

}

if (pc_OTDOA_multiPrbNprs != ispresent (v_OtdoaProvideCapabilities.multiPrbNprs_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for multiPrbNprs_r14")

}

if (pc_OTDOA_idleStateForMeasurements != ispresent (v_OtdoaProvideCapabilities.idleStateForMeasurements_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for idleStateForMeasurements_r14")

}

if (not (ispresent (v_OtdoaProvideCapabilities.numberOfRXantennas_r14) and (v_OtdoaProvideCapabilities.numberOfRXantennas_r14 == pc_OTDOA_numberOfRXantennas_r14))){ //@sic R5-214181 sic@

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for numberOfRXantennas_r14")

}

if (pc_OTDOA_motionMeasurements != ispresent (v_OtdoaProvideCapabilities.motionMeasurements_r15)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for motionMeasurements_r15")

}

if (pc_OTDOA_interRAT_RSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interRAT_RSTDmeasurement_r15)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interRAT_RSTDmeasurement_r15")

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_OTDOA_CheckEutraBand(OTDOA_ProvideCapabilities p_OtdoaProvideCapabilities) runs on NR_BASE_PTC

{

var OTDOA_ProvideCapabilities v_OtdoaProvideCapabilities;

var integer i;

var integer v_BandEUTRA;

var integer v_BitStringIndex;

var bitstring v_SuppEutraBandsAccToPICS := f_DeriveSuppEutraBandsFromPics();

var bitstring v_SuppEutraBandsAccToPICS_v9a0 := f_DeriveSuppEutraBandsFromPics_v9a0();

var bitstring v_SuppEutraBandsAccToPICS_Chunk;

v_OtdoaProvideCapabilities := p_OtdoaProvideCapabilities;

if (not isvalue(v_OtdoaProvideCapabilities.supportedBandListEUTRA)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "SupportedBandListEUTRA not included in OTDOA capabilities");

}

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

// Check only the bands for which there are PICS; do not check other bands which may or may not be reported

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 0, 14); // bands 1 - 14

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 0, 14);

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 16, 30); // bands 17 - 46

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 16, 30);

v_SuppEutraBandsAccToPICS_Chunk := substr(v_SuppEutraBandsAccToPICS, 47, 1); // band 48

f_CheckUE_SupportedEUTRA_Bands_ChunkOtdoa (v_OtdoaProvideCapabilities, v_SuppEutraBandsAccToPICS_Chunk, 47, 1);

if (lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA) == maxFBI) {

//Check supportedBandListEUTRA-v9a0

if (isvalue (v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0)) {

// @sic R5-150483 sic@

// It is no longer checked that number of supported EUTRA bands is at least a subset of the PICS parameters (pc_eBanda_Supp)

// There may be more bands reported than there are PICS defined for bands

// Check that all OTDOA reported bands are also set to true in the EUTRA band PICS

for (i := 0; i < lengthof(v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0); i := i + 1) {

v_BandEUTRA := v_OtdoaProvideCapabilities.supportedBandListEUTRA_v9a0[i].bandEUTRA_v9a0;

v_BitStringIndex := v_BandEUTRA - 1;

if (not f_Bitstring_BitIsSet(v_SuppEutraBandsAccToPICS_v9a0, v_BitStringIndex)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for band nr. " & int2str(v_BandEUTRA));

}

}

}

}

//Checking of interFreqRSTDmeasurement-r10.

if (pc_InterFreq_RSTD_WithGaps) {

if (not(v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10 == supported)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "interFreqRSTDmeasurement_r10 not reported")

}

}

//Checking of OTDOA measurements

//@sic R5-201624 sic@

if (pc_OTDOA_interFreqRSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interFreqRSTDmeasurement_r10)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interFreqRSTDmeasurement_r10")

}

if (pc_OTDOA_additionalNeighbourCellInfoList != ispresent (v_OtdoaProvideCapabilities.additionalNeighbourCellInfoList_r10)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalNeighbourCellInfoList_r10")

}

if (pc_OTDOA_prs_id != ispresent (v_OtdoaProvideCapabilities.prs_id_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_id_r14")

}

if (pc_OTDOA_tp_separation_via_muting != ispresent (v_OtdoaProvideCapabilities.tp_separation_via_muting_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for tp_separation_via_muting_r14")

}

if (pc_OTDOA_additional_prs_config != ispresent (v_OtdoaProvideCapabilities.additional_prs_config_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additional_prs_config_r14")

}

if (pc_OTDOA_prs_based_tbs != ispresent (v_OtdoaProvideCapabilities.prs_based_tbs_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prs_based_tbs_r14")

}

if (pc_OTDOA_additionalPathsReport != ispresent (v_OtdoaProvideCapabilities.additionalPathsReport_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for additionalPathsReport_r14")

}

if (pc_OTDOA_densePrsConfig != ispresent (v_OtdoaProvideCapabilities.densePrsConfig_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for densePrsConfig_r14")

}

if (pc_OTDOA_maxSupportedPrsBandwidth != ispresent (v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14)){

if (not (v_OtdoaProvideCapabilities.maxSupportedPrsBandwidth_r14 == pc_OTDOA_maxSupportedPrsBandwidth_r14)){ //@sic R5-214181 sic@

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsBandwidth_r14")

}

}

if (pc_OTDOA_prsOccGroup != ispresent (v_OtdoaProvideCapabilities.prsOccGroup_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsOccGroup_r14")

}

if (pc_OTDOA_prsFrequencyHopping != ispresent (v_OtdoaProvideCapabilities.prsFrequencyHopping_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for prsFrequencyHopping_r14")

}

if (pc_OTDOA_maxSupportedPrsConfigs!= ispresent (v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14)){

if (not (v_OtdoaProvideCapabilities.maxSupportedPrsConfigs_r14 == pc_OTDOA_maxSupportedPrsConfigs_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for maxSupportedPrsConfigs_r14")

}

}

if (pc_OTDOA_periodicalReporting != ispresent (v_OtdoaProvideCapabilities.periodicalReporting_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for periodicalReporting_r14")

}

if (pc_OTDOA_multiPrbNprs != ispresent (v_OtdoaProvideCapabilities.multiPrbNprs_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for multiPrbNprs_r14")

}

if (pc_OTDOA_idleStateForMeasurements != ispresent (v_OtdoaProvideCapabilities.idleStateForMeasurements_r14)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for idleStateForMeasurements_r14")

}

if (pc_OTDOA_numberOfRXantennas != ispresent (v_OtdoaProvideCapabilities.numberOfRXantennas_r14)){

if (not (v_OtdoaProvideCapabilities.numberOfRXantennas_r14 == pc_OTDOA_numberOfRXantennas_r14)){ //@sic R5-214181 sic@

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for numberOfRXantennas_r14")

}

}

if (pc_OTDOA_motionMeasurements != ispresent (v_OtdoaProvideCapabilities.motionMeasurements_r15)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for motionMeasurements_r15")

}

if (pc_OTDOA_interRAT_RSTDmeasurement != ispresent (v_OtdoaProvideCapabilities.interRAT_RSTDmeasurement_r15)){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for interRAT_RSTDmeasurement_r15")

}

}<<SKIPPED CODE>>

## 1.2 Correction to f_POS_NR_CheckCapabilities_NR_UL_SRS

| Function name | f_POS_NR_CheckCapabilities_NR_UL_SRS |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function f_POS_NR_CheckCapabilities_NR_UL_SRS is not doing the capability check properly for: maxNumberSRS_PosPathLossEstimateAllServingCells_r16 and maxNumberSRS_PosSpatialRelationsAllServingCells_r16 This leads into fail verdicts if UE does not report the aforementioned |
| Summary of change | Change the implementation of the capability check in f_POS_NR_CheckCapabilities_NR_UL_SRS to avoid faild verdicts in case the IEs are not reported |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_NR_UL_SRS (NR_UL_SRS_Capability_r16 p_NrUlSrsCapability) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

f_POS_NR_CheckCapabilities_Srs_CapabilityBandList(p_NrUlSrsCapability.srs_CapabilityBandList_r16);

if (ispresent(p_NrUlSrsCapability.srs_PosResourceConfigCA_BandList_r16)) {

f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList (p_NrUlSrsCapability.srs_PosResourceConfigCA_BandList_r16);

}

if (not(ispresent(p_NrUlSrsCapability.maxNumberSRS_PosPathLossEstimateAllServingCells_r16) and (p_NrUlSrsCapability.maxNumberSRS_PosPathLossEstimateAllServingCells_r16 == pc_maxNumberSRS_PosPathLossEstimateAllServingCells))){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR UL SRS maxNumberSRS-PosPathLossEstimateAllServingCells-r16");

}

if (not(ispresent(p_NrUlSrsCapability.maxNumberSRS_PosSpatialRelationsAllServingCells_r16) and (p_NrUlSrsCapability.maxNumberSRS_PosSpatialRelationsAllServingCells_r16 == pc_maxNumberSRS_PosSpatialRelationsAllServingCell))){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR UL SRSmaxNumberSRS-PosSpatialRelationsAllServingCells-r16");

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_NR_UL_SRS (NR_UL_SRS_Capability_r16 p_NrUlSrsCapability) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

f_POS_NR_CheckCapabilities_Srs_CapabilityBandList(p_NrUlSrsCapability.srs_CapabilityBandList_r16);

if (ispresent(p_NrUlSrsCapability.srs_PosResourceConfigCA_BandList_r16)) {

f_POS_NR_CheckCapabilities_Srs_PosResourceConfigCA_BandList (p_NrUlSrsCapability.srs_PosResourceConfigCA_BandList_r16);

}

if (ispresent(p_NrUlSrsCapability.maxNumberSRS_PosPathLossEstimateAllServingCells_r16)){

if(p_NrUlSrsCapability.maxNumberSRS_PosPathLossEstimateAllServingCells_r16 != pc_maxNumberSRS_PosPathLossEstimateAllServingCells){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR UL SRS maxNumberSRS-PosPathLossEstimateAllServingCells-r16");

}

}

if (ispresent(p_NrUlSrsCapability.maxNumberSRS_PosSpatialRelationsAllServingCells_r16)){

if(p_NrUlSrsCapability.maxNumberSRS_PosSpatialRelationsAllServingCells_r16 != pc_maxNumberSRS_PosSpatialRelationsAllServingCell){

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for NR UL SRSmaxNumberSRS-PosSpatialRelationsAllServingCells-r16");

}

}

}

<<SKIPPED CODE>>

## 1.3 Correction to f_POS_NR_CheckCapabilities_Srs_CapabilityBandList

| Function name | f_POS_NR_CheckCapabilities_Srs_CapabilityBandList |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function f_POS_NR_CheckCapabilities_Srs_CapabilityBandList is not doing the capability check properly for: olpc_SRS_Pos_r16 and spatialRelationsSRS-Pos-r16 This leads into fail verdicts if UE does not report the aforementioned |
| Summary of change | Change the implementation of the capability check in f_POS_NR_CheckCapabilities_Srs_CapabilityBandList to avoid faild verdicts in case the IEs are not reported |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_Srs_CapabilityBandList (NR_UL_SRS_Capability_r16.srs_CapabilityBandList_r16 p_NrUlSrsCapabilityBandList) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer v_Index;

var boolean v_Found_Olpc_SRS_PosBasedOnPRS_Serving := false;

var boolean v_Found_Olpc_SRS_PosBasedOnSSB_Neigh := false;

var boolean v_Found_Olpc_SRS_PosBasedOnPRS_Neigh := false;

var boolean v_Found_MaxNumberPathLossEstimatePerServing := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnSSB_Serving := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnCSI_RS_Serving := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnPRS_Serving := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnSRS := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnSSB_Neigh := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnPRS_Neigh := false;

for (v_Index := 0; v_Index < lengthof(p_NrUlSrsCapabilityBandList); v_Index:=v_Index+1) {

fl_CheckNRBandCapability(p_NrUlSrsCapabilityBandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS

if (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16)) {

if (pc_olpc_SRS_PosBasedOnPRS_Serving == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.olpc_SRS_PosBasedOnPRS_Serving_r16)))  {

v_Found_Olpc_SRS_PosBasedOnPRS_Serving := true;

}

if (pc_olpc_SRS_PosBasedOnSSB_Neigh == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.olpc_SRS_PosBasedOnSSB_Neigh_r16)))  {

v_Found_Olpc_SRS_PosBasedOnSSB_Neigh := true;

}

if (pc_olpc_SRS_PosBasedOnPRS_Neigh == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.olpc_SRS_PosBasedOnPRS_Neigh_r16)))  {

v_Found_Olpc_SRS_PosBasedOnPRS_Neigh := true;

}

if (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.maxNumberPathLossEstimatePerServing_r16) and

(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.maxNumberPathLossEstimatePerServing_r16 == pc_maxNumberPathLossEstimatePerServing)) {

v_Found_MaxNumberPathLossEstimatePerServing := true;

}

}

if (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16)) {

if (pc_spatialRelation_SRS_PosBasedOnSSB_Serving == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnSSB_Serving_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnSSB_Serving := true;

}

if (pc_spatialRelation_SRS_PosBasedOnCSI_RS_Serving == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnCSI_RS_Serving_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnCSI_RS_Serving := true;

}

if (pc_spatialRelation_SRS_PosBasedOnPRS_Serving == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnPRS_Serving_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnPRS_Serving := true;

}

if (pc_spatialRelation_SRS_PosBasedOnSRS == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnSRS_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnSRS := true;

}

if (pc_spatialRelation_SRS_PosBasedOnSSB_Neigh == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnSSB_Neigh_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnSSB_Neigh := true;

}

if (pc_spatialRelation_SRS_PosBasedOnPRS_Neigh == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnPRS_Neigh_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnPRS_Neigh := true;

}

}

}

if (not (v_Found_Olpc_SRS_PosBasedOnPRS_Serving and v_Found_Olpc_SRS_PosBasedOnSSB_Neigh and v_Found_Olpc_SRS_PosBasedOnPRS_Neigh and v_Found_MaxNumberPathLossEstimatePerServing)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for olpc-SRS-Pos-r16");

}

if (not (v_Found_SpatialRelation_SRS_PosBasedOnSSB_Serving and v_Found_SpatialRelation_SRS_PosBasedOnCSI_RS_Serving and v_Found_SpatialRelation_SRS_PosBasedOnPRS_Serving and

v_Found_SpatialRelation_SRS_PosBasedOnSRS and v_Found_SpatialRelation_SRS_PosBasedOnSSB_Neigh and v_Found_SpatialRelation_SRS_PosBasedOnPRS_Neigh)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability spatialRelationsSRS-Pos-r16");

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CheckCapabilities_Srs_CapabilityBandList (NR_UL_SRS_Capability_r16.srs_CapabilityBandList_r16 p_NrUlSrsCapabilityBandList) runs on NR_BASE_PTC

{ //@sic POS Rel-16 R5-213638 sic@

var integer v_Index;

var boolean v_Found_Olpc_SRS_PosBasedOnPRS_Serving := false;

var boolean v_Found_Olpc_SRS_PosBasedOnSSB_Neigh := false;

var boolean v_Found_Olpc_SRS_PosBasedOnPRS_Neigh := false;

var boolean v_Found_MaxNumberPathLossEstimatePerServing := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnSSB_Serving := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnCSI_RS_Serving := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnPRS_Serving := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnSRS := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnSSB_Neigh := false;

var boolean v_Found_SpatialRelation_SRS_PosBasedOnPRS_Neigh := false;

for (v_Index := 0; v_Index < lengthof(p_NrUlSrsCapabilityBandList); v_Index:=v_Index+1) {

fl_CheckNRBandCapability(p_NrUlSrsCapabilityBandList[v_Index].freqBandIndicatorNR_r16); // checks freqBandIndicatorNR-r16 against capability PICS

if (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16)) {

if (pc_olpc_SRS_PosBasedOnPRS_Serving == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.olpc_SRS_PosBasedOnPRS_Serving_r16)))  {

v_Found_Olpc_SRS_PosBasedOnPRS_Serving := true;

}

if (pc_olpc_SRS_PosBasedOnSSB_Neigh == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.olpc_SRS_PosBasedOnSSB_Neigh_r16)))  {

v_Found_Olpc_SRS_PosBasedOnSSB_Neigh := true;

}

if (pc_olpc_SRS_PosBasedOnPRS_Neigh == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.olpc_SRS_PosBasedOnPRS_Neigh_r16)))  {

v_Found_Olpc_SRS_PosBasedOnPRS_Neigh := true;

}

if (ispresent(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.maxNumberPathLossEstimatePerServing_r16) and

(p_NrUlSrsCapabilityBandList[v_Index].olpc_SRS_Pos_r16.maxNumberPathLossEstimatePerServing_r16 == pc_maxNumberPathLossEstimatePerServing)) {

v_Found_MaxNumberPathLossEstimatePerServing := true;

}

if (not (v_Found_Olpc_SRS_PosBasedOnPRS_Serving and v_Found_Olpc_SRS_PosBasedOnSSB_Neigh and v_Found_Olpc_SRS_PosBasedOnPRS_Neigh and v_Found_MaxNumberPathLossEstimatePerServing)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability for olpc-SRS-Pos-r16");

}

}

if (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16)) {

if (pc_spatialRelation_SRS_PosBasedOnSSB_Serving == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnSSB_Serving_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnSSB_Serving := true;

}

if (pc_spatialRelation_SRS_PosBasedOnCSI_RS_Serving == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnCSI_RS_Serving_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnCSI_RS_Serving := true;

}

if (pc_spatialRelation_SRS_PosBasedOnPRS_Serving == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnPRS_Serving_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnPRS_Serving := true;

}

if (pc_spatialRelation_SRS_PosBasedOnSRS == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnSRS_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnSRS := true;

}

if (pc_spatialRelation_SRS_PosBasedOnSSB_Neigh == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnSSB_Neigh_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnSSB_Neigh := true;

}

if (pc_spatialRelation_SRS_PosBasedOnPRS_Neigh == (ispresent(p_NrUlSrsCapabilityBandList[v_Index].spatialRelationsSRS_Pos_r16.spatialRelation_SRS_PosBasedOnPRS_Neigh_r16)))  {

v_Found_SpatialRelation_SRS_PosBasedOnPRS_Neigh := true;

}

if (not (v_Found_SpatialRelation_SRS_PosBasedOnSSB_Serving and v_Found_SpatialRelation_SRS_PosBasedOnCSI_RS_Serving and v_Found_SpatialRelation_SRS_PosBasedOnPRS_Serving and

v_Found_SpatialRelation_SRS_PosBasedOnSRS and v_Found_SpatialRelation_SRS_PosBasedOnSSB_Neigh and v_Found_SpatialRelation_SRS_PosBasedOnPRS_Neigh)) {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "PICS does not match reported capability spatialRelationsSRS-Pos-r16");

}

}

}

}<<SKIPPED CODE>>