3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220171

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2209 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 7.1.1.1.1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In this test case at Step0A SS transmits RRCReconfiguration message and updates BWP-UplinkCommon - > RACH-ConfigCommon When configuring the SS entity TTCN is providing the RACHConfigDedicated parameter update - The reconfiguration parameters on the SS entity is resulting in NR_UplinkBWP_ConfigCommon update done separately for v_NR_UplinkBWP and v_TDD_UL_DL_ConfigCommon due to pattern change. This reconfiguration can be comined to one procedure to avoid additional reconfiguration on ss entity - In cads_NR_TDD_UL_DL_ConfigCommon_REQ there are IEs that are not initialised and will result in runtime error |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed TTCN code related to v_RACH_ConfigDedicated and cads_NR_CellConfigPhysicalLayerUplink and added code to update f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); - Moved the v_NR_UplinkBWP to be done as part of v_TDD_UL_DL_ConfigCommon - Updated the unitialised IEs to omit |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.1a.NR5GC, 7.1.1.1.1a.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required to test case 7.1.1.1.1a	3

1.1	f_TC_7_1_1_1_1a_NR5GC	3

1.2	cads_NR_TDD_UL_DL_ConfigCommon_REQ	8

1.3	cads_NR_Common_UplinkBWP_ConfigCommon_REQ	9

# Corrections required to test case 7.1.1.1.1a

## f_TC_7_1_1_1_1a_NR5GC

| Function name | f_TC_7_1_1_1_1a_NR5GC |
| --- | --- |
| Reason for change | - In this test case at Step0A SS transmits RRCReconfiguration message and updates BWP-UplinkCommon - > RACH-ConfigCommon When configuring the SS entity TTCN is providing the RACHConfigDedicated parameter update - The reconfiguration parameters on the SS entity is resulting in NR_UplinkBWP_ConfigCommon update done separately for v_NR_UplinkBWP and v_TDD_UL_DL_ConfigCommon due to pattern change. This reconfiguration can be comined to one procedure to avoid additional reconfiguration on ss entity |
| Summary of change | - Removed TTCN code related to v_RACH_ConfigDedicated and cads_NR_CellConfigPhysicalLayerUplink and added code to update f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); - Moved the v_NR_UplinkBWP to be done as part of v_TDD_UL_DL_ConfigCommon update using the new template cads_NR_Common_UplinkBWP_ConfigCommon_REQ |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_1_1a_NR5GC() runs on NR5GC_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; //@sic R5-198888 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing; //@sic R5s200526 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP; var template (value) ReconfigurationWithSync.rach_ConfigDedicated v_RACH_ConfigDedicated; … … … f_NR_CellInfo_SetTDD_UL_DL_ConfigCommon(nr_Cell1,v_TDD_UL_DL_ConfigCommon); }; //@sic R5s210088 sic@ v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_NR_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_Mac_71111a(v_SubcarrierSpacing)); f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id); //@sic r5s201469 sic@ f_NR_SS_ConfigureSysinfo(nr_Cell1, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s190511 sic@ v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n4; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //Configure SS CFRA @sic R5s200526 sic@ //v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1,tsc_NR_BWP_Id), tsc_PreambleIndex,f_NR_CellInfo_GetSSB_Index(nr_Cell1))); v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(f_NR_GetRACHConfigDedicated_Def( nr_Cell1, tsc_PreambleIndex)); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, cs_TimingInfo_Now, cs_NR_CellConfigPhysicalLayerUplink_Def(f_ConvertRACH_ConfigDedicated(v_RACH_ConfigDedicated.uplink)))); // Send message to UE f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); //@sic R5s200891 sic@ if (ischosen(v_NR_DuplexMode.TDD)) { f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_TDD_UL_DL_ConfigCommon_REQ(nr_Cell1, -, v_TDD_UL_DL_ConfigCommon)); }; SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); f_TC_7_1_1_1_1a_NR_TestBody(); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_1_1a_NR5GC() runs on NR5GC_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; //@sic R5-198888 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing; //@sic R5s200526 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP; var NR_PhysicalParameters_Type v_NR_PhysicalParameters; //RACH procedure var template (value) ReconfigurationWithSync.rach_ConfigDedicated v_RACH_ConfigDedicated; var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //RACH procedure … … f_NR_CellInfo_SetTDD_UL_DL_ConfigCommon(nr_Cell1,v_TDD_UL_DL_ConfigCommon); }; //@sic R5s210088 sic@ v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_NR_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_Mac_71111a(v_SubcarrierSpacing)); f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id); //@sic r5s201469 sic@ f_NR_SS_ConfigureSysinfo(nr_Cell1, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); // RACH procedure called later //Removed f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s190511 sic@ v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n4; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //Configure SS CFRA @sic R5s200526 sic@ //v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1,tsc_NR_BWP_Id), tsc_PreambleIndex,f_NR_CellInfo_GetSSB_Index(nr_Cell1))); /*v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(f_NR_GetRACHConfigDedicated_Def( nr_Cell1, tsc_PreambleIndex)); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, cs_TimingInfo_Now, cs_NR_CellConfigPhysicalLayerUplink_Def(f_ConvertRACH_ConfigDedicated(v_RACH_ConfigDedicated.uplink)))); */ // REMOVED out as RACH config dedicated is not updated in this procedure. //update RACH procedure configuration to perform CBRA v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // Send message to UE f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); //@sic R5s200891 sic@ //moved v_NR_UplinkBWP configuration here. if (ischosen(v_NR_DuplexMode.TDD)) { //moved uplink configuration here. f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_Common_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP},v_TDD_UL_DL_ConfigCommon)); } else { f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); }; SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); f_TC_7_1_1_1_1a_NR_TestBody(); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## cads_NR_TDD_UL_DL_ConfigCommon_REQ

| Function name | cads_NR_TDD_UL_DL_ConfigCommon_REQ |
| --- | --- |
| Reason for change | In this template there are IEs that are not initialised and will result in runtime error |
| Summary of change | Updated the unitialised IEs to omit |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

{ /* @status APPROVED (ENDC, NR5GC) */ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := { DuplexMode := { TDD:= { Config := { Common :={ R15 := p_TDD_UL_DL_ConfigCommon } } } } } } } } } };

After Change:

{ /* @status APPROVED (ENDC, NR5GC) */ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := { PhysicalCellId := omit, // uninitalised value DuplexMode := { TDD:= { Config := { Common :={ R15 := p_TDD_UL_DL_ConfigCommon }, Dedicated := omit //WA#WI=892505 uninitalised value } } } }, Downlink := omit, // uninitalised value Uplink := omit // uninitalised value } } } } };

## cads_NR_Common_UplinkBWP_ConfigCommon_REQ

| Function name | cads_NR_Common_UplinkBWP_ConfigCommon_REQ |
| --- | --- |
| Reason for change | See Change 1.1 |
| Summary of change | New template to configure NR_UplinkBWP_List_Type and TDD_UL_DL_ConfigCommon |
| TTCN module | NR_CellCfg_Templates |
| MCC160 Comment |  |

New Change:

template (value) NR_SYSTEM_CTRL_REQ cads_NR_Common_UplinkBWP_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_UplinkBWP_List_Type p_NR_UplinkBWP_List, template (value) TDD_UL_DL_ConfigCommon p_TDD_UL_DL_ConfigCommon) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status APPROVED (ENDC, NR5GC) */ //@sic R5s190360_r1 R5s200549 R5s210396 sic@ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := { PhysicalCellId := omit, DuplexMode := { TDD:= { Config := { Common :={ R15 := p_TDD_UL_DL_ConfigCommon }, Dedicated := omit } } } }, Downlink := omit, Uplink := { Uplink := { Config:= { FrequencyInfoUL := omit, BWPs := { ActiveBWP := omit, BwpArray := p_NR_UplinkBWP_List }, RACH_ConfigDedicated := omit, SI_RequestConfig := omit } }, SupplementaryUplink := omit, TimingAdvance := omit, PUSCH_ServingCellConfig := omit, PUSCH_ServingCellConfigSUL := omit } } } } } };

## f_TC_7_1_1_1_1a_ENDC_NR

| Function name | f_TC_7_1_1_1_1a_ENDC_NR |
| --- | --- |
| Reason for change | - In this test case at Step0A SS transmits RRCReconfiguration message and updates BWP-UplinkCommon - > RACH-ConfigCommon When configuring the SS entity TTCN is providing the RACHConfigDedicated parameter update |
| Summary of change | - Removed TTCN code related to v_RACH_ConfigDedicated and cads_NR_CellConfigPhysicalLayerUplink and added code to update f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); |
| TTCN module | MAC_ENDC_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_1_1a_ENDC_NR() runs on ENDC_NR_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / //contention free random access procedure var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; //@sic R5-198888 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing;//@sic R5s201022 sic@ var template (value) ReconfigurationWithSync.rach_ConfigDedicated v_RACH_ConfigDedicated; //Init Cell parameters f_ENDC_NR_Init(); … … //@siclog "Steps 0A-0B" siclog@ v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n4; v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); //@sic R5s190541 sic@ // Now send octetstrings to EUTRA v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(f_NR_GetRACHConfigDedicated_Def( nr_Cell1, tsc_PreambleIndex)); f_NR_CellInfo_SetRACH_ConfigDedicated (nr_Cell1,v_RACH_ConfigDedicated.uplink); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, cs_TimingInfo_Now, cs_NR_CellConfigPhysicalLayerUplink_Def(f_ConvertRACH_ConfigDedicated(v_RACH_ConfigDedicated.uplink)))); f_TC_7_1_1_1_1a_NR_TestBody(); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_1_1a_ENDC_NR() runs on ENDC_NR_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / //contention free random access procedure var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; //@sic R5-198888 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing;//@sic R5s201022 sic@ var template (value) ReconfigurationWithSync.rach_ConfigDedicated v_RACH_ConfigDedicated; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //Init Cell parameters f_ENDC_NR_Init(); … … //@siclog "Steps 0A-0B" siclog@ v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n4; //added setPreambletransMax and uplinkBWP to update the rach config common rather than RACH_ConfigDedicated f_NR_CellInfo_SetPreambleTransMax(nr_Cell1, n4); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_NR_UplinkBWP_List[0].Dedicated := omit; f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); //@sic R5s190541 sic@ // Now send octetstrings to EUTRA /* //REMOVED v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(f_NR_GetRACHConfigDedicated_Def( nr_Cell1, tsc_PreambleIndex)); f_NR_CellInfo_SetRACH_ConfigDedicated (nr_Cell1,v_RACH_ConfigDedicated.uplink); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, cs_TimingInfo_Now, cs_NR_CellConfigPhysicalLayerUplink_Def(f_ConvertRACH_ConfigDedicated(v_RACH_ConfigDedicated.uplink)))); */ f_TC_7_1_1_1_1a_NR_TestBody(); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }