3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220172

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4654 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of EUTRA procedures to support MCX EUTRA test model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some LTE common functions need to be updated to allow EUTRA IPCAN implementation for MCX according to 36.579-1 clause 5.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | LTE common functions updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCX EUTRA test model cannot be implemented |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All test cases with EUTRA PTC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## Change 1 – f_EUTRA_InitialRegistration_Step14_15

f_EUTRA_InitialRegistration_Step14_15 is split into a common function f_EUTRA_InitialRegistration_Step14_15_Common, which is independent form the 36.508 specific PDN mapping and the LTE specific EPS bearer contexts, and the LTE specific f_EUTRA_InitialRegistration_Step14_15.

function f_EUTRA_InitialRegistration_Step14_15_Common(EUTRA_CellId_Type p_CellId, EUTRA_ATTACH_Type p_ForcedAttach, template (omit) AdditionalUpdateType p_AdditionalUpdateType, ProcedureTransactionIdentifier p_EPS_TI, boolean p_IPv4AllocationViaNasFlag, template (omit) ProtocolConfigOptions p_PcoToUE, AccessPointName p_APN, PDN_Index_Type p_PdnIndex, EPS_BearerIdentity p_EpsDefaultBearerId, BEARER_CONTEXT_TYPE p_BearerContextNumber, template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact, template (omit) GPRS_Timer p_T3402 := omit, template (omit) GPRS_Timer p_T3423 := omit, template (omit) PLMN_List p_EquivalentPlmnList := omit, integer p_NoOfEmergencyNumbers := 0, // @sic R5-115673 sic@ template (omit) EMM_Cause p_EMM_Cause := omit, template (omit) DCN_ID p_DCN_Id := omit, template (omit) EUTRA_NR_PduSessionInfo_Type p_PduSessionInfo := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPco := omit) runs on EUTRA_PTC { var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed(); /* @sic R5s170307 sic@ */ var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(p_ForcedAttach); // @sic R5-103673, R5s100795 sic@ var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity); var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(p_CellId); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, p_ForcedAttach); // @sic R5s110176 sic@ var TrackingAreaCode v_TAC := f_EUTRA_CellInfo_GetTAC(p_CellId); var template (omit) EmergNumList v_EmergNumList := omit; var template (omit) MobileIdentity v_MSId := f_GetMSId(p_ForcedAttach); // @sic R5-103673 sic@ var template (omit) ESM_Cause v_ESM_Cause := omit; // @sic R5s120136 sic@ var template (value) PDN_Address v_PDN_Address; var EmergencyNumList v_EmergencyNumList; if (pc_IPv6 and pc_IPv4) { /* @sic R5-120234, R5-120541, R5-120542 sic@ */ select (v_PDN_TypeToBeUsed) { case (forceIPv4only) { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); } // PDN type IPv4 only allowed case (forceIPv6only) { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); } // PDN type IPv6 only allowed case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; } } } if (p_NoOfEmergencyNumbers > 0) { v_EmergencyNumList := f_Get_EmergencyNumList (p_NoOfEmergencyNumbers, f_EUTRA_MobileInfo_GetLocalEmergencyNumberList()); /* @sic R5s141315 sic@ */ f_EUTRA_MobileInfo_SetLocalEmergencyNumberList (v_EmergencyNumList); // save the list of locally usable emergency numbers v_EmergNumList := f_Build_EmergNumList(p_NoOfEmergencyNumbers, v_EmergencyNumList); } v_PDN_Address := f_EUTRA_GetPDNAddress(p_IPv4AllocationViaNasFlag, p_PdnIndex); f_EUTRA_MobileInfo_SetEPS_BearerContext(p_EpsDefaultBearerId, p_PduSessionInfo); // @sic R5-185166 sic@ f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId, p_EpsDefaultBearerId, // @sic R5-144797 sic@ tsc_RRC_TI_Def, p_EPS_TI, v_AttachType, v_PDN_Address, v_Guti, cs_TAIListNonConsecutive_lv(v_PLMN, {bit2oct(v_TAC)}), v_LAI, v_MSId, v_ESM_Cause, // @sic R5-104703 sic@ p_APN, p_PcoToUE, f_GetAdditionalUpdateResult(p_AdditionalUpdateType, p_ForcedAttach), // @sic R5s110176 sic@ p_T3412, p_T3402, p_T3423, p_EquivalentPlmnList, v_EmergNumList, // @sic R5s130681 sic@ -, -, p_BearerContextNumber, // @sic R5-144797 sic@ p_EMM_Cause, // @sic R5-145971 sic@ omit, p_DCN_Id, // @sic R5-177082 sic@ p_ExtdPco); // @sic R5-200649 sic@ }

function f_EUTRA_InitialRegistration_Step14_15(EUTRA_CellId_Type p_CellId, EUTRA_ATTACH_Type p_ForcedAttach, template (omit) AdditionalUpdateType p_AdditionalUpdateType, ProcedureTransactionIdentifier p_EPS_TI, boolean p_IPv4AllocationViaNasFlag, template (omit) ProtocolConfigOptions p_PcoToUE, AccessPointName p_APN, template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact, template (omit) GPRS_Timer p_T3402 := omit, template (omit) GPRS_Timer p_T3423 := omit, template (omit) PLMN_List p_EquivalentPlmnList := omit, integer p_NoOfEmergencyNumbers := 0, // @sic R5-115673 sic@ template (omit) EMM_Cause p_EMM_Cause := omit, EUTRA_InitialRegistrationPDN_Type p_InitialRegistrationPDN := firstPDN, template (omit) DCN_ID p_DCN_Id := omit, template (omit) EUTRA_NR_PduSessionInfo_Type p_PduSessionInfo := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPco := omit) runs on EUTRA_PTC { /* @sic R5-112600: IP address allocation sic@ */ /* @sic R5-110796: split into f_EUTRA_InitialRegistration_Step5_13 and f_EUTRA_InitialRegistration_Step14_15 sic@ */ /* @sic R5-120234, R5-120541, R5-120542 sic@ */ /* @sic R5-144370, R5-144797: p_APN shall not be omit sic@ */ /* @sic R5-145971: new parameter p_EMM_Cause sic@ */ /* @sic R5s150510: new parameter p_InitialRegistrationPDN sic@ */ /* @sic R5s170307: p_PDN_TypeToBeUsed removed sic@ */ /* @sic R5s180648: got NetworkFeatureSupport from global variable sic@*/ /* @sic R5-199103: p_PduSessionInfo sic@ */ var PDN_Index_Type v_PdnIndex := f_EUTRA_InitialRegistrationPdnIndex(p_InitialRegistrationPDN); // @sic R5-144797, R5s150510, R5s180526 sic@ var EPS_BearerIdentity v_EpsDefaultBearerId := f_EUTRA_InitialRegistrationEpsBearerId(p_InitialRegistrationPDN); // @sic R5-144797, R5s150510, R5-183247 sic@ var BEARER_CONTEXT_TYPE v_BearerContextNumber := DEF_1; if (tsc_EUTRA_Registration_PDN1isIMS) { // @sic R5-144797, R5s141367 sic@ v_BearerContextNumber := DEF_2; } f_EUTRA_InitialRegistration_Step14_15_Common(p_CellId, p_ForcedAttach, p_AdditionalUpdateType, p_EPS_TI, p_IPv4AllocationViaNasFlag, p_PcoToUE, p_APN, v_PdnIndex, v_EpsDefaultBearerId, v_BearerContextNumber, p_T3412, p_T3402, p_T3423, p_EquivalentPlmnList, p_NoOfEmergencyNumbers, p_EMM_Cause, p_DCN_Id, p_PduSessionInfo, p_ExtdPco); }

## Change 2 – f_EUTRA_RRCConnectionReconfiguration_Common

f_EUTRA_RRCConnectionReconfiguration_Common split into f_EUTRA_RRCConnectionReconfiguration_Common and f_EUTRA_RRCConnectionReconfiguration_Send.

function f_EUTRA_RRCConnectionReconfiguration_Send(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, UInt_Type p_NoOfAddDRBs_UM := 0, template (omit) SRB_ToAddModList p_SRB_ToAddModList := omit, template (value) DRB_ToAddModList p_DRB_ToAddModList, template (omit) MAC_MainConfig_Type p_MAC_MainConfig := omit, template (omit) PhysicalConfigDedicated p_PhysicalConfigDedicated := omit, template (omit) NAS_MSG_RequestList_Type p_NAS_MsgList := omit, template (omit) RLF_TimersAndConstants_r9 p_RLF_TimersAndConstants_r9 := omit, template (omit) DownlinkAntennaGroupConfig_Type p_DownlinkAntennaGroupConfig := omit, template (omit) RRCConnectionReconfiguration_v890_IEs p_RRCConnectionReconfiguration_v890_IEs := omit) runs on EUTRA_PTC { var template (value) EUTRA_FDD_TDD_CellInfo_Type v_EUTRA_FDD_TDD_Info := f_EUTRA_CellInfo_GetFDD_TDD_Info (p_CellId); //Get FDD or TDD mode from Cell-configuration var template (value) MAC_MainConfig_Type v_MAC_MainConfig; var template (omit) PhysicalConfigDedicated v_PhysicalConfigDedicated; var template (value) RadioResourceConfigDedicated v_RadioResourceConfigDedicated; var template (value) DRX_Config v_DRX_Config; var template (value) PhysicalConfigDedicated_AntennaInfo_Type v_AntennaInfo := f_EUTRA_CellInfo_GetAntennaInfoDedicated (p_CellId); //@sic R5-100786 sic@ var template (value) TimingInfo_Type v_TimingInfo := cs_TimingInfo_Now; /* @sic R5-104796 DRX support sic@ */ var integer v_NoOfNASPDUs; var SubFrameTiming_Type v_SubFrameTiming; if (isvalue(p_MAC_MainConfig)) { v_MAC_MainConfig := valueof(p_MAC_MainConfig); } else { if (p_NoOfAddDRBs_UM > 0) { /* @sic R5-104796 DRX support sic@ */ v_MAC_MainConfig := cs_508_MAC_MainConfig_Explicit_RBC_DrxS; // short DRX cycle acc. to note in 36.508 table 4.6.3-16 } else { v_MAC_MainConfig := cs_508_MAC_MainConfig_Explicit_RBC_DrxL; // long DRX cycle acc. to note in 36.508 table 4.6.3-16 } } if (isvalue(p_PhysicalConfigDedicated)) { v_PhysicalConfigDedicated := valueof(p_PhysicalConfigDedicated); } else { if (isvalue(p_SRB_ToAddModList)) { // @sic R5-130491, R5-121736, R5-121737 - support of MO, MT video call: per default PhysicalConfigDedicated are needed only if SRB2 is configured too sic@ v_PhysicalConfigDedicated := cs_508_PhysicalConfigDedicated_Default_RBC(v_EUTRA_FDD_TDD_Info.cqi_ReportConfig, v_EUTRA_FDD_TDD_Info.soundingRS_UL_Config, v_AntennaInfo); } else { v_PhysicalConfigDedicated := omit; } } v_RadioResourceConfigDedicated := cds_RadioResourceConfigDedicated_SrbDrbConfig(p_SRB_ToAddModList, // @sic R5-130491, R5-121736, R5-121737 - support of MO, MT video call sic@ p_DRB_ToAddModList, v_MAC_MainConfig, v_PhysicalConfigDedicated, p_RLF_TimersAndConstants_r9); // @sic R5s120808 sic@ if (p_NoOfAddDRBs_UM > 0) { /* @sic R5-104796 DRX support sic@ */ if (ispresent(v_MAC_MainConfig.explicitValue.drx_Config)) { /* @sic R5120095 sic@ */ v_DRX_Config := valueof(v_MAC_MainConfig.explicitValue.drx_Config); /* valueof is ok as v_MAC_MainConfig.explicitValue.drx_Config is checked with 'ispresent' beforehand */ f_EUTRA_SS_DrxCtrlConfig(p_CellId, cs_DrxCtrl_Config(v_DRX_Config)); /* configure DRX at the SS; @sic R5s100630 R5s120054 sic@ */ if (isvalue(p_NAS_MsgList)) { v_NoOfNASPDUs := lengthof(valueof(p_NAS_MsgList)); v_SubFrameTiming := f_EUTRA_GetNextSendOccasion(p_CellId, 100 + ((v_NoOfNASPDUs - 1) * 20)); /* @sic R5-115770 change 5 sic@ */ } else { /* @sic R5s211690: when there is no NAS signalling, no additional delay is needed sic@ */ v_SubFrameTiming := f_EUTRA_GetNextSendOccasion(p_CellId); } v_TimingInfo := cs_TimingInfo_EUTRA_NB(v_SubFrameTiming); } } else { /* do nothing; for the time being we don't need any DRX configuration at the SS for DRX_L */ } /* Step 8 */ SRB.send(cas_SRB1_RrcNasPduList_REQ(p_CellId, v_TimingInfo, cs_RRCConnectionReconfiguration_Common(p_RRC_TI, omit, omit, v_RadioResourceConfigDedicated, omit, p_RRCConnectionReconfiguration_v890_IEs), // @sic R5-175050 sic@ p_NAS_MsgList)); if (ischosen(v_AntennaInfo.explicitValue)) { f_EUTRA_SS_CommonCellConfig(p_CellId, cas_PhysicalLayerConfigDL_CcchDcchDtchDL_Config_REQ(p_CellId, cs_TimingInfo_Now, cs_PhysicalLayerConfigDL_AntennaInfo(p_DownlinkAntennaGroupConfig), cs_DciDLInfo_Auto(cs_DciInfo_CcchDcchDtchDL_Dci1A(f_EUTRA_CellInfo_GetDL_ChBandwidth(p_CellId))), cs_AntennaInfoDedicated_r8andLater(v_AntennaInfo.explicitValue))) } }

function f_EUTRA_RRCConnectionReconfiguration_Common(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, UInt_Type p_NoOfAddDRBs_AM := 0, UInt_Type p_NoOfAddDRBs_UM := 0, template (omit) SRB_ToAddModList p_SRB_ToAddModList := omit, template (omit) DRB_ToAddModList p_DRB_ToAddModList := omit, template (omit) MAC_MainConfig_Type p_MAC_MainConfig := omit, template (omit) PhysicalConfigDedicated p_PhysicalConfigDedicated := omit, template (omit) EPS_QoSList_Type p_QoS := omit, template (omit) Tft_List_Type p_Tft := omit, boolean p_PDCP_StatusReportRequired := true, template (omit) RLF_TimersAndConstants_r9 p_RLF_TimersAndConstants_r9 := omit, template (omit) DownlinkAntennaGroupConfig_Type p_DownlinkAntennaGroupConfig := omit, template (omit) DRB_Identity p_DrbFor2ndPDN := omit, template (omit) RRCConnectionReconfiguration_v890_IEs p_RRCConnectionReconfiguration_v890_IEs := omit, template (omit) UplinkDataCompression_r15_Type p_UDC_r15 := omit) runs on EUTRA_PTC { /* @sic R5-130491, R5-121736, R5-121737 - support of MO, MT video call: new parameter p_SRB_ToAddModList; function renamed to f_EUTRA_RRCConnectionReconfiguration_Common sic@ */ /* @sic R5-144797: new parameter p_DrbFor2ndPDN sic@ */ var integer v_DRB_Id; var integer i; var integer m; var integer n; var DRB_Identity v_DrbFor2ndPDN; var EPS_BearerIdentity v_EpsBearerId; var integer v_NoOfDefaultBearers := 1; /* @sic R5-144797 sic@ */ var integer v_NoOfQos := 0; var integer v_NoOfTft := 0; var integer v_NoOfAddDRBs := p_NoOfAddDRBs_AM + p_NoOfAddDRBs_UM; /* number of bearers to be configured additionally (i.e. apart from the default bearer) if v_NoOfAddDRBs=0 => only the default DRB is established */ var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (omit) NAS_MSG_RequestList_Type v_NAS_MsgList; if (isvalue(p_DrbFor2ndPDN)) { v_NoOfDefaultBearers := v_NoOfDefaultBearers + 1; } /* @sic R5-144797 sic@ */ // ----------------------------------------------------- // Check parameters are valid if (isvalue(p_QoS)) { v_NoOfQos := lengthof(p_QoS); } if (isvalue(p_Tft)) { v_NoOfTft := lengthof(p_Tft); } if (v_NoOfQos != v_NoOfAddDRBs) { FatalError (__FILE__, __LINE__, "invalid no. of QoS"); } if (v_NoOfTft != v_NoOfAddDRBs) { FatalError (__FILE__, __LINE__, "invalid no. of TFT"); } // ----------------------------------------------------- // initialise v_DRB_ToAddModList if (isvalue(p_DRB_ToAddModList)) { // explicit setting for DRB_ToAddMod if (lengthof(p_DRB_ToAddModList) != v_NoOfAddDRBs + v_NoOfDefaultBearers) { FatalError (__FILE__, __LINE__, "invalid no. of DRB_ToAddMod"); } v_DRB_ToAddModList := valueof(p_DRB_ToAddModList); } else { // use default configuration v_DRB_Id := tsc_DRB1; // default bearer for (n := 0; n < (p_NoOfAddDRBs_AM + 1); n := n + 1) { // default bearer included in the list (=> p_NoOfAddDRBs_AM + 1) if (p_PDCP_StatusReportRequired == true) { if (isvalue(p_UDC_r15)) { v_DRB_ToAddModList[n] := cs_DRB_ToAddMod_UDC (v_DRB_Id, p_UDC_r15); } else { v_DRB_ToAddModList[n] := cs_508_DRB_ToAddMod_DEFAULT_AM(v_DRB_Id); } } else { if (isvalue(p_UDC_r15)) {FatalError (__FILE__, __LINE__, "UDC not foreseen with No Status Report");} v_DRB_ToAddModList[n] := cs_DRB_ToAddMod_NoPDCP_StatusReport(v_DRB_Id); } v_DRB_Id := v_DRB_Id + 1; } for (m := 0; m < p_NoOfAddDRBs_UM; m := m + 1) { v_DRB_ToAddModList[n+m] := cs_508_DRB_ToAddMod_DEFAULT_UM (v_DRB_Id); v_DRB_Id := v_DRB_Id + 1; } } if (isvalue(p_DrbFor2ndPDN)) { /* @sic R5-144797 sic@ */ v_DrbFor2ndPDN := valueof(p_DrbFor2ndPDN); if (v_DrbFor2ndPDN < v_DRB_Id) { FatalError (__FILE__, __LINE__, "2nd default bearer is within the range of dedicated bearers"); } v_DRB_ToAddModList[v_NoOfAddDRBs+1] := cs_508_DRB_ToAddMod_DEFAULT_AM(v_DrbFor2ndPDN); } // ----------------------------------------------------- // initialise NAS messages if (v_NoOfAddDRBs == 0) { v_NAS_MsgList := omit; // no NAS messages to be sent } else { for (i := 0; i < v_NoOfAddDRBs; i := i+1) { // only for the additional DRBs (there is no ActDedEPSBearerCxtReq for the already existing default bearer) // get EpsBearerId from the corresponding DRB_ToAddModList; DRB_ToAddModList[0] is the default bearer which needs to be skipped v_EpsBearerId := int2hex(valueof(v_DRB_ToAddModList[i+1].eps_BearerIdentity), 1); // i+1 to skip default DRB; valueof cannot be avoided here v_NAS_MsgList[i] := cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_ActDedEPSBearerCxtReq_QoS(v_EpsBearerId, valueof(p_QoS[i]), valueof(p_Tft[i]))); // valueof to avoid compiler warnings: neither p_QoS nor p_Tft can be omit as v_NoOfAddDRBs > 0 } } f_EUTRA_RRCConnectionReconfiguration_Send(p_CellId, p_RRC_TI, p_NoOfAddDRBs_UM, p_SRB_ToAddModList, v_DRB_ToAddModList, p_MAC_MainConfig, p_PhysicalConfigDedicated, v_NAS_MsgList, p_RLF_TimersAndConstants_r9, p_DownlinkAntennaGroupConfig, p_RRCConnectionReconfiguration_v890_IEs); }

As consequence of the above changes there are further changes to hand over a DRB list instead of a single DRB in

- f_EUTRA_InitialRegistration_ReleaseInternetPDN

- f_EUTRA38_ENDC_UE_DetachMO

- f_EUTRA_IPCAN_XCAP_Signalling

- fl_EUTRA_RbEst_With2ndDefPDN

- f_EUTRA_OptionalMultiPDN_IMSRegistration

- f_EUTRA_508_ProseCommunicationProcedure

## Change 3 – f_Get_508_EPSQoS

Configuration DEF_3 has not been implemented yet

function f_Get_508_EPSQoS(template (omit) BEARER_CONTEXT_TYPE p_BearerContextNumber) return template (value) EPS_QualityOfService { /* @sic R5-144797: parameter p_BearerContextNumber sic@ */ var template (value) EPS_QualityOfService v_QoS; var BEARER_CONTEXT_TYPE v_BearerContextNumber := fl_BearerContextNumber_Def(p_BearerContextNumber); select (v_BearerContextNumber){ /* @sic R5-144797: v_QoS does not depend on pc_IMS anymore sic@ */ case (DEF_1) { v_QoS := cs_508_EPS_QoS_Default_1; /* Qos acc. to 36.508 Table 6.6.1-1 column 1 */ } case (DEF_3) { v_QoS := cds_508_EPS_QoS_Default_3; /* Qos acc. to 36.508 Table 6.6.1-1 column 3 */ } case else { v_QoS := cds_EPS_QoS_Def_nonGBR_QCI; /* Qos acc. to 36.508 Table 6.6.1-1 column 2 */ } } return v_QoS; }

template (value) EPS_QualityOfService cds_508_EPS_QoS_Default_3 modifies cs_508_EPS_QoS_Default_1 := { qci := '01000101'B // 69 for MCPTT };

## Change 4 – f_EUTRA_ActivateDedicatedEpsBearer_Speech, f_EUTRA_ActivateDedicatedEpsBearer_SpeechVideo, f_EUTRA_508_ActDedicatedEPSBearerCxtReq_Speech

f_EUTRA_508_ActDedicatedEPSBearerCxtReq_Speech needs to distinguish Reference dedicated EPS bearer context #4 and #5 of 36.508 Table 6.6.2-1

type enumerated BEARER_CONTEXT_TYPE { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NBIOT, NR5GC, NR5GC_IRAT, POS) */ DEF_1, // EPS Default Bearer#1 defined in the first column of table 6.6.1 DEF_2, // EPS Default Bearer#2 defined in the second column of table 6.6.1 DEF_3, // EPS Default Bearer#3 defined in the third column of table 6.6.1 DED_1, // EPS Dedicated Bearer#1 defined in the first column of table 6.6.2 DED_2, // EPS Dedicated Bearer#2 defined in the second column of table 6.6.2 DED_3, // EPS Dedicated Bearer#3 defined in the third column of table 6.6.2 DED_4, // EPS Dedicated Bearer#4 defined in the fourth column of table 6.6.2 @sic R5-123632 sic@ DED_5 // EPS Dedicated Bearer#5 defined in the fifth column of table 6.6.2 };

function f_EUTRA_ActivateDedicatedEpsBearer_Speech(EUTRA_CellId_Type p_CellId, EPS_BearerIdentity p_EpsDefaultBearerId := tsc_EpsDefaultBearerId, EPS_BearerIdentity p_EpsDedicatedBearerId := tsc_EpsDedicatedBearerId, PDN_Index_Type p_PdnIndex := PDN_1, boolean p_IsSRVCC := false, BEARER_CONTEXT_TYPE p_BearerContextNumber := DED_4) runs on EUTRA_PTC { var DRB_Identity v_DrbId := f_EUTRA_NB_EpsBearerAssociatedDRB(p_EpsDedicatedBearerId); var template (value) IP_DrbInfo_Type v_DrbInfo := cs_DrbInfo_EUTRA(p_CellId, v_DrbId); /* @sic R5s160131 sic@ */ var template (value) MAC_MainConfig_Type v_MAC_MainConfig := cs_508_MAC_MainConfig_Explicit_RBC_DrxS; var template (value) DRB_ToAddModList v_DRB_ToAddModList := {cs_508_DRB_ToAddMod_DEFAULT_UM(v_DrbId)}; var template (value) NAS_DL_Message_Type v_ActDedicatedEPSBearerCxtReq := f_EUTRA_508_ActDedicatedEPSBearerCxtReq_Speech(p_EpsDefaultBearerId, p_EpsDedicatedBearerId, p_PdnIndex, p_IsSRVCC, p_BearerContextNumber); var template (value) NAS_MSG_RequestList_Type v_NAS_MsgList := {cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_ActDedicatedEPSBearerCxtReq)}; f_EUTRA_SS_RtpRtcpLoopback(p_PdnIndex, tsc_IMS_MediaPort_M1, v_DrbInfo); /* @sic R5-153746: RTP/RTCP loopback; R5s160131: v_DrbInfo sic@ */ fl_EUTRA_ActivateDedicatedEpsBearer_UM(p_CellId, v_DRB_ToAddModList, v_MAC_MainConfig, v_NAS_MsgList); }

function f_EUTRA_ActivateDedicatedEpsBearer_SpeechVideo(EUTRA_CellId_Type p_CellId, EPS_BearerIdentity p_EpsDefaultBearerId := tsc_EpsDefaultBearerId, EPS_BearerIdentity p_EpsDedicatedBearerIdSpeech := tsc_EpsDedicatedBearerId, EPS_BearerIdentity p_EpsDedicatedBearerIdVideo := tsc_EpsDedicatedBearerId2, PDN_Index_Type p_PdnIndex := PDN_1, BEARER_CONTEXT_TYPE p_BearerContextNumber := DED_4) runs on EUTRA_PTC { var DRB_Identity v_DrbIdSpeech := f_EUTRA_NB_EpsBearerAssociatedDRB(p_EpsDedicatedBearerIdSpeech); /* @sic R5s160131 sic@ */ var DRB_Identity v_DrbIdVideo := f_EUTRA_NB_EpsBearerAssociatedDRB(p_EpsDedicatedBearerIdVideo); /* @sic R5s160131 sic@ */ var template (value) IP_DrbInfo_Type v_DrbInfoSpeech := cs_DrbInfo_EUTRA(p_CellId, v_DrbIdSpeech); var template (value) IP_DrbInfo_Type v_DrbInfoVideo := cs_DrbInfo_EUTRA(p_CellId, v_DrbIdVideo); var template (value) MAC_MainConfig_Type v_MAC_MainConfig := cs_508_MAC_MainConfig_Explicit_RBC_DrxS; var template (value) DRB_ToAddModList v_DRB_ToAddModList := { cs_508_DRB_ToAddMod_DEFAULT_UM(v_DrbIdSpeech), cs_508_DRB_ToAddMod_DEFAULT_UM(v_DrbIdVideo) }; var template (value) NAS_DL_Message_Type v_ActDedicatedEPSBearerCxtReqSpeech := f_EUTRA_508_ActDedicatedEPSBearerCxtReq_Speech(p_EpsDefaultBearerId, p_EpsDedicatedBearerIdSpeech, p_PdnIndex, -, p_BearerContextNumber); var template (value) NAS_DL_Message_Type v_ActDedicatedEPSBearerCxtReqVideo := f_EUTRA_508_ActDedicatedEPSBearerCxtReq_Video(p_EpsDefaultBearerId, p_EpsDedicatedBearerIdVideo, p_PdnIndex); var template (value) NAS_MSG_RequestList_Type v_NAS_MsgList := { cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_ActDedicatedEPSBearerCxtReqSpeech), cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_ActDedicatedEPSBearerCxtReqVideo) }; f_EUTRA_SS_RtpRtcpLoopback(p_PdnIndex, tsc_IMS_MediaPort_M1, v_DrbInfoSpeech); /* @sic R5-153746: RTP/RTCP loopback; R5s160131: v_DrbInfo sic@ */ f_EUTRA_SS_RtpRtcpLoopback(p_PdnIndex, tsc_IMS_MediaPort_M2, v_DrbInfoVideo); /* @sic R5-153746: RTP/RTCP loopback; R5s160131: v_DrbInfo sic@ */ f_EUTRA_ActivateDedicatedEpsBearer_UM(p_CellId, v_DRB_ToAddModList, v_MAC_MainConfig, v_NAS_MsgList); }

function f_EUTRA_508_ActDedicatedEPSBearerCxtReq_Speech(EPS_BearerIdentity p_EpsDefaultBearerId := tsc_EpsDefaultBearerId, EPS_BearerIdentity p_EpsDedicatedBearerId := tsc_EpsDedicatedBearerId, PDN_Index_Type p_PdnIndex := PDN_1, boolean p_IsSRVCC := false, BEARER_CONTEXT_TYPE p_BearerContextNumber := DED_4) runs on EUTRA_PTC return template (value) NAS_DL_Message_Type { var template (value) LinkedEpsBearerIdentity v_LinkedId := cs_LinkedId(p_EpsDefaultBearerId); var template (value) EPS_QualityOfService v_QoS; var template (value) TrafficFlowTemplate v_Tft; var template (value) NAS_DL_Message_Type v_ActDedicatedEPSBearerCxtReq; select (p_BearerContextNumber) { case (DED_5) { v_QoS := cds_EPS_QoS_Dedicated_5; /* EPS bearer context #5 (QCI 65) for MCPTT according to 36.508 table 6.6.2-1*/ v_Tft := f_EUTRA_Get_508_TrafficFlowTemplate(5, p_EpsDedicatedBearerId, -, p_PdnIndex); /* EPS bearer context #5 (QCI 65) for MCPTT according to 36.508 table 6.6.2-1 */ } case else { v_QoS := cds_EPS_QoS_Dedicated_4; /* EPS bearer context #4 (QCI 1) according to 36.508 table 6.6.2-1; @sic R5s130897 change 5 sic@ */ v_Tft := f_EUTRA_Get_508_TrafficFlowTemplate(4, p_EpsDedicatedBearerId, -, p_PdnIndex); /* EPS bearer context #4 (QCI 1) according to 36.508 table 6.6.2-1; @sic R5s130897 change 5 sic@ */ } } if (p_IsSRVCC) { v_ActDedicatedEPSBearerCxtReq := cs_508_ActDedicatedEPSBearerCxtReq_SRVCC(p_EpsDedicatedBearerId, tsc_PTI_Unassigned, v_LinkedId, v_QoS, v_Tft); } else { v_ActDedicatedEPSBearerCxtReq := cs_508_ActDedicatedEPSBearerCxtReq(p_EpsDedicatedBearerId, tsc_PTI_Unassigned, v_LinkedId, v_QoS, v_Tft); } return v_ActDedicatedEPSBearerCxtReq; }

function f_EUTRA_NB_Get_508_TrafficFlowTemplate(integer p_BearerContext, EPS_BearerIdentity p_EPS_BearerId, B3_Type p_PdnTypeValue, PDN_Index_Type p_PdnIndex := PDN_1) return template (value) TrafficFlowTemplate { /* @sic R5-112600: to replace cs_508_Tft_Dedicated_1 and cs_508_Tft_Dedicated_2 sic@ */ var integer v_PacketFilterLen1ByRef; var integer v_PacketFilterLen2ByRef; var template (value) PacketFilter v_PacketFilter1ByRef; var template (value) PacketFilter v_PacketFilter2ByRef; var template (value) TrafficFlowTemplate.packetFilterList v_PacketFilterList; var integer v_IeLength; select (p_BearerContext) { case (1) { // @sic R5-123632 sic@ fl_Get_508_PacketFilter(v_PacketFilter1ByRef, v_PacketFilterLen1ByRef, 1, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #1 acc. to 36.508 Table 6.6.2-2 @sic R5s130495 sic@ fl_Get_508_PacketFilter(v_PacketFilter2ByRef, v_PacketFilterLen2ByRef, 2, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #2 acc. to 36.508 Table 6.6.2-3 @sic R5s130495 sic@ v_IeLength := v_PacketFilterLen1ByRef + v_PacketFilterLen2ByRef + 1; // packet filter 1 + packet filter 2 + one octet for tftOperationCode, eBit and noOfPktFilter v_PacketFilterList := {v_PacketFilter1ByRef, v_PacketFilter2ByRef}; } case (2) { // @sic R5-123632, R5-132085 sic@ fl_Get_508_PacketFilter(v_PacketFilter1ByRef, v_PacketFilterLen1ByRef, 3, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #3 acc. to 36.508 Table 6.6.2-4 @sic R5s130495 sic@ v_IeLength := v_PacketFilterLen1ByRef + 1; // packet filter 1 + one octet for tftOperationCode, eBit and noOfPktFilter v_PacketFilterList := {v_PacketFilter1ByRef}; } case (3) { // @sic R5-132085 sic@ fl_Get_508_PacketFilter(v_PacketFilter1ByRef, v_PacketFilterLen1ByRef, 5, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #3 acc. to 36.508 Table 6.6.2-4 @sic R5s130495 sic@ v_IeLength := v_PacketFilterLen1ByRef + 1; // packet filter 1 + one octet for tftOperationCode, eBit and noOfPktFilter v_PacketFilterList := {v_PacketFilter1ByRef}; } case (4) { // @sic R5-132085 sic@ fl_Get_508_PacketFilter(v_PacketFilter1ByRef, v_PacketFilterLen1ByRef, 4, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #4 acc. to 36.508 Table 6.6.2-4 @sic R5s130495 sic@ v_IeLength := v_PacketFilterLen1ByRef + 1; // packet filter 1 + one octet for tftOperationCode, eBit and noOfPktFilter v_PacketFilterList := {v_PacketFilter1ByRef}; } case (5) { // @sic R5-210201 sic@ fl_Get_508_PacketFilter(v_PacketFilter1ByRef, v_PacketFilterLen1ByRef, 4, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #4 acc. to 36.508 Table 6.6.2-1 fl_Get_508_PacketFilter(v_PacketFilter2ByRef, v_PacketFilterLen2ByRef, 10, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #10 acc. to 36.508 Table 6.6.2-1 v_IeLength := v_PacketFilterLen1ByRef + v_PacketFilterLen2ByRef + 1; // packet filter 1 + packet filter 2 + one octet for tftOperationCode, eBit and noOfPktFilter @sic R5s180640 sic@ v_PacketFilterList := {v_PacketFilter1ByRef, v_PacketFilter2ByRef}; } case (6) { // @sic R5-183239 sic@ fl_Get_508_PacketFilter(v_PacketFilter1ByRef, v_PacketFilterLen1ByRef, 6, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #6 acc. to 36.508 Table 6.6.2-1A fl_Get_508_PacketFilter(v_PacketFilter2ByRef, v_PacketFilterLen2ByRef, 7, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #7 acc. to 36.508 Table 6.6.2-1A v_IeLength := v_PacketFilterLen1ByRef + v_PacketFilterLen2ByRef + 1; // packet filter 1 + packet filter 2 + one octet for tftOperationCode, eBit and noOfPktFilter @sic R5s180640 sic@ v_PacketFilterList := {v_PacketFilter1ByRef, v_PacketFilter2ByRef}; // @sic R5s180640 sic@ } case (7) { // @sic R5-187765 sic@ fl_Get_508_PacketFilter(v_PacketFilter1ByRef, v_PacketFilterLen1ByRef, 8, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #8 acc. to 36.508 Table 6.6.2-1A v_IeLength := v_PacketFilterLen1ByRef + 1; // packet filter 1 + one octet for tftOperationCode, eBit and noOfPktFilter v_PacketFilterList := {v_PacketFilter1ByRef}; } case (8) { // @sic R5-187765 sic@ fl_Get_508_PacketFilter(v_PacketFilter1ByRef, v_PacketFilterLen1ByRef, 9, p_EPS_BearerId, p_PdnTypeValue, p_PdnIndex); // get packet filter #9 acc. to 36.508 Table 6.6.2-1A v_IeLength := v_PacketFilterLen1ByRef + 1; // packet filter 1 + one octet for tftOperationCode, eBit and noOfPktFilter v_PacketFilterList := {v_PacketFilter1ByRef}; } case else { FatalError(__FILE__, __LINE__, "invalid bearer context"); } } return cs_TrafficFlowTemplate(v_IeLength, v_PacketFilterList); }

function fl_Get_508_PacketFilter(out template (value) PacketFilter p_PacketFilterByRef, out integer p_PacketFilterLengthByRef, integer p_PacketFilterNumber, HalfOctet_Type p_EPS_BearerId, B3_Type p_PdnTypeValue, PDN_Index_Type p_PdnIndex) { /* @sic R5-112600 new parameter p_PdnTypeValue to determine whether IPv4 or IPv6 address to be used sic@ */ /* @sic R5s130362 - MCC160 Implementation: tsc_PDN_AddressInfo; new parameter p_PdnIndex sic@ */ var integer v_ContentsLength := 0; var template (value) PacketFilter v_Filter; var PDN_AddressInfo_Type v_PDN_AddressInfo := f_PDN_AddressInfo_Get(p_PdnIndex); var charstring v_NetworkIpAddrv4 := v_PDN_AddressInfo.PCSCF_IPAddressIPv4; var charstring v_NetworkIpAddrv6 := v_PDN_AddressInfo.PCSCF_IPAddressIPv6; if ((p_PacketFilterNumber != 4) and (p_PacketFilterNumber != 5) and (p_PacketFilterNumber != 10)) { // @sic R5-132085 sic@ in case (4, 5, 10) no addresses used @sic R5s130495 sic@ if (p_PdnTypeValue == tsc_PdnType_IPv4) { // IPv4 only; @sic R5s110168 change 8.3; R5-112600 sic@ v_Filter.contents[0] := cs_PktFilterCompIPv4RemoteAddress(v_NetworkIpAddrv4); v_ContentsLength := v_ContentsLength + 9; } else { // IPv4v6 or IPv6 only v_Filter.contents[0] := cs_PktFilterCompIPv6RemoteAddress(v_NetworkIpAddrv6); v_ContentsLength := v_ContentsLength + 33; } } select (p_PacketFilterNumber){ case (1, 6) { // @sic R5-183239 sic@ v_Filter.spare := '00'B; // @sic R5s130195 Baseline Moving sic@ v_Filter.direction := '01'B; // @sic R5s130195 Baseline Moving sic@ v_Filter.iei := '0'H; // @sic R5s130195 Baseline Moving sic@ if (hex2int(p_EPS_BearerId) > 5) { v_Filter.precedence := int2bit((hex2int(p_EPS_BearerId)-6), 8); // @sic R5-174469 sic@ } else { v_Filter.precedence := int2bit(0, 8); } if (p_PacketFilterNumber == 6) { // @sic R5-183239, R5-193982 sic@ if (p_PdnTypeValue == tsc_PdnType_IPv4) { // IPv4 only v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderICMP; } else { v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderICMPv6; } } else { v_Filter.contents[1] := cs_PktFilterCompSingleRemotePort31160(p_EPS_BearerId); v_ContentsLength := v_ContentsLength + 3; v_Filter.contents[2] := cs_PktFilterCompProtocolIdNextHeaderUDP; } v_ContentsLength := v_ContentsLength + 2; } case (2, 7) { // @sic R5-183239 sic@ // Update to values in Reference packet filter #2 // UL only filter, ID=1 @sic R5-104702 sic@ v_Filter.spare := '00'B; // @sic R5s130195 Baseline Moving sic@ v_Filter.direction := '10'B; // @sic R5s130195 Baseline Moving sic@ v_Filter.iei := '1'H; // @sic R5s130195 Baseline Moving sic@ v_Filter.precedence := int2bit((hex2int(p_EPS_BearerId)-6+8), 8); // @sic R5-104702 sic@ if (p_PacketFilterNumber == 7) {// @sic R5-183239, R5-193982 sic@ if (p_PdnTypeValue == tsc_PdnType_IPv4) { // IPv4 only v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderICMP; } else { v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderICMPv6; } } else { v_Filter.contents[1] := cs_PktFilterCompSingleRemotePort61000(p_EPS_BearerId); v_ContentsLength := v_ContentsLength + 3; v_Filter.contents[2] := cs_PktFilterCompProtocolIdNextHeaderUDP; } v_ContentsLength := v_ContentsLength + 2; } case (3){ // Update to values in Reference packet filter #3 // Bidirectional filter, ID=2 @sic R5-104702 sic@ v_Filter.spare := '00'B; // @sic R5s130195 Baseline Moving sic@ v_Filter.direction := '11'B; // @sic R5s130195 Baseline Moving sic@ v_Filter.iei := '2'H; // @sic R5s130195 Baseline Moving sic@ v_Filter.precedence := '00001111'B; } case (4){ // @sic R5-132085 sic@ // Update to values in Reference packet filter #4 // Bidirectional filter, ID=3 v_Filter.spare := '00'B; v_Filter.direction := '11'B; v_Filter.iei := '3'H; if (hex2int(p_EPS_BearerId)>5) { v_Filter.precedence := int2bit((hex2int(p_EPS_BearerId)-6), 8); // @sic R5-174469 sic@ } else { v_Filter.precedence := int2bit(0, 8); } v_Filter.contents[0] := cs_PktFilterCompRemotePortRange(tsc_IMS_MediaPort_M1, tsc_IMS_MediaPort_M1+1); // @sic R5s130495 sic@ v_ContentsLength := v_ContentsLength + 5; v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderUDP; v_ContentsLength := v_ContentsLength + 2; } case (5){ // @sic R5-132085 sic@ // Update to values in Reference packet filter #5 // Bidirectional filter, ID=3 v_Filter.spare := '00'B; v_Filter.direction := '11'B; v_Filter.iei := '4'H; if (hex2int(p_EPS_BearerId)>5) { v_Filter.precedence := int2bit((hex2int(p_EPS_BearerId)-6), 8); // @sic R5-174469 sic@ } else { v_Filter.precedence := int2bit(0, 8); } v_Filter.contents[0] := cs_PktFilterCompRemotePortRange(tsc_IMS_MediaPort_M2, tsc_IMS_MediaPort_M2+1); // @sic R5s130495 sic@ v_ContentsLength := v_ContentsLength + 5; v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderUDP; v_ContentsLength := v_ContentsLength + 2; } case (8) { // @sic R5-187765 sic@ // Update to values in Reference packet filter #8 // UL only filter, ID=1 v_Filter.spare := '00'B; v_Filter.direction := '10'B; v_Filter.iei := '1'H; v_Filter.precedence := int2bit((hex2int(p_EPS_BearerId)-6+8), 8); } case (9) { // @sic R5-187765 sic@ // Update to values in Reference packet filter #9 // Bidirectional filter, ID=5 v_Filter.spare := '00'B; v_Filter.direction := '11'B; v_Filter.iei := '5'H; if (hex2int(p_EPS_BearerId)>5) { v_Filter.precedence := int2bit((hex2int(p_EPS_BearerId)-6), 8); } else { v_Filter.precedence := int2bit(0, 8); } v_Filter.contents[0] := cs_PktFilterCompRemotePortRange(tsc_IMS_MediaPort_M2, tsc_IMS_MediaPort_M2+1); v_ContentsLength := v_ContentsLength + 5; v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderUDP; v_ContentsLength := v_ContentsLength + 2; } case (10) { // @sic R5-210201 sic@ // Update to values in Reference packet filter #10 // Bidirectional filter, ID=4 v_Filter.spare := '00'B; v_Filter.direction := '11'B; // Bidirectional v_Filter.iei := '4'H; // ID=4 if (hex2int(p_EPS_BearerId)>5) { v_Filter.precedence := int2bit((hex2int(p_EPS_BearerId)-6), 8); } else { v_Filter.precedence := int2bit(0, 8); } v_Filter.contents[0] := cs_PktFilterCompSingleRemotePort(tsc_IMS_MediaPort_M3); // same portnumber as tsc_MCX_MediaControlPortNW v_ContentsLength := v_ContentsLength + 3; v_Filter.contents[1] := cs_PktFilterCompProtocolIdNextHeaderUDP; v_ContentsLength := v_ContentsLength + 2; } } v_Filter.iel := int2oct(v_ContentsLength, 1); p_PacketFilterByRef := v_Filter; p_PacketFilterLengthByRef := v_ContentsLength + 3; // length of the filter contents + 3 octets for iei, precedence and iel }