3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220180

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2331 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC Logged MDT test case 8.1.6.1.2.12 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add MDT TC 8.1.6.1.2.12 Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to MDT test case 8.1.6.1.2.12 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	Mediatek MT6983	3

5	References	4

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.2.12 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.6.1.2.12

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Mediatek MT6983

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | TC_8_1_6_1_2_12_NR5GC () |
| --- | --- |
| Reason for change | t_GuardTimer 300sec is less for this case as step18 takes10min. |
| Summary of change | Updated t_GaurdTimer to 900sec. |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comment |  |

Before Change

testcase TC_8_1_6_1_2_12_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_8_1_6_1_2_12_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change

testcase TC_8_1_6_1_2_12_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(900); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_8_1_6_1_2_12_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

## Change 2

| Function name | f_TC_8_1_6_1_2_12_TestBody () |
| --- | --- |
| Reason for change | - Since the test case expect at step18 that t330 should be expired but T330(loggingDuration) by default set to 120 min so modified it to 10min [TS38.331 cl 5.5a.1.3] start timer T330 with the timer value set to the loggingDuration. #draft proseCR would be raised for RAN5#94-e meeting - At step16 Cell 1 should be handling UEInformationResponse. - UE use SRB2 when it will be having logMeasReport_r16 as per 38.331 clause 5.7.10.3 at step 14 and 29. “1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else: - submit the UEInformationResponse message to lower layers for transmission via SRB1.” - At step 29 exception fo UE was given for logMeasAvailable in logMeasReport is True. #draft proseCR would be raised for RAN5#94-e meeting - As per prose step 29a1 and 29a2 should be there. |
| Summary of change | - loggedDuration is modified to 10min at step1. - Cell_ID updated to cell1 - UEInformationResponse reception modified for SRB2. - Added anyoromit option for logMeasAvailable in logMeasReport . - Steps 29a1 and 29a2 added. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_8_1_6_1_2_12_TestBody() runs on NR5GC_PTC { var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var integer i:=0; //@siclog "Step 1" siclog@ //The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical/* added arguments for loggedduration here*/))); //@siclog "Step 2" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30 seconds for the UE to perform the logging at regular time intervals. f_Delay(30.0); //@siclog "Step 4-12" siclog@ //Steps 1 to 9 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, -, -, -, -, -, -, -, -, -, -, cr_RRCSetupComplete_v1610_IEs(-,-,cr_UE_MeasurementsAvailable_r16(true_))); //@siclog "Step 13" siclog@ //The SS sends an UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Step 14" siclog@ //The UE sends an UEInformationResponse message including logMeasReport. v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14: Not enough LogMeasurementInfo entries") }else{ while(i < v_FieldCnt) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16( -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), *, omit))) { if (i == v_FieldCnt-1) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14: None of the LogMeasurementInfo matched the template") } }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 19 PASS"); break; } i := i +1; } } //@siclog "Step 15" siclog@ //The SS sends an UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Step 16" siclog@ //Check: Does the UE send an UEInformationResponse message without logMeasReport? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-,omit))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 15 : UE send an UEInformationResponse message not include logMeasReport."); //@siclog "Step 17" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 18" siclog@ //Wait 10 minutes for the UE to perform the logging at regular time intervals as to ensure timer T330 has expired. f_Delay(600.0); //@siclog "Step 19-27" siclog@ //Steps 1 to 9 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, -, -, -, -, -, -, -, -, -, -, cr_RRCSetupComplete_v1610_IEs(-,-,cr_UE_MeasurementsAvailable_r16(true_))); //@siclog "Step 28" siclog@ //The SS sends an UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Step 29" siclog@ //The UE sends an UEInformationResponse message including logMeasReport. v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 29: Not enough LogMeasurementInfo entries") }else{ while(i < v_FieldCnt) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16( -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), *, omit))) { if (i == v_FieldCnt-1) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 29: None of the LogMeasurementInfo matched the template") }; }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 29 PASS"); break; } i := i +1; } } /* added step 29a1 and 29a2 here*/ //@siclog "Step 30" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 31" siclog@ //Wait 30 seconds. f_Delay(30.0); //@siclog "Step 32" siclog@ //The SS transmits a Paging message on NR Cell 1. //@siclog "Step 33" siclog@ //The UE transmits an RRCRequest message on NR Cell 1. //@siclog "Step 34" siclog@ //The SS transmits an RRCSetup message on NR Cell 1. //@siclog "Step 35" siclog@ //Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? //@siclog "Step 36-40" siclog@ //Steps 5 to 9 of the generic procedure in TS 38.508-1[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, " Step 35"); }//End of f_TC_8_1_6_1_2_12_NR5GC_TestBody

After Change

function f_TC_8_1_6_1_2_12_TestBody() runs on NR5GC_PTC { var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var integer i:=0; //@siclog "Step 1" siclog@ //The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical,-,min10))); //@siclog "Step 2" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30 seconds for the UE to perform the logging at regular time intervals. f_Delay(30.0); //@siclog "Step 4-12" siclog@ //Steps 1 to 9 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, -, -, -, -, -, -, -, -, -, -, cr_RRCSetupComplete_v1610_IEs(-,-,cr_UE_MeasurementsAvailable_r16(true_))); //@siclog "Step 13" siclog@ //The SS sends an UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Step 14" siclog@ //The UE sends an UEInformationResponse message including logMeasReport. v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1,tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14: Not enough LogMeasurementInfo entries") }else{ while(i < v_FieldCnt) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16( -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), *, omit))) { if (i == v_FieldCnt-1) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14: None of the LogMeasurementInfo matched the template") } }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 19 PASS"); break; } i := i +1; } } //@siclog "Step 15" siclog@ //The SS sends an UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Step 16" siclog@ //Check: Does the UE send an UEInformationResponse message without logMeasReport? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-,omit))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 15 : UE send an UEInformationResponse message not include logMeasReport."); //@siclog "Step 17" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 18" siclog@ //Wait 10 minutes for the UE to perform the logging at regular time intervals as to ensure timer T330 has expired. f_Delay(600.0); //@siclog "Step 19-27" siclog@ //Steps 1 to 9 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, -, -, -, -, -, -, -, -, -, -, cr_RRCSetupComplete_v1610_IEs(-,-,cr_UE_MeasurementsAvailable_r16(true_))); //@siclog "Step 28" siclog@ //The SS sends an UEInformationRequest message to get logMeasReport. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); //@siclog "Step 29" siclog@ //The UE sends an UEInformationResponse message including logMeasReport. v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1,tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, -,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 29: Not enough LogMeasurementInfo entries") }else{ while(i < v_FieldCnt) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16( -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), *, omit))) { if (i == v_FieldCnt-1) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 29: None of the LogMeasurementInfo matched the template") }; }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 29 PASS"); break; } i := i +1; } } //@siclog "Step29a" siclog@ if (isvalue (v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasAvailable_r16)) { SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_NR_UEInformationRequest_LOG)); v_MeasQuantityResults := f_NR_Get_MeasQuantityResults() SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1,tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit ))))-> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); while(i < v_FieldCnt) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (not match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16( -, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), *, omit))) { if (i == v_FieldCnt-1) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 29a2: None of the LogMeasurementInfo matched the template") }; }else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 29a2 PASS"); break; } i := i +1; } } //@siclog "Step 30" siclog@ //The SS transmits a RRCRelease message to release the RRC connection. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 31" siclog@ //Wait 30 seconds. f_Delay(30.0); //@siclog "Step 32" siclog@ //The SS transmits a Paging message on NR Cell 1. //@siclog "Step 33" siclog@ //The UE transmits an RRCRequest message on NR Cell 1. //@siclog "Step 34" siclog@ //The SS transmits an RRCSetup message on NR Cell 1. //@siclog "Step 35" siclog@ //Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? //@siclog "Step 36-40" siclog@ //Steps 5 to 9 of the generic procedure in TS 38.508-1[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, " Step 35"); }//End of f_TC_8_1_6_1_2_12_NR5GC_TestBody }

## Change 3

| Template name | cs_38508_LoggedMeasurementConfiguration |
| --- | --- |
| Reason for change | Since the test case expect at step18 that t330 should be expired and SS waits for 10min but T330(logingDuration) by default set to 120 min so modified it to 10min [TS38.331 cl 5.5a.1.3] start timer T330 with the timer value set to the loggingDuration. |
| Summary of change | loggedDuration is modified to 10min at step1. Updated template for this change. |
| TTCN module | NR5GC_MDT_Specific_Templates.ttcn |
| MCC160 Comment |  |

Before Change

template (value) DL_DCCH_Message cs_38508_LoggedMeasurementConfiguration (PLMN_Identity p_PLMN_Identity, AbsoluteTimeInfo_r16 p_AbsoluteTimeInfo_r16, template (value) LoggedMeasurementConfiguration_r16_IEs.reportType p_LoggedMeasurementConfiguration_r16, template (omit) AreaConfiguration_r16 p_AreaConfiguration_r16 := omit /*added template here*/) := { /* 38.508-1 clause 4.6.1 table 4.6.1-5AA LoggedMeasurementConfiguration */ /* @status */ message_ := { c1 := { loggedMeasurementConfiguration_r16 := { criticalExtensions := { loggedMeasurementConfiguration_r16 := { traceReference_r16 := { plmn_Identity_r16 := p_PLMN_Identity, traceId_r16 := '0000EF'O }, traceRecordingSessionRef_r16 := '001A'O, tce_Id_r16 := '05'O, absoluteTimeInfo_r16 := p_AbsoluteTimeInfo_r16, areaConfiguration_r16 := p_AreaConfiguration_r16, plmn_IdentityList_r16 := omit, bt_NameList_r16 := omit, wlan_NameList_r16 := omit, sensor_NameList_r16 := omit, loggingDuration_r16 := min120, reportType := p_LoggedMeasurementConfiguration_r16, lateNonCriticalExtension := omit, nonCriticalExtension := omit } } } } } };

After Change

template (value) DL_DCCH_Message cs_38508_LoggedMeasurementConfiguration (PLMN_Identity p_PLMN_Identity, AbsoluteTimeInfo_r16 p_AbsoluteTimeInfo_r16, template (value) LoggedMeasurementConfiguration_r16_IEs.reportType p_LoggedMeasurementConfiguration_r16, template (omit) AreaConfiguration_r16 p_AreaConfiguration_r16 := omit, template (omit) LoggingDuration_r16 p_LoggingDuration_r16 := min120 ) := { /* 38.508-1 clause 4.6.1 table 4.6.1-5AA LoggedMeasurementConfiguration */ /* @status */ message_ := { c1 := { loggedMeasurementConfiguration_r16 := { criticalExtensions := { loggedMeasurementConfiguration_r16 := { traceReference_r16 := { plmn_Identity_r16 := p_PLMN_Identity, traceId_r16 := '0000EF'O }, traceRecordingSessionRef_r16 := '001A'O, tce_Id_r16 := '05'O, absoluteTimeInfo_r16 := p_AbsoluteTimeInfo_r16, areaConfiguration_r16 := p_AreaConfiguration_r16, plmn_IdentityList_r16 := omit, bt_NameList_r16 := omit, wlan_NameList_r16 := omit, sensor_NameList_r16 := omit, loggingDuration_r16 := p_LoggingDuration_r16, reportType := p_LoggedMeasurementConfiguration_r16, lateNonCriticalExtension := omit, nonCriticalExtension := omit } } } } } };

# Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Mediatek MT6983

The Mediatek MT6983 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_6_1_2_12_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_6_1_2_12_PIXIT.txt

# References

| [1] | R5s220181 - Supporting information for addition of NR5GC Logged MDT test case 8.1.6.1.2.12 in FR1 |
| --- | --- |