3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220217

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2355 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.6.1.2.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current TTCN code does not handle all the SS procedure related to reestablishment procedure definied in 38.523-3. The function f_NR5GC_RRCReestablishment_SON_MDT already handles this and is used in other MDT test cases. The check for restablishment message related to logMeasAvailable can be done via this function Therefore it is proposed to use f_NR5GC_RRCReestablishment_SON_MDT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reworked the restablishment procedure to use function f_NR5GC_RRCReestablishment_SON_MDT and removed code that is not required |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reestalbishment procedure will not be inline with 38.523-3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections to test case 8.1.6.1.2.7	2

1.1	f_TC_8_1_6_1_2_7_NR5GC_TestBody	2

# Corrections to test case 8.1.6.1.2.7

## f_TC_8_1_6_1_2_7_NR5GC_TestBody

| Function name | f_TC_8_1_6_1_2_7_NR5GC_TestBody |
| --- | --- |
| Reason for change | The current TTCN code does not handle all the SS procedure related to reestablishment procedure definied in 38.523-3. The function f_NR5GC_RRCReestablishment_SON_MDT already handles this and is used in other MDT test cases. The check for restablishment message related to logMeasAvailable can be done via this function Therefore it is proposed to use f_NR5GC_RRCReestablishment_SON_MDT |
| Summary of change | Reworked the restablishment procedure to use function f_NR5GC_RRCReestablishment_SON_MDT and removed code that is not required |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_2_7_NR5GC_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var float v_WaitToLogging := 30.0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),//@sic R5-216303, R5s211676 sic@ cs_NR_CellPower (nr_Cell2, -91, -91) }; //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfigurationmessage with reportType is set to periodical and LoggingInterval is set to ms10240, to configure the UE to perform logging of measurement results while in RRC_IDLE. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(v_WaitToLogging); //@siclog "Step 4-11" siclog@ //Steps 1 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on Cell 1. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); //@siclog "Step 12" siclog@ //The SS changes Cell 1 and Cell 2 levels according to the row "T1" in table 8.1.6.1.2.7.3.2-1 for FR1 or table 8.1.6.1.2.7.3.2-1 for FR2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 13" siclog@ //The UE send RRCReestablishmentRequest message on NR Cell 2. f_NR5GC_RRCReestablishment_Steps1To2(nr_Cell1, nr_Cell2, otherFailure, tsc_C_RNTI_Value4); //@siclog "Step 14" siclog@ //The SS transmits RRCReestablishmentmessage. f_NR_ULGrantConfiguration_Start(nr_Cell2, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //Target Cell: Activate Security f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2, 0, -, true); SRB.send (cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def))); //@siclog "Step 15" siclog@ //Check: Does The UE transmits RRCReestablishmentCompletemessage including logMeasAvailable IE? SRB.receive ( car_NR_SRB1_RrcPdu_IND ( nr_Cell2, cr_38508_RRCReestablishmentComplete(-,cr_RRCReestablishmentComplete_v1610_IEs_LogMeas)) ); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.6.1.2.7 Step 15"); //@siclog "Step 16-17" siclog@ //The SS transmits an RRCReconfigurationmessage to resume existing radio bearer. //The UE transmit an RRCConnectionReconfigurationComplete message. f_NR5GC_RRCReestablishment_Steps8To13(nr_Cell1, nr_Cell2, -, cs_NR_RLF_TimersAndConstantsRelease); //@siclog "Step 18" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell2); }//end of f_TC_8_1_6_1_2_7_NR5GC_TestBody

After Change:

function f_TC_8_1_6_1_2_7_NR5GC_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var float v_WaitToLogging := 30.0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),//@sic R5-216303, R5s211676 sic@ cs_NR_CellPower (nr_Cell2, -91, -91) }; //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfigurationmessage with reportType is set to periodical and LoggingInterval is set to ms10240, to configure the UE to perform logging of measurement results while in RRC_IDLE. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(v_WaitToLogging); //@siclog "Step 4-11" siclog@ //Steps 1 to 8 of the generic procedure in TS 38.508[4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on Cell 1. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); //@siclog "Step 12" siclog@ //The SS changes Cell 1 and Cell 2 levels according to the row "T1" in table 8.1.6.1.2.7.3.2-1 for FR1 or table 8.1.6.1.2.7.3.2-1 for FR2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 13" siclog@ //The UE send RRCReestablishmentRequest message on NR Cell 2. //WA#reworked reestablishment procedure to use common function, as the current code is not inline with 38523-3 f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell1, nr_Cell2, logMeasAvailable,otherFailure); // Removed f_NR5GC_RRCReestablishment_Steps1To2(nr_Cell1, nr_Cell2, otherFailure, tsc_C_RNTI_Value4); //@siclog "Step 14" siclog@ //The SS transmits RRCReestablishmentmessage. // Removed f_NR_ULGrantConfiguration_Start(nr_Cell2, -, -, -, cs_NR_UplinkTimeAlignment_Stop); //Target Cell: Activate Security // Removed f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2, 0, -, true); // Removed SRB.send (cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, cs_TimingInfo_Now,cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def))); //@siclog "Step 15" siclog@ //Check: Does The UE transmits RRCReestablishmentCompletemessage including logMeasAvailable IE? // Removed SRB.receive ( car_NR_SRB1_RrcPdu_IND ( nr_Cell2, cr_38508_RRCReestablishmentComplete(- ,cr_RRCReestablishmentComplete_v1610_IEs_LogMeas)) ); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.6.1.2.7 Step 15"); //@siclog "Step 16-17" siclog@ //The SS transmits an RRCReconfigurationmessage to resume existing radio bearer. //The UE transmit an RRCConnectionReconfigurationComplete message. // Removed f_NR5GC_RRCReestablishment_Steps8To13(nr_Cell1, nr_Cell2, -, cs_NR_RLF_TimersAndConstantsRelease); //@siclog "Step 18" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell2); }//end of f_TC_8_1_6_1_2_7_NR5GC_TestBody