3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220222

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2357 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of R16 eNS test case 9.1.10.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-16 RACS test case 9.1.10.6 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Rel-16 RACS test case 9.1.10.6 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 2	3

2.2	Change 2	6

2.3	Change 3	11

3	Branches executed	12

4	Execution Log Files	12

4.1	Qualcomm SM8450 + SDX65	12

5	References	12

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.1.10.6 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Narendra Kalahasti

Narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	9.1.10.6.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk49

System Simulator used:	Keysight S8704A Protocol Conformance Toolset, Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8450 + SDX65

Verification Status:	PASS

# Corrections required

## Change 2

| Function name | fl_TC_9_1_10_6_TestBody() |
| --- | --- |
| Reason for change | - One closing brace is not correct which causes incorrect steps to be executed. - Step 28a1-a21 repeats steps 2-22, but only till step 20 is repeated - Steps28a22 is to remove the USIM, but TTCN is missing the user prompt of this - Since Step 28a22 is USIM removal without power down, function f_NR_UE_DeRegisterOnSwitchOff cannot be used here - Step 28a25 is missing to prompt for reinserting the USIM and instead, Switch On is incorrectly used |
| Summary of change | - Moved closing brace - Added steps 21 and 22 as repetition in Steps 28a1-a213 - Added the user prompt to remove USIM - Added steps to handle RRC_Idle DEREGISTRATION without the Switch Off prompt Note: A Prose CR on 38.523-1 will be raised at RAN5#94-e - Added the function to prompt the uset to re-insert the default USIM |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_9_1_10_6_TestBody() runs on NR5GC_PTC { … … //@siclog "Step 27A" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //EXCEPTION: Steps 28a1 to 28a27 take place if pc_USIM_Removal = TRUE if(pc_USIM_Removal){ //@siclog "Step 28a1 - 28a21" siclog@ fl_TC_9_1_10_6_Steps2To20(); } //@siclog "Step 28a22 - 28a23" siclog@ //Switch off procedure in RRC_IDLE specified in TS 38.508-1 subclause 4.9.6.1 is performed. f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A, -, true); //@siclog "Step 28a24" siclog@ //The SS configures NGC Cell A as the "Non-suitable cell" f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); //@siclog "Step 28a25" siclog@ //The USIM is inserted into the UE. f_UT_SwitchOnUE(UT, true); //@siclog "Step 28a26" siclog@ /*verify that S-NSSAI=2 is not in the Rejected NSSAI list associated with current PLMN */ v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28a26"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28a26"); //@siclog "Step 28a27" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); } //@siclog "Step 29 - 47" //Steps 2 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA); }

After Change

function fl_TC_9_1_10_6_TestBody() runs on NR5GC_PTC { … … //@siclog "Step 27A" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //EXCEPTION: Steps 28a1 to 28a27 take place if pc_USIM_Removal = TRUE if(pc_USIM_Removal){ //@siclog "Step 28a1 - 28a19" siclog@ fl_TC_9_1_10_6_Steps2To20(); //MOVED } //@siclog "Step 28a20" siclog@ /*Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause 'S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization' associated with current PLMN */ v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } //@siclog "Step 28a21" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=2 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //Cause removal of USIM from the UE without powering down. //@siclog "Step 28a22" siclog@ f_UT_USIM_Remove (UT); //@siclog "Step 28a23" siclog@ v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); v_DetachReq := f_NR5GC_OptionalIMSDeregisterOnSwitchOff(v_NGC_NASCellA, v_ReceivedMsg); // @sic R5-204389 sic@ f_NR_RRCRelease(v_NGC_NASCellA); //@siclog "Step 28a24" siclog@ //The SS configures NGC Cell A as the "Non-suitable cell" f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableCellSSS_EPRE_FR1, tsc_NR_NonSuitableCellSSS_EPRE_FR2); //@siclog "Step 28a25" siclog@ //The USIM is inserted into the UE. f_UT_USIM_Insert (UT, "38.508-1, USIM containing default values"); //@siclog "Step 28a26" siclog@ /*verify that S-NSSAI=2 is not in the Rejected NSSAI list associated with current PLMN */ v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28a26"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 28a26"); } //@siclog "Step 28a27" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); } //@siclog "Step 29 - 47" //Steps 2 to 20a1 of the generic procedure for NR RRC_IDLE specified in TS 38.508-1 subclause 4.5.2 are performed. f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA); }

## Change 2

| Function name | fl_TC_9_1_10_6_Steps2To20() |
| --- | --- |
| Reason for change | - Incorrect iel for pendingNSSAI IE in Registration Accept at Step 12 - Invalid iel, s_nssai_length, s_nssai_sd values for rejectedNSSAI IE in Configuration Update Command at Step 18. See associated prose CR for more details. - Intiialisation step is missing in calculation of security parameters for EAP Authentication - ConfigUpdateInd IE in Configuraion Update Command at Step 18 is omitted by default. Prose requires this IE to be present as per TS 36.523-1 Table 9.1.10.6.3.3-6 |
| Summary of change | - Changed iel for v_PendingNSSAI from ‘04’O to ‘02’O - Changed some of the values of v_RejectedNSSAI: - Iel from ‘02’O to ‘05’O - rejectS_NSSAI[0] length from ‘0001’B to ‘0100’B - rejectS_NSSAI[0] sd from ‘omit’ to ‘FFFFFF’O Note : An associated prose CR on 38.523-1 will be raised at RAN5#94 - initialized v_SecurityParams using PLMN and IMSI values prior to calculating EAP authentication parameters - enabled configUpdateInd IE in Confioguration Update Command at Step 18 as per values in prose |
| TTCN module | NetworkingSlicingEnh_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_10_6_Steps2To20() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var NG_NAS_SecurityParams_Type v_SecurityParams; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) EAP_Message v_EAP; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit), cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; var template (value) NSSAI v_PendingNSSAI := { iei := '39'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; var template (value) RejectedNSSAI v_RejectedNSSAI := { iei := '11'O, iel := '02'O, rejectS_NSSAI := {cs_RejectedS_NSSAI('0001'B,'0010'B, '02'O, omit)} }; //@siclog "Step 2" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message with NSSAA set to '1'B ? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B, -, -, -, -, -, -, v_PendingNSSAI); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, v_SRB); v_SRB := tsc_NR_RbId_SRB2; // SRB2 is established during PDU Session Establishment, hence further NAS signalling will happen on SRB2 } v_SecurityParams := f_NR5GC_Security_Get(); v_EAP := cs_EAP_Message (f_NG_EAP_ChallengeReq (v_SecurityParams, v_PLMN), omit); //@siclog "Step 15" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMMAND(cs_S_NSSAI(omit, '01'O, '02'O, omit),v_EAP)))); //@siclog "Step 16" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMPLETE(cs_S_NSSAI(omit, '01'O, '02'O, omit),?)))) -> value v_ReceivedAsp; //@siclog "Step 17" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT(cs_S_NSSAI(omit, '01'O, '02'O, omit),cs_EAP_Message(bit2oct(encvalue(cs_EAP_Failure)), omit))))); //@siclog "Step 17" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(-, -, -, -, -, -, -, -, -, -, -, -, -, -, v_RejectedNSSAI)))); //@siclog "Step 19" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); //SS releases the RRC connection. //@siclog "Step 20" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); }

After Change:

function fl_TC_9_1_10_6_Steps2To20() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var NG_NAS_SecurityParams_Type v_SecurityParams; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) EAP_Message v_EAP; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit), cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; var template (value) NSSAI v_PendingNSSAI := { iei := '39'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; var template (value) RejectedNSSAI v_RejectedNSSAI := { iei := '11'O, iel := '05'O, rejectS_NSSAI := {cs_RejectedS_NSSAI('0100'B,'0010'B, '02'O, 'FFFFFF'O)} }; //@siclog "Step 2" siclog@ //Check: Does the UE transmit a REGISTRATION REQUEST message with NSSAA set to '1'B ? v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 12" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B, -, -, -, -, -, -, v_PendingNSSAI); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, v_SRB); v_SRB := tsc_NR_RbId_SRB2; // SRB2 is established during PDU Session Establishment, hence further NAS signalling will happen on SRB2 } v_SecurityParams := f_NR5GC_Security_Get(); v_SecurityParams := f_NG_EAP_AuthenticationInit(v_SecurityParams, v_PLMN, hex2str(px_IMSI_Def)); v_EAP := cs_EAP_Message (f_NG_EAP_ChallengeReq (v_SecurityParams, v_PLMN), omit); //@siclog "Step 15" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMMAND(cs_S_NSSAI(omit, '01'O, '02'O, omit),v_EAP)))); //@siclog "Step 16" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_COMPLETE(cs_S_NSSAI(omit, '01'O, '02'O, omit),?)))) -> value v_ReceivedAsp; //@siclog "Step 17" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT(cs_S_NSSAI(omit, '01'O, '02'O, omit),cs_EAP_Message(bit2oct(encvalue(cs_EAP_Failure)), omit))))); //@siclog "Step 17" siclog@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, v_SRB, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(cs_ConfigUpdateInd('0'B, '1'B), -, -, -, -, -, -, -, -, -, -, -, -, -, v_RejectedNSSAI)))); //@siclog "Step 19" siclog@ SRB.receive (car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_CONFIGURATION_UPDATE_COMPLETE))); //SS releases the RRC connection. //@siclog "Step 20" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); }

## Change 3

| Function name | cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT |
| --- | --- |
| Reason for change | Incorrect messageType used for this template |
| Summary of change | Changed the messageType from tsc_MT_NG_NetworkSliceSpecificAuthenticationComplete to tsc_MT_NG_NetworkSliceSpecificAuthenticationResult Note : This change was previously included in R5s211718 and was accepted as “Accepted (Already implemented in 21wk49)” but the correction is actually not in IWD-21wk49, so is included here again. |
| TTCN module | NetworkingSlicingEnh_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) NG_NAS_DL_Message_Type cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT (template (value) S_NSSAI_Type p_S_NSSAI, template (value) EAP_Message p_EAP) := { /* 24.501 cl. 8.2.33 */ network_Slice_Specific_Authentication_Result := { protocolDiscriminator := tsc_EPD_GMM, /* cl. 9.2 M V 1 */ spareHalfOctet := tsc_SpareHalfOctet, /* cl. 9.5 M V 1 */ securityHeaderType := tsc_SHT_NoSecurityProtection, messageType := tsc_MT_NG_NetworkSliceSpecificAuthenticationComplete, /* cl. 9.7 M V 1 */ sNSSAI := p_S_NSSAI, eapMessage := p_EAP } };

After Change:

template (value) NG_NAS_DL_Message_Type cs_NG_NETWORK_SLICE_SPECIFIC_AUTHENTICATION_RESULT (template (value) S_NSSAI_Type p_S_NSSAI, template (value) EAP_Message p_EAP) := { /* 24.501 cl. 8.2.33 */ network_Slice_Specific_Authentication_Result := { protocolDiscriminator := tsc_EPD_GMM, /* cl. 9.2 M V 1 */ spareHalfOctet := tsc_SpareHalfOctet, /* cl. 9.5 M V 1 */ securityHeaderType := tsc_SHT_NoSecurityProtection, messageType := tsc_MT_NG_NetworkSliceSpecificAuthenticationResult, /* cl. 9.7 M V 1 */ sNSSAI := p_S_NSSAI, eapMessage := p_EAP } };

# Branches executed

The test case was executed on NR band n41 uing NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Qualcomm SM8450 + SDX65

The Qualcomm SM8450 + SDX65 UE passed this test case on Keysight S8704A Protocol Conformance Toolset solution. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_9_1_10_6_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_9_1_10_6_PIXIT.xml

The Qualcomm SM8450 + SDX65UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR solution. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_9_1_10_6_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_9_1_10_6_PIXIT.txt

# References

| [1] | R5s220223 : NR5GC FR1 : Addition of R16 eNS test case 9.1.10.6 |
| --- | --- |