3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220336

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0780 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS Call Control test case 7.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify IMS 5GS test case 7.23 using IWD 21wk49 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS test case 7.23 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.23.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Prose CR R5-220564 raised for RAN5#94e |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	13

4	Execution Log Files	14

HiSilicon	14

Samsung SM-G996U	14

5	References	14

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 7.23 which is part of the iwd-TTCN3-B2020-09_D21wk49.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	7.23.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49.zip

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000 UE, Samsung SM-G996U UE

Verification Status:	PASS

# Corrections required

  - Correction to function f_IMS_MOCallSetup_Video_Step2

| Function name | f_IMS_MOCallSetup_Video_Step2 |
| --- | --- |
| Reason for change | Addition of video is performed with a re-INVITE hence condition A11 is not met. |
| Summary of change | Set condition A11 to false. |
| TTCN module | IMS_Procedures_Video.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

select (p_VideoCallType) {

case (SetupVideoCall) {

v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAudioVideo_RX(p_AudioCodecList, p_VideoCodecList, p_WithPreconditions);

v_Context := A_2_1_A4;

}

case (AddVideo) {

v_AudioCodec := p_AudioCodecList[0];                /* @sic R5-213073 sic@ */

v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAddVideo_RX(v_AudioCodec, p_VideoCodecList, p_WithPreconditions);

v_Context := A_2_1_A5;

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

select (p_VideoCallType) {

case (SetupVideoCall) {

v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAudioVideo_RX(p_AudioCodecList, p_VideoCodecList, p_WithPreconditions);

v_Context := A_2_1_A4;

}

case (AddVideo) {

v_A11 := false;  //WA#WI=918780 Change 1: Add video is a re-INVITE hence condition A11 is not met. v_A11 is used to add a check to AcceptContact.

v_AudioCodec := p_AudioCodecList[0];                /* @sic R5-213073 sic@ */

v_SDP_Message_Step2 := f_IMS_BuildSDP_InitialOfferAddVideo_RX(v_AudioCodec, p_VideoCodecList, p_WithPreconditions);

v_Context := A_2_1_A5;

}

}

<<SKIPPED CODE>>

  - Correction to function f_IMS_MTCallSetup_Video_SendInvite

| Function name | f_IMS_MTCallSetup_Video_SendInvite |
| --- | --- |
| Reason for change | With current implementation INVITE will only be setup with preconditions. |
| Summary of change | Added input paramterer to f_IMS_InviteRequest_MessageHeaderTX to include preconditinns or not depending of test case needs. |
| TTCN module | IMS_Procedures_Video.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

select (p_VideoCallType) {

case (SetupVideoCall) { v_Context := A_2_1_A4; } /* @sic R5-170782; R5-172930: SetupVideoCall sic@ */

case else             { v_Context := A_2_1_A5; }

}

if (isvalue(p_ContactUrl)) {

v_ContactUrl := valueof(p_ContactUrl);

} else {

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl();

}

if (not f_SDP_MediaDescr_CheckWithPreconditions(p_SdpOffer.media_list[0])) {       /* @sic R5-216215: no precondition tag in case of A.16.2 sic@ */

v_OptionTagsForSupported := v_OptionTagsForSupported & tsc_OptionTagList_precondition;

}

v_MessageHeader_Invite  := f_IMS_InviteRequest_MessageHeaderTX(-, v_Context);                        /* @sic R5-176209: p_SrvccAlertingSupported := pc_IMS_SRVCCAlert for video too (i.e. v_SrvccAlerting removed) sic@ */

v_InviteRequest := cs_INVITE_Request(v_ContactUrl, v_MessageHeader_Invite, cs_MessageBody_SDP(p_SdpOffer));

v_InviteRequestWithSdp := f_IMS_InviteRequestWithSdpMT(v_InviteRequest, p_SdpOffer);

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

select (p_VideoCallType) {

case (SetupVideoCall) { v_Context := A_2_1_A4; } /* @sic R5-170782; R5-172930: SetupVideoCall sic@ */

case else             { v_Context := A_2_1_A5; }

}

if (isvalue(p_ContactUrl)) {

v_ContactUrl := valueof(p_ContactUrl);

} else {

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl();

}

if (f_SDP_MediaDescr_CheckWithPreconditions(p_SdpOffer.media_list[0])) {       /* @sic R5-216215: no precondition tag in case of A.16.2 sic@ */ //WA#WI=918780 Change 2: Preconditions tags are always included indenpendant previous SDP message.

v_OptionTagsForSupported := v_OptionTagsForSupported & tsc_OptionTagList_precondition;

}

v_MessageHeader_Invite  := f_IMS_InviteRequest_MessageHeaderTX(v_OptionTagsForSupported, v_Context);                        /* @sic R5-176209: p_SrvccAlertingSupported := pc_IMS_SRVCCAlert for video too (i.e. v_SrvccAlerting removed) sic@ */

v_InviteRequest := cs_INVITE_Request(v_ContactUrl, v_MessageHeader_Invite, cs_MessageBody_SDP(p_SdpOffer));

v_InviteRequestWithSdp := f_IMS_InviteRequestWithSdpMT(v_InviteRequest, p_SdpOffer);

<<SKIPPED CODE>>

  - Correction to function f_BuildDefaultQoSRules

| Function name | f_BuildDefaultQoSRules |
| --- | --- |
| Reason for change | According to TS 38.508-1 Table 4.8.2.1-7: Reference QoS rule #7, Rule operationcode should be ‘001’B. |
| Summary of change | Change Rule operation code to value specified in TS 38.508-1 Table 4.8.2.1-7. |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_BuildDefaultQoSRules (charstring p_RuleNum,

template (omit) IEI8_Type p_IEI := omit) return template (value) QoS_Rules

{

var template (value) QoS_Rules v_QoS_Rules;

var template (value) QoS_Rule v_VoiceQoS := cs_QoS_Rule('03'O, int2bit(7, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B); // @sic R5-210627 sic@;

if (p_RuleNum == "7_Voice") {

v_QoS_Rules := cs_QoS_Rules ({v_VoiceQoS}, p_IEI) // @sic R5-213440 sic@

} else if (p_RuleNum == "7_Video") {

v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('04'O, int2bit(8, 6), '02'O, '011'B, cs_PacketFilterList_MediaUDP('0111'B), '0'B)}, p_IEI) // @sic R5-217796 sic@

} else if (p_RuleNum == "7_VoiceVideo") {

v_QoS_Rules := cs_QoS_Rules ({v_VoiceQoS, cs_QoS_Rule('04'O, int2bit(8, 6), '02'O, -, cs_PacketFilterList_MediaUDP_VideoCall('0111'B), '0'B)}, p_IEI) // @sic R5-210627, R5-216161 sic@

} else {

v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI);

}

return v_QoS_Rules;

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_BuildDefaultQoSRules (charstring p_RuleNum,

template (omit) IEI8_Type p_IEI := omit) return template (value) QoS_Rules

{

var template (value) QoS_Rules v_QoS_Rules;

var template (value) QoS_Rule v_VoiceQoS := cs_QoS_Rule('03'O, int2bit(7, 6), '01'O, -, cs_PacketFilterList_MediaUDP('0110'B), '0'B); // @sic R5-210627 sic@;

if (p_RuleNum == "7_Voice") {

v_QoS_Rules := cs_QoS_Rules ({v_VoiceQoS}, p_IEI) // @sic R5-213440 sic@

} else if (p_RuleNum == "7_Video") {//WA#WI=918780 Change 3: Rule operation code should be '001'B.

v_QoS_Rules := cs_QoS_Rules ({cs_QoS_Rule('04'O, int2bit(8, 6), '02'O, -, cs_PacketFilterList_MediaUDP('0111'B), '0'B)}, p_IEI) // @sic R5-217796 sic@

} else if (p_RuleNum == "7_VoiceVideo") {

v_QoS_Rules := cs_QoS_Rules ({v_VoiceQoS, cs_QoS_Rule('04'O, int2bit(8, 6), '02'O, -, cs_PacketFilterList_MediaUDP_VideoCall('0111'B), '0'B)}, p_IEI) // @sic R5-210627, R5-216161 sic@

} else {

v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule(p_RuleNum)}, p_IEI);

}

return v_QoS_Rules;

}

<<SKIPPED CODE>>

  - Correction to function f_NR5GC_ModifyPDUSession_AddVideoToSpeech

| Function name | f_NR5GC_ModifyPDUSession_AddVideoToSpeech |
| --- | --- |
| Reason for change | Test case requires two DRBs, for audio and video. Current number of elements added to list to assign DRB ID for video is one. This will provoke a runtime error as script is trying to access a position with no element. |
| Summary of change | Created a DRB List with 2 elements. One for audio and another one for video. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

v_DRBList := f_NR_GetNextDRBIds(v_GSM_MobilityInfo, false, 1);

v_NewDRBIdVideo := v_DRBList[1];

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

v_DRBList := f_NR_GetNextDRBIds(v_GSM_MobilityInfo, false, 2);

v_NewDRBIdVideo := v_DRBList[1];

<<SKIPPED CODE>>

  - Correction to function f_IMS_InviteResponse_183_MessageHeaderTX

| Function name | f_IMS_InviteResponse_183_MessageHeaderTX |
| --- | --- |
| Reason for change | Change 1: It may happen that HostPort is included by UE in INVITE. In that case SS should include it also in 183 message Contact header. Change 2: Test case has the singularity that being a MT, UE sends the reINVITE to add video media to the call. This will cause RouteBody to be wrongly set, Dialog is MT but adding video call flow is UE initiated. |
| Summary of change | Change 1: Added input parameter to add HostPort to Contact header. Change 2:Added an statement to invert RouteBody in case that initial dialog is of MT nature. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_IMS_InviteResponse_183_MessageHeaderTX(INVITE_Request p_InviteRequest,

charstring p_ContactUri := px_IMS_CalleeContactUri,

template (omit) SemicolonParam_List p_AditionalContactParams := omit,

OptionTag_List p_OptionTagsForRequire := tsc_OptionTagList_100rel_precondition,

IMS_CallType_Type p_CallType := NORMAL_CALL,

boolean p_A6 := pc_IMS_Video_FeatureTag,

template (omit) ContentType p_ContentType := cs_ContentTypeSDP) runs on IMS_PTC return template (value) MessageHeader

{ /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */

/* @sic R5s150180: tsc_IMS_VideoFeature replaced by pc_IMS_Video_FeatureTag sic@ */

/* @sic R5s150692 change 7: parameter p_CallType sic@ */

/* @sic R5-174544: SessionId removed sic@ */

/* @sic R5s190882: p_OptionTagsForRequire := tsc_OptionTagList_100rel_precondition instead of p_AdditionalOptionTagsForRequire := tsc_OptionTagList_Empty; p_Earlymedia removed; p_ContentType added  sic@ */

var MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader;

var template (value) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderTX(v_MessageHeader_Invite);  /* @sic R5s190882: f_IMS_BaseResponse_MessageHeaderTX sic@ */

var template (omit) SemicolonParam_List v_ContactParams := p_AditionalContactParams;

var template (value) RouteBody_List v_RouteBodyList := f_IMS_RouteSet_MO_Call_TX(p_CallType);     /* @sic R5s150692 change 7 sic@ */

v_MessageHeader_Response.recordRoute     := cs_RecordRoute(v_RouteBodyList);

v_MessageHeader_Response.contentType     := p_ContentType;

if (lengthof(p_OptionTagsForRequire) > 0) {                                              /* @sic R5s190882 sic@ */

v_MessageHeader_Response.require       := cs_Require(p_OptionTagsForRequire);          // @sic R5s130333 change 3 sic@  @sic R5s190882 sic@

if (f_MessageHeader_CheckRequire(valueof(v_MessageHeader_Response), c_tag100rel)) {    /* -> reliable provisional responses acc. RFC 3262; condition A3 @sic R5s190882 sic@ */

v_MessageHeader_Response.rSeq        := cs_RSeq(tsc_IMS_RSeqNumFor183);

}

}

if (p_CallType == ASSERTED_EMERGENCY) {    // Condition A5 @sic R5s150692 change 7 sic@

v_MessageHeader_Response.pAssertedID   := cs_PAssertedId(f_SIP_BuildSipUri_lr_TX(tsc_IMS_Emergency_TelUri));

}

if (pc_BSRVCC) {  // @sic R5s140962 sic@

v_MessageHeader_Response.featureCaps   := cs_FeatureCaps({ cs_GenericParam(tsc_IMS_FeatureCap_SrvccPreAlerting)});  // @sic R5-145796 sic@

}

if (p_A6) { // @sic R5s130109 sic@

v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("video"));

}

v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("audio"));                           // @sic R5-165943 sic@

v_MessageHeader_Response.contact         := cs_Contact(f_SIP_BuildSipUri_TX(p_ContactUri), v_ContactParams);

return v_MessageHeader_Response;

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_IMS_InviteResponse_183_MessageHeaderTX(INVITE_Request p_InviteRequest,

charstring p_ContactUri := px_IMS_CalleeContactUri,

template (omit) SemicolonParam_List p_AditionalContactParams := omit,

OptionTag_List p_OptionTagsForRequire := tsc_OptionTagList_100rel_precondition,

IMS_CallType_Type p_CallType := NORMAL_CALL,

boolean p_A6 := pc_IMS_Video_FeatureTag,

template (omit) ContentType p_ContentType := cs_ContentTypeSDP) runs on IMS_PTC return template (value) MessageHeader

{ /* @sic R5s130756 additional changes: p_IsGIBA removed sic@ */

/* @sic R5s150180: tsc_IMS_VideoFeature replaced by pc_IMS_Video_FeatureTag sic@ */

/* @sic R5s150692 change 7: parameter p_CallType sic@ */

/* @sic R5-174544: SessionId removed sic@ */

/* @sic R5s190882: p_OptionTagsForRequire := tsc_OptionTagList_100rel_precondition instead of p_AdditionalOptionTagsForRequire := tsc_OptionTagList_Empty; p_Earlymedia removed; p_ContentType added  sic@ */

var MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader;

var template (value) MessageHeader v_MessageHeader_Response := f_IMS_BaseResponse_MessageHeaderTX(v_MessageHeader_Invite);  /* @sic R5s190882: f_IMS_BaseResponse_MessageHeaderTX sic@ */

var template (omit) SemicolonParam_List v_ContactParams := p_AditionalContactParams;

var template (value) RouteBody_List v_RouteBodyList;     /* @sic R5s150692 change 7 sic@ */

var template (omit) integer v_HostPort := omit;

if (f_IMS_PTC_ImsInfo_DialogIsMO()) {

v_RouteBodyList := f_IMS_RouteSet_MO_Call_TX(p_CallType);

} else {

v_RouteBodyList := f_IMS_RouteSet_ReverseOrder(f_IMS_RouteSet_MT_Call_TX());

}

if (isvalue (p_InviteRequest.requestLine.requestUri.components.sip.hostPort.portField)){

v_HostPort := p_InviteRequest.requestLine.requestUri.components.sip.hostPort.portField;

}

v_MessageHeader_Response.recordRoute     := cs_RecordRoute(v_RouteBodyList);

v_MessageHeader_Response.contentType     := p_ContentType;

if (lengthof(p_OptionTagsForRequire) > 0) {                                              /* @sic R5s190882 sic@ */

v_MessageHeader_Response.require       := cs_Require(p_OptionTagsForRequire);          // @sic R5s130333 change 3 sic@  @sic R5s190882 sic@

if (f_MessageHeader_CheckRequire(valueof(v_MessageHeader_Response), c_tag100rel)) {    /* -> reliable provisional responses acc. RFC 3262; condition A3 @sic R5s190882 sic@ */

v_MessageHeader_Response.rSeq        := cs_RSeq(tsc_IMS_RSeqNumFor183);

}

}

if (p_CallType == ASSERTED_EMERGENCY) {    // Condition A5 @sic R5s150692 change 7 sic@

v_MessageHeader_Response.pAssertedID   := cs_PAssertedId(f_SIP_BuildSipUri_lr_TX(tsc_IMS_Emergency_TelUri));

}

if (pc_BSRVCC) {  // @sic R5s140962 sic@

v_MessageHeader_Response.featureCaps   := cs_FeatureCaps({ cs_GenericParam(tsc_IMS_FeatureCap_SrvccPreAlerting)});  // @sic R5-145796 sic@

}

if (p_A6) { // @sic R5s130109 sic@

v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("video"));

}

v_ContactParams := f_SemicolonParam_List_Add_TX(v_ContactParams, cs_GenericParam("audio"));                           // @sic R5-165943 sic@

v_MessageHeader_Response.contact         := cs_Contact(f_SIP_BuildSipUri_TX(p_ContactUri, v_HostPort), v_ContactParams);

return v_MessageHeader_Response;

}

<<SKIPPED CODE>>

  - Correction to function f_IMS_MOCallSetup_NR5GC_Video_Step6_7

| Function name | f_IMS_MOCallSetup_NR5GC_Video_Step6_7 |
| --- | --- |
| Reason for change | f_IMS_MOCallSetup_NR5GC_Video_Step6_7 2nd input parameter p_contacUrl is a parameter passed through few functions to finally be used to macht received messages hence nature should be of reception not transmission. |
| Summary of change | Changed assignment to p_ContactURL to the output of f_SIP_BuildSipUri_RX |
| TTCN module | IMS_Procedures_Video.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_IMS_MOCallSetup_NR5GC_Video_Step6_7(VideoCallTypeMO_Type p_VideoCallType,

IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp,

template (value) SipUrl p_ContactUrl := f_SIP_BuildSipUri_TX(px_IMS_CalleeContactUri),

IMS_CallType_Type p_CallType := NORMAL_CALL) runs on IMS_PTC return SDP_Message

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_IMS_MOCallSetup_NR5GC_Video_Step6_7(VideoCallTypeMO_Type p_VideoCallType,

IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp,

template (value) SipUrl p_ContactUrl := f_SIP_BuildSipUri_RX(px_IMS_CalleeContactUri),

IMS_CallType_Type p_CallType := NORMAL_CALL) runs on IMS_PTC return SDP_Message

<<SKIPPED CODE>>

  - Correction to function f_SIP_BuildSipUri_RX

| Function name | f_SIP_BuildSipUri_RX |
| --- | --- |
| Reason for change | Being definition of Hostport: type record HostPort { charstring host optional, // hostname, IPv4 or IPv6 integer portField optional // represented as an integer } Input parameter should reflect that and be assigned with “*”. |
| Summary of change | Assigned “*” to p_Port later assignet to portField to admit any or none value sent by the UE. |
| TTCN module | IMS_SIP_Templates.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_SIP_BuildSipUri_RX(charstring p_URI,

template integer p_Port := omit,

template SemicolonParam_List p_UrlParameters := *,

template AmpersandParam_List p_Headers := omit) return template (present) SipUrl

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_SIP_BuildSipUri_RX(charstring p_URI,

template integer p_Port := *,

template SemicolonParam_List p_UrlParameters := *,

template AmpersandParam_List p_Headers := omit) return template (present) SipUrl

<<SKIPPED CODE>>

  - Correction to function fl_NR5GC_ModifyPDUSession_RemoveIMS_Media

| Function name | fl_NR5GC_ModifyPDUSession_RemoveIMS_Media |
| --- | --- |
| Reason for change | To remove video media QoS rule and flow for video should be deleted. Current implementation modify Rule and delete rule. That can cause UE to reject the PDU session Modification. It will require also a Prose change for TS 38.508-1 Table 4.9.28.2.3-2 |
| Summary of change | Replaced modfy QoS rule with Delete QoS rule to be sent in PDU Session modification to remove video media from call. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

// @sic R5-216150 sic@

if (v_RemoveVoice) { // @sic R5-217798 sic@

v_QoS_Rules[j] := cs_QoS_DeleteRule('03'O); // @sic R5s210073, R5-213439 sic@

v_QoS_Flows[j] := cs_QoS_DeleteFlow('000111'B); // @sic R5-213439 sic@

j := j+1;

}

if (v_RemoveVideo) {

if (v_RemoveVoice) {

v_QoS_Rules[j] := cs_QoS_DeleteRule('04'O);

} else {  // @sic R5-217798 sic@ FFS: error in prose

v_QoS_Rules[j] := cs_QoS_ModifyRule('04'O);

}

v_QoS_Flows[j] := cs_QoS_DeleteFlow('001000'B);

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

// @sic R5-216150 sic@

if (v_RemoveVoice) { // @sic R5-217798 sic@

v_QoS_Rules[j] := cs_QoS_DeleteRule('03'O); // @sic R5s210073, R5-213439 sic@

v_QoS_Flows[j] := cs_QoS_DeleteFlow('000111'B); // @sic R5-213439 sic@

j := j+1;

}

if (v_RemoveVideo) {

<<COMMENTED OUT>>

/*if (v_RemoveVoice) {

v_QoS_Rules[j] := cs_QoS_DeleteRule('04'O);

} else {  // @sic R5-217798 sic@ FFS: error in prose

v_QoS_Rules[j] := cs_QoS_ModifyRule('04'O);

}*/

<<COMMENTED OUT>>

v_QoS_Rules[j] := cs_QoS_DeleteRule('04'O);

v_QoS_Flows[j] := cs_QoS_DeleteFlow('001000'B);

}

<<SKIPPED CODE>>

  - Correction to function f_IMS_VideoCall_NR5GC_ReleaseVideoMT

| Function name | f_IMS_VideoCall_NR5GC_ReleaseVideoMT |
| --- | --- |
| Reason for change | Function f_IMS_VideoCall_NR5GC_ReleaseVideoMT is hardcoded to only support preconditions. |
| Summary of change | Added input parameters to add the possibility to set preconditions or not depending on test cases needs. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_IMS_VideoCall_NR5GC_ReleaseVideoMT(template (value) SDP_Message p_LatestSdpMessageFromSS) runs on IMS_PTC

{

var SipUrl v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);   // The UE's Contact URI

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var template (value) SDP_Message v_SdpOffer := p_LatestSdpMessageFromSS;

var template (value) MessageHeader v_MessageHeaderINVITE;

var template (present) MessageHeader v_MessageHeader200OK;

var template (value) MessageHeader v_MessageHeaderACK;

// Send INVITE

v_SdpOffer.origin.session_version := f_IMS_PTC_ImsInfo_DialogIncrAndGetSessVersionTX();

v_SdpOffer.media_list[1].media_field.ports.port_number := 0;

v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(RemoveVideo, v_SdpOffer, v_ContactUrlUE);

// Receive 100 Trying (optional) and 200 OK

v_MessageHeaderINVITE := v_InviteRequestWithSdp.Invite.msgHeader;

v_MessageHeader200OK := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeaderINVITE, cr_ContentTypeSDP);

f_IMS_ReceiveResponse_OptionalTrying(v_InviteRequestWithSdp.Invite.msgHeader,

v_InviteRequestWithSdp.RoutingInfo.Protocol,

cr_Response(c_statusLine200, v_MessageHeader200OK, cr_MessageBody_SDP));

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_ReleaseVideo);

f_IMS_IPCAN_WaitForTrigger(IPCAN);

// Send ACK

v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite);

IMS_Client.send(cas_IMS_Ack_Request(cs_IMS_RoutingInfo(v_InviteRequestWithSdp.RoutingInfo.Protocol), cs_ACK_Request(v_ContactUrlUE, v_MessageHeaderACK)));

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_IMS_VideoCall_NR5GC_ReleaseVideoMT(template (value) SDP_Message p_LatestSdpMessageFromSS, boolean p_PreconditionRequiredForSDP := true) runs on IMS_PTC

{

var SipUrl v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);   // The UE's Contact URI

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var template (value) SDP_Message v_SdpOffer := p_LatestSdpMessageFromSS;

var template (value) MessageHeader v_MessageHeaderINVITE;

var template (present) MessageHeader v_MessageHeader200OK;

var template (value) MessageHeader v_MessageHeaderACK;

// Send INVITE

v_SdpOffer.origin.session_version := f_IMS_PTC_ImsInfo_DialogIncrAndGetSessVersionTX();

v_SdpOffer.media_list[1].media_field.ports.port_number := 0;

v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(RemoveVideo, v_SdpOffer, v_ContactUrlUE);

// Receive 100 Trying (optional) and 200 OK

v_MessageHeaderINVITE := v_InviteRequestWithSdp.Invite.msgHeader;

v_MessageHeader200OK := f_IMS_OtherResponse_200_MessageHeaderRX(v_MessageHeaderINVITE, cr_ContentTypeSDP, p_PreconditionRequiredForSDP);

f_IMS_ReceiveResponse_OptionalTrying(v_InviteRequestWithSdp.Invite.msgHeader,

v_InviteRequestWithSdp.RoutingInfo.Protocol,

cr_Response(c_statusLine200, v_MessageHeader200OK, cr_MessageBody_SDP));

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_ReleaseVideo);

f_IMS_IPCAN_WaitForTrigger(IPCAN);

// Send ACK

v_MessageHeaderACK := f_IMS_AckRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite);

IMS_Client.send(cas_IMS_Ack_Request(cs_IMS_RoutingInfo(v_InviteRequestWithSdp.RoutingInfo.Protocol), cs_ACK_Request(v_ContactUrlUE, v_MessageHeaderACK)));

}

<<SKIPPED CODE>>

  - Correction to function f_TC_7_23_NR5GC_IMS1

| Function name | f_TC_7_23_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Test case is implemented to run with preconditions when should be run without. |
| Summary of change | Added input parameter to configurestep 9-13 to be run without preconditions. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

// @siclog "Step 7 - 8" siclog@             200 OK, ACK

f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8");

// @siclog "Step 9 - 13" siclog@            Release Video

f_IMS_VideoCall_NR5GC_ReleaseVideoMT(v_ResponseWithSdp.SdpMessage);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12");

// @siclog "Step 14 - 16" siclog@           MMI (Release Call), BYE, 200 OK

f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite);

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

// @siclog "Step 7 - 8" siclog@             200 OK, ACK

f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8");

// @siclog "Step 9 - 13" siclog@            Release Video

f_IMS_VideoCall_NR5GC_ReleaseVideoMT(v_ResponseWithSdp.SdpMessage, tsc_SDP_WithoutPreconditions);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12");

// @siclog "Step 14 - 16" siclog@           MMI (Release Call), BYE, 200 OK

f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite);

<<SKIPPED CODE>>

  - Correction to function f_NR5GC_IPCAN_AddVideo

| Function name | f_NR5GC_IPCAN_AddVideo |
| --- | --- |
| Reason for change | Function f_NR5GC_IPCAN_AddVideo is missing coordination message to inform IMS PTC that video media has been added successfully. |
| Summary of change | Added coordination message to inform IMS PTC that video media has been added. |
| TTCN module | IPCAN_NR5GC.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_NR5GC_IPCAN_AddVideo(IMS_IPCAN_CO_ORD_PORT p_Port,

NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type

{

var TriggerResult_Type v_Result := NORMAL;

// Steps 2 - 8 the same as for speech call

f_NR5GC_ModifyPDUSession_AddVideoToSpeech(p_NR_CellId);

return v_Result;

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_NR5GC_IPCAN_AddVideo(IMS_IPCAN_CO_ORD_PORT p_Port,

NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type

{

var TriggerResult_Type v_Result := NORMAL;

// Steps 2 - 8 the same as for speech call

f_NR5GC_ModifyPDUSession_AddVideoToSpeech(p_NR_CellId);

f_IMS_IPCAN_SendCoOrdMsg(p_Port);

return v_Result;

}

<<SKIPPED CODE>>

  - Correction to function f_NR5GC_IPCAN_ReleaseVideo

| Function name | f_NR5GC_IPCAN_ReleaseVideo |
| --- | --- |
| Reason for change | Function f_NR5GC_IPCAN_ReleaseVideo is missing coordination message to inform IMS PTC that video media has been released successfully. |
| Summary of change | Added coordination message to inform IMS PTC that video media has been released. |
| TTCN module | IPCAN_NR5GC.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_NR5GC_IPCAN_ReleaseVideo(IMS_IPCAN_CO_ORD_PORT p_Port,

NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type

{

var TriggerResult_Type v_Result := NORMAL;

// Steps 2 - 8 the same as for speech call

f_NR5GC_ModifyPDUSession_RemoveVideoFromSpeech(p_NR_CellId);

return v_Result;

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_NR5GC_IPCAN_ReleaseVideo(IMS_IPCAN_CO_ORD_PORT p_Port,

NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type

{

var TriggerResult_Type v_Result := NORMAL;

// Steps 2 - 8 the same as for speech call

f_NR5GC_ModifyPDUSession_RemoveVideoFromSpeech(p_NR_CellId);

f_IMS_IPCAN_SendCoOrdMsg(p_Port);

return v_Result;

}

<<SKIPPED CODE>>

# Branches executed

The test case was executed on NR band n41 using NR ciphering algorithm nea1 and integrity algorithm nia1 on Rohde & Schwarz 5G Protocol Conformance Test platform with Samsung SM-G996U UE and Hisilicon Balong 5000 UE.

# Execution Log Files

## HiSilicon

The HiSilicon Balong5000 IMS UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7.23_Hisilicon.log

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

## Samsung SM-G996U

The Samsung SM-G996U UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7.23_Samsung.log

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220337: Supporting information for addition of IMS 5GS Call Control test case 7.23 |
| --- | --- |